| PRODUCT DATA |  |  |
| --- | --- | --- |
| PN: 8010457/AZD8630 Affinity (CaptureSelect CH1-XL, GPFN, 500 L, Process 1) |  |  |
| General Supplies Process Order No. | MPR Process Order No.: | Recorded by /Date: |

| This record is an accurate reproduction of the Master. | Issued by: | Date Issued: |
| --- | --- | --- |

| Completed Record Reviewed By: | Production: | Date: |
| --- | --- | --- |
|  | Quality Assurance: | Date: |

| PERSONNEL IDENTIFICATION |  |  |
| --- | --- | --- |
| Print Name | Signature | Initials |

# Process Summary

This master production record (MPR) describes the procedure for the CaptureSelect Chromatography step in the AZD8630 downstream process outlined in the downstream process flowchart below.

# Downstream Process Flowchart

MABR-0023642 / PN: 8010441 / AZD8630 – Pod Harvest (GPFN, 500 L, Process 1)



MABR-0023643 / PN: 8010457 / AZD8630 – Affinity (CaptureSelect CH1-XL, GPFN, 500 L, Process 1)



MABR-0023644 / PN: 8010458 / AZD8630 – Virus Inactivation (Low pH, GPFN, 500 L, Process 1)



MABR-0023645 / PN: 8010459 / AZD8630 – Anion Exchange Chromatography (CaptoQ, GPFN, 500 L, Process 1)



MABR-0023646 / PN: 8010460 / AZD8630 – Cation Exchange Chromatography (POROS 50 HS, GPFN, 500 L, Process 1)



MABR-0023647 / PN: 8010461 / AZD8630 – Virus Filtration (Viresolve Pro+, GPFN, 500 L, Process 1)



MABR-0023648 / PN: 8010462 / AZD8630 – Concentration and Diafiltration (Manual P&F, GPFN, 500 L, Process 1)



MABR-0023649 /PN: 8010463 / AZD8630 – Formulation and Bulk Filtration (Bottles, GPFN, 500 L, Process 1)



Storage

# Referenced Documents

## FORM-0064069:  Column Packing Data Sheet – Parent Document SOP-0107109

## SOP-0080295:  DLIMS Sample Submission for Manufacturing Sciences

## SOP-0107091: Operation and Maintenance of the SoloVPE System for Protein Concentration Determination

## SOP-0107097:  Sampling from Process Vessels

## SOP-0080423:  Operation and Maintenance of the LevTech and LevMixer Mixing Systems

## SOP-0080297:  Benchtop pH and Conductivity Meter Operation

## SOP-0080154:  General Gowning Procedure

## SOP-0107219:  General Cleaning of Equipment, Supplies, and Materials for Entry into GPF-North

## SOP-0080854:  WFI Point of Use Operations in the GPF-North Facility

## SOP-0107297: Operation and Maintenance of the Sartorius Biowelder

## SOP-0107414: Operation of Masterflex Pumps

## SOP-0080622:  Operation and Cleaning of the Millipore Mobius Disposable Mixing System

## SOP-0080155:  General GPF-N Facility Information and Flow

## SOP-0107212:  AKTA Ready Gradient Chromatography Skid Setup, Operation, Cleaning and Storage

## SOP-0107109:  Column Packing Data Sheets and Chromatography Column Performance Evaluation

## SOP-0107056:  cGMP Labeling Practices for Controlled Materials, Equipment and Product

## SOP-0080429: POD Harvest Operations at GPF-N Facility

## MABR-0023642 / PN: 8010441 / AZD8630 – Pod Harvest (GPFN, 500 L, Process 1)

## MABR-0023644 / PN: 8010458 / AZD8630 – Virus Inactivation (Low pH, GPFN, 500 L, Process 1)

# Bill of Materials – Equipment and Components

| Part No./ Equipment ID | Quantity | Description |
| --- | --- | --- |
|  | 1 | CaptureSelect CH1-XL Column (45cm diameter, 16-22 cm bed height) |
| PK-4500 | 1 | AKTA Ready Chromatography Skid |
|  | As Needed | Mixer Drive |
|  | 1 | pH meter |
|  | 1 | Conductivity Meter |
|  | 1 | Thermometer |
|  | 1 | Floor Scale |
|  | 1 | Filter Integrity Tester |
|  |  | WFI Filter |
| 6002650 | As Needed | Sartobran P Capsule Filter (Size 7) |
|  |  | Chromatography Column Guard Filter |
| 4101737 | As Needed | Filter – SHC 0.5+0.2 μm Capsules with C-Flex (3”) |
| 4101736 |  | Filter – SHC 0.5+0.2 μm Capsules with C-Flex (10”) |
| 4100987 |  | Filter – 30 in., Opticap® XLT 30 Capsule Filter with Millipore Express® SHC Hydrophilic Membrane, 0.5/0.2 μm |
| 6004084 |  | Filter - OPTICAP XLT Capsule w/SHC Membrane, Sterile (20”) |
| 6001342 |  | Filter – 6 in. Kleenpak Capsule with Fluorodyne EX Grade EDF Membrane |
| 6001343 |  | Filter – High Area Kleenpak Nova Capsules with Fluorodyne EX Grade EDF Membrane (10 in.) |
| 6001344 |  | Filter – High Area Kleenpak Nova Capsules with Fluorodyne EX Grade EDF Membrane (20 in.) |
| 6001345 |  | Filter – High Area Kleenpak Nova Capsules with Fluorodyne EX Grade EDF Membrane (30 in.) |

4         Bill of Materials – Equipment and Components

| Part No./ Equipment ID | Quantity | Description |
| --- | --- | --- |
| 6002673 | 1 | Chromatography Kit AKTA Ready – Flow Kit (High Flow Kit) |
| 6003953 | 1 | AKTA Ready – pH Probe |
| 8004948 | As Needed | 1% (v/v) Acetone, 1 M Sodium Chloride |
|  | As Needed | Product Filters for Bags |
| 4101737 |  | Filter – SHC 0.5+0.2 μm Capsules with C-Flex (3”) |
| 4101736 4100987* |  | Filter – SHC 0.5+0.2 μm Capsules with C-Flex (10”) Filter – 30 in., Opticap® XLT 30 Capsule Filter with Millipore Express® SHC Hydrophilic Membrane, 0.5/0.2 μm |
| 6001342* 6001345* |  | Filter – 6 in. Kleenpak Capsule with Fluorodyne EX Grade EDF Membrane Filter – High Area Kleenpak Nova Capsules with Fluorodyne EX Grade EDF Membrane (30 in.) |
|  | As Needed | Product Mixing Bags |
| 6003471 |  | Custom Mobius Bag with Disposable Assembly (50 L) |
| 6003472 |  | Custom Mobius Bag with Disposable Assembly (100 L) |
| 6003473 |  | Custom Mobius Bag with Disposable Assembly (200 L) |
| 6003474 |  | Custom Mobius Bag with Disposable Assembly (500 L) |
| 4102059 |  | LevTech Mix Bag (50 L) |
| 4102062 |  | LevTech Mix Bag (100 L) |
| 4102060 |  | LevTech Mix Bag (200 L) |
| 4102032 |  | LevTech Mix Bag (300 L) |
| 4102061 |  | LevTech Mix Bag (560 L) |
| 4101684 | As Needed | Bags – Stedim 3D Bags with C-Flex (500 L) |
| 4101685 |  | Bags – Stedim 3D Bags with C-Flex (300 L) |
| 4101687 |  | Bags – Stedim 3D Bags with C-Flex (200 L) |
| 6003424 | As Needed | Gamma Irradiated Braided Silicone Tubing (3/8” x 5/8”) |
| 6003425 |  | Gamma Irradiated Braided Silicone Tubing (1/2” x 7/8”) |
| 6003426 |  | Gamma Irradiated Braided Silicone Tubing (1” x 1-3/8”) |
| 4101552 | As Needed | Filter for Depth Filtration Filter- Millistak POD Filter, 1.1 m2 (C0HC grade) |
| 4102309 4102310 | As Needed | POD Holder Disposable POD Adapter Kit 3 through 3 Blind Disposable POD Adapter Kit 6 through |

* Materials with bolded filter part numbers require autoclaving prior to use.

Additional Manufacturing Supplies Used in Process:

If additional materials (filter, bag, etc.) need to be used during processing:

- Prior to use, ensure the material is acceptable for use based on Section 4.

- Record the applicable material information.

- Document the reason for the change in Attachment 6: Comments Section.

| MPR Step No. | Material Description | Part No. | Batch No. | Serial No., if applicable | Exp. Date | Autoclave Assembly ID., if applicable | Autoclave Exp. Date, if applicable | Performed by/Date | Witnessed by/Date | SAP Consumption by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Process Flowchart

| Equilibration 1 | 8004949: 50 mM Tris, pH 7.4 |  |  |
| --- | --- | --- | --- |
| Batch No. |  | Exp. Date |  |
| Batch No. |  | Exp. Date |  |
| Batch No. |  | Exp. Date |  |



| Load | 8010441: Conditioned Medium Product |
| --- | --- |
| Batch No. |  |



| POD Filtration | 8004949: 50 mM Tris, pH 7.4 |
| --- | --- |



| Equilibration 2 | 8004949: 50 mM Tris, pH 7.4 |
| --- | --- |



| Wash | 8004950: 50 mM Tris, 0.5 M Sodium Chloride, pH 7.4 |  |  |
| --- | --- | --- | --- |
| Batch No. |  | Exp. Date |  |
| Batch No. |  | Exp. Date |  |



| Equilibration 3 | 8004949: 50 mM Tris, pH 7.4 |
| --- | --- |



| Elution | 8010534: 25 mM Sodium Acetate, pH 3.5 |  |  |
| --- | --- | --- | --- |
| Batch No. |  | Exp. Date |  |
| Batch No. |  | Exp. Date |  |

↓

| Strip 1 | 8008812: 103 mM acetic acid, 2 M urea, pH 3.1 |  |  |
| --- | --- | --- | --- |
| Batch No. |  | Exp. Date |  |



Continued on the next page

Continued from the previous page



| Equilibration 4 | 8004949: 50 mM Tris, pH 7.4 |
| --- | --- |



| Strip 2 | 8008813: 12.5 mM sodium hydroxide |  |  |
| --- | --- | --- | --- |
| Batch No. |  | Exp. Date |  |



| Equilibration 5 | 8004949: 50 mM Tris, pH 7.4 |
| --- | --- |



| Strip 3 | 8006409:120 mM phosphoric acid, 167 mM acetic acid, 2.2% (v/v) benzyl alcohol |  |  |
| --- | --- | --- | --- |
| Batch No. |  | Exp. Date |  |



Additional Cycles, if applicable: Equilibration through Strip 2



Column Sanitization HOLD



| Pre – Storage Equilibration | 8004949: 50 mM Tris, pH 7.4 |
| --- | --- |



| Column Storage | 8004953: 2% benzyl alcohol, 100 mM sodium acetate, pH 5.0 |
| --- | --- |
| Batch No. and Exp. Date recorded in Step 15.1. |  |
| NOTE: Storage is Performed only after the completion of the last cycle of each processing day. |  |

| Recorded by/ Date |  | Verified by/ Date |  |
| --- | --- | --- | --- |

Additional Solution Batches Used in Process:

If a solution (of a different batch noted in Section 5) needs to be replaced during processing:

- Indicate the switch at the step it is occurring. Ensure the new solution is the same part number as the solution being replaced.

- Record the solution part number, batch number (and/or process order number) and expiration date below.

- Document the change in Attachment 6: Comments Section.

| Applicable Step Number | Part Number | Batch Number | Exp. Date | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |

# Process Notes

## All activities are performed at 15 C to 27C.

## For all calculations, assume 1 L = 1 kg and 1 mL = 1 g unless otherwise noted. PK-4500 absorbance units are in mAU. The conversion from OD to mAU is: mAU = OD x 0.15 x 1000.

## The target linear velocity for CaptureSelect CH1-XL Chromatography is 200 cm/hr except for the ELUTION step where the linear velocity is 150 cm/hr. Alert limits are +/- 5% of target and the Normal Operating Range is +10% of the target.

## Unless otherwise noted, flow rate ranges are for steady state operation and do not consider pump ramping strategy.

## NOTE: Temporary flowrate excursions resulting from the skid adjusting to changing process conditions are acceptable.

## Verify that all solutions, raw materials, and standards are within expiration prior to use.

## When an equipment ID is written in the MPR, the operator is indicating they have visually inspected and verified calibration prior to use.

## During this process if PAUSE is pushed on the chromatography skid HMI the chromatography skid pumps will stop. If PAUSE is used at any time during this process the details as to why PAUSE was pushed must be documented in Attachment 6: Comments Section. To continue processing, push the CONTINUE button.

## During the load step, next breakpoint may be used to proceed to the next chromatography step in the recipe. This should be used when the operator is sure that the load step is complete. Note the use of this in the MPR at the step in which it is used.

## If the next breakpoint is used for any other reason a supervisor, or designee, signature is required. Record the reason in Attachment 6: Comments Section.

NOTE:  This function does not apply to the elution step and should not be used during elution unless directed by the area supervisor to do so.

## The chromatography process, outlined in this MPR, may be run manually if the automated chromatography recipe is not functioning correctly. (Move to the recipe section)

## If a non-routine sample is requested by MEMO, complete Attachment 2: Non-Routine Sampling Record. Complete record by recording the Process Section No., Contents, Container Type, Quantity, Designation, Perform by/Date, and Witness by/Date fields.

## NOTE: If Non-Routine samples are taken, re-adjust the weight of the product sampled.

## All parameters listed in this document are Targets/Alert Limits unless otherwise noted. When alert limits are exceeded, contact Supervisor and Downstream Tech Transfer. Document any actions taken in Attachment 6: Comments Section.

## To avoid over pressurization during process, flow rates may be lowered manually or as part of the method. Record adjusted flowrate information in Attachment 4: Adjusted Flowrate Table.

# AKTA Ready Kit Installation and WFI Charge Information

| Step | Description |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- |
|  | - Contact a Supervisor/designee and verify that the data logger is on. - Verify the process trend for PK-4500 is activated. |  | Day 1, First Cycle | Day 1, First Cycle |
|  |  |  | Day 2, First Cycle | Day 2, First Cycle |
|  | Record the room where this process will occur. |  |  |  |
|  | Room Number |  |  |  |
|  | Step 7.4: Day 1 WFI Container - Set Up Set up per SOP-0080854: - Inspect the collection vessel for any visible defects. - Record the WFI collection vessel information. - Obtain a WFI filter per Section 4 and record the WFI filter information in Step 7.4. - Verify that the WFI port has been flushed per SOP-0080854. - Install the WFI filter per SOP-0080854. - Charge into the vessel and record the charge information in Step 7.4. NOTE: The charged WFI expiration date and time is 24 hours from the WFI Charge date and time. |  |  |  |

| ## Day 1 WFI Container - Set Up |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Part No. | Batch No. |  | Exp. Date |  |  | SAP Consumption Performed by/Date |
| Bag Info |  |  |  |  |  |  |  |
| WFI Filter |  |  |  |  |  |  |  |
| WFI Valve ID |  |  |  |  |  |  |  |
| WFI Charge Date/Time | Date: |  |  |  | Time: |  |  |
| WFI Expiration Date/Time | Date: |  |  |  | Time: |  |  |
| Recorded By / Date |  |  | Verified By / Date |  |  |  |  |

7	AKTA Ready Kit Installation and WFI Charge Information (continued)

| Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | Step 7.6: Day 2 WFI Container - Set Up Set up per SOP-0080854: - Inspect the collection vessel for any visible defects. - Record the WFI collection vessel information. - Obtain a WFI filter per Section 4 and record the WFI filter information in Step 7.6. - Verify that the WFI port has been flushed per SOP-0080854. - Install the WFI filter per SOP-0080854. - Charge into the vessel and record the charge information in Step 7.6. NOTE: The charged WFI expiration date and time is 24 hours from the WFI Charge date and time. |  |  |

| ## Day 2 WFI Container - Set Up |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Part No. | Batch No. |  | Exp. Date |  |  | SAP Consumption Performed by/Date |
| Bag Info |  |  |  |  |  |  |  |
| WFI Filter |  |  |  |  |  |  |  |
| WFI Valve ID |  |  |  |  |  |  |  |
| WFI Charge Date/Time | Date: |  |  |  | Time: |  |  |
| WFI Expiration Date/Time | Date: |  |  |  | Time: |  |  |
| Recorded By / Date |  |  | Verified By / Date |  |  |  |  |

7	AKTA Ready Kit Installation and WFI Charge Information (continued)

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Obtain the AKTA Ready kit required for processing. - Install the AKTA Ready kit per SOP-0107212. |  |  |  |  |  |  |
|  | Kit Required | AKTA High Flow Kit |  |  |  |  |  |
|  | Part Number | 6002673 |  |  |  |  |  |
|  | Batch Number |  |  |  |  |  |  |
|  | Expiration Date |  |  |  |  |  |  |
|  | Skid ID | PK-4500 |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |  |
|  | Obtain the buffer PN: 8004948: 1% (v/v) Acetone, 1 M Sodium Chloride. |  |  |  |  |  |  |
|  | Batch Number |  |  |  |  |  |  |
|  | Exp. Date |  |  |  |  |  |  |
|  | From the AKTA Ready Wizard, run the Flow Kit Installation with Component Test. Record results below. NOTE: The component test uses 5 L of PN: 8004948 per component test attempted. |  |  |  |  |  |  |
|  | Test Results |  | PASS | FAIL |  |  |  |
|  | Test Results |  | PASS | FAIL | N/A |  |  |
|  | Test Results |  | PASS | FAIL | N/A |  |  |
|  | 8004948 Volume used |  | L |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |  |

# Column Information

| Step | Description |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # Obtain the CaptureSelect XL Column and the column packing data sheet (CPDS) FORM-0064069. # Verify that the column was packed with CaptureSelect CH1-XL for AZD8630 processing. # From the CPDS: - Assign “Column Pressure Rating” as the Column Inlet Pressure below. - Assign “Packing Pressure” as the Column Differential Pressure below. Note: As needed, label the column with ‘AZD8630’, ‘CaptureSelect CH1-XL’, and initial/date. |  |  |  |  |  |  |  |
|  | Chromatography Media Name |  |  |  |  |  |  |  |
|  | Column ID |  |  |  |  |  |  |  |
|  | CPDS Lot No. |  |  |  |  |  |  |  |
|  | CPDS Part Number |  |  |  |  |  |  |  |
|  | Column Bed Volume |  |  | L |  |  |  |  |
|  | Column Inlet Pressure |  |  | bar |  |  |  |  |
|  | Column Differential Pressure |  |  | barD |  |  |  |  |
|  | Column bed height (16-22 cm) |  |  | cm |  |  |  |  |
|  | SAP Consumption Performed by/ Date |  |  |  |  |  |  |  |
|  | Calculate the column inlet pressure to be used for chromatography. |  |  |  |  |  |  |  |
|  | Column Inlet Pressure (Step 8.1) |  | Safety Factor |  |  | Column Inlet Pressure Max |  |  |
|  | bar | x | 0.9 |  | = | bar |  |  |

8	Column Information (continued)

| ## Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | Verify from the column’s logbook that the COLUMN was both: # Sanitized with 120 mM Phosphoric Acid, 167 mM Acetic Acid, 2.2 %(v/v) Benzyl Alcohol (8006409) Stored in 2% (v/v) benzyl alcohol, 100 mM Sodium Acetate, pH 5.0 (8004953) | Day 1 |  |
|  | # Sanitized with 120 mM Phosphoric Acid, 167 mM Acetic Acid, 2.2 %(v/v) Benzyl Alcohol (8006409) Stored in 2% (v/v) benzyl alcohol, 100 mM Sodium Acetate, pH 5.0 (8004953) | Day 2 |  |
|  | # Verify that the AZD8630_ProteinA method on the chromatography skid has been Wet Tested and Ran As-Expected. | Day 1 | Day 1 |

# Chromatography Calculations

| Step | Description |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Load Concentration and Volume Obtain the protein concentration and volume results for the Conditioned Medium (CM) from the AZD8630 Pod Harvest MPR (MABR-0023642) and record the results below. |  |  |  |  |  |  | Bag 1 | Bag 1 |
|  | Bag 1 |  |  |  |  |  |  |  |  |
|  | CM Product Net Volume |  |  |  | Kg=L |  |  |  |  |
|  | CM Concentration |  |  |  | g/L |  |  |  |  |
|  | DLIMS Information |  |  |  | Project Number | Sample Number |  |  |  |
|  | Bag 2, If Applicable |  |  |  |  |  |  | Bag 2 | Bag 2 |
|  | CM Product Net Volume |  |  |  | Kg=L |  |  |  |  |
|  | CM Concentration |  |  |  | g/L |  |  |  |  |
|  | DLIMS Information |  |  |  | Project Number | Sample Number |  |  |  |
|  | SAP Consumption performed by/Date |  |  |  |  |  |  |  |  |
|  | Calculate the Total Amount of Protein in Bag 1. |  |  |  |  |  |  |  |  |
|  | CM Product Net Volume - Bag 1 (Step 9.1) | x |  | CM Concentration – Bag 1 (Step 9.1) |  | = | Total Amount of Protein - Bag 1 |  |  |
|  | L |  |  | g/L |  |  | g |  |  |
|  | Calculate the Total Amount of Protein in Bag 2, if applicable. |  |  |  |  |  |  |  |  |
|  | CM Product Net Volume – Bag 2 (Step 9.1) | x | CM Concentration – Bag 2 (Step 9.1) |  |  | = | Total Amount of Protein - Bag 2 |  |  |
|  | L |  | g/L |  |  |  | g |  |  |

9	Chromatography Calculations (continued)

| Step | Description |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the Total Amount of Protein. If there is only one bag of CM, enter “0.0” for the Product Mass of Bag 2. |  |  |  |  |  |  |  |  |  |
|  | Total Amount of Protein Bag 1 (Step 9.2) | + |  | Total Amount of Protein Bag 2 (Step 9.3) |  | = | Total Amount of Protein |  |  |  |
|  | g |  |  | g |  |  | g |  |  |  |
|  | Calculate the maximum load for the column. |  |  |  |  |  |  |  |  |  |
|  | Column Bed Volume (Step 8.1) |  | x | Max Column binding capacity |  |  | = | Maximum Load |  |  |
|  | L |  |  | 15 g/L/cycle |  |  |  | g/cycle |  |  |
|  | Calculate the number of chromatography cycles. |  |  |  |  |  |  |  |  |  |
|  | Total Amount of Protein (Step 9.4) |  | ÷ | Maximum Load (Step 9.5) |  |  | = | Total number of cycles |  |  |
|  | g |  |  | g/cycle |  |  |  | cycle(s) |  |  |
|  | Round the total number of cycles in Step 9.6 up to the next whole number. |  |  |  |  |  |  |  |  |  |
|  | Number of Cycles |  |  |  | cycle(s) |  |  |  |  |  |

9	Chromatography Calculations (continued)

| Step | Description |  |  |  |  |  |  |  |  | Performed by/Date |  | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Contact management to determine if process will occur over 1 or 2 Days. - If process will occur over 2 Days, mark Steps 9.9 and 9.10 as “N/A” and proceed to step 9.11. NOTE: Bag 1 will be processed on Day 1 and Bag 2 will be processed on Day 2. - If process will occur over 1 day, proceed to the next step and mark steps 9.11 - 9.16 as “N/A”. |  |  |  |  |  |  |  |  |  |  |  |
|  | Calculate the total volume of CM. |  |  |  |  |  |  |  |  |  |  |  |
|  | CM Product Net Volume - Bag 1 (Step 9.1) |  | + | CM Product Net Volume - Bag 2 (Step 9.1) |  |  |  | = | Total CM Product Net Volume |  |  |  |
|  | L |  |  | L |  |  |  |  | L |  |  |  |
|  | Calculate the load volume per cycle for 1 day processing. |  |  |  |  |  |  |  |  |  |  |  |
|  | Total CM Product Net Volume (Step 9.9) |  | ÷ | Expected Number of cycles (Step 9.7) |  |  |  | = | Load Volume for 1 day processing |  |  |  |
|  |  |  |  |  |  |  |  |  | L/cycle |  |  |  |
|  | Calculate the number of chromatography cycles for Bag 1. |  |  |  |  |  |  |  |  |  |  |  |
|  | Total Amount of Protein, Bag 1 (Step 9.2) |  | Maximum load (Step 9.5) |  |  | = | Number of cycles for Bag 1 |  |  |  |  |  |
|  | g |  | g/cycle |  |  |  | cycles |  |  |  |  |  |
|  | Round the number of cycles for Bag 1 in Step 9.11 to the next whole number. |  |  |  |  |  |  |  |  |  |  |  |
|  | Number of cycles for Bag 1 |  |  |  | cycles |  |  |  |  |  |  |  |

9	Chromatography Calculations (continued)

| ## Step | Description |  |  |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the volume of load per cycle. |  |  |  |  |  |  |  |  |  |  |  |
|  | CM Product Net Volume – Bag 1 (Step 9.1) |  |  |  | Number of cycles for Bag 1 (Step 9.12) |  |  | = |  | Volume of CM Bag 1 per cycle |  |  |
|  | L |  |  |  | cycles |  |  |  |  | L/cycle |  |  |
|  | Calculate the number of chromatography cycles for Bag 2. |  |  |  |  |  |  |  |  |  |  |  |
|  | Total Amount of Protein, Bag 2 (Step 9.3) |  |  | Maximum load (Step 9.5) |  |  | = |  | Number of cycles for Bag 2 |  |  |  |
|  | g |  |  | g/cycle |  |  |  |  | cycles |  |  |  |
|  | Round the number of cycles in Step 9.14 to the next whole number. |  |  |  |  |  |  |  |  |  |  |  |
|  | Number of cycles for Bag 2 |  |  |  |  | cycles |  |  |  |  |  |  |
|  | Calculate the volume of load per cycle. |  |  |  |  |  |  |  |  |  |  |  |
|  | CM Product Net Volume – Bag 2 (Step 9.1) |  |  |  | Number of cycles for Bag 2 (Step 9.15) |  |  | = |  | Volume of CM Bag 2 per cycle |  |  |
|  | L |  |  |  | cycles |  |  |  |  | L/cycle |  |  |

# Filtered Product Collection Vessel Information and Setup

| Step | Description |  |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the estimated Viral Inactivation (VI) Product Volume. Estimated Product volume: - Enter 2.3 CV/cycle. |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Column Bed Volume (Step 8.1) | x | Estimated Product Volume | x | Number of cycles (Step 9.7) | x | Estimated Viral Inac. (VI) additions | = | Estimated VI Product Volume |  |  |
|  | L/CV |  | _______ CV/Cycle |  | Cycle(s) |  | 1.10 |  | L |  |  |
|  | - Obtain an appropriately sized product vessel based on the Estimated VI Product Volume in Step 10.1. - Label the vessel with “AZD8630”, “CaptureSelect CH1-XL Product” batch number, part number, and initials and date. - Obtain the appropriate product filter for Section 4. |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | For Steps 10.5 and 10.6: - Record required information for the Filtered CaptureSelect CH1-XL Product Collection Vessel in Step 10.5 and the Day 1 Product Filter in Step 10.6. - Attach the product filter after the tare weight of the collection vessel is obtained. |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day 2 Product Filter - N/A this step if only one day of CaptureSelect CH1-XL process is needed. Otherwise, continue below: - Obtain appropriate product filter(s) to attach to the collection vessel that was set up on Day 1. - Record the information in Step 10.6. - NOTE: The Day 2 CaptureSelect CH1-XL product will be collected into the same collection vessel and on top of the Day 1 CaptureSelect CH1-XL product. |  |  |  |  |  |  |  |  | Day 2 | Day 2 |

| ## Filtered CaptureSelect CH1-XL Chromatography Product Collection Vessel Information |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scale/Balance ID |  |  | Calibration Due Date |  |  |  |  |  |
| Tare weight of the Product Vessel | kg |  |  |  |  |  |  |  |
| Collection Bag Info | Part No. | Batch No. |  | Exp. Date |  |  | SAP Consumption Performed by/Date |  |
| Mark the applicable attachments connected to the LevTech Bag during the vessel tare weight. If LevTech bag not used, N/A information. | Attached: Mixer Drive | Top Base |  | Magnetic Clamp |  |  |  |  |
| Recorded By / Date |  |  | Witnessed by / Date |  |  |  |  |  |

| ## Product Filter Information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Day 1 |  |  |  |  |  |  |  |  |
| Part Number | Batch Number | Exp. Date |  |  | Serial No |  |  | SAP Consumption Performed by / Date |
| Autoclave Information (if applicable) | Autoclave Assembly ID: |  |  |  | Autoclave Exp. Date. |  |  |  |
| Recorded by / Date |  |  |  | Witnessed by / Date |  |  |  |  |
| Day 2 (if applicable) |  |  |  |  |  |  |  |  |
| Part Number | Batch Number | Exp. Date |  |  | Serial No. |  |  | SAP Consumption Performed by / Date |
| Autoclave Information (if applicable) | Autoclave Assembly ID |  |  |  | Autoclave Exp. Date |  |  |  |
| Recorded By / Date |  |  | Witnessed by / Date |  |  |  |  |  |

# Depth Filter Setup

| Step | Description |  |  |  |  |  | Results |  |  |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the C0HC surface area needed for the day. *Bag 1 volume will be used for Day 1 processing. Bag 2 volume will be used for Day 2 processing. |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day | CM Volume per day* (Step 9.1) |  |  |  | C0HC Capacity |  |  |  | C0HC Surface Area Needed |  |  |  |
|  | 1 | L |  | ÷ |  | 500 L/m2 |  |  | = | m2 |  |  |  |
|  | 2 | L |  |  |  |  |  |  |  | m2 |  | Day 2 | Day 2 |
|  | Calculate the number of depth filter units to be installed for the day. Note: Round up to the nearest whole number |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day | C0HC Surface Area Needed (Step 11.1) |  |  |  | C0HC Surface Area |  |  |  | Number of C0HC Filters |  |  |  |
|  | 1 | m2 |  | ÷ |  | 1.1 m2 |  |  | = | (round up) |  |  |  |
|  | 2 | m2 |  |  |  |  |  |  |  | (round up) |  | Day 2 | Day 2 |
|  | - Obtain POD adaptors and filters to be used. - Attach the SAP labels to the top of the Pod units. |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Description |  | Quantity Used |  | Batch No. |  |  | Exp. Date |  |  | Serial No. |  |  |
|  | C0HC, 1.1 m2 (4101552) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pod Adapter Kit 3 through 3 Blind (4102309) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pod Adapter Kit 6 Through (4102310) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SAP Consumption Performed (Day 1) |  |  |  |  |  |  |  |  |  |  | Day 1 |  |
|  | C0HC, 1.1 m2 (4101552) |  |  |  |  |  |  |  |  |  |  | Day 2 | Day 2 |
|  | Pod Adapter Kit 3 through 3 Blind (4102309) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pod Adapter Kit 6 Through (4102310) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SAP Consumption Performed (Day 2) |  |  |  |  |  |  |  |  |  |  | Day 2 |  |

11	Depth Filter Set Up (continued)

| Step | Description |  |  |  |  |  |  |  | Results |  |  |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the total surface area of the C0HC units. |  |  |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day | Number of C0HC Filters (Step 11.3) |  |  |  |  | Surface area per unit |  |  |  |  | Total surface area of C0HC units |  |  |  |
|  | 1 |  |  |  | X |  | 1.1 m2 |  |  | = |  | m2 |  |  |  |
|  | 2 |  |  |  |  |  |  |  |  |  |  | m2 |  | Day 2 | Day 2 |
|  | Calculate the flowrate range to flush the depth filtration unit at. |  |  |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day |  | Flux (LMH) |  |  |  | Total surface area of C0HC units (Step 11.4) |  |  |  |  | Flow rate Range |  |  |  |
|  | 1 |  | 500 (min)* |  | X |  | m2 |  |  | = |  | L/hr (min) |  |  |  |
|  |  |  | 600 (target) |  |  |  |  |  |  |  |  | L/hr (target) |  |  |  |
|  |  |  | 700 (max) |  |  |  |  |  |  |  |  | L/hr (max) |  |  |  |
|  | 2 |  | 500 (min)* |  | X |  | m2 |  |  | = |  | L/hr (min) |  | Day 2 | Day 2 |
|  |  |  | 600 (target) |  |  |  |  |  |  |  |  | L/hr (target) |  |  |  |
|  |  |  | 700 (max) |  |  |  |  |  |  |  |  | L/hr (max) |  |  |  |
|  | * NOR allows for flow rates less than the minimum of 500 LMH if the equipment used is not able to meet the range. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Calculate hold-up of the depth filters. |  |  |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day |  |  | Number of C0HC Filters (Step 11.3) |  | X |  | Internal Void Volume of a 1.1m2 C0HC Depth Filter |  |  | = |  | Hold-Up Volume of Installed Depth Filters |  |  |
|  | 1 |  |  |  |  |  |  | 10.3 L |  |  |  |  | L |  |  |
|  | 2 |  |  |  |  |  |  | 10.3 L |  |  |  |  | L | Day 2 | Day 2 |
|  | Calculate the minimum volume of WFI to flush the depth filtration units |  |  |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day | WFI Min |  |  | x |  | Total surface area of C0HC units (Step 11.4) |  |  | = |  | Minimum WFI flush volume |  |  |  |
|  | 1 | 105 L/ m2 |  |  |  |  | m2 |  |  |  |  | L |  |  |  |
|  | 2 |  |  |  |  |  | m2 |  |  |  |  | L |  | Day 2 | Day 2 |

| ## Step | Description |  |  |  |  |  |  |  |  | Results |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the minimum volume of Equilibration Buffer (PN 8004949: 50 mM Tris, pH 7.4) to flush the depth filtration units |  |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day |  | Buffer Min | x | Total surface area of C0HC units (Step 11.4) |  |  | + | Hold-Up Volume of Installed Depth Filters (Step 11.6) |  | = | Minimum Buffer flush volume |  |  |
|  | 1 |  | 30 L/ m2 |  | m2 |  |  |  | L |  |  | L |  |  |
|  | 2 |  |  |  | m2 |  |  |  | L |  |  | L | Day 2 | Day 2 |
|  |  | Confirm that the pH/conductivity meter has been standardized per SOP-0080297. |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  |  | Meter ID (record on Day 1 only) |  |  |  |  |  |  |  |  |  |  | Day 2 | Day 2 |
|  |  | Confirm that the thermometer is within its calibration interval. Record the equipment ID of the thermometer used. |  |  |  |  |  |  |  |  |  |  | Day 1 | Day 1 |
|  |  | Thermometer ID (record on Day 1 only) |  |  |  |  |  |  |  |  |  |  | Day 2 | Day 2 |

| ## Step | Description |  |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Obtain and document the holder ID for the depth filter Pods and the pressure sensors to be used (Day 1) |  |  |  | Day 1 | Day 1 |
|  | Pod Holder ID Day 1 |  |  |  |  |  |
|  | Pod Pressure Sensors | Part No. | Batch No. | Exp. Date |  |  |
|  | Inlet |  |  |  |  |  |
|  | Outlet |  |  |  |  |  |
|  | Pendotech ID |  |  |  |  |  |
|  | Pendotech Calibration Due Date |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |
|  | Obtain and document the holder ID for the depth filter Pods and the pressure sensors to be used. (Day 2) |  |  |  | Day 2 | Day 2 |
|  | Pod Holder ID |  |  |  |  |  |
|  | Pod Pressure Sensors | Part No. | Batch No. | Exp. Date |  |  |
|  | Inlet |  |  |  |  |  |
|  | Outlet |  |  |  |  |  |
|  | Pendotech ID |  |  |  |  |  |
|  | Pendotech Calibration Due Date |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |

| Step | Description | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | Install the depth filtration units per SOP-0107204: Per the diagram in this step: - Install the depth filter units into the column position of the chromatography skid - Skid Column Top → POD Inlet T - POD Inlet T →POD Holder Inlet Pressure Sensor T - POD Holder Outlet Pressure Sensor T → POD Outlet T - POD Outlet T → Skid Column Bottom - Create a bypass line by connecting tubing between the two Ts. - Using pinch clamps, adjust both T’’s to set flow through the POD(s). | Day 1 | Day 1 |
|  |  | Day 2 | Day 2 |

# Depth Filter Pre-Flush

| Step | Description |  |  | Results | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Make the connections: - Inlet 2: 50 mM Tris, pH 7.4 (8004949) - Inlet 1: WFI - Outlet 1: Waste |  |  |  | Day 1 | Day 1 |
|  |  |  |  |  | Day 2 | Day 2 |
|  | Depth Filter WFI Flush - Flush the filter pods per SOP-0080429. - Ensure the filter vent valves are bled. - Flush the filters with at least the amount calculated in Step 11.7. - Perform Manual Operation on the AKTA Skid using the parameters listed below per SOP-0107212. Initiate Manual Run and label file name “Batch ID: PartNumber BatchNumber DepthFilterFlush Date” - Inlet: 1 - Airtrap: Inline - Column: Inline - Outlet: 1 - Alarm_Press_PT111: High Alarm: 3.4 bar - Alarm_Delta_PT111_PT112: High Alarm 1.3 bar - Flow: 25%* - After filter is bled, target flowrate per Step 11.5* |  |  |  | Day 1 | Day 1 |
|  | # Day 1 | # Batch ID |  |  |  |  |
|  |  | # POD WFI Flush Volume (≥Step 11.7) | # L |  |  |  |
|  | # Day 2 | # Batch ID |  |  | Day 2 | Day 2 |
|  |  | # POD WFI Flush Volume (≥Step 11.7) | # L |  |  |  |

*NOTE: Adjust the flow as needed to run as close to the target flow rate without exceeding the pressure alarms.

12	Depth Filter Flush (continued)

| Step | Description |  |  | Results | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
| ## 12.3 | Depth Filter Equil Flush - Flush the filter pods per SOP-0080429. - Ensure the filter vent valves are bled. - Flush the filters with at least the amount calculated in Step 11.8. - Perform Manual Operation on the AKTA Skid using the parameters listed below per SOP-0107212. Initiate Manual Run and label file name “Batch ID: PartNumber BatchNumber DepthFilterEquil Date” - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Alarm_Press_PT111: High Alarm: 3.4 bar - Alarm_Delta_PT111_PT112: High Alarm 1.3 bar - Flow: 25%* - After filter is bled, target flowrate per Step 11.5* |  |  |  | Day 1 | Day 1 |
|  | # Day 1 | # Batch ID |  |  |  |  |
|  |  | # POD Equil Flush Volume (≥Step 11.8) | # L |  |  |  |
|  |  | # SAP Consumption Performed (Day 1) |  |  |  |  |
|  | # Day 2 | # Batch ID |  |  | Day 2 | Day 2 |
|  |  | # POD Equil Flush Volume (≥Step 11.8) | # L |  |  |  |
|  |  | # SAP Consumption Performed (Day 2) |  |  |  |  |

*NOTE: Adjust the flow as needed to run as close to the target flow rate without exceeding the pressure alarms.

# Skid Connections

| ## Step | Description |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | - Verify that the chromatography skid pH probe has been user standardized per SOP-0107212. Record pH probe information below. |  |  | Day 1 | Day 1 |
|  |  | Day 1 | Day 2, If Applicable |  |  |
|  | Part Number | 6003953 | 6003953 |  |  |
|  | Batch Number |  |  | Day 2 | Day 2 |
|  | Expiration Date |  |  |  |  |
|  | SAP Consumption – Day 1 Performed By/Date |  |  |  |  |
|  | SAP Consumption – Day 2 Performed By/Date |  |  |  |  |
|  | - Day 1 – Continue to Step 13.3. - Day 2 – If applicable, connect new tubing to the outlet of the Load Vessel and proceed to Step 13.5. |  |  | Day 1 | Day 1 |
|  |  |  |  | Day 2 | Day 2 |

13	Skid Connections (continued)

| ## Step | Description |  |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Connect the buffers to the inlets of the chromatography skid per SOP-0107212. - Inlet 6 will require a Y connection: - Refer to Step 13.4 for instruction to connect to Inlet 6. |  |  |  | Day 1 | Day 1 |
|  | Day 1 |  |  |  |  |  |
|  | Inlet No. | Buffer P/N | Buffer Name | Processing Step |  |  |
|  | 6 | 8008813 | 12.5 mM sodium hydroxide | Strip 2 |  |  |
|  |  | 8006409 | 120 mM phosphoric acid, 167 mM acetic acid, 2.2% (v/v) benzyl alcohol | Strip 3 |  |  |
|  | 5 | 8008812 | 103 mM acetic acid, 2 M urea, pH 3.1 | Strip 1 |  |  |
|  | 4 | 8010534 | 25 mM Sodium Acetate, pH 3.5 | Elution |  |  |
|  | 3 | 8004950 | 50 mM Tris, 0.5 M Sodium Chloride, pH 7.4 | Wash |  |  |
|  | 2 | 8004949 | 50 mM Tris, pH 7.4 | Equilibration 1, 2, 3, 4, 5 |  |  |
|  | 1* | 8010441 | Conditioned Medium | Load |  |  |
|  | * If process will occur over 1 Day and CM Product Bags 1 and 2 will both be processed, connect both bags to Inlet 1 using a Y connector. |  |  |  |  |  |
|  | For Inlet 6, perform the following: - Obtain a Y and connect to Inlet 6. - Connect PN 8006409: 120 mM phosphoric acid, 167 mM acetic acid, 2.2% (v/v) benzyl alcohol to one of the Y legs and PN 8008813: 12.5 mM sodium hydroxide to the other leg. - Ensure the clamp on the PN 8008813: 12.5 mM sodium hydroxide is Open. - Ensure the clamp on the PN 8006409: 120 mM phosphoric acid, 167 mM acetic acid, 2.2% (v/v) benzyl alcohol is Closed. |  |  |  |  |  |
|  | Proceed to Step 13.8. |  |  |  | Day 1 | Day 1 |

13	Skid Connections (continued)

| ## Step | Description |  |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Connect the buffers to the inlets of the chromatography skid per SOP-0107212. - Inlet 6 will require a Y connection: - Refer to Step 13.7 for instruction to connect to Inlet 6. |  |  |  | Day 2 | Day 2 |
|  | Day 2 |  |  |  |  |  |
|  | Inlet No. | Buffer P/N | Buffer Name | Processing Step |  |  |
|  | 6 | 8008813 | 12.5 mM sodium hydroxide | Strip 2 |  |  |
|  |  | 8006409 | 120 mM phosphoric acid, 167 mM acetic acid, 2.2% benzyl alcohol | Strip 3 |  |  |
|  | 5 | 8008812 | 103 mM acetic acid, 2 M urea, pH 3.1 | Strip 1 |  |  |
|  | 4 | 8010534 | 25 mM Sodium Acetate, pH 3.5 | Elution |  |  |
|  | 3 | 8004950 | 50 mM Tris, 0.5 M Sodium Chloride, pH 7.4 | Wash |  |  |
|  | 2 | 8004949 | 50 mM Tris, pH 7.4 | Equilibration 1, 2, 3, 4, 5 |  |  |
|  | 1 | 8010441 | Conditioned Medium | Load |  |  |
|  | - For Inlet 6, perform the following: - Obtain a Y and connect to Inlet 6. - Connect PN 8006409: 120 mM phosphoric acid, 167 mM acetic acid, 2.2% (v/v) benzyl alcohol to one of the Y legs and PN 8008813: 12.5 mM sodium hydroxide to the other. - Ensure the clamp on the PN 8008813: 12.5 mM sodium hydroxide is Open. - Ensure the clamp on the PN 8006409: 120 mM phosphoric acid, 167 mM acetic acid, 2.2% (v/v) benzyl alcohol is Closed. |  |  |  | Day 2 | Day 2 |

13	Skid Connections (continued)

| Step | Description |  |  |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Column Guard Filter - Obtain a column guard filter, refer to Section 4. - Install the inlet of the column guard filter to the POD outlet T. - Install the outlet of the column guard filter to the inlet of the column tubing. - Install the column outlet to the skid per SOP-0107212. - Using pinch clamps, adjust both T’s to bypass the POD(s) NOTE: The column guard filter does not need to be autoclaved prior to use. NOTE: A 30” SHC filter (4100987) is preferred for use as the column guard filter. |  |  |  | Day 1 | Day 1 |
|  | Day 1 |  |  |  |  |  |
|  | Part Number | Batch Number | Exp. Date | Serial No. |  |  |
|  | SAP Consumption Performed by/ Date |  |  |  |  |  |
|  | Day 2 (If Applicable) |  |  |  | Day 2 | Day 2 |
|  | Part Number | Batch Number | Exp. Date | Serial No. |  |  |
|  | SAP Consumption Performed by/ Date |  |  |  |  |  |

# CaptureSelect CH1-XL Chromatography

| Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | - Verify from AZD8630 Pod Harvest MPR (MABR-0023642) that the CM product was held for at least 12 hours at 2-8 °C prior to being placed at room temperature. - Note: Contact supervisor if CM product was not held for at least 12 hours at 2-8 °C prior to being placed at room temperature. | Day 1, First Cycle | Day 1, First Cycle |
|  | - Connect the CaptureSelect CH1-XL column obtained from Step 8.1 to the skid according to SOP-0107212. - Ensure the CM Product is connected to Inlet 1. - Ensure the outlet from the CM Product Vessel is open. | First Cycle, Day 1 | First Cycle, Day 1 |
|  |  | First Cycle, Day 2 | First Cycle, Day 2 |
|  | - Ensure the Guard filter vent valve is open. - Ensure that the column inlet is closed. - Ensure that the bleed valve to the column is open. - Ensure that the column outlet is open. | CYCLE 1 | CYCLE 1 |
|  |  | CYCLE 2 | CYCLE 2 |
|  |  | CYCLE 3 | CYCLE 3 |
|  |  | CYCLE 4 | CYCLE 4 |
|  |  | CYCLE 5 | CYCLE 5 |
|  | - Ensure that the Filtered CaptureSelect CH1-XL Product Collection Vessel is set up and connected to Outlet 4 on the AKTA. | CYCLE 1 | CYCLE 1 |
|  |  | CYCLE 2 | CYCLE 2 |
|  |  | CYCLE 3 | CYCLE 3 |
|  |  | CYCLE 4 | CYCLE 4 |
|  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| ## Step | Description |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the POD chase volume. |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Day | Number of C0HC Filters (Step 11.3) |  | x | POD Hold Up Volume | = | POD Chase Volume |  |  |
|  | 1 |  |  |  | 10.6 L |  | L |  |  |
|  | 2 |  |  |  |  |  | L | Day 2 | Day 2 |
|  | - Confirm that the thermometer is within its calibration interval. Record the equipment ID of the thermometer used. |  |  |  |  |  |  | Day 1 | Day 1 |
|  | Thermometer ID |  |  |  |  |  |  | Day 2 | Day 2 |
|  | - Confirm that the pH meter has been standardized between pH 4.0 and pH 10.0 per SOP-0080297. - Confirm that the conductivity meter has been standardized per SOP-0080297. |  |  |  |  |  |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  |  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  |  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  |  |  |  |  | CYCLE 4 | CYCLE 4 |
|  | Meter ID |  |  |  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Initiate the AZD8630_ProteinA method per SOP-0107212. Document the variables below and input into the method. |  |  |  |  |  |  |  |  |

14	CaptureSelect CH1-XL Chromatography (continued)

| ## AZD8630_ProteinA Method Overview |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Volumes | Target Flowrate | Step |  |  |  |
| 3 CV | 318 L/hr | Equilibration 1 |  |  |  |
| ***Step 9.13 or Step 9.16 or Step 9.10 |  | Load |  |  |  |
| 3 CV |  | Equilibration 2 |  |  |  |
| 3 CV |  | Wash |  |  |  |
| 3 CV |  | Equilibration 3 |  |  |  |
| N/A | 239 L/hr | Elution - Collection Start: 37.5 mAU - Collection End: 37.5 mAU |  |  |  |
| 3 CV | 318 L/hr | Strip 1 |  |  |  |
| 1 CV |  | Equilibration 4 |  |  |  |
| 3 CV |  | Strip 2 |  |  |  |
| 1 CV |  | Equilibration 5 |  |  |  |
| 3 CV |  | Strip 3 |  |  |  |
| Variables | Cycle 1 | Cycle 2, if needed | Cycle 3, if needed | Cycle 4, if needed | Cycle 5, if needed |
| Column Bed Volume (Step 8.1) | L | L | L | L | L |
| Column Inlet Pressure Max (Step 8.2)a | bar | bar | bar | bar | bar |
| Column Differential Pressure (Step 8.1)b | barD | barD | barD | barD | barD |
| Load Volumec (Step 9.13 or Step 9.16 or Step 9.10) | L | L | L | L | L |
| Performed By/Date |  |  |  |  |  |
| Verified By/ Date |  |  |  |  |  |

- If CPDS value is greater than the skid rating, record the skid max pressure (5 bar).

- If CPDS value is greater than the skid rating, record the skid max differential pressure (4 barD).

- Note:

- Step 9.13 for Day 1 of 2

- Step 9.16 for Day 2 of 2

- Step 9.10, if both bags are processed in one day.

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  | Performed by/ Date |  | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Start AZD8630_ProteinA per SOP-0107212. During the process, the following ranges are allowed: - Process CV Ranges: -0.1 CV to +1.0 CV from target - Flowrate Range: 302 – 334 L/hr. NOR: 0.0 – 350 L/hr. - Elution Flowrate Range is 226 – 251 L/hr. NOR: 0.0 – 350 L/hr - Record the start time below and batch Name ID below. - Verify flow paths are correct during the execution of the chromatography recipe. If there are any discrepancies between the recipe and the flow paths outlined below, immediately contact area management. - The chromatography process, as outlined in this MPR, may be run manually if the automated chromatography method is not functioning correctly. NOTE: To avoid over pressurization, flow rates may be lowered manually or as part of the method. Record adjusted flowrate information in Attachment 4: Adjusted Flowrate Table. (Format: PartNumber_ProductBatchNumber_Cycle#_AZD8630_ProteinA_Date) |  |  | CYCLE 1 |  | CYCLE 1 |
|  | Cycle 1 | Name ID |  |  |  |  |
|  |  | Start Time |  |  |  |  |
|  | Cycle 2, If Applicable | Name ID |  | CYCLE 2 |  | CYCLE 2 |
|  |  | Start Time |  |  |  |  |
|  | Cycle 3, If Applicable | Name ID |  | CYCLE 3 |  | CYCLE 3 |
|  |  | Start Time |  |  |  |  |
|  | Cycle 4, If Applicable | Name ID |  | CYCLE 4 | CYCLE 4 |  |
|  |  | Start Time |  |  |  |  |
|  | Cycle 5, If Applicable | Name ID |  | CYCLE 5 | CYCLE 5 |  |
|  |  | Start Time |  |  |  |  |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Skid Flush (8004949: 50 mM Tris, pH 7.4) | - Inlet: 2 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flowrate: 50% - Volume: 5 L | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Column Guard Filter Purge (8004949: 50 mM Tris, pH 7.4) Set the following valves: - Column Inlet Valve: Closed - Column Bleed Valve: Open - Guard Filter Vent: Open NOTE: Once the air from the guard filter is adequately purged, Close the vent on the guard filter. | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flowrate: 25% - Duration: 60 seconds | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Column Bleed Line Purge (8004949: 50 mM Tris, pH 7.4) When prompted, adjust the following valves: - Column Inlet Valve: Open - Column Bleed Valve: Closed - Column Outlet Valve: Open | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flowrate: 25% - Duration: 60 seconds | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| ## Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Equilibration 1 (8004949: 50 mM Tris, pH 7.4) NOTE: After 2 CVs, column effluent results can be Performed per Step 14.15. |  |  | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - At 2 CVs: UV Autozero - Volume: 3 CV |  |  | CYCLE 1 | CYCLE/ 1 |
|  |  |  |  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Equilibration Effluent Results - Confirm the equilibration effluent is in range using an offline meter and record the results for pH, conductivity, and temperature for each cycle below. - Verify that the pH is within the range of 7.2 – 7.6. NOR: 7.0 – 7.8. NOTE: Contact the area supervisor/designee if the pH is not within specification. |  |  |  |  |  | CYCLE 1 | CYCLE 1 |
|  | Cycle | Offline pH (7.2 – 7.6) | Conductivity |  | Temperature (15 – 25 °C) | CV where taken |  |  |
|  | 1 |  | mS/cm |  | °C | CV |  |  |
|  | 2 |  | mS/cm |  | °C | CV | CYCLE 2 | CYCLE 2 |
|  | 3 |  | mS/cm |  | °C | CV | CYCLE 3 | CYCLE 3 |
|  | 4 |  | mS/cm |  | °C | CV | CYCLE 4 | CYCLE 4 |
|  | 5 |  | mS/cm |  | °C | CV | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | - After the equilibration effluent has been confirmed to be in range, remove a sample of the Affinity Chromatography Column Effluent (8010457-20-1 for Day 1) & (8010457-20-2 for Day 2) according to Attachment 1: Sampling Record. NOTE: This is required for the first cycle of each processing day only. For all other cycles continue to Step 14.20. |  |  | DAY 1 | DAY 1 |
|  | Sampling Date and Time- Day 1 | Date: | Time: |  |  |
|  | Sampling Date and Time- Day 2 | Date | Time: | DAY 2 | DAY 2 |
|  | - Aliquot the sample (sample designation: (8010457-20-1 for Day 1) & (8010457-20-2 for Day 2) per Attachment 1: Sampling Record - If applicable, aliquot non-routine samples per Attachment 2: Non-Routine Sampling Record. - Label the sample per SOP-0107056. |  |  | DAY 1 | DAY 1 |
|  |  |  |  | DAY 2 | DAY 2 |
|  | - Submit the sample to the storage location indicated in DLIMS per SOP-0080295. NOTE: Ensure the sampling time and location are updated in DLIMS. |  |  | DAY 1 | DAY 1 |
|  |  |  |  | DAY 2 | DAY 2 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - The skid will pause - Reposition the clamps to place the POD(s) inline. - Press continue once complete NOTE: the POD(s) will stay in line for Load through Equilibration 2. |  |  |  |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Load (8010441: Conditioned Medium) For each cycle: - Record the load start time and end time for each cycle. - Record the Actual Volume Loaded from the Skid Totalizers. |  |  | - Inlet: 1 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: See step (Step 9.13, 9.16 or 9.10)* *Step 9.13 for Day 1 Step 9.16 for Day 2 Step 9.10, if both bags are processed in one day |  | CYCLE 1 | CYCLE 1 |
|  | Cycle | Load Start Time | Load End Time |  | Actual Volume Loaded per Skid Totalizer |  |  |
|  | 1 |  |  |  | L |  |  |
|  | 2 |  |  |  | L | CYCLE 2 | CYCLE 2 |
|  | 3 |  |  |  | L | CYCLE 3 | CYCLE 3 |
|  | 4 |  |  |  | L | CYCLE 4 | CYCLE 4 |
|  | 5 |  |  |  | L | CYCLE 5 | CYCLE 5 |
|  | Total Load Volume (All Cycles) |  |  |  | L | All Cycles | All Cycles |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Equilibration 2 (8004949: 50 mM Tris, pH 7.4) | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 3 CV | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | - The skid will pause - Adjust both T’s to place the POD(s) in bypass. - Press Continue once complete. NOTE: The POD(s) will stay in bypass for the remainder of this cycle. |  | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Skid Flush Wash (8004950: 50 mM Tris, 0.5 M Sodium Chloride, pH 7.4) | - Inlet: 3 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Wash (8004950: 50 mM Tris, 0.5 M Sodium Chloride, pH 7.4) | - Inlet: 3 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 3 CV | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Skid Flush Equilibration 3 (8004949: 50 mM Tris, pH 7.4) | - Inlet: 2 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Equilibration 3 (8004949: 50 mM Tris, pH 7.4) | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 3 CV | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | - Ensure that the inlet to the product collection vessel is open. - Ensure the floor scale is tared and reading zero. |  | Day 1, First Cycle | Day 1, First Cycle |
|  |  |  | Day 2, First Cycle | Day 2, First Cycle |
|  | Skid Flush Elution (8010534: 25 mM Sodium Acetate, pH 3.5) | - Inlet: 4 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Elution (8010534: 25 mM Sodium Acetate, pH 3.5) - Collection will begin when the absorbance is 37.5 mAU and the outlet will switch to Outlet 4. - Bleed Air from the Product Filter at the start of collection. | - Inlet: 4 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 239 L/hr - Collection Start: 37.5 mAU - Outlet: 4 - Collection End: 37.5 mAU | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Elution (continued) - Once the UV returns to 37.5 mAU, the Outlet will switch to Outlet 1. - Record the volume of the collected product per cycle from Outlet 4 Totalizer. If Outlet 4 Totalizer is not available, weigh the product eluted from the cycle. - Record the end time for the Elution per cycle. |  |  | CYCLE 1 | CYCLE 1 |
|  | Cycle | Product Net Weight/Volume | Elution End Time |  |  |
|  | 1 | L=kg |  |  |  |
|  | 2 | L=kg |  | CYCLE 2 | CYCLE 2 |
|  | 3 | L=kg |  | CYCLE 3 | CYCLE 3 |
|  | 4 | L=kg |  | CYCLE 4 | CYCLE 4 |
|  | 5 | L=kg |  | CYCLE 5 | CYCLE 5 |
|  | Skid Flush Strip 1 (8008812: 103 mM acetic acid, 2 M urea, pH 3.1) |  | - Inlet: 5 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L | CYCLE 1 | CYCLE 1 |
|  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| ## Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Strip 1 (8008812: 103 mM acetic acid, 2 M urea, pH 3.1) | - Inlet: 5 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 3 CV | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Skid Flush Equilibration 4 (8004949: 50 mM Tris, pH 7.4) | - Inlet: 2 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Equilibration 4 (8004949: 50 mM Tris, pH 7.4) | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 1 CV | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | - Prior to the start of Skid Flush Strip 2 perform the following: - Close the outlet to the Strip 3 Buffer (8006409). - Open the outlet to the Strip 2 Buffer (8008813). |  |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Skid Flush Strip 2 (8008813: 12.5 mM sodium hydroxide) | - Inlet: 6 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Strip 2 (8008813: 12.5 mM sodium hydroxide) At the completion of Strip 2 for each cycle, record the volume from the Inlet 6 totalizer. |  | - Inlet: 6 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 3 CV | CYCLE 1 | CYCLE 1 |
|  | Inlet 6 Totalizer Volume |  | L |  |  |
|  | Inlet 6 Totalizer Volume |  | L | CYCLE 2 | CYCLE 2 |
|  | Inlet 6 Totalizer Volume |  | L | CYCLE 3 | CYCLE 3 |
|  | Inlet 6 Totalizer Volume |  | L | CYCLE 4 | CYCLE 4 |
|  | Inlet 6 Totalizer Volume |  | L | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Strip 2 Results - Verify the effluent conductivity is within the NOR of 2.2-3.8 mS/cm. - Verify the temperature is in range. NOTE: Contact a supervisor if the conductivity is not within specification. |  |  |  | CYCLE 1 | CYCLE 1 |
|  | Cycle | Conductivity (2.2-3.8 mS/cm) |  | Temperature (15-27 °C) |  |  |
|  |  |  |  | °C |  |  |
|  |  |  |  | °C | CYCLE 2 | CYCLE 2 |
|  |  |  |  | °C | CYCLE 3 | CYCLE 3 |
|  |  |  |  | °C | CYCLE 4 | CYCLE 4 |
|  |  |  |  | °C | CYCLE 5 | CYCLE 5 |
|  | Skid Flush Equilibration 5 (8004949: 50 mM Tris, pH 7.4) |  | - Inlet: 2 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Equilibration 5 (8004949: 50 mM Tris, pH 7.4) |  | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 1 CV |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Prior to the start of Skid Flush Strip 3 perform the following: - Close the outlet to the Strip 2 Buffer (8008813). - Open the outlet to Strip 3 Buffer (8006409). |  |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Skid Flush Strip 3 (8006409: 120 mM Phosphoric Acid, 167 mM Acetic Acid, 2.2 % (v/v) Benzyl Alcohol) |  | - Inlet: 6 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% - Volume: 5 L | CYCLE 1 | CYCLE 1 |
|  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Strip 3 (8006409: 120 mM Phosphoric Acid, 167 mM Acetic Acid, 2.2 % (v/v) Benzyl Alcohol) |  | - Inlet: 6 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 3 CV | CYCLE 1 | CYCLE 1 |
|  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  |  | CYCLE 5 | CYCLE 5 |
|  | Confirm that the pH meter has been standardized between pH 2.0 and pH 7.0 per SOP-0080297. |  |  | CYCLE 1 | CYCLE 1 |
|  |  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  |  | CYCLE 4 | CYCLE 4 |
|  | Meter ID |  |  | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Strip 3 Results - Verify the effluent pH is ≤ 2.4 (NOR: ≤ 2.5) using an offline meter. - Verify the temperature is in range. NOTE: Contact a supervisor if the pH is not within specification. |  |  |  | Cycle 1 | Cycle 1 |
|  | Cycle |  | pH (≤ 2.4) | Temperature (15 – 27 °C) |  |  |
|  | 1 |  |  | °C |  |  |
|  | 2 |  |  | °C | CYCLE 2 | CYCLE 2 |
|  | 3 |  |  | °C | CYCLE 3 | CYCLE 3 |
|  | 4 |  |  | °C | CYCLE 4 | CYCLE 4 |
|  | 5 |  |  | °C | CYCLE 5 | CYCLE 5 |

14	CaptureSelect CH1-XL Chromatography (continued)

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | After the completion of the Strip 3 step, the chromatography skid pumps will stop. This recipe is now complete. Record buffer volumes from skid totalizers in Attachment 5: Buffer Totalizer Volumes. |  | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | Continue to Step 14.3 if more cycles are required. Continue to Step 14.48 if no additional cycles are needed on the current processing day. |  | CYCLE 1 | CYCLE 1 |
|  |  |  | CYCLE 2 | CYCLE 2 |
|  |  |  | CYCLE 3 | CYCLE 3 |
|  |  |  | CYCLE 4 | CYCLE 4 |
|  |  |  | CYCLE 5 | CYCLE 5 |
|  | At the completion of the Strip 3 step for the final cycle, hold the column for 35 minutes (NOR: 30 -120 minutes). NOTE: At the end of the last day of processing, sampling may be performed concurrently with this step. |  | Final Cycle (Day 1) | Final Cycle (Day 1) |
|  | Sanitization HOLD Start Time, FINAL Cycle (Day 1) |  |  |  |
|  | Sanitization HOLD Start Time, FINAL Cycle (Day 2) |  | Final Cycle (Day 2) | Final Cycle (Day 2) |

# Storage of the Column

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Record the information for the Column Storage buffer below. |  |  |  | Day 1 | Day 1 |
|  | Column Storage Buffer | Part No. | Batch No. | Exp. Date |  |  |
|  | 2% (v/v) Benzyl alcohol, 100 mM sodium acetate, pH 5.0 | 8004953 |  |  |  |  |
|  |  |  |  |  | Day 2 | Day 2 |
|  | - Close the Outlets and disconnect buffers 8006409 and 8008813 from Inlet 6. - Connect the storage buffer (8004953) 2% (v/v) benzyl alcohol, 100 mM sodium acetate, pH 5.0 to inlet 6. - Ensure the outlet from the (8004953) 2% (v/v) benzyl alcohol, 100 mM sodium acetate, pH 5.0 is Open. |  |  |  | Day 1 | Day 1 |
|  |  |  |  |  | Day 2 | Day 2 |
|  | Initiate the AZD8630 Affinity Storage method per SOP-0107212. Verify the method parameters per Step 15.4 |  |  |  | Day 1 | Day 1 |
|  |  |  |  |  | Day 2 | Day 2 |

15	Storage of the Column (continued)

| ## AZD8630 CaptureSelect CH1-XL Store Storage Method Overview |  |  |  |
| --- | --- | --- | --- |
| Volumes | Step |  |  |
| 5L | Skid Flush – Pre-Storage Equilibration |  |  |
| 1 CV | Pre-Storage Equilibration |  |  |
| 5 L | Skid Flush – Column Storage |  |  |
| 3 CV | Column Storage |  |  |
| Variables |  | Day 1 | Day 2 (If Applicable) |
| Column Bed Volume (Step 8.1) |  | L | L |
| Column Inlet Pressure Max (Step 8.2)* |  | bar | bar |
| Column Differential Pressure (Step 8.1)** |  | barD | barD |
| Pre-Storage Equilibration and Column Storage Flow Rates*** |  | 318 L/hr | 318 L/hr |
| Performed By/Date |  |  |  |
| Verified By/ Date |  |  |  |

*If CPDS value is greater than the skid’s rating, record the skid’s max pressure (5 bar).

**If CPDS value is greater than the skid’s rating, record the skid’s max differential pressure (4 barD).

***To avoid over pressurization flow rates may be lowered manually or as a part of the recipe.
The NOR for column storage is 0.0 – 350 L/hr.

15	Storage of the Column (continued)

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | Start AZD8630 Affinity Storage method per SOP-0107212. - Record the batch Name ID and Start Time below. - Verify flow paths are correct during the execution of the storage method. - If there are any discrepancies between the method and the flow paths outlined below, immediately contact area supervisor. |  |  | Day 1 | Day 1 |
|  | Name ID, Day 1 |  |  |  |  |
|  | Start Time, Day 1 |  |  |  |  |
|  | Name ID, Day 2 (if applicable) |  |  | Day 2 | Day 2 |
|  | Start Time, Day 2 (if appliable) |  |  |  |  |
|  | Skid Flush Pre-Storage Equilibration (8004949: 50 mM Tris, pH 7.4) |  | - Inlet: 2 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% Volume: 5 L | Day 1 | Day 1 |
|  |  |  |  | Day 2 | Day 2 |

15	Storage of the Column (continued)

| Step | Description |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- |
|  | Pre-Storage Equilibration (8004949 50 mM Tris, pH 7.4) Record the column sanitization Hold End Time and calculate the Hold Duration below: - The Hold End Time is the time when the pump starts for this step. - The sanitization Hold Duration is the time in minutes that has elapsed between the Sanitization Hold Start Time (Step 14.48) and the Hold End Time. - NOR for the sanitization Hold Duration is 30 – 120 minutes. | - Inlet: 2 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr - Volume: 1 CV | Day 1 | Day 1 |
|  | Day 1 Hold End Time* |  |  |  |
|  | * The Pre-Storage Flush start time is the sanitization hold end time. |  |  |  |
|  | Day 1 Hold Duration (30 – 60 minutes, Target 35 minutes) | min |  |  |
|  | Day 2 Hold End Time* (if applicable) |  | Day 2 | Day 2 |
|  | * The Pre-Storage Flush start time is the sanitization hold end time. |  |  |  |
|  | Day 2 Hold Duration (if appliable) (30 – 60 minutes, Target 35 minutes) | min |  |  |

15	Storage of the Column (continued)

| ## Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Skid Flush Storage (8004953: 2% (v/v) benzyl alcohol, 100 mM sodium acetate, pH 5.0) |  | - Inlet: 6 - Airtrap: Inline - Column: Bypass - Outlet: 1 - Flow: 50% Volume: 5 L |  | Day 1 | Day 1 |
|  |  |  |  |  | Day 2 | Day 2 |
|  | Column Storage (8004953: 2% (v/v) benzyl alcohol,100 mM sodium acetate, pH 5.0) |  | - Inlet: 6 - Airtrap: Inline - Column: Inline - Outlet: 1 - Flow: 318 L/hr Volume: 3 CV |  | Day 1 | Day 1 |
|  |  |  |  |  | Day 2 | Day 2 |
|  | Column Storage Effluent Results - At the end of Column Storage, verify the Column Storage effluent is in range using an offline meter and record the results pH and temperature below. - Verify that the pH is within the range of 4.8-5.2. NOTE: Contact the area supervisor/designee if the pH is not within specification. |  |  |  | Day 1 | Day 1 |
|  |  | pH (4.8-5.2) |  | Temperature (15 – 25°C) |  |  |
|  | Column Storage Effluent – Day 1 |  |  |  |  |  |
|  | Column Storage Effluent – Day 2 |  |  |  | Day 2 | Day 2 |

15	Storage of the Column (continued)

| ## Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | - After the completion of the Storage step, the chromatography skid pumps will stop. This recipe is now complete. - Store the pH probe in KCI storage solution at the end of day 1, if applicable. - Record buffer volumes from the skid Totalizers in Attachment 5: Buffer Totalizer Volumes. | Day 1 | Day 1 |
|  |  | Day 2 | Day 2 |
|  | - If Day 1 of processing is complete and Day 2 of processing is required, perform the following: - Disconnect the column from the skid and dispose of the column guard filter. - Disconnect the load from the chromatography skid inlet and discard tubing. - If only Day 1 of processing is required or Day 2 processing is complete, perform the following: - Disconnect the column from the skid and dispose of the column guard filter. - Disconnect the load from the chromatography skid inlet and discard tubing. - Disconnect and dispose of the AKTA Ready Kit. | Day 1 | Day 1 |
|  |  | Day 2 | Day 2 |
|  | - Close the inlet to the product collection vessel. - Disconnect the product collection vessel from Outlet 4. - Remove and dispose of the product filter. | Day 1 | Day 1 |
|  |  | Day 2 | Day 2 |

15	Storage of the Column (continued)

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | Proceed based on how many days of CaptureSelect CH1-XL processing is needed: - 2 days: - On Day 1: Go to the next step to record the storage of the Day 1 product. - On Day 2: Go to Section 16 to sample the CaptureSelect CH1-XL product. - 1 day N/A the next step, then go to Section 16 to sample the CaptureSelect CH1-XL product. |  |  | Day 1 | Day 1 |
|  |  |  |  | Day 2 | Day 2 |
|  | Day 1 Storage Start Store the Day 1 CaptureSelect CH1-XL Product and record the storage start time/date below—the storage start time is the Elution End Time of the last cycle on Day 1 (Step 14.30). |  |  | Day 1 | Day 1 |
|  | Day 1 Product Storage Start Time and Date | Time | Date |  |  |
|  | Storage Temperature | 2 – 8 ºC 15 – 25 ºC |  |  |  |

# CaptureSelect CH1-XL Chromatography Product – Sampling

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Weigh the product vessel. Record the weight and relevant scale information. |  |  | Final Day | Final Day |
|  | Scale ID |  |  |  |  |
|  | Calibration Due Date |  |  |  |  |
|  | Gross Weight of the CaptureSelect CH1-XL Product Vessel before sampling |  | kg |  |  |
|  | CaptureSelect CH1-XL Net Weight before sampling |  | kg = L |  |  |
|  | CaptureSelect CH1-XL Net Weight before sampling = Gross Weight of the CaptureSelect CH1-XL Product Vessel before sampling (above) – Tare weight of the CaptureSelect CH1-XL Product Vessel (Step 10.5). |  |  |  |  |
|  | Mix the product for at least 15 minutes and record mixing information below. - Follow SOP-0080423 for the LevTech mixer, if applicable. - Follow SOP-0080622 for mixing in a Mobius bag. - Mix the product at a speed that will effectively mix without causing foaming. NOTE: The agitation setting may be adjusted, as needed, record changes in Attachment 6: Comments Section. |  |  | Final Day | Final Day |
|  | Mixer ID |  |  |  |  |
|  | Agitation Setting | RPM |  |  |  |
|  | Mixing Start Time |  |  |  |  |
|  | Mixing End time |  |  |  |  |
|  | Mixing Duration (≥ 15 minutes) | Minutes |  |  |  |

16	CaptureSelect CH1-XL Chromatography Product – Sampling (continued)

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Remove product from the product collection vessel per Attachment 1: Sampling Record per SOP-0080622. |  |  |  |  | Final Day | Final Day |
|  | Sampling Date and Time |  | Date: |  | Time: |  |  |
|  | - Aliquot the sample (sample designation: 8009938-30) per Attachment 1: Sampling Record - If applicable, aliquot non-routine samples per Attachment 2: Non-Routine Sampling Record. - Label the sample per SOP-0107056. NOTE: If using a non-general supply item for samples, record material information in Section 4 – Additional Manufacturing Supplies Used in Process page. |  |  |  |  | Final Day | Final Day |
|  | - Submit the sample to the storage location indicated in DLIMS per SOP-0080295. NOTE: Ensure the sampling time and location are updated in DLIMS |  |  |  |  | Final Day | Final Day |
|  | Place 10 mL of sample in a 50 ml conical tube and measure the pH, conductivity, and temperature. |  |  |  |  | Final Day | Final Day |
|  | pH |  |  |  |  |  |  |
|  | Conductivity | mS/cm |  |  |  |  |  |
|  | Temperature | °C |  |  |  |  |  |
|  | Obtain the gross weight and net weight of the CaptureSelect CH1-XL collected product after sampling. |  |  |  |  | Final Day | Final Day |
|  | Gross Weight of the CaptureSelect CH1-XL Product vessel after sampling |  |  | kg |  |  |  |
|  | CaptureSelect CH1-XL Net Weight after sampling |  |  | kg = L |  |  |  |
|  | CAPTURESELECT CH1-XL Net Weight after sampling = Gross Weight of the CAPTURESELECT CH1-XL Product vessel after sampling (above) – Tare weight of the CAPTURESELECT CH1-XL Product Vessel (Step 10.5). |  |  |  |  |  |  |

16	CaptureSelect CH1-XL Chromatography Product – Sampling (continued)

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Store the CaptureSelect CH1-XL product and document storage information below. - For a 2-day CaptureSelect CH1-XL process, transcribe the storage start time/date and temperature from Step 15.15 in the table below. - For a 1-day CaptureSelect CH1-XL process: - For product stored at 2 – 8ºC, the storage start time is the time when the product is transferred into 2 – 8ºC storage. - For product stored at 15 – 25ºC, the storage start time is the time when final sampling occurred in Step 16.3 |  |  |  |  | Final Day | Final Day |
|  | Initial Storage Condition | 2 – 8ºC 15 – 25ºC |  |  |  |  |  |
|  | Storage Start Time/Date | Time |  | Time |  |  |  |
|  |  | Date |  | Date |  |  |  |
|  | Record the following values in the AZD8630 in the AZD8630 Pod Harvest Record (MABR-0023642) |  |  |  |  |  |  |
|  | MPR Name |  | Name in Attachment |  | Step No. |  |  |
|  | Load Start Time (First Cycle, Day 1) |  | Conditioned Medium Storage End (Time/Date) |  | Step 14.20 |  |  |
|  | Load Start Time (First Cycle, Day 2), If Applicable |  | Conditioned Medium Storage End (Time/Date) |  | Step 14.20 |  |  |
|  | * Use Day 1 if only one day of CaptureSelect CH1-XL processing is needed. |  |  |  |  |  |  |

# Batch Production Record Review

| Step | Description | Production Initial/Date |
| --- | --- | --- |
|  | Ensure any additional supporting documentation are listed and attached. |  |
|  | Verify that the product net weight after sampling (Step 16.7) is correctly inputted into SAP. |  |
|  | Complete the calculations in Attachment 3: Yield Calculations. |  |
|  | Verify all starting materials, buffers, and CPDS (Column Packing Data Sheet) have been consumed into SAP. |  |
|  | Review the Batch Production Record for completeness and submit for review. |  |

# Attachment 1: Sampling Record

| AZD8630 CaptureSelect CH1-XL |  |  |  | 8009938 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dev Samples DLIMS Project # |  |  |  |  |  |  |  |
| Pre-Load Performed By/Date |  |  |  |  |  |  |  |
| DLIMS Test Name | Sample Volume | Location | Tube Type | Intermediate Storage Temp b | DLIMS Storage Temp. | Sample Number |  |
| -10 (Starting Product Samples) |  |  |  |  |  |  |  |
| N/A |  |  |  |  |  |  |  |
| Column Effluent Samples |  |  |  |  |  | -20-1 (Day 1) | -20-2 (Day 2, If Applicable) |
| BIOBURDEN_IP | 25 mL | QC | PP | 2-8˚C | 2-8˚C |  |  |
| ENDOTOXIN_IP_GPF1 | 10 mL |  | PET | 2-8˚C | 2-8˚C |  |  |
| Product Quality Samples |  |  |  |  |  | -30 (CaptureSelect CH1-XL Product) |  |
| SOLO_A280 a | 10 mL | MS | PP | 2-8˚C | 2-8˚C |  |  |
| IEC_PRE_MAIN_POST | 10 mL | AS | PP | 2-8˚C | 2-8˚C |  |  |
| CHO_SYBR | 10 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| EL_CH1_LIG | 10 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| EL_CHO_GEN | 10 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| HPSEC_PUR | 10 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| CGE_N_RED | 10 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| PLRONC_F68_ELSD | 10 mL |  | PET | 2-8˚C | 2-8˚C |  |  |
| CGE_RED | 10 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| SILICONE | 10 mL |  | PET | 2-8˚C | 2-8˚C |  |  |
| RP_LC_CONC | 10 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| ABT Retain | 5 x 14 mL |  | PP | 2-8˚C | 2-8˚C |  |  |
| PPS Small Retain | 3 x 14 mL |  | PP | 2-8˚C | 2-8˚C |  |  |

- Test SOLO_A280 sample per SOP-0107091. If sample is not tested immediately, store sample at 2-8 °C.

- The intermediate storage temperature applies to both the shipment and storage of the sample prior to reaching the Sample Management team.

Definitions/Terminology:

- Use GREENFORM Template for Development DLIMS sample submissions

- Sample Designation “-10”:  For sampling of starting material.

- Sample Designation “-20”:  For sampling column effluent.

- Sample Designation “-30”:  For product quality samples.

- Bold (STAT): Bold text indicates STAT sample submission.

# Attachment 2: Non-Routine Sampling Record

- If non-routine samples are taken, re-adjust weight of the product sampled.

- Log into DLIMS per SOP-0080295 as required by the memo. Record DLIMS Project number and sample number if submitted per DLIMS.

| Pre-Load Performed by/Date |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Process Section No. | Contents | DLIMS Form | DLIMS Project Number | Container Type | Sample Volume | DLIMS Test Name | Sample Number | Storage Temp. (oC) | Location | Performed By/Date | Witnessed By/Date |

# Attachment 3: Yield Calculations

| Net Volume of CM – Bag 1 | X | Concentration of CM - Bag 1 | = | Load Mass, Pre-Sampling (a1) | Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- |
| ___________L_ (Step 9.1) |  | ________g/L__ (Step 9.1) |  | g |  |  |
| Net Volume of CM – Bag 2 | X | Concentration of CM - Bag 2 | = | Load Mass, Pre-Sampling (a2) | Performed by/Date | Verified by/Date |
| ___________L_ (Step 9.1) |  | ________g/L__ (Step 9.1) |  | g |  |  |
| Load Mass, Pre-Sampling (a1) | + | Load Mass, Pre-Sampling (a2) | = | Load Mass, Pre-Sampling (a) | Performed by/Date | Verified by/Date |
| g |  | g |  | g |  |  |
| Final Product Volume Pre-Sampling | x | Final Product Concentration | = | Final Product Mass Pre-Sampling (b) | Performed by/Date | Verified by/Date |
| __________L_ (Step 16.1) |  | ________g/L__ (8009938-30)1 |  | g |  |  |
| Final Product Volume, Post-Sampling | X | Final Product Concentration | = | Final Product Mass, Post-Sampling (c) | Performed by/Date | Verified by/Date |
| ___________L_ (Step 16.7) |  | ________g/L__ (8009938-30)1 |  | g |  |  |

1TP_A280 should be used for concentration.

# Attachment 3: Yield Calculations (Continued)

| Final Product Mass Pre-Sampling (b) | ÷ | Load Mass, Pre-Sampling (a) | x | 100 | = | % Yield | Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Final Product Mass, Post-Sampling (c) | ÷ | Load Mass, Pre-Sampling (a) | x | 100 | = | % Yield | Performed by/Date | Verified by/Date |

# Attachment 4: Adjusted Flowrate Table

| Step Number | Cycle Number | Adjusted Flow Rate Achieved | Reason for Flow Rate Change | Performed By/Date | Witnessed By/Date |
| --- | --- | --- | --- | --- | --- |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |

Attachment 4: Adjusted Flowrate Table (Continued)

| Step Number | Cycle Number | Adjusted Flow Rate Achieved | Reason for Flow Rate Change | Performed By/Date | Witnessed By/Date |
| --- | --- | --- | --- | --- | --- |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |

Attachment 4: Adjusted Flowrate Table (Continued)

| Step Number | Cycle Number | Adjusted Flow Rate Achieved | Reason for Flow Rate Change | Performed By/Date | Witnessed By/Date |
| --- | --- | --- | --- | --- | --- |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |

Attachment 4: Adjusted Flowrate Table (Continued)

| Step Number | Cycle Number | Adjusted Flow Rate Achieved | Reason for Flow Rate Change | Performed By/Date | Witnessed By/Date |
| --- | --- | --- | --- | --- | --- |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |
|  |  | L/hr |  |  |  |

# Attachment 5: Buffer Totalizer Volumes

| Inlet | Buffer |  | Cycle 1 | Cycle 2 | Cycle 3 | Cycle 4 |  | Cycle 5 |  | Total | Consumption Performed By/Date |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6* | 12.5 mM sodium hydroxide (8008813) |  | L | L | L | L |  | L |  | L |  |  |
| 6 | 120 mM phosphoric acid, 167 mM acetic acid, 2.2% (v/v) benzyl alcohol (8006409) |  | L | L | L | L |  | L |  | L |  |  |
| 5 | 103 mM acetic acid, 2 M urea, pH 3.1 (8008812) |  | L | L | L | L |  | L |  | L |  |  |
| 4 | 25 mM Sodium Acetate, pH 3.5(8010534) |  | L | L | L | L |  | L |  | L |  |  |
| 3 | 50 mM Tris, 0.5 M Sodium Chloride, pH 7.4 (8004950) |  | L | L | L | L |  | L |  | L |  |  |
| 2 | 50 mM Tris, pH 7.4 (8004949) |  | L | L | L | L |  | L |  | L |  |  |
| Performed By / Date |  |  |  |  |  |  |  |  |  |  |  |  |
| Verified By / Date |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | *Transcribe Buffer Totalizer Volumes per cycle for 8008813 from Step 14.37. |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | (Day 1 Final Cycle) |  | (Day 2 Final Cycle) |  | Total |  |  | Consumption Performed By/Date |
| 2 | 50 mM Tris, pH 7.4 (8004949) |  |  |  | L |  | L |  | L |  |  |  |
| 6 | 2% (v/v) benzyl alcohol, 100 mM sodium acetate, pH 5.0 (8004953) |  |  |  | L |  | L |  | L |  |  |  |
| Performed By / Date |  |  |  |  |  |  |  |  |  |  |  |  |
| Verified By / Date |  |  |  |  |  |  |  |  |  |  |  |  |

# Attachment 6: Comments Section

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

# Attachment 6: Comments Section (Continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

# Attachment 6: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

# MASTER PRODUCTION RECORD REVISION HISTORY

The following is included for reference only and is not part of the executable record.

| Version | Change | Reason |
| --- | --- | --- |
| 3.0 | This is a Full Approval per SOP-0107045: - TP_A280 sample name updated to SOLO_A280. - Added Step 7.1, turn on data loggers at beginning of process, each day. Moved from Step 14.1. - Added Step 14.1, operators to verify that CM product was held at 2-8°C for at least 12 hours prior to being placed at room temperature. - Added Step 17.1, 17.3, and 17.5. - Step 9.5: Max Column binding capacity set to 15g/L/cycle. Set to match NOR Max. - Step 11.6: Added step to calculate hold-up volume of depth filters. - Step 11.8: Added depth filter hold-up volume addition for calculation. - Updated POD depth filter diagram throughout the document. | - Per PLAN-0119352. - Template update to ensure data loggers are turned on at the beginning of the process. - Per PRO-0204129, to ensure CM product was held at 2-8°C for at least 12 hours prior to being placed at room temperature. - Template update to ensure documentation is complete, attachments are attached, and materials are consumed to SAP. - Per PRO-0204129. - Step is needed for calculation in Step 11.8. - Needed for calculation. - Updated diagram to accurately represent current setup. |
| 4.0 | This is an abbreviated approval per SOP-0107045. - Step 11.6 – Added ‘L’ units to “Hold-Up Volume of Installed Depth Filters.” - Step 12.3 – Changed the format of the Batch ID from “PartNumber BatchNumber DepthFilterFlush Date” to “PartNumber BatchNumber DepthFilterEquil Date. - Step 14.22 – Corrected the Visio to put the pods in bypass. - Attachment 1 – Changed the Location for SOLO_A280 from ‘QC’ to ‘MS.’ - Attachment 2 – Added a “Pre-Load Performed by/Date” row and bulleted instructions. - Removed the previous two revision histories. | - Correction to doc defect – volume units were missing. - Correction to doc defect – this was a copy and paste error because the previous format was a duplicate of Step 12.2. - The previous Visio diagram had the pods inline, but this was a doc defect. Per the step instructions, the pods are bypassed. - Correction to doc defect – SOLO_A280 gets tested by MS, not QC. - To provide the non-routine samples pre-loader to sign for pre-loading and to provide additional instruction for filling out Attachment 2. - Per SOP-0107043. |