V 1.0

May 2026

Design XStep Specification

EFS Exhaust Filter Info

Table of Contents

Functional Requirements	3

1 Purpose	3

2 Overview	3

3 Reasons for developing	4

4 Authorization	4

5 Assumptions/ Dependencies	5

6 Validation Checks	6

7 XStep Layout Design	6

8 Configuration Specification	7

9 Test Scenarios	7

10 Document References	7

11 Revision History	7

# Functional Requirements

# Purpose

The document details the functional and technical design, along with the configuration specifications for the process step SiMPL: EFS Exhaust Filter Info. The process step allows the user to capture Exhaust Filter part data.

The process step contains a second “Tubing Assembly” table that can be activated or deactivated based on what is selected from a dropdown function. If tubing parts are used then the table is activated and the user can capture the part data.

# Overview

The process step allows the user to record Exhaust Filter information. The “Exhaust Filter Assembly” table captures the following part data.

- # - Output: Line number is automatically captured in the table

- Part No – Input: Capture part number in the line item of the table

- Description – Output: The part description is automatically pulled from SAP table MAKT based on the part number that was entered into the table. The field in the table is known as MAKTX.

- Serial Number – Input: Capture the serial number of the selected part.

- Batch – Input: Capture the batch of the selected part.

- Expiry Date – Output: Expiry information is pulled from SAP table MCH1. The field in the table is known as VFDAT. Information from this SAP table is pulled into the relevant output field of the process step.

- Get Welder – Functional Button: The functional button pulls welder data into the output fields of the table.

- Welder ID – Output: The functional button pulls Welder data into the table of the process step.

- Description – Output: The functional button pulls Welder data into the table of the process step.

- Get Date – Functional Button: The functional button pulls information into the output fields of the table.

- Installation Date – Output: The functional button pulls data into the table of the process step.

- Port Attached to – Input: Capture data in the process step.

- Performed By – Input: Verify the data captured in the table is correct and complete.

The process step contains a second Tubing Assembly table. The table can be activated or deactivated based on what the user selected from the dropdown function “Tubing Assembly Used”. If the table is activated the user is required to capture the following data:

- # - Output: Line number is automatically captured in the table

- Part No – Input: Capture part number in the line item of the table

- Description – Output: The part description is automatically pulled from SAP table MAKT based on the part number that was entered into the table. The field in the table is known as MAKTX.

- Batch – Input: Capture the batch of the selected part.

- Expiration Date – Output: Expiry information is pulled from SAP table MCH1. The field in the table is known as VFDAT. Information from this SAP table is pulled into the relevant output field of the process step.

- Performed By – Input: Verify the data captured in the table is correct and complete. The data is saved to SAP upon user signature.

A second operator confirms that the data captured in the tables is completed and correct by signing off in the Witnessed By field.

# Reasons for developing

The process step EFS Exhaust Filter Info is developed to provide a controlled and standardized electronic method for capturing Exhaust Filter Assembly and Tubing Assembly component information during manufacturing execution within the SAP PP-PI environment.

The process step is required to ensure that all critical Exhaust Filter and Tubing Assembly component data is accurately recorded, verified, and stored within SAP as part of the electronic batch record. The XStep supports traceability, data integrity, GMP compliance, and operational efficiency by eliminating manual paper-based recording processes and ensuring that component information is captured directly within the PI Sheet during process execution.

The solution integrates directly with SAP backend tables such as:

- MAKT for automatic material description retrieval (MAKTX)

- MCH1 for automatic batch expiry retrieval (VFDAT)

This integration reduces manual transcription activities, minimizes operator error, and ensures that master data maintained in SAP is consistently used during manufacturing operations.

The development of the dynamic “Tubing Assembly” table functionality allows the process step to conditionally activate or deactivate additional data capture requirements based on operational process needs.

The functional buttons developed within the XStep automate the retrieval of:

- Welder information

- Installation date information

This automation improves process efficiency, standardization, and data accuracy while reducing manual operator interaction.

# Authorization

The Signature Strategies SS000201 - SiMPL Done By and SS000202 - SiMPL Checked By are created and assigned to the Authorization Groups AGDS0001 PI step Done By SS and AGDS0002 PI step Checked By SS. This ensures that at least two users are required to successfully sign off the process step. The first user signs off in the Performed By field and a second user signs the Witnessed By field.

| Object Name | Description |
| --- | --- |
| Signature validation | The user is required to provide a password validating their ID before their digital signature is accepted. Validation Checks: - SAP User ID exists - Password is correct - User is not locked - User is signed into PI sheet - User session is valid |
| Signature Strategy | A signature strategy will be implemented so that operators or/and verifiers must sign off before the process step is locked and the data is automatically saved in the SAP system. The Performed By signature checks that: - Mandatory fields are completed - Numeric Range checks passed - No invalid entries - No open deviations/ errors remain - Calculation results are valid The Witnessed By signature checks that: - Required number of signatures are met - A second user signs the Witnessed By signature activating the Signature strategy SS000202 - SiMPL Checked By - Data is verified |
| Authorization | Appropriate authorization is required for users to execute the XStep/PI sheet. - Signature Strategy SS000201 - SiMPL Done By is linked to authorization group AGDS0001 PI step Done By SS - Signature Strategy SS000202 - SiMPL Checked By is linked to authorization group AGDS0002 PI step Checked By SS |

# Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | The process step contains a table that allows the user to capture sample data to the SAP system |
| 2 | The process step contains numeric fields that allows the user to capture and record data in the system |
| 3 | The output fields of the table are automatically populated with relevant information after the part is identified |
| 4 | The process step contains functional buttons that pull information into the output fields of the table |
| 5 | The process step contains a dropdown function that can activate or deactivate a table based on the selection |
| 6 | The user signing the signature fields is an approved user in the system with the correct access configuration |
| 7 | The signature fields are set up with the correct dependencies that lock the applicable steps when executed |

# Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Mandatory “Part No” field not completed in Exhaust Filter Assembly table | Display validation error and prevent save |
| Invalid Part Number entered in Exhaust Filter Assembly table | Display SAP material validation error |
| Material description not retrieved from SAP table MAKT-MAKTX | Display backend retrieval error |
| Mandatory fields are left blank | Display mandatory field validation error |
| Expiry Date not retrieved from SAP table MCH1-VFDAT | Display backend retrieval validation error |
| User attempts to manually edit output fields | Prevent modification and display read-only message |
| Functional button fails to populate the output fields | Display functional button execution error |
| “Performed By” signature field not completed | Prevent process continuation |
| Invalid SAP credentials entered during Performed By signature | Reject signature and display authentication error |
| Unauthorized SAP user attempts Performed By sign-off | Reject signature and display authorization error |
| Same user attempts both Performed By and Witnessed By signatures | Reject second signature |
| Witnessed By signature attempted before Performed By signature completed | Prevent Witnessed By sign-off |
| Witnessed By signature not completed | Prevent process step completion |
| Tubing Assembly Used dropdown left on “Select” | Display mandatory dropdown validation error |
| Tubing Assembly table activated incorrectly after dropdown selection | Display activation/deactivation validation error |
| Tubing Assembly table remains editable when dropdown selection deactivates table | Prevent editing and display read-only message |
| Tubing Assembly table not activated when required | Display dynamic activation validation error |
| Data fails to save to SAP backend after user signature | Display backend save error |
| Expired batch selected in Exhaust Filter Assembly table | Display expiry validation error and prevent completion |
| Expired batch selected in Tubing Assembly table | Display expiry validation error and prevent completion |
| SAP backend connection failure during material retrieval | Display SAP integration error |
| SAP backend connection failure during expiry retrieval | Display SAP integration error |
| Data mismatch between entered Part Number and retrieved Description | Display master data consistency validation error |

# XStep Layout Design

# Configuration Specification

The process step contains two functional tables that allow the users to capture part information of exhaust filters and tubing.

The Exhaust Filter Assembly table contains a functional push button “Get Welder”. When the functional button is selected a function call is triggered and the output fields are populated with equipment master data.

| Item | Requirement |
| --- | --- |
| Source | Equipment Master |
| Welder ID Field | Pulled from EQUI-EQUNR |
| Welder Description Field | Pulled from EQUI-EQKT |
| Population Method | XStep Function Module |

The Exhaust Filter Assembly table contains another functional push button “Get Date”.  When the functional button is selected an XStep function call is triggered and the output field is populated with the current system time. The output field for the timestamp becomes read only and can not be changed.

| Item | Requirement |
| --- | --- |
| Source | SAP Application Server Time |
| Format | HH:MM:SS |
| Population Method | XStep Function Module |
| Audit Requirement | Timestamp recorded in audit trail |

The primary table Exhaust Filter Assembly is made up of the following table fields:

| Field Name | Field Type | Mandatory | Editable | Description |
| --- | --- | --- | --- | --- |
| Line number (#) | Output field | Yes | No | Automatically increments per line item |
| Part No | Character Input | Yes | Yes | User captures material/ part number. The data is verified according Material Master |
| Description | Output field | Yes | No | Auto-populated when part number is entered. Data pulled from MAKT-MAKTX |
| Serial Number | Character Input | Yes | Yes | Operator captures serial number |
| Batch | Character Input | Yes | Yes | The user enters batch information. Validation executed on Batch Master |
| Expiry Date | Output field | Yes | No | Expiry date pulled from Batch Master MCH1-VFDAT |
| Welder ID | Output field | Yes | No | Populated from the functional button |
| Welder Description | Output field | Yes | No | Populated from the functional button |
| Installation Date | Output field | Yes | No | Populated from the functional button |
| Port Attached To | Character Input | Yes | Yes | The installation point is captured |
| Performed By | Signature Input field | Yes | No | Signatory data captured |

The process step contains a dropdown selection that acts as an activation control for the field “Tubing Assembly used”. The dropdown selection is made up of characteristics:

- 1-Yes

- 2-No

When the step is activated the table “Tubing Assembly” becomes editable. If the step is deactivated the sample table becomes read-only.

The secondary table “Tubing Assembly” is made up of the following table fields:

| Field Name | Field Type | Mandatory | Editable | Description |
| --- | --- | --- | --- | --- |
| Line number (#) | Output field | Yes | No | Automatically increments per line item |
| Part No | Character Input | Yes | Yes | User captures material/ part number. The data is verified according Material Master |
| Description | Output field | Yes | No | Auto-populated when part number is entered. Data pulled from MAKT-MAKTX |
| Batch | Character Input | Yes | Yes | The user enters batch information. Validation executed on Batch Master |
| Expiration Date | Output field | Yes | No | Data pulled from SAP batch data MCH1-VFDAT |
| Performed By | Signature Input field | Yes | No | Signatory data captured |

The step requires two users to sign off on the data captured in the XStep. A signature strategy will be implemented so that operators or/and verifiers must sign off before the process step is locked and the data is automatically saved in the SAP system.

The Performed By signature checks that:

- Mandatory fields are completed

- Numeric Range checks passed

- No invalid entries

- No open deviations/ errors remain

- Calculation results are valid

The Witnessed By signature checks that:

- Required number of signatures are met

- A second user signs the Witnessed By signature activating the Signature strategy  SS000202 - SiMPL Checked By

- Data is verified

Appropriate authorization are setup for users to execute the XStep/PI sheet.

- Signature Strategy SS000201 - SiMPL Done By is linked to authorization group AGDS0001 PI step Done By SS

- Signature Strategy SS000202 - SiMPL Checked By is linked to authorization group AGDS0002 PI step Checked By SS

# Test Scenarios

| # | Scenario | Type | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- | --- |
| 1 | Open EFS Exhaust Filter Info XStep in PI Sheet | Functional | XStep loads successfully with primary and secondary table structures |  |  |
| 2 | Verify Exhaust Filter Assembly table displays by default | Functional | Primary table is visible and editable |  |  |
| 3 | Verify auto-line numbering in Exhaust Filter table | Functional | Line number auto-increments for each new row |  |  |
| 4 | Enter valid Part No in Exhaust Filter table | Positive | Material accepted and validated against Material Master |  |  |
| 5 | Verify Description auto-populates from MAKT-MAKTX | Integration | Correct material description displayed in read-only field |  |  |
| 6 | Attempt to edit Description field manually | Validation | Field remains read-only |  |  |
| 7 | Enter valid Serial Number | Positive | Serial Number accepted |  |  |
| 8 | Enter valid Batch | Positive | Batch accepted and validated against Batch Master |  |  |
| 9 | Verify Expiry Date retrieval from MCH1-VFDAT | Integration | Expiry Date auto-populates correctly |  |  |
| 10 | Attempt to edit Expiry Date manually | Validation | Output field remains locked |  |  |
| 11 | Click “Get Welder” functional push button | Functional | Function module executes successfully |  |  |
| 12 | Verify Welder ID retrieval from EQUI-EQUNR | Integration | Correct Welder ID populates |  |  |
| 13 | Verify Welder Description retrieval from EQUI-EQKT | Integration | Correct Welder Description populates |  |  |
| 14 | Attempt to edit Welder ID manually | Validation | Field remains read-only |  |  |
| 15 | Attempt to edit Welder Description manually | Validation | Field remains read-only |  |  |
| 16 | Click “Get Date” functional push button | Functional | Function module retrieves current SAP Application Server time |  |  |
| 17 | Verify Installation Date populated in HH:MM:SS format | Validation | Timestamp displayed in correct format |  |  |
| 18 | Verify Installation Date becomes read-only after population | Validation | Field locked from editing |  |  |
| 19 | Verify timestamp recorded in audit trail | Compliance | Audit log captures timestamp event |  |  |
| 20 | Enter valid Port Attached To value | Positive | Data accepted successfully |  |  |
| 21 | Verify “Tubing Assembly Used” dropdown loads | Functional | Dropdown values display as 1-Yes / 2-No |  |  |
| 22 | Select “Yes” in Tubing Assembly Used | Functional | Tubing Assembly table becomes editable |  |  |
| 23 | Select “No” in Tubing Assembly Used | Functional | Tubing Assembly table becomes read-only |  |  |
| 24 | Verify Tubing table line numbering | Functional | Line number auto-increments |  |  |
| 25 | Enter valid Part No in Tubing table | Positive | Description auto-populates |  |  |
| 26 | Verify Description retrieval from MAKT-MAKTX in Tubing table | Integration | Correct description displayed |  |  |
| 27 | Attempt manual edit of Tubing Description | Validation | Read-only behavior enforced |  |  |
| 28 | Enter valid Batch in Tubing table | Positive | Batch accepted |  |  |
| 29 | Verify Expiration Date retrieval from MCH1-VFDAT | Integration | Correct date displayed |  |  |
| 30 | Attempt manual edit of Expiration Date | Validation | Field remains locked |  |  |
| 31 | Perform Performed By signature | Security | Signature Strategy executed successfully |  |  |
| 32 | Attempt Performed By with invalid SAP password | Security | Authentication rejected |  |  |
| 33 | Attempt Performed By with unauthorized SAP user | Security | Authorization rejected |  |  |
| 34 | Verify invalid entries block Performed By | Validation | Signature rejected |  |  |
| 35 | Verify open deviations/errors block Performed By | Validation | Signature blocked |  |  |
| 36 | Verify data save to SAP after Performed By | Integration | Data saved successfully |  |  |
| 37 | Execute Witnessed By signature | Security | Signature Strategy triggered successfully |  |  |
| 38 | Same user attempts both signatures | Security | Second signature rejected |  |  |
| 39 | Verify second-user enforcement | Security | Different SAP user required |  |  |

# Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

# Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.0 | Initial Document | Chane Ebersohn | 12-05-2026 |