V 1.0

April 2026

Functional & Technical XStep Specification

SMPL: Cell Culture / Medium Addition

Table of Contents

Functional Requirements	3

1 Purpose	3

2 Overview	3

3 Reasons for developing	4

4 Authorization	4

5 Assumptions/ Dependencies	5

6 Security Objects	5

7 Development Type	5

8 Processing Options	5

9 Functional Mapping Rules for Fields	6

10 Validation Checks	6

General Technical Specification	7

11 Technical XStep Layout Design	7

12 XStep Technical Specification	7

12.1 Parameters:	7

12.2 Development Architecture	11

12.3 Parameter Configuration	12

12.4 Technical Implementation	13

13 Function Module(s)	14

14 Pseudocode	15

15 Configuration Specifications	19

16 Test Scenarios	19

17 Document References	22

18 Revision History	22

# Functional Requirements

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: Cell Culture / Medium Addition”.

## Overview

The SMPL: Cell Culture / Medium Addition XStep allows operators to transfer cell cultures into new flasks either by weight or by volume. The process step is configured to validate that the correct amount of cell culture and medium is added by comparing the entered values with the predetermined target values for each addition.

Functional Design

Mockup 1

OR

Mockup 2

*Need to confirm with Business if medium added is calculated by subtracting cell culture from the total or if total is calculated by adding Cell Culture and Medium together. Or do they need both fields to be entry fields.

A new characteristic will be created ZSMPL_CHAR_ADD_METHOD which will be configured as a CHAR parameter with 1 value. In the Values tab the information will be added as follows, Actual Value - Description.

- 0 - By Weight (g)

- 1 - Volumetric (mL)

The Select Addition Method dropdown will determine which UoM displays in the table using ZSMPL_FM_ADDITION_UOM. g for addition by weight and mL for Volumetric.

The Tare Table will deactivate if Volumetric (mL) is selected from the dropdown above the table. If the dropdown is changed to “By Weight (g)” the table will reactivate. The xstep is designed to be executed using only a single method of addition per xstep. Changing the addition method in the middle of execution will not properly work.

The Dropdown can be changed if incorrectly selected at the start and the table values for lines that remain open will update according to the above logic. Lines that have been completed will not update when the dropdown is changed.

The Flask No. column will populate with name configured in MBR followed by the number for example MBR parameter IV_FLASK set to “1L/400” will populate as 1L/400/1 for line 1 and 1L/400/2 for line two etc. This will update sequentially as lines are added using the custom function ZSMPL_FM_CUSTOM_INDEX. Adding a table line will also add an additional table line to the Tare Table if the addition method is set to “By Weight (g)”

The Target Tare, Target Cell Culture, Target Medium (If Mockup 1) and Target Total (if mockup 2) columns will populate based on the MBR parameters IV_TARE, IV_CELL, IV_MED (If Mockup 1) and IV_TOTAL (if mockup 2)  respectively. Using ZSMPL_FM_CALC_TOLERANCE

Validation will occur for the entry fields, Tare weight, Cell Culture Added, Medium Added and Total Amount, where the entered value must be within the target value plus/minus 5%. The percentage will be defaulted to 5% within the step configuration but can be changed by setting the IV_TOL_PCT parameter in MBR. USING ZSMPL_FM_VALIDATE_QUANTITY

The Medium added or Total Added field will populate automatically based on the information of the other two fields. Using ZSMPL_FM_BASIC_CALCULATIONS.

Once an entry field value is accepted, by hitting the enter key for the field or other standard sap functionality, it will only be modifiable through manual entry.

The performed by signature in the table will validate that the user is signed into the PI Sheet before allowing for the signature to continue. The Performed by signature strategy will be used for this field.

## Reasons for developing

The SMPL: Cell Culture / Medium Addition XStep was developed to record the transfer process of cell culture and medium into additional flasks while ensuring all quantities are within the specified tolerances.

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | Master Data has been set up correctly |

## Security Objects

| Object Name | Description |
| --- | --- |
| Signature validation | Optional |

## Development Type

| Development Type: |  | Report |  | Interface |  | Conversion |  | Enhancement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Form |  | Workflow |  | Portal |  | EDI |
|  |  | Screen |  | Data Cleansing |  | Data Dependencies |  |  |
|  | X | Other: | XStep |  |  |  |  |  |

## Processing Options

| Processing Options |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processing Mode: |  | Batch | X | Online |  |  |  |  |
| Frequency |  | Annually |  | Quarterly |  | Monthly |  | Weekly |
|  |  | Daily |  | Minutes |  |  |  |  |
|  | X | Other: | Executed per EBR |  |  |  |  |  |

## Functional Mapping Rules for Fields

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Entered Tare Weight is within Tolerance | Validation Triggers stating the value is outside of the tolerance |
| Entered Cell Culture quantity is within Tolerance | Validation Triggers stating the value is outside of the tolerance |
| Entered Medium quantity or total quantity is within Tolerance | Validation Triggers stating the value is outside of the tolerance |

# General Technical Specification

## Technical XStep Layout Design

Screenshot of Final Design

## XStep Technical Specification

### 12.1 Parameters:

Screenshot of parameters page in Xstep

### 12.2 Development Architecture

The SMPL: Cell Culture / Medium Addition XStep is developed to facilitate the transfer process of cell culture and medium into additional flasks while ensuring all quantities are within the specified tolerances. Tolerance values are configurable through MBR as needed.

### 12.3 Parameter Configuration

This XStep includes input and local parameters to manage content display, input validation and tolerance values / range.

Configuration Parameters

- IV_HEADER (PPPI_SHORT_TEXT) – Configurable header text displayed at the top of the XStep via the SiMPL MBR

- IV_INSTR (PPPI_FRAGMENT_HTML) – Instructional content for operators, displayed under the header

- IV_ACTIVE / IV_ACT_FLP (ZSMPL_CHAR_YES_NO) – Controls whether the step is active and visible in execution

- IV_SIGN (ZSMPL_CHAR_SIGNATURE_STRAT) – Signature strategy to authorize and finalize equipment assignment

- IV_PRINT / IV_PL_TXT (PPPI_SHORT_TEXT / PPPI_FRAGMENT_HTML) – Controls print version and instruction print configuration

- IV_TITLE1 (PPPI_SHORT_TEXT) – Optional title for secondary display context

- IV_VERIFY (ZSMPL_CHAR_SIGNATURE_STRAT) – Optional verification signature strategy

- SIMPL_UUID (ZSMPL_CHAR_UUID) – Unique XStep identifier for traceability

- STEP_INDEX (PPPI_MATERIAL_QUANTITY) – Indicates the XStep’s position in the execution sequence

- IV_FLASK (PPPI_SHORT_TEXT)-flask description for table column ie 1L/400

- IV_TARE (PPPI_MATERIAL_QUANTITY)-Target tare value for addition by weight

- IV_CELL(PPPI_MATERIAL_QUANTITY)-Target cell culture value for addition

- IV_MED(PPPI_MATERIAL_QUANTITY)-Target medium value for addition

- IV_TOTAL(PPPI_MATERIAL_QUANTITY)-Target total value for addition

- IV_TOL_PCT(PPPI_MATERIAL_QUANTITY)-Tolerance percentage to add and subtract from target

Output and Local Parameters

- LAV_ORDER (PPPI_PROCESS_ORDER) – Auto-filled with process order number

- LAV_PHASE (PPPI_PHASE) – Displays the current process phase

- LAV_PLANT (PPPI_PLANT_OF_BATCH) – Displays the production plant

- LAV_CRID (PPPI_CONTROL_RECIPE) – Displays the control recipe number

XStep Dependency Configuration

The following fields are used to link the dependency configuration in SiMPL MBR to the XStep structure:

- LV_LOCK(ZSMPL_CHAR_YES_NO) – Locking parameter

- EV_LOCK(ZSMPL_CHAR_YES_NO) - Output parameter that indicates locking decision

These parameters work in conjunction with the /SMPL/MBR_DEP_CHECK_ACTIVE function module, which checks to see if the current XStep has any dependencies.  If there are any active dependencies linked to the XStep, the XStep will either be locked, or execution will be prevented, until the dependent step has been completed.

No additional setup is required from the XStep itself regarding the parameters listed above.  The solution is already included for each XStep, so only the MBR configuration is required on the master recipe level for dependency information, if any inter-dependencies are needed throughout the PI Sheet.

### 12.4 Technical Implementation

Function Button Integration

NA

Field Behavior and Flexibility

Add description of field logic if any (validation, triggers etc)

Signature Validation Mechanism

After the step is fully executed, operators are required to sign off using the configured signature strategy (IV_SIGN), confirming the assignment for audit and compliance purposes.

Applicability and Execution Control

- The IV_ACTIVE parameter determines whether the XStep is enabled during execution.

- The IV_ACT_FLP parameter ensures mutual exclusivity or alternate routing by acting as an inverse flag.

Data Integration and Traceability

- UUID-based tracking supports full traceability in the electronic batch record.

- Signature events are logged and auditable.

- Local parameters such as LAV_ORDER, LAV_PLANT, and LAV_PHASE are used to… If needed

## Function Module(s)

Function: ZSMPL_FM_VALIDATE_QUANTITY

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_QTY_NUM | ERFMG | Quantity numeric (optional) |
| IM_QTY_CHAR | CHAR30 | Quantity Char (optional) |
| IM_MIN | ERFMG | Min Quantity (optional) |
| IM_MAX | ERFMG | Max Quantity (optional) |
| IM_TARGET | ERFMG | Target Quantity (optional) |
| IM_FLAG | BOOLE_D | True for exact value validation |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_OUTSIDE_TOLERANCE | The entered value is outside the expected tolerance range |
| TARGET_VAL_NOT_FOUND | The target value was not found. |

Function: ZSMPL_FM_CALC_TOLERANCE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_TARGET_1 | ERFMG | Target Value for first parameter |
| IM_TARGET_2 | ERFMG | Target Value for second parameter (Optional) |
| IM_TARGET_3 | ERFMG | Target Value for third parameter (Optional) |
| IM_PERCENT | ERFMG | Percent for MIN/Max Calculation. % format not decimal |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_MIN_1 | ERFMG | Min Value for target 1 |
| EX_MAX_1 | ERFMG | Max Value for target 1 |
| EX_MIN_2 | ERFMG | Min Value for target 2 |
| EX_MAX_2 | ERFMG | Max Value for target 2 |
| EX_MIN_3 | ERFMG | Min Value for target 3 |
| EX_MAX_3 | ERFMG | Max Value for target 3 |

Function: ZSMPL_FM_ADDITION_UOM

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_DROPDOWN | CHAR1 | Selection result of dropdown menu |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_UOM | ERFME | UoM for Addition Method |

Function: ZSMPL_FM_BASIC_CALCULATIONS

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_QTY | ERFMG | First Value |
| IM_QTY_2 | ERFMG | Second Value |
| IM_OPERAND | CHAR3 | Operand( SUB, ADD, MUL, DIV ) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_RESULT | ERFMG | Result of calculation |

Function: ZSMPL_FM_CUSTOM_INDEX

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_PRE_TEXT | CHAR10 | Label to cocatenate at start of line entry (Optional) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_INDEX_2 | ZSMPL_TT_CHAR12 | Copy of Index 1 |
| EX_INDEX_3 | ZSMPL_TT_CHAR12 | Copy of Index 1 |

Changing Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| CH_INDEX_1 | ZSMPL_TT_CHAR12 | Table Index 1 |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

## Pseudocode

Function: ZSMPL_FM_VALIDATE_QUANTITY

1. Initialization

a. Check if IM_QTY_NUM or IM_QTY_CHAR is initial

b. If both are initial return from the FM.

c. check if value imported is a number, IM_QTY_NUM, or a char, IM_QTY_CHAR, then move it to a local erfmg parameter lv_qty for later processing.

d. Check if IV_FLAG is set to true

- If true, check that IM_TARGET is not initial. If Initial raise an error

  - then compare lv_qty to IM_TARGET. If not exactly the same raise an error

- If false, check that IM_MIN and IM_MAX are not initial, if initial raise an error

  - Then check that lv_qty is equal to or within IM_MIN and IM_MAX, if not equal to or within the two parameters raise an error

Dev to add changes as needed

Function: ZSMPL_FM_CALC_TOLERANCE

1. Initialization

a. Convert IM_PERCENT to a decimal format by dividing by 100, i.e 5 / 100 = 0.05

b. Multiply IM_TARGET by percentage in decimal format, then add/subtract the result to get the minimum and maximum values for that target parameter.

- If the minimum value is less than 0 set minimum parameter to 0.

c. Repeat for IM_TARGET_2 and IM_TARGET_3 if that values are not initial

Dev to add changes as needed

Function: ZSMPL_FM_ADDITION_UOM

1. Initialization

a. Check which dropdown option was selected

- Assign EX_UOM = g if By Weight was selected

- Assign EX_UOM = mL if Volumetric was selected

Dev to add changes as needed

Function: ZSMPL_FM_BASIC_CALCULATIONS

1. Initialization

a. Check that both Value parameters have values. If either is initial return immediately.

b. Check which operand is used

- IF ADD add the two values

- IF SUB subtract value 1 from value 2

- IF MUL multiply value 1 by value 2

- IF DIV check that divider is not equal to zero

  - If valid then divide first value by second value

c. Assign result  to EX_RESULT

Dev to add changes as needed

Function: ZSMPL_FM_CUSTOM_INDEX

a. Run through each line of CH_INDEX_1

b. For each line check if IM_PRE_TEXT is initial

- If initial continue

- If not initial concatenate this value with the index number. i.e 1 becomes IM_PRE_TEXT1, 2 becomes IM_PRE_TEXT2

c. Assign index number to the value for this table row in CH_INDEX_1

d. After looping through CH_INDEX_1 copy the results to EX_INDEX_2 and EX_INDEX_3

Dev to add changes as needed

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.
b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

- If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

- If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

- This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps

- The use of constants (gc_yes, gc_no, and gc_inside_range) allows the logic to be aligned with broader application settings or standards.

- Helpful in scenarios where input values may vary but need to result in consistent "active" or "inactive" behavior.

- Input is optional, so defaulting behavior can be configured outside this function.

- This function improves clarity and consistency in determining whether an XStep should be active when rendered in the PI Sheet.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR (Manufacturing Batch Record) step can be activated:

- IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

- IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

- IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

- IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

- IM_PLANT (WERKS_D) – Plant code for process context.

b. The function first validates that IM_CONTROL_RECIPE is not initial.

- If it is initial, the exception CONTROL_RECIPE_INITIAL is raised.

c. Internal data is initialized:

- lt_ord_sigs: Internal table for storing order signature data from /SMPL/MBR_OR_SIG.

- ls_step_dep: Work area for step dependency data.

- Routine LOAD_INTERNAL_DATA is called to populate internal buffers (g_ord_data-t_ord_deps) using IM_AUFNR, IM_CONTROL_RECIPE, and IM_STEP_UUID.

2. Signature Data Retrieval

a. The function retrieves signature records for the process order from database table /SMPL/MBR_OR_SIG.
b. Selected fields include: AUFNR, SOG_UUID, TYPE, CREATED_DATE, CREATED_TIME, DEST_NAME, SIGN_ID, SIGN_DATE, and SIGN_TIME.
c. Only signatures related to the specified order (AUFNR = IM_AUFNR) are selected.
d. These records are stored in the internal table lt_ord_sigs for later evaluation.

3. Dependency Evaluation

a. The function loops through dependencies in g_ord_data-t_ord_deps where:

- TARGET_UUID = IM_STEP_UUID, and

- DEP_RULE is either gc_perform_before_start_rule or gc_verify_before_start_rule.

b. For each dependency record:

- If the rule is PERFORM BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending perform signature (gc_perform_signature) with SIGN_ID = abap_false.

- If the rule is VERIFY BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending verify signature (gc_verify_signature) with SIGN_ID = abap_false.

c. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive).
d. If all prerequisite signatures are complete, the step is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in the export parameter EV_ACTIVE.

- gc_step_active – All prerequisite steps are completed; the current step can start.

- gc_step_inactive – At least one prerequisite step is incomplete; the current step remains blocked.

Additional Notes

This function determines whether a dependent step in a Manufacturing Batch Record is eligible to start based on the completion of prerequisite steps.

Key Features:

- Evaluates Perform Before Start and Verify Before Start dependency rules.

- Dynamically validates signature data from /SMPL/MBR_OR_SIG.

- Integrates with existing MBR dependency structures via global table g_ord_data-t_ord_deps.

Error Handling:

- Raises CONTROL_RECIPE_INITIAL if the control recipe input is missing.

- Handles empty dependency or signature data without runtime errors.

Dependencies:

- Tables: /SMPL/MBR_OR_SIG (Signatures), g_ord_data-t_ord_deps (Step Dependencies).

- Constants: gc_perform_before_start_rule, gc_verify_before_start_rule, gc_perform_signature, gc_verify_signature, gc_step_active, gc_step_inactive.

- Routine: LOAD_INTERNAL_DATA for initializing order context.

Assumptions:

- All prerequisite steps are recorded in the dependency table.

- Signature records are current and synchronized with MBR step progression.

Activation status is recalculated each time the function is called to reflect live signature data.

## Configuration Specifications

N/A

## Test Scenarios

Variant 1: Equipment Select

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: Cell Culture / Medium Addition |  |  |
| 2 | Open the EBR and navigate to Phase 0030 and then navigate to SMPL: Cell Culture / Medium Addition | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Cell Culture / Medium Addition step is visible in Phase 0030 Output parameter fields displaying correctly |  |  |
| 3 | Select “Volumetric (mL)” from the Addition method dropdown | Tare Table Deactivates. UoM field in the Cell Culture / Medium Addition table populates with “mL” |  |  |
| 4 | Select “By Weight (g)” from the Addition method dropdown | Tare Table activates. UoM field in the Cell Culture / Medium Addition table populates with “g” |  |  |
| 5 | Enter a tare weight that is outside the tolerance range shown. | Validation popup occurs. |  |  |
| 6 | Discard the validation popup | Popup is closed, value is not accepted. |  |  |
| 7 | Enter a value exactly equal to the minimum tare field | Value is accepted |  |  |
| 8 | Enter a value exactly equal to the maximum tare field | Value is accepted |  |  |
| 9 | Sign for the line in the tare table | Validation popup triggers stating user is not signed into the PI Sheet |  |  |
| 10 | Discard validation popup | Popup is closed, signature is not completed. |  |  |
| 11 | Enter a value into the Cell Culture Added field that is outside the tolerance range shown. | Validation popup occurs. |  |  |
| 12 | Discard the validation popup | Popup is closed, value is not accepted. |  |  |
| 13 | Enter a value into the Cell Culture Added field equal to the minimum Cell field | Value is accepted. |  |  |
| 14 | Enter a value into the Cell Culture Added field exactly equal to the maximum Cell field | Value is accepted |  |  |
| 15 | Enter a value into the Cell Culture Added field that is outside the tolerance range shown. | Validation popup occurs. |  |  |
| 16 | Discard the validation popup | Popup is closed, value is not accepted. |  |  |
| 17 | Enter a value into the Cell Culture Added field equal to the minimum Cell field | Value is accepted. |  |  |
| 18 | Enter a value into the Cell Culture Added field exactly equal to the maximum Cell field | Value is accepted |  |  |
| 19 | Enter a value into the Total Added field that is outside the tolerance range shown. | Validation popup occurs. |  |  |
| 20 | Discard the validation popup | Popup is closed, value is not accepted. |  |  |
| 21 | Enter a value into the Total Added field equal to the minimum Total field | Value is accepted. Medium Added Output Field updates correctly |  |  |
| 22 | Enter a value into the Total Added field exactly equal to the maximum Total field | Value is accepted. Medium Added Output field updates correctly |  |  |
| 23 | Sign for the line in the Cell Culture/ Medium Addition table | Validation popup triggers stating user is not signed into the PI Sheet |  |  |
| 24 | Discard validation popup | Popup is closed, signature is not completed. |  |  |
| 25 | Sign into the PI Sheet using the SMPL:Signature Table Xstep | User signed in successfully |  |  |
| 26 | Sign for the line in the tare table | Table Line completes |  |  |
| 27 | Sign for the line in the Cell Culture/ Medium Addition table | Table Line completes |  |  |
| 28 | Select the Add table row button in the Cell Culture /Medium Addition table | A new table line is added for both the Tare Table and Cell Culture / Medium Addition Table. The Flask No. Output fields increment the counter correctly for both tables. |  |  |
| 29 | Select the Add table row button in the Cell Culture /Medium Addition table | A new table line is added for both the Tare Table and Cell Culture / Medium Addition Table. The Flask No. Output fields increment the counter correctly for both tables. |  |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1 | Initial Functional Design Information | Tom Rautenbach | 01APR2026 |
| 1.1 | Technical Design Information | NAME | DATE |