V 1.0

April 2026

Functional & Technical XStep Specification

SMPL: Viable Cell Density

Table of Contents

Functional Requirements	3

1 Purpose	3

2 Overview	3

3 Reasons for developing	4

4 Authorization	4

5 Assumptions/ Dependencies	5

6 Security Objects	5

7 Development Type	5

8 Processing Options	5

9 Functional Mapping Rules for Fields	6

10 Validation Checks	6

General Technical Specification	7

11 Technical XStep Layout Design	7

12 XStep Technical Specification	7

12.1 Parameters:	7

12.2 Development Architecture	11

12.3 Parameter Configuration	12

12.4 Technical Implementation	13

13 Function Module(s)	14

14 Pseudocode	15

15 Configuration Specifications	19

16 Test Scenarios	19

17 Document References	22

18 Revision History	22

# Functional Requirements

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: Viable Cell Density”.

## Overview

The SMPL: Viable Cell Density XStep allows operators to record viable cell density and viability percentage during the process.

The XStep captures the Seed Day, Min Cell Density, Max Cell Density, Viable Cell Density, UoM, Viability %, Performed By, and Witnessed By fields.

The Min Cell Density and Max Cell Density values will display as output values. Operators will enter the Seed Day, Viable Cell Density, and Viability %. The Performed By signature will validate that the user is signed into the PI Sheet before allowing the signature to continue.

The XStep includes instructional guidance for the operator. When the viable cell density is >= 14.60 x 10^6 cells/mL, the operator is instructed to prepare to add medium via the long text instruction.

The XStep also includes additional instruction-based validation and decision logic:

If viability is less than 90%, a validation popup will occur that allows the operator to record a comment before accepting the value.

If VCD is still not achieved by Perfusion Seed Day 4.5, the operator proceeds with comment after contacting the supervisor/designee.

If VCD is greater than 29.2 x 10^6 cells/mL, a validation popup occurs.

If VCD is outside the Min/Max Cell Density range.

Functional Design

The XStep captures the Seed Day, Min Cell Density, Max Cell Density, Viable Cell Density, UoM, Viability %, Performed By, and Witnessed By fields.

The Min Cell Density and Max Cell Density values will display as output values. Operators will enter the Seed Day, Viable Cell Density, and Viability %. The Performed By signature will validate that the user is signed into the PI Sheet before allowing the signature to continue.

The XStep includes instructional guidance for the operator. When the viable cell density is >= 14.60 x 10^6 cells/mL, the operator is instructed to prepare to add medium via the long text instruction.

The XStep also includes additional instruction-based validation and decision logic:

If viability is less than 90%, a validation popup will occur that allows the operator to record a comment before accepting the value.

If VCD is still not achieved by Perfusion Seed Day 4.5, the operator proceeds with comment after contacting the supervisor/designee.

If VCD is greater than 29.2 x 10^6 cells/mL, a validation popup occurs.

If VCD is outside the Min/Max Cell Density range.

## Reasons for developing

The SMPL: Viable Cell Density XStep was developed to record viable cell density and viability percentage in the EBR.

This XStep is required to capture the seed day, viable cell density result, viability result, operator signature, and witness review when required. It also provides instruction-based process guidance when viability or VCD values fall outside expected limits.

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | Master Data has been set up correctly |
| 2 | The min and max cell density values are available |
| 3 | Validation available for viability below 90% |
| 4 | Validation available when viable cell density is out of range |

## Security Objects

| Object Name | Description |
| --- | --- |
| Signature validation | Optional |

## Development Type

| Development Type: |  | Report |  | Interface |  | Conversion |  | Enhancement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Form |  | Workflow |  | Portal |  | EDI |
|  |  | Screen |  | Data Cleansing |  | Data Dependencies |  |  |
|  | X | Other: | XStep |  |  |  |  |  |

## Processing Options

| Processing Options |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processing Mode: |  | Batch | X | Online |  |  |  |  |
| Frequency |  | Annually |  | Quarterly |  | Monthly |  | Weekly |
|  |  | Daily |  | Minutes |  |  |  |  |
|  | X | Other: | Executed per EBR |  |  |  |  |  |

## Functional Mapping Rules for Fields

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Viable Cell Density blank | Entry required |
| Viability % less than 90% | Deviation popup, must be signed by supervisor to accept |
| Viable Cell Density > 29.2 x 10^6 cells/mL | Popup that VCD is out of range |
| Viable cell density is outside Min/Max | Validation popup, can be voided with supervisor signature |

# General Technical Specification

## Technical XStep Layout Design

Screenshot of Final Design

## XStep Technical Specification

### 12.1 Parameters:

Screenshot of parameters page in Xstep

### 12.2 Development Architecture

The SMPL: Viable Cell Density XStep is developed to facilitate the recording of viable cell density and viability percentage results during the process.

The XStep allows users to enter the seed day, viable cell density, and viability percentage, display the applicable min/max density range, validate entered results against instruction-based thresholds, require comments when needed, and capture signatures for operator confirmation and witness review where applicable.

### 12.3 Parameter Configuration

This XStep includes input and local parameters to manage content display, input validation and etc.

Configuration Parameters

- IV_HEADER (PPPI_SHORT_TEXT) – Configurable header text displayed at the top of the XStep via the SiMPL MBR

- IV_INSTR (PPPI_FRAGMENT_HTML) – Instructional content for operators, displayed under the header

- IV_ACTIVE / IV_ACT_FLP (ZSMPL_CHAR_YES_NO) – Controls whether the step is active and visible in execution

- IV_SIGN (ZSMPL_CHAR_SIGNATURE_STRAT) – Signature strategy to authorize and finalize equipment assignment

- IV_PRINT / IV_PL_TXT (PPPI_SHORT_TEXT / PPPI_FRAGMENT_HTML) – Controls print version and instruction print configuration

- IV_TITLE1 (PPPI_SHORT_TEXT) – Optional title for secondary display context

- IV_VERIFY (ZSMPL_CHAR_SIGNATURE_STRAT) – Optional verification signature strategy

- SIMPL_UUID (ZSMPL_CHAR_UUID) – Unique XStep identifier for traceability

- STEP_INDEX (PPPI_MATERIAL_QUANTITY) – Indicates the XStep’s position in the execution sequence

Output and Local Parameters

- LAV_ORDER (PPPI_PROCESS_ORDER) – Auto-filled with process order number

- LAV_PHASE (PPPI_PHASE) – Displays the current process phase

- LAV_PLANT (PPPI_PLANT_OF_BATCH) – Displays the production plant

- LAV_CRID (PPPI_CONTROL_RECIPE) – Displays the control recipe number

- LT_SEED (ZSMPL_CHAR_GI_INITIALS) - Seed Day

- LT_MIN (PPPI_MATERIAL_QUANTITY) - Min Cell Density

- LT_MAX (PPPI_MATERIAL_QUANTITY) - Max Cell Density

- LT_VCD (PPPI_MATERIAL_QUANTITY) - Viable Cell Density

- LT_UoM (PPPI_SHORT_TEXT) - UoM

- LT_VPERC (PPPI_MATERIAL_QUANTITY) - Viability Percentage

XStep Dependency Configuration

The following fields are used to link the dependency configuration in SiMPL MBR to the XStep structure:

- LV_LOCK(ZSMPL_CHAR_YES_NO) – Locking parameter

- EV_LOCK(ZSMPL_CHAR_YES_NO) - Output parameter that indicates locking decision

These parameters work in conjunction with the /SMPL/MBR_DEP_CHECK_ACTIVE function module, which checks to see if the current XStep has any dependencies.  If there are any active dependencies linked to the XStep, the XStep will either be locked, or execution will be prevented, until the dependent step has been completed.

No additional setup is required from the XStep itself regarding the parameters listed above.  The solution is already included for each XStep, so only the MBR configuration is required on the master recipe level for dependency information, if any inter-dependencies are needed throughout the PI Sheet.

### 12.4 Technical Implementation

Function Button Integration

Add description of button logic if any

Field Behavior and Flexibility

Add description of field logic if any

Signature Validation Mechanism

After the step is fully executed, operators are required to sign off using the configured signature strategy (IV_SIGN), confirming the assignment for audit and compliance purposes.

Applicability and Execution Control

- The IV_ACTIVE parameter determines whether the XStep is enabled during execution.

- The IV_ACT_FLP parameter ensures mutual exclusivity or alternate routing by acting as an inverse flag.

Data Integration and Traceability

- UUID-based tracking supports full traceability in the electronic batch record.

- Signature events are logged and auditable.

- Local parameters such as LAV_ORDER, LAV_PLANT, and LAV_PHASE are used to… If needed

## Function Module(s)

Function: ZSMPL_FM_VALIDATE_QUANTITY

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_QTY_NUM | ERFMG | Quantity numeric (optional) |
| IM_QTY_CHAR | CHAR30 | Quantity Char (optional) |
| IM_MIN | ERFMG | Min Quantity (optional) |
| IM_MAX | ERFMG | Max Quantity (optional) |
| IM_TARGET | ERFMG | Target Quantity (optional) |
| IM_FLAG | BOOLE_D | True for exact value validation |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_OUTSIDE_TOLERANCE | The entered value is outside the expected tolerance range |
| TARGET_VAL_NOT_FOUND | The target value was not found. |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

## Pseudocode

Function: ZSMPL_FM_VALIDATE_QUANTITY

1. Initialization

a. Check if IM_QTY_NUM or IM_QTY_CHAR is initial

b. If both are initial return from the FM.

c. check if value imported is a number, IM_QTY_NUM, or a char, IM_QTY_CHAR, then move it to a local erfmg parameter lv_qty for later processing.

d. Check if IV_FLAG is set to true

- If true, check that IM_TARGET is not initial. If Initial raise an error

  - then compare lv_qty to IM_TARGET. If not exactly the same raise an error

- If false, check that IM_MIN and IM_MAX are not initial, if initial raise an error

  - Then check that lv_qty is equal to or within IM_MIN and IM_MAX, if not equal to or within the two parameters raise an error

Dev to add changes as needed

.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.
b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

- If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

- If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

- This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps

- The use of constants (gc_yes, gc_no, and gc_inside_range) allows the logic to be aligned with broader application settings or standards.

- Helpful in scenarios where input values may vary but need to result in consistent "active" or "inactive" behavior.

- Input is optional, so defaulting behavior can be configured outside this function.

- This function improves clarity and consistency in determining whether an XStep should be active when rendered in the PI Sheet.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR (Manufacturing Batch Record) step can be activated:

- IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

- IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

- IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

- IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

- IM_PLANT (WERKS_D) – Plant code for process context.

b. The function first validates that IM_CONTROL_RECIPE is not initial.

- If it is initial, the exception CONTROL_RECIPE_INITIAL is raised.

c. Internal data is initialized:

- lt_ord_sigs: Internal table for storing order signature data from /SMPL/MBR_OR_SIG.

- ls_step_dep: Work area for step dependency data.

- Routine LOAD_INTERNAL_DATA is called to populate internal buffers (g_ord_data-t_ord_deps) using IM_AUFNR, IM_CONTROL_RECIPE, and IM_STEP_UUID.

2. Signature Data Retrieval

a. The function retrieves signature records for the process order from database table /SMPL/MBR_OR_SIG.
b. Selected fields include: AUFNR, SOG_UUID, TYPE, CREATED_DATE, CREATED_TIME, DEST_NAME, SIGN_ID, SIGN_DATE, and SIGN_TIME.
c. Only signatures related to the specified order (AUFNR = IM_AUFNR) are selected.
d. These records are stored in the internal table lt_ord_sigs for later evaluation.

3. Dependency Evaluation

a. The function loops through dependencies in g_ord_data-t_ord_deps where:

- TARGET_UUID = IM_STEP_UUID, and

- DEP_RULE is either gc_perform_before_start_rule or gc_verify_before_start_rule.

b. For each dependency record:

- If the rule is PERFORM BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending perform signature (gc_perform_signature) with SIGN_ID = abap_false.

- If the rule is VERIFY BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending verify signature (gc_verify_signature) with SIGN_ID = abap_false.

c. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive).
d. If all prerequisite signatures are complete, the step is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in the export parameter EV_ACTIVE.

- gc_step_active – All prerequisite steps are completed; the current step can start.

- gc_step_inactive – At least one prerequisite step is incomplete; the current step remains blocked.

Additional Notes

This function determines whether a dependent step in a Manufacturing Batch Record is eligible to start based on the completion of prerequisite steps.

Key Features:

- Evaluates Perform Before Start and Verify Before Start dependency rules.

- Dynamically validates signature data from /SMPL/MBR_OR_SIG.

- Integrates with existing MBR dependency structures via global table g_ord_data-t_ord_deps.

Error Handling:

- Raises CONTROL_RECIPE_INITIAL if the control recipe input is missing.

- Handles empty dependency or signature data without runtime errors.

Dependencies:

- Tables: /SMPL/MBR_OR_SIG (Signatures), g_ord_data-t_ord_deps (Step Dependencies).

- Constants: gc_perform_before_start_rule, gc_verify_before_start_rule, gc_perform_signature, gc_verify_signature, gc_step_active, gc_step_inactive.

- Routine: LOAD_INTERNAL_DATA for initializing order context.

Assumptions:

- All prerequisite steps are recorded in the dependency table.

- Signature records are current and synchronized with MBR step progression.

Activation status is recalculated each time the function is called to reflect live signature data.

## Configuration Specifications

N/A

## Test Scenarios

Variant 1: Equipment Select

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and release a process order with the pre-configured SiMPL Master Recipe and relevant Production Version. | Process Order is created and released successfully containing SMPL: Viable Cell Density. |  |  |
| 2 | Open the EBR for the released process order and navigate to the phase containing SMPL: Viable Cell Density. | The EBR opens successfully and the SMPL: Viable Cell Density XStep is visible in the applicable phase. |  |  |
| 3 | Review the instruction text displayed in the XStep. | Instructions for viable cell density, viability %, supervisor contact, and witness requirements are displayed. |  |  |
| 4 | Confirm that the Min Cell Density, Max Cell Density, and UoM fields are displayed. | Min Cell Density, Max Cell Density, and UoM display as output fields. |  |  |
| 5 | Leave Viable Cell Density blank and attempt to complete the Performed By signature. | Validation triggers stating that Viable Cell Density must be entered. |  |  |
| 6 | Leave Viability % blank and attempt to complete the Performed By signature. | Validation triggers stating that Viability % must be entered. |  |  |
| 7 | Enter a valid Viable Cell Density value within the configured Min/Max range. | The entered value is accepted without triggering a validation popup. |  |  |
| 8 | Enter a Viability % value less than 90. | A validation popup is displayed allowing the operator to record a comment before accepting the value. |  |  |
| 9 | Enter a Viable Cell Density value greater than 29.2 x 10^6 cells/mL. | A validation popup is displayed for supervisor acceptance before the value can be accepted. |  |  |
| 10 | Enter a Viable Cell Density value below the configured Min Cell Density. | A validation popup is displayed |  |  |
| 11 | Enter a Viable Cell Density value above the configured Max Cell Density. | A validation popup is displayed |  |  |
| 12 | Complete the Performed By signature after entering valid Viable Cell Density and Viability % values. | The Performed By signature is captured successfully and the XStep can proceed. |  |  |
| 13 | Complete Witnessed By with valid user credentials | XStep is locked and data is accepted |  |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1 | Initial Functional Design Information | Carlos Redekopp | 05/04/2026 |
| 1.1 | Technical Design Information | NAME | DATE |