V 1.0

April 2026

Functional & Technical XStep Specification

SMPL: Bag Replacment Volumetric

Table of Contents

Functional Requirements	3

1 Purpose	3

2 Overview	3

3 Reasons for developing	4

4 Authorization	4

5 Assumptions/ Dependencies	5

6 Security Objects	5

7 Development Type	5

8 Processing Options	5

9 Functional Mapping Rules for Fields	6

10 Validation Checks	6

General Technical Specification	7

11 Technical XStep Layout Design	7

12 XStep Technical Specification	7

12.1 Parameters:	7

12.2 Development Architecture	11

12.3 Parameter Configuration	12

12.4 Technical Implementation	13

13 Function Module(s)	14

14 Pseudocode	15

15 Configuration Specifications	19

16 Test Scenarios	19

17 Document References	22

18 Revision History	22

# Functional Item

Technical Item

# Functional Requirements

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: Bag Replacement Volumetric”.

## Overview

The SMPL: Bag Replacement Volumetric XStep allows operators to record the information pertaining to the bag replacement process including the day of replacement. A consumption of the expended bags is also performed.

Functional Design

The Header text, Long text instruction and Optional signatures located at the end of each xstep will be configurable as usual with MBR parameters.

Bag Information Table

The # column will contain the table line number, if an additional line is added the number will automatically increment. If additional lines are added in the first table, additional lines will be added to the second table automatically to match the first.

Part No. will be an entry field, validation will be performed to ensure the material exists in SAP. The Description of the part will populate automatically if the material is accepted.

Batch will be an entry field, validation will be performed to ensure the batch is not entered before a material is entered. It will also check that the batch exists in SAP for the material entered.

The Expiry date for the batch will automatically populate once the batch is entered.

The “Get Welder” button will call the existing  equipment select function which is configured to use the equipment category assigned to welders. The Welder ID and Description fields will populate as a result of the selected equipment from this function call

Day of Replacement Table

Work Day or Process will be an entry field.

The Get Time button will call GET_SYSTEM_TIME_REMOTE to populate the date and time fields when pressed.

Volume will be an entry field.

UoM will be an output field defaulted to L.

The “Get Sealer” button will call the existing  equipment select function which is configured to use the equipment category assigned to welders. The Sealer ID and Description fields will populate as a result of the selected equipment from this function call.

The table line will complete upon signing for the table line using the performed by  signature field.

Goods Issue

A goods issue will be performed when a table line is completed. The quantity will always be 1EA for the bag that is to be consumed. The Goods Issue Process message will be used to facilitate the consumption.

## Reasons for developing

The SMPL: Bag Replacement Volumetric XStep was developed to facilitate the replacement and consumption of bags

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | Master Data has been set up correctly |
| 2 | The function module(s) are available and returns the relevant information. |
| 3 | The function module can successfully pull the data into the output fields |
| 4 | Equipment needs to be assigned to the order before it can be selected. |

## Security Objects

| Object Name | Description |
| --- | --- |
| Signature validation | Optional |

## Development Type

| Development Type: |  | Report |  | Interface |  | Conversion |  | Enhancement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Form |  | Workflow |  | Portal |  | EDI |
|  |  | Screen |  | Data Cleansing |  | Data Dependencies |  |  |
|  | X | Other: | XStep |  |  |  |  |  |

## Processing Options

| Processing Options |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Processing Mode: |  | Batch | X | Online |  |  |  |  |
| Frequency |  | Annually |  | Quarterly |  | Monthly |  | Weekly |
|  |  | Daily |  | Minutes |  |  |  |  |
|  | X | Other: | Executed per EBR |  |  |  |  |  |

## Functional Mapping Rules for Fields

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Material entered does not exist in SAP | Raise a validation popup stating that the material was not found in SAP |
| Batch was entered before the material field | Raise validation popup stating that the material entry needs to be completed before the batch entry |
| Batch entered does not exist as a combination with the material previously entered | Raise a validation popup stating that the Material/Batch combination was not found in SAP |
| Standard Signature Validation | Raise Validation popup |

# General Technical Specification

## Technical XStep Layout Design

Screenshot of Final Design

## XStep Technical Specification

### 12.1 Parameters:

Screenshot of parameters page in Xstep

### 12.2 Development Architecture

The SMPL: Bag Replacement Volumetric XStep allows operators to record the information pertaining to the bag replacement process including the day of replacement. A consumption of the expended bags is also performed.

### 12.3 Parameter Configuration

This XStep includes input and local parameters to manage content display, input validation and etc.

Configuration Parameters

- IV_HEADER (PPPI_SHORT_TEXT) – Configurable header text displayed at the top of the XStep via the SiMPL MBR

- IV_INSTR (PPPI_FRAGMENT_HTML) – Instructional content for operators, displayed under the header

- IV_ACTIVE / IV_ACT_FLP (ZSMPL_CHAR_YES_NO) – Controls whether the step is active and visible in execution

- IV_SIGN (ZSMPL_CHAR_SIGNATURE_STRAT) – Signature strategy to authorize and finalize equipment assignment

- IV_PRINT / IV_PL_TXT (PPPI_SHORT_TEXT / PPPI_FRAGMENT_HTML) – Controls print version and instruction print configuration

- IV_TITLE1 (PPPI_SHORT_TEXT) – Optional title for secondary display context

- IV_VERIFY (ZSMPL_CHAR_SIGNATURE_STRAT) – Optional verification signature strategy

- SIMPL_UUID (ZSMPL_CHAR_UUID) – Unique XStep identifier for traceability

- STEP_INDEX (PPPI_MATERIAL_QUANTITY) – Indicates the XStep’s position in the execution sequence

Output and Local Parameters

- LAV_ORDER (PPPI_PROCESS_ORDER) – Auto-filled with process order number

- LAV_PHASE (PPPI_PHASE) – Displays the current process phase

- LAV_PLANT (PPPI_PLANT_OF_BATCH) – Displays the production plant

- LAV_CRID (PPPI_CONTROL_RECIPE) – Displays the control recipe number

XStep Dependency Configuration

The following fields are used to link the dependency configuration in SiMPL MBR to the XStep structure:

- LV_LOCK(ZSMPL_CHAR_YES_NO) – Locking parameter

- EV_LOCK(ZSMPL_CHAR_YES_NO) - Output parameter that indicates locking decision

These parameters work in conjunction with the /SMPL/MBR_DEP_CHECK_ACTIVE function module, which checks to see if the current XStep has any dependencies.  If there are any active dependencies linked to the XStep, the XStep will either be locked, or execution will be prevented, until the dependent step has been completed.

No additional setup is required from the XStep itself regarding the parameters listed above.  The solution is already included for each XStep, so only the MBR configuration is required on the master recipe level for dependency information, if any inter-dependencies are needed throughout the PI Sheet.

### 12.4 Technical Implementation

Function Button Integration

Add description of button logic if any

Field Behavior and Flexibility

Add description of field logic if any

Signature Validation Mechanism

After the step is fully executed, operators are required to sign off using the configured signature strategy (IV_SIGN), confirming the assignment for audit and compliance purposes.

Applicability and Execution Control

- The IV_ACTIVE parameter determines whether the XStep is enabled during execution.

- The IV_ACT_FLP parameter ensures mutual exclusivity or alternate routing by acting as an inverse flag.

Data Integration and Traceability

- UUID-based tracking supports full traceability in the electronic batch record.

- Signature events are logged and auditable.

- Local parameters such as LAV_ORDER, LAV_PLANT, and LAV_PHASE are used to… If needed

## Function Module(s)

Function: NAME

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

## Pseudocode

Function: NAME

1. Initialization

Add Info Dev to add changes as needed

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.
b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

- If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

- If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

- This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps

- The use of constants (gc_yes, gc_no, and gc_inside_range) allows the logic to be aligned with broader application settings or standards.

- Helpful in scenarios where input values may vary but need to result in consistent "active" or "inactive" behavior.

- Input is optional, so defaulting behavior can be configured outside this function.

- This function improves clarity and consistency in determining whether an XStep should be active when rendered in the PI Sheet.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR (Manufacturing Batch Record) step can be activated:

- IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

- IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

- IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

- IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

- IM_PLANT (WERKS_D) – Plant code for process context.

b. The function first validates that IM_CONTROL_RECIPE is not initial.

- If it is initial, the exception CONTROL_RECIPE_INITIAL is raised.

c. Internal data is initialized:

- lt_ord_sigs: Internal table for storing order signature data from /SMPL/MBR_OR_SIG.

- ls_step_dep: Work area for step dependency data.

- Routine LOAD_INTERNAL_DATA is called to populate internal buffers (g_ord_data-t_ord_deps) using IM_AUFNR, IM_CONTROL_RECIPE, and IM_STEP_UUID.

2. Signature Data Retrieval

a. The function retrieves signature records for the process order from database table /SMPL/MBR_OR_SIG.
b. Selected fields include: AUFNR, SOG_UUID, TYPE, CREATED_DATE, CREATED_TIME, DEST_NAME, SIGN_ID, SIGN_DATE, and SIGN_TIME.
c. Only signatures related to the specified order (AUFNR = IM_AUFNR) are selected.
d. These records are stored in the internal table lt_ord_sigs for later evaluation.

3. Dependency Evaluation

a. The function loops through dependencies in g_ord_data-t_ord_deps where:

- TARGET_UUID = IM_STEP_UUID, and

- DEP_RULE is either gc_perform_before_start_rule or gc_verify_before_start_rule.

b. For each dependency record:

- If the rule is PERFORM BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending perform signature (gc_perform_signature) with SIGN_ID = abap_false.

- If the rule is VERIFY BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending verify signature (gc_verify_signature) with SIGN_ID = abap_false.

c. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive).
d. If all prerequisite signatures are complete, the step is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in the export parameter EV_ACTIVE.

- gc_step_active – All prerequisite steps are completed; the current step can start.

- gc_step_inactive – At least one prerequisite step is incomplete; the current step remains blocked.

Additional Notes

This function determines whether a dependent step in a Manufacturing Batch Record is eligible to start based on the completion of prerequisite steps.

Key Features:

- Evaluates Perform Before Start and Verify Before Start dependency rules.

- Dynamically validates signature data from /SMPL/MBR_OR_SIG.

- Integrates with existing MBR dependency structures via global table g_ord_data-t_ord_deps.

Error Handling:

- Raises CONTROL_RECIPE_INITIAL if the control recipe input is missing.

- Handles empty dependency or signature data without runtime errors.

Dependencies:

- Tables: /SMPL/MBR_OR_SIG (Signatures), g_ord_data-t_ord_deps (Step Dependencies).

- Constants: gc_perform_before_start_rule, gc_verify_before_start_rule, gc_perform_signature, gc_verify_signature, gc_step_active, gc_step_inactive.

- Routine: LOAD_INTERNAL_DATA for initializing order context.

Assumptions:

- All prerequisite steps are recorded in the dependency table.

- Signature records are current and synchronized with MBR step progression.

Activation status is recalculated each time the function is called to reflect live signature data.

## Configuration Specifications

N/A

## Test Scenarios

Variant 1: Equipment Select

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: Bag Replacement Volumetric | Process Order created and Released containing SMPL: Bag Replacement Volumetric |  |
| 2 | Open the EBR and navigate to Phase 0030 and then navigate to SMPL: Bag Replacement Volumetric | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Bag Replacement Volumetric step is visible in Phase 0030 | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Bag Replacement Volumetric step is visible in Phase 0030 |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1 | Initial Functional Design Information | NAME | DATE |
| 1.1 | Technical Design Information | NAME | DATE |