V 1.0

May 2026

XStep Design Specification

SMPL: Bioreactor Sampling

Table of Contents

Right-click and choose "Update Field" to build the table of contents.

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep "SMPL: Bioreactor Sampling". It defines the necessary inputs, process parameter recordings, calculated fields, validation rules, and backend function modules required to digitize the daily bioreactor sampling process in SAP.

## Overview

The XStep contains several main input parameters:

- IV_HEADER - This can be changed in SiMPL MBR to display specific header text for the XStep

- IV_INSTR - This can be changed in SiMPL MBR to display a specific set of instructions for the XStep that will be displayed below the header

- IV_AGMAX: Agitation maximum validation value can be authored in SiMPL MBR.

- IV_AGMIN: Agitation minimum validation value can be authored in SiMPL MBR.

- IV_ASMAX: Air Sparge maximum validation value can be authored in SiMPL MBR.

- IV_ASMIN: Air Sparge minimum validation value can be authored in SiMPL MBR.

- IV_BPMAX: Bioreactor pH maximum validation value can be authored in SiMPL MBR.

- IV_BPMIN: Bioreactor pH minimum validation value can be authored in SiMPL MBR.

- IV_BTMAX: Bioreactor Temp maximum validation value can be authored in SiMPL MBR.

- IV: BTMIN: Bioreactor Temp minimum validation value can be authored in SiMPL MBR.

- IV_DMAX: Prod Days maximum validation value can be authored in SiMPL MBR.

- IV_DMIN: Prod Days minimum validation value can be authored in SiMPL MBR.

- IV_OAMAX: Overlay Air maximum validation value can be authored in SiMPL MBR.

- IV_OAMIN: Overlay Air minimum minimum validation value can be authored in SiMPL MBR.

- IV_PO2MAX: pO2 maximum validation value can be authored in SiMPL MBR.

- IV_PO2MIN: pO2 minimum validation value can be authored in SiMPL MBR.

- IV_PRMAX: Pressure maximum validation value can be authored in SiMPL MBR.

- IV_PRMIN: Pressure minimum validation value can be authored in SiMPL MBR.

The xStep is structured as six sequential data-capture tables.

The operator selects from the dropdown input the sample day type and inputs the Day # and Sample Time

- “ Sample Day Type” - Input dropdown with the options of Seed Day, Production Day, Seed Perfusion Day.

- “ Day #” - Input Text

- “ Sample Time” - Input Time

The following fields will only be available to input values when the IDB Production option is selected in SiMPL MBR:

- “ TFF Number” - Input Text

- “ Time of Permeate Sample” - Input Time

Bioreactor Conditions

The following values are recorded:

- “Bioreactor Temp (°C)” - Numeric Input (XX.X)

- “pO2 Primary” - Numeric Input (X.X)

- “Air Sparge (slpm)” - Numeric Input (X.X)

- “CO2 Sparge (slpm)” - Numeric Input (X.X)

- “Agitation (rpm)” - Numeric Input (X)

- “pO2 Secondary” - Numeric Input (X.X)

- “O2 Sparge (slpm)” - Numeric Input (X.X)

- “Base Weight (kg)” - Numeric Input (X.X)

- “Bioreactor pO2 AIT ID” - Dropdown Input with options of AIT07004 and AIT07005

- “Pressure (psig)” - Numeric Input (X.XX)

- “Overlay Air (slpm)” - Numeric Input (X.X)

- “Bioreactor Weight (kg)” - Numeric Input (X)

Bioreactor Conditions

The following values are recorded:

- “pCO2 (T) (mmHg)” - Numeric Input (X.X)

- “pO2(T) (mmHg)” - Numeric Input (X.X)

- “Na+ (mmol/L)” - Numeric Input (X.X)

- “K+ (mmol/L)” - Numeric Input (X.X)

- “Ca++ (mmol/L)” - Numeric Input (X.X)

- “Cl- (mmol/L)” - Numeric Input (X.X)

Bioreactor pH Conditions

The operator selects from the dropdown input the probe and enter the following details:

- “Bioreactor pH AIT ID” - Dropdown Input with the options of AIT07006 or AIT07007 to select the primary probe

- “Bioreactor pH Prim” - Numeric Input (X.XX)

- “Bioreactor pH Sec” - Numeric Input (X.XX)

- “BGA/Offline pH Meter ID” - Text Input

- “Offline pH” - Numeric Input (X.XX)

- “pH Diff Prim” - Calculation Output: Bioreactor pH Prim - Offline pH (X.XX)

- “pH Diff Sec” - Calculation Output: Bioreactor pH Sec - Offline pH (X.XX)

- “One Point Calib Req?” - Output

- “2nd Sample Req” - Output

The One Point Calib Req? Is automatically calculated based on the pH Diff result.

If pH difference is ﹤0.05 pH unit: no correction required

If ≥ 0.05 or ≤ 0.10 pH unit: use the offline pH to perform a one-point

If ＞±0.10 pH unit: collect a 2nd pH sample immediately

The 2nd Sample Req also auto-populates with the result linked to the above validation.

Bioreactor Dilution

The operator enters the following values:

Glucose and Lactate:

- “Value (mg/dL)” - Text Input

- “Dilution” - Text Input

- “Calculated Value (mg/dL)” - Numeric Input (X.X)

Bioreactor Dilution

The operator executes the “Get Equipment” button:

- Get Equipment - Function Button

The button is linked to the function module ZSMPL_FM_GET_ASSIGNED_EQUI_EBR that verifies the equipment number against the Equipment Master and retrieves the following information:

- “Device ID” - Input short text field

The operator inputs the following information:

- “Vi-Cell Dilution” - Text Numeric (X)

- “VCD” - Text Numeric (X)

- “Viability %” - Text Input

- “Osmolality (mOsm/kg)” - Text Input

The following two tables will only be available when selected as such from SiMPL MBR.

TFF Pressure Check & Swap

The operator inputs the following information

- “P1 (psig)” - Numeric Input (X.X)

- “P2 (psig)” - Numeric Input (X.X)

- “P3 (psig)” - Numeric Input (X.X)

- “P1-P3 (psig)” - Numeric Output as calculated by subtracting P3 from P1 values. (X.X)

Raman Glucose

The following values are recorded:

- “Raman Glucose Result” - Numeric Input (X.X)

- “Current Offset” - Numeric Input (X.X)

- “Current Glucose Result” - Output calculation: sum of the Current Glucose Offset and Raman Glucose Result.

- “BGA Permeate Glu.” - Numeric Input (X.X)

- “Current Glucose Result” - Output

- “Update Offset (＞ᐩ-50)” - Output calculation: Current Glucose Result is subtracted from BGA Permeate Glu.

There should be validation on the Updated Offset value to ensure it is equal to or above 50 mg/dL.

The operator selects from the dropdown input whether a Complete Offset has been completed or N/A.

- “Complete Offset” - Input Dropdown with options of Not Applicable, Completed, Not Completed.

The sample time is populated in the following field:

- “Raman Sample Time” - Time Output using the function module /SMPL/PPPI_FM_GET_DATE_TIME

A custom Z-table, ZTC_BIO_SMPL_DATA will be created to utilize a composite primary key consisting of the Process Order number and the Production Day number to allow daily sampling tracking to be recorded and uniquely identified per batch campaign. The Z-table will need to contain the following fields:

- MANDT - MANDT: Client, Key field

- AUFNR - AUFNR: Order number, Key field

- PROD_DAY - CHAR30: Production day of entry, Key Field

- BIO_WT - ERFMG: Bioreactor weight

- GLUCOSE_VAL - ERFMG: Glucose value

- VCD - ERFMG: Viable Cell Density

- VIABILITY_PCT - CHAR5: Viability percentage

Functional Design

## Reasons for developing

The "SMPL: Bioreactor Sampling" XStep was developed to digitize, standardize, and streamline the daily bioreactor sampling workflow within the Electronic Batch Record (EBR), shifting away from error-prone paper-based logging. The system enforces automated validation checks—such as real-time calculation of primary and secondary pH differentials against offline analytical meters—and dynamically flags when mandatory actions like a 1-point calibration or an immediate second sample are required. Furthermore, it safeguards data integrity by programmatically restricting specific parameters (e.g., TFF and Raman glucose data) to their appropriate operational contexts and verifying critical analytical instruments against the Equipment Master at the point of use, establishing a robust and transparent digital audit trail.

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | User signing signature fields is an approved user in the system with the correct access configuration |
| 2 | Master Data has been set up correctly |
| 3 | The function modules are available and return the correct system time. |
| 4 | The function module can successfully pull the data into the output fields |

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| User Signature rejected due to an incorrect username or password | User accepts error message and re-enter the correct username and password |
| Equipment item not found | Check Equipment Master |
| Mandatory field data not captured | A validation error is displayed - capture mandatory fields. |
| Value outside of the tolerance range is accepted. | Verify if tolerance ranges have been correctly authored in SiMPL MBR. |

## XStep Layout Design

*EBR screenshot will be inserted once developed.

## Function Module(s)

Function: ZSMPL_FM_GET_ASSIGNED_EQUI_EBR

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_ORDER | AUFNR | Order Number |
| IM_EQUIP_TYPE | EQTYP | Equipment Category |
| IM_ALLOW_NA | BOOLE_D | Boolean variable (allow N/A) |
| IM_VORNR | VORNR | Operation/Activity Number |
| IM_OBJ_TYP | EQART | Object Type of Technical Object |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_EQUI | CHAR30 | Equipment ID |
| EX_EQUI_TXT | CHAR30 | Equipment Description |

Exceptions:

| Exception | Short Text |
| --- | --- |
| INTERNAL_ERROR | Internal processing error |

Function: /SMPL/ELB_FM_GET_ASS_EQ_VALID

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_ORDER | AUFNR | Order Number |
| IM_EQUIP_TYPE | EQTYP | Equipment Category |
| IM_ALLOW_NA | BOOLE_D | Boolean variable (allow N/A) |
| IM_EQUI | CHAR30 | Equipment ID to validate |
| IM_VORNR | VORNR | Operation/Activity Number |
| IM_MULTI_PO | ABAP_BOOL | Boolean (multi-PO assignment) |
| IM_CRID | POC_DOCID | Control Recipe Number |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_EQUI_TXT | CHAR30 | Equipment Description |

Exceptions:

| Exception | Short Text |
| --- | --- |
| EQUI_NOT_ASSIGNED | Equipment is not assigned to the order |
| EQUI_NOT_CLEAN | Equipment is not in a clean status |
| STATUS_NOT_CONFIGURED | Equipment status configuration is missing |

Function: ZSMPL_FM_STORE_DAILY_SAMPLE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_AUFNR | AUFNR | Order Number |
| IV_DAYTYPE | CHAR4 | Character 4 (day-type indicator) |
| IV_DAY | INT1 | Single-byte integer (sample day) |
| IV_GLUCOSE | /SMPL/STR_DE_ONE_DEC | One-decimal value (glucose) |
| IV_BRX_WT | /SMPL/STR_DE_ONE_DEC | One-decimal value (BRX weight) |
| IV_VCD | /SMPL/STR_DE_ONE_DEC | One-decimal value (VCD) |
| IV_VIAB | /SMPL/STR_DE_ONE_DEC | One-decimal value (viability) |
| IV_RETRIEVE | FLAG | General flag (retrieve mode) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_DAY | INT1 | Single-byte integer (sample day) |
| EV_GLUCOSE | /SMPL/STR_DE_ONE_DEC | One-decimal value (glucose) |
| EV_BRX_WT | /SMPL/STR_DE_ONE_DEC | One-decimal value (BRX weight) |
| EV_VCD | /SMPL/STR_DE_ONE_DEC | One-decimal value (VCD) |

Exceptions:

| Exception | Short Text |
| --- | --- |
| TABLE_LOCK_NOT_POSSIBLE | Daily-sample table could not be locked |
| INSERT_FAILED | Insert of the daily-sample record failed |
| DAY_TYPE_REQUIRED | A valid day type (Seed/Production/Perfusion) was not supplied |

Function: ZSMPL_FM_OPS

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_PHDIFFP | F | Floating point (Probe pH difference) |
| IV_PHDIFFS | F | Floating point (Sample pH difference) |
| IV_TOLERANCE | F | Floating point (tolerance) |
| IV_SAMPLE_IND | CHAR1 | Character 1 (sample indicator) |
| IV_REF_VALUE | F | Floating point (reference value) |
| IV_VALUE | F | Floating point (value to validate) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_STD6 | CHAR3 | Character 3 (standardization result – probe) |
| EV_STD7 | CHAR3 | Character 3 (standardization result – sample) |

Exceptions:

| Exception | Short Text |
| --- | --- |
| BAD_TOLERANCE | Value is not within the allowed tolerance range |

Function: /SMPL/PPPI_FM_CALC_EXECUTE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_VAL1 | F | Floating point (value 1) |
| IV_TYPE1 | CHAR1 | Character 1 (operation 1) |
| IV_VAL2 | F | Floating point (value 2) |
| IV_TYPE2 | I | Integer (operation 2) |
| IV_VAL3 | F | Floating point (value 3) |
| IV_RESULT_UOM | ERFME | Unit of measure of entry (result UOM) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_RESULT | F | Floating point (calculated result) |
| EV_RESULT_OUT | CHAR30 | Character 30 (formatted result) |

Function: /SMPL/PPPI_FM_MIN_MAX

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| I_MIN | F | Floating Point Number (minimum) |
| I_MAX | F | Floating Point Number (maximum) |
| I_VALUE | F | Floating Point Number (value to check) |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_OUT_OF_RANGE | Entered value falls outside the defined range |
| MIN_GREATER_THAN_MAX | Minimum value is greater than or equal to the maximum |

Function: /SMPL/PPPI_FM_GET_DATE_TIME

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_COMPLETE | CHAR1 | Character 1 |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_DATE | DATS | Date |
| EV_TIME | TIMS | Time |
| EV_DATE_DEFAULT | DATS | Date |
| EV_TIME_DEFAULT | TIMS | Time |

Exceptions:

| Exception | Short Text |
| --- | --- |
| PERIOD_COMPLETED | Raised/handled when the period has already been completed |

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_IDX | INT4 | XStep Step Index Number |
| IM_STEP | CHAR30 | XStep Header |
| IM_SIGTYPE | CHAR20 | Signature Type |
| IM_USERID | XUBNAME | User Name |
| IM_DB_UP_DOWN | CHAR1 | Done By Up/Down |
| IM_VALIDATION_REQ | CHAR3 | Validation Required |
| IM_STEP_UUID | ZSMPL_DE_SMPL_SOG_UUID | XStep SOG UUID |

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_STEP_UUID | /SMPL/STR_DE_SOG_UUID | XStep SOG UUID |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| TABLE_LOCK_NOT_POSSIBLE | Structure table could not be locked |

Function: /SMPL/PPPI_FM_SIG_VALIDATION

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | XStep SOG UUID |
| IM_CB_VALIDATE | CHAR1 | Checked By Validation Flag |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User ID is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not registered on PI sheet |
| USER_SIGNED_DONE_BY | User already signed the Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed the Perform By |
| XSTEP_CONFIG | XStep / structure configuration error |

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | SYSUUID_22 | 16 Byte UUID in 22 Characters |
| IM_CB_VALIDATE | CHAR1 | Checked By Validation Flag |
| IM_PLANT | WERKS_D | Plant |
| IV_MN | F | Minimum Value |
| IV_MN_DAY | I | Minimum Day Offset |
| IV_MN_TIM | TIMS | Minimum Time |
| IV_MT | F | Target Value |
| IV_MT_DY | I | Target Day Offset |
| IV_MT_TM | TIMS | Target Time |
| IV_MX | F | Maximum Value |
| IV_MX_DAY | I | Maximum Day Offset |
| IV_MX_TIM | TIMS | Maximum Time |
| IV_INSIDE_RANGE | CHAR1 | Inside Range Indicator (default 1) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_SIG_REQ | CHAR1 | Signature Required Indicator |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_NOT_SET | Value to validate not set |
| RANGE_BELOW_MIN | Value below minimum |
| RANGE_ABOVE_MAX | Value above maximum |
| RANGE_INSIDE_RANGE | Value inside range |
| DOESNT_WORK_FOR_TIME | Not supported for time values |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User ID is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not registered on PI sheet |
| USER_SIGNED_DONE_BY | User already signed the Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed the Perform By |
| UNEXPECTED_ERROR | Unexpected error |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

## Pseudocode

Function: ZSMPL_FM_GET_ASSIGNED_EQUI_EBR

1. Purpose

Returns the equipment (e.g. welder, room, or additional equipment) currently assigned to the active process order from the SAP Equipment Master / SiMPL equipment logbook. If a single qualifying equipment is assigned it is returned automatically; if multiple are assigned the operator is prompted with a selection pop-up; if none are assigned a warning is displayed.

2. Input Parameters

•  IM_ORDER – Process order number used to look up assigned equipment.

•  IM_EQUIP_TYPE – Equipment category (EQTYP), e.g. ‘R’ room or ‘A’ additional equipment.

•  IM_ALLOW_NA – Optional flag; when set, an ‘N/A’ entry is offered so the operator can record that no equipment applies.

•  IM_VORNR – Optional operation/phase number to restrict the search to an order-phase combination.

•  IM_OBJ_TYP – Optional technical object type (EQART) for further filtering.

3. Status Determination

a. The active “IN USE” and “IN USE - CLEANED” equipment statuses are resolved dynamically from the SiMPL status configuration table /SMPL/ELB_EQSTAT via subroutine GET_EQ_STAT_BY_CLASS (status classes ‘U1’ and ‘U2’), allowing status text translation.

4. Equipment Retrieval

a. Export parameters and working tables are cleared. If IM_ALLOW_NA is set, an ‘N/A’ row is appended to the candidate list.

b. Equipment is selected from /SMPL/ELB_EQUIP joined to the Equipment Master (EQUI) and description table (EQKT) where:

•  order number = IM_ORDER

•  (optional) activity = IM_VORNR

•  equipment category = IM_EQUIP_TYPE

•  the confirmation/update signature has been completed (UPDAT_CB <> space)

•  (optional) object type = IM_OBJ_TYP

c. Adjacent duplicates are removed (latest entry per equipment kept), and rows whose status is neither “IN USE” nor “IN USE - CLEANED” are deleted.

5. Result Handling

a. Single match: the equipment ID and description are returned directly.

b. No match (or only the N/A row): a configured SiMPL enhancement-framework class/method is invoked and a warning pop-up (“No room is currently assigned…” / “…has not been assigned…”) is shown; the export values are set to ‘N/A’.

c. Multiple matches: for the SiMPL EBR (websocket) context a generic value-help dialog is built (/SMPL/EBR_CL_GENERIC_VAL_HELP); for the SAP GUI context an ALV pop-up (CL_SALV_TABLE) lists the equipment for single selection. The selected row is read.

6. Standardization Check

a. The selected equipment number is ALPHA-converted and passed to subroutine STANDARDIZATION_CHECK.

b. If the equipment is not standardized (‘S1’) or has not yet been standardized (‘S2’), a warning pop-up is displayed and the export values are reset to ‘N/A’.

7. Output

•  EX_EQUI – Selected equipment ID (leading zeros removed).

•  EX_EQUI_TXT – Equipment description.

Additional Notes

Key Features:

•  Supports automatic single-equipment return, multi-equipment pop-up selection, and N/A handling.

•  Dynamically resolves equipment status text for translation support.

•  Works in both the SiMPL EBR websocket UI and the classic SAP GUI.

Error Handling:

•  INTERNAL_ERROR is raised if a configured enhancement class/method cannot be called.

Dependencies:

•  Tables /SMPL/ELB_EQUIP, EQUI, EQKT, /SMPL/ELB_EQSTAT; classes /SMPL/EBR_CL_APC_WS, /SMPL/EBR_CL_GENERIC_VAL_HELP, /SMPL/EBR_CL_ENH_FWK.

Assumptions:

•  Equipment has been assigned to the order through the appropriate SiMPL assignment step with a completed confirmation signature.

Function: /SMPL/ELB_FM_GET_ASS_EQ_VALID

1. Purpose

Validates that a specific equipment ID is assigned (and in an in-use / cleaned status) to the process order, and returns its description. Used to confirm a scanned/entered equipment number is the correct, currently-assigned equipment before it is recorded.

2. Input Parameters

•  IM_ORDER – process order number.

•  IM_EQUIP_TYPE – equipment category.

•  IM_EQUI – the equipment ID to validate.

•  IM_ALLOW_NA – optional flag to permit an ‘N/A’ result.

•  IM_VORNR – optional operation/phase for an order-phase assignment check.

•  IM_MULTI_PO – optional flag to also accept the ‘in use cleaned’ status (multi-PO).

•  IM_CRID – optional control recipe for a control-recipe assignment check.

3. Export Parameters

•  EX_EQUI_TXT – the validated equipment description.

4. Processing Logic

a. Resolve the active ‘IN USE’ status (class U1) from /SMPL/ELB_EQSTAT; if missing, RAISE status_not_configured. When IM_MULTI_PO is set, also resolve the ‘IN USE - CLEANED’ status (class U2).

b. ALPHA-convert IM_EQUI and SELECT from /SMPL/ELB_EQUIP (joined to EQUI and EQKT) for the order, equipment ID, category, and optional phase / control recipe.

c. Keep the latest record per equipment and delete rows whose status is not an accepted ‘in use’ status.

d. Read the first remaining row; if none, display a message and RAISE equi_not_assigned. Otherwise set EX_EQUI_TXT to its description.

e. Run the standardization check; if the equipment is not standardized (‘S1’) or not yet standardized (‘S2’), display a warning and set EX_EQUI_TXT to ‘N/A’.

Output

•  EX_EQUI_TXT – the description of the validated assigned equipment (or ‘N/A’ when standardization fails).

Additional Notes

Error Handling:

•  EQUI_NOT_ASSIGNED – the equipment ID is not assigned to the order.

•  STATUS_NOT_CONFIGURED – the equipment status configuration is missing.

•  EQUI_NOT_CLEAN – declared for the not-clean status case.

Dependencies:

•  Tables /SMPL/ELB_EQUIP, EQUI, EQKT, /SMPL/ELB_EQSTAT; standardization-check subroutine.

Assumptions:

•  The equipment was assigned through the appropriate SiMPL assignment step and its status is maintained in the status configuration.

Function: ZSMPL_FM_STORE_DAILY_SAMPLE

1. Purpose

Stores or retrieves the daily bioreactor sampling values (sample day, glucose, BRX weight, VCD and viability) in the custom table ZTC_DAILY_SAMPLE, keyed by process order, sample day and day type (Seed / Production / Perfusion).

2. Input Parameters

•  IV_AUFNR – process order number.

•  IV_DAYTYPE – day-type indicator (‘1’ = Seed, ‘2’ = Production, ‘3’ = Perfusion).

•  IV_DAY – the sample day number.

•  IV_GLUCOSE / IV_BRX_WT / IV_VCD / IV_VIAB – the sampling measurement values (store mode).

•  IV_RETRIEVE – when set, the function reads instead of writes.

3. Export Parameters

•  EV_DAY / EV_GLUCOSE / EV_BRX_WT / EV_VCD – the stored values for the requested day (retrieve mode).

4. Processing Logic

a. Map the day-type indicator to its code (1→SEED, 2→PROD, 3→PERF); if none matches, RAISE day_type_required.

b. Store mode (IV_RETRIEVE = false): build a ZTC_DAILY_SAMPLE record from the inputs, lock the table (ENQUEUE_E_TABLE; on failure RAISE table_lock_not_possible), INSERT the record (on failure unlock and RAISE insert_failed), then unlock the table (DEQUEUE_E_TABLE).

c. Retrieve mode (IV_RETRIEVE = true): SELECT the record for the order + sample day + day type; when found, populate EV_DAY, EV_GLUCOSE, EV_BRX_WT and EV_VCD.

Output

•  A persisted daily-sample record (store mode) or the previously stored values (retrieve mode).

Additional Notes

Error Handling:

•  DAY_TYPE_REQUIRED – the day-type indicator was not 1/2/3.

•  TABLE_LOCK_NOT_POSSIBLE – the table could not be locked for insert.

•  INSERT_FAILED – the record could not be inserted.

Dependencies:

•  Table ZTC_DAILY_SAMPLE; lock object ENQUEUE/DEQUEUE_E_TABLE.

Assumptions:

•  Order + sample day + day type uniquely identify a daily-sample record.

Function: ZSMPL_FM_OPS

1. Purpose

A tolerance-check helper for the bioreactor sampling pH standardization. Depending on which inputs are supplied it operates in one of three modes: comparing probe/sample pH differences against a configured tolerance, comparing them against fixed mid/high thresholds, or validating a measured value against a percentage tolerance band around a reference value.

2. Input Parameters

•  IV_PHDIFFP / IV_PHDIFFS – the probe and sample pH differences (absolute values are used).

•  IV_TOLERANCE – the configured tolerance.

•  IV_SAMPLE_IND – sample indicator selecting the mid/high comparison mode.

•  IV_REF_VALUE / IV_VALUE – reference value and measured value for the range-check mode.

3. Export Parameters

•  EV_STD6 / EV_STD7 – standardization results (‘YES’/‘NO’) for the probe and sample.

4. Processing Logic

a. The pH differences and tolerance are converted to absolute packed decimals; mid (0.05) and high (0.10) thresholds are prepared.

b. Mode 1 – when neither the sample indicator nor a value is supplied: compare each pH difference to the tolerance and set EV_STD6/EV_STD7 to ‘YES’/‘NO’ per the four combinations (both below → NO/NO, both above → YES/YES, mixed → one YES one NO).

c. Mode 2 – when the sample indicator is set but no value: compare the differences to the mid and high thresholds (both below mid → NO/NO; either between mid and high → YES/NO; both above high → No/Yes).

d. Mode 3 – when a value is supplied: compute the upper limit as (tolerance × reference + reference); if the value exceeds it, display a message stating the allowed tolerance percentage and RAISE bad_tolerance.

Output

•  EV_STD6 / EV_STD7 standardization flags (modes 1–2), or an exception when the value is out of range (mode 3).

Additional Notes

Error Handling:

•  BAD_TOLERANCE – the measured value exceeds the allowed tolerance band around the reference value.

Dependencies:

•  None (self-contained arithmetic / comparison).

Assumptions:

•  The supplied parameter combination selects exactly one of the three modes; mid/high thresholds of 0.05 / 0.10 are fixed business rules.

Function: /SMPL/PPPI_FM_CALC_EXECUTE

1. Purpose

A generic two-step calculation engine for XStep formulas. It applies one arithmetic operation to two values and, optionally, a second operation with a third value, then formats the result with a unit of measure for display.

2. Input Parameters

•  IV_VAL1 / IV_VAL2 – the first operands.

•  IV_TYPE1 – the first operation (add, subtract, multiply, divide, or absolute-difference).

•  IV_TYPE2 / IV_VAL3 – optional second operation and its operand, applied to the intermediate result.

•  IV_RESULT_UOM – optional unit of measure used when formatting the output.

3. Export Parameters

•  EV_RESULT – the numeric calculation result.

•  EV_RESULT_OUT – the formatted result string (value with UOM).

4. Processing Logic

a. If IV_VAL1 > 0, apply the first operation (IV_TYPE1) to IV_VAL1 and IV_VAL2 — add, subtract, multiply, divide (guarding against divide-by-zero → 0), or absolute difference.

b. If a result was produced and IV_TYPE2 is supplied, apply the second operation with IV_VAL3 to the intermediate result (same operation set, same divide-by-zero guard).

c. Call /SMPL/PPPI_FM_OUTPUT_VALUE with the result and IV_RESULT_UOM to produce the formatted display string EV_RESULT_OUT.

Output

•  EV_RESULT (numeric) and EV_RESULT_OUT (formatted with UOM).

Additional Notes

Error Handling:

•  No exceptions; division by zero yields a result of 0.

Dependencies:

•  Function module /SMPL/PPPI_FM_OUTPUT_VALUE; calculation-type constants (plus/minus/multiply/divide/abs-minus).

Assumptions:

•  IV_VAL1 greater than zero indicates a calculation should be attempted; the operation-type codes match the configured calculation constants.

Function: /SMPL/PPPI_FM_MIN_MAX

1. Purpose

Validates a numeric value against an optional minimum and/or maximum limit and raises an exception (after displaying an explanatory message) when the value falls outside the defined range, or when the configured minimum and maximum are inconsistent. Used to enforce the K setpoint and other numeric entry limits.

2. Input Parameters

•  I_MIN – optional lower bound.

•  I_MAX – optional upper bound.

•  I_VALUE – the value to validate.

3. Processing Logic

a. The min, max and value are copied into packed decimals (3 decimal places) for use in the operator messages.

b. If BOTH I_MIN and I_MAX are supplied:

•  If I_MIN >= I_MAX, display "The Minimum value is set larger than the Maximum value" and RAISE min_greater_than_max.

•  Else if I_VALUE lies between I_MIN and I_MAX (inclusive), the value is accepted (no exception).

•  Otherwise display a message stating the entered value should be between the min and max, and RAISE value_out_of_range.

c. If ONLY I_MAX is supplied: accept when I_VALUE <= I_MAX; otherwise display a message and RAISE value_out_of_range.

d. If ONLY I_MIN is supplied: accept when I_VALUE >= I_MIN; otherwise display a message and RAISE value_out_of_range.

e. If neither bound is supplied, no validation is performed.

Output

•  No export parameters; a successful validation is indicated by no exception being raised.

Additional Notes

Error Handling:

•  VALUE_OUT_OF_RANGE – the value is outside the configured bound(s); an explanatory message naming the entered value and the limit(s) is shown first.

•  MIN_GREATER_THAN_MAX – the supplied minimum is greater than or equal to the supplied maximum.

Dependencies:

•  None – no tables or external function modules; messages are issued as type 'S' displayed like an error.

Assumptions:

•  Both bounds are optional; when both are blank the range check is intentionally skipped.

Function: /SMPL/PPPI_FM_GET_DATE_TIME

1. Purpose

Returns the current local date and time for use in a process instruction (XStep) entry, unless the period/XStep has already been completed, in which case it issues an information message and does nothing.

2. Input Parameters

•  IV_COMPLETE – completion flag; '1' means the period has already been completed.

Export Parameters

•  EV_DATE / EV_TIME – current local date and time.

•  EV_DATE_DEFAULT / EV_TIME_DEFAULT – default copies of the same date and time.

3. Processing Logic

a. If IV_COMPLETE = '1', issue information message 008 ('Period has been completed, remove completion before updating') and EXIT without setting any values.

b. Otherwise set EV_DATE = sy-datlo and EV_TIME = sy-timlo (current local date/time).

c. Copy those values into EV_DATE_DEFAULT and EV_TIME_DEFAULT.

Output

•  Current local date/time in both the working and default export fields.

Additional Notes

Error Handling:

•  PERIOD_COMPLETED – declared exception; in practice completion is signalled via information message 008 and an early EXIT rather than RAISE.

Dependencies:

•  System fields sy-datlo / sy-timlo; message class message 008; constant gc_type_i (message type 'I').

Assumptions:

•  IV_COMPLETE = '1' reliably indicates a completed period that must not be re-dated.

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

•  Signature Type Identification

•  Assign the incoming signature type (IM_SIGTYPE) to local variable lv_sigtype.

•  A CASE structure is used to determine behavior based on the type: ‘ ’ (Done By), ‘C’ (Checked By), ‘O’ (Validation Flag Only).

•  Done By Signature (lv_sigtype = ‘ ’)

•  a. Optional Step Name Update – if IM_STEP is provided, update the XStep header in table /SMPL/PPPI_PISST.

b. Determine Position for Signature Insertion

•  If IM_DB_UP_DOWN is empty: insert the new signature above the current item by fetching MIN(UNIQUE_SEQID) and inserting one row before it.

•  If IM_DB_UP_DOWN = ‘B’: insert the new signature below the current item by fetching MAX(UNIQUE_SEQID) and inserting one row after it.

•  In both cases use IM_USERID as the signer (SIGN_ID), the signature type description from TEXT-001, and insert a new row into /SMPL/PPPI_PISST.

•  Checked By Signature (lv_sigtype = ‘C’)

•  a. Get Signature Start Point – select the UNIQUE_SEQID of the “Checked By” step matching the given input.

b. Determine Signature End Point

•  Fetch all prior ‘C’ type signatures for the same control recipe and determine the highest UNIQUE_SEQID lower than the start point to use as the end.

c. Update Checked By Step – set the SIGN_ID and mark the step as COMPLETED = ‘X’.

d. Complete All Steps Between Start and End

•  Loop over all steps with UNIQUE_SEQID between the start and end boundaries and mark each row as COMPLETED = ‘X’; update the database using MODIFY.

•  Validation Flag Update (lv_sigtype = ‘O’)

•  a. Determine Validation Value – if IM_VALIDATION_REQ = ‘2’, set to ‘No’, else ‘Yes’.

b. Update the VALIDATION_REQUIRED field for the current control recipe, phase, and step index in /SMPL/PPPI_PISST.

Additional Notes:

•  Purpose: dynamically inserts or updates signature records in /SMPL/PPPI_PISST depending on the role of the signer (Done By, Checked By, or Validator).

•  Supports step name overrides and UUID tracking, and allows insertion before or after specific records using IM_DB_UP_DOWN.

•  Ensures Checked By signatures cascade completion to preceding steps and accommodates flexible validation logic.

•  Error Handling: no explicit RAISE statements; relies on CHECK and SY-SUBRC evaluations to exit silently if lookup conditions fail.

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

1. Purpose

Builds (once per control recipe) the EBR signature structure stored in /SMPL/PPPI_PISST that drives the Signature Table, then maintains the “Checked By” header/type for the current step. The structure lists every signature step for the order in sequence with a unique index used for ordering and dependency checks.

2. Input Parameters

•  IM_LABEL_ID – Signature label (‘C’ = Checked By).

•  IM_STEP_NAME – Optional XStep header text to display.

•  IM_STEP_INDEX / IM_STEP_UUID – Step sequence index and SOG UUID.

•  IM_CONTROL_RECIPE – Control recipe (PI sheet) number.

•  IM_PHASE, IM_PO_NR, IM_PLANT – Phase, process order and plant context.

3. Structure Build (first execution only)

a. SELECT COUNT(*) from /SMPL/PPPI_PISST for the control recipe + order. If rows already exist the build is skipped.

b. When empty:

•  Read master recipe (PLNNR/PLNAL from AFKO) and control recipe destination (PHSEQ from COCH).

•  Determine whether the order is Production or Process order (AFKO-PLNAW) to pick the SOG data type.

•  Retrieve the most recent SiMPL SOG data via /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA and filter to the current control recipe destination.

•  Lock /SMPL/PPPI_PISST (ENQUEUE_E_TABLE); on failure raise TABLE_LOCK_NOT_POSSIBLE.

•  Insert one /SMPL/PPPI_PISST row per step (step index, name, UUID, master recipe, control recipe, order, date, phase, VALIDATION_REQUIRED = ‘Yes’); then unlock the table.

•  Re-read the structure, sort by phase + step sequence, and assign a UNIQUE_SEQID in increments of 1000.

4. Checked By Maintenance

a. If IM_LABEL_ID = ‘C’, update the step’s TYPE to the “Checked By” text; if IM_STEP_NAME is supplied also update the displayed step name, keyed by step index + control recipe + phase.

5. Output

No export parameters; the persisted /SMPL/PPPI_PISST structure is the result.

Additional Notes

Error Handling:

•  TABLE_LOCK_NOT_POSSIBLE – the structure table could not be locked for the initial build.

Dependencies:

•  Tables /SMPL/PPPI_PISST, AFKO, COCH; class /SMPL/MBR_CL_DB_SERVICE; lock objects ENQUEUE/DEQUEUE_E_TABLE.

Assumptions:

•  SOG/dependency data has been loaded for the order; the first step to execute builds the structure for all subsequent steps.

Function: /SMPL/PPPI_FM_SIG_VALIDATION

1. Purpose

Validates a Done By / Checked By electronic signature before it is committed. It enforces user validity, predecessor-step completion, signature-table registration, the rule that a Checked By user cannot have performed the Done By in the same range, and a final-step “mass dependency” completion check. It also delegates to the MBR dependency validation FM.

2. Input Parameters

•  IM_LABEL_ID – Signature label (‘C’ Checked By, otherwise Done By/Perform).

•  IM_STEP_INDEX / IM_STEP_UUID – Step identifiers.

•  IM_CONTROL_RECIPE, IM_PHASE, IM_PO_NR – Control recipe, phase and process order.

•  IM_USERID – User performing the signature.

•  IM_CB_VALIDATE – Flag to run the Checked-By-vs-Done-By conflict check.

•  IM_PLANT – Plant.

3. Basic Validation

a. If IM_CONTROL_RECIPE is initial → raise CONTROL_RECIPE_INITIAL.

b. If IM_USERID is initial → raise USER_INITIAL.

4. Predecessor / Configuration Checks

a. Read master recipe (AFKO) and control recipe destination (COCH). If either is missing → raise XSTEP_CONFIG.

b. Determine Production vs Process order and retrieve dependency data via /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA.

c. Filter dependencies to the current destination and step UUID and check predecessor completion in /SMPL/PPPI_PISST (predecessor enforcement is delegated to MBR).

5. Signature Table Validation

a. The user must be registered for the PI sheet in /SMPL/PPPI_PIBSI; otherwise → raise USER_NOT_SIGNED_INTO_SIG_TAB.

6. Checked By vs Done By Conflict

a. When IM_LABEL_ID = ‘C’ and IM_CB_VALIDATE is set, the unique sequence range of the current Checked By is determined and the structure is searched for a Done By (‘Perform’) signature by the same user within that range; if found → raise USER_SIGNED_PERF_BY.

7. Final Step Mass Dependency Check

a. The last step (max UNIQUE_SEQID) of the control recipe is read. If the current signature is that final step, all other required (validation required = ‘Yes’, Checked By) steps must be completed; otherwise → raise REQ_EBR_STEPS_NOT_COMPLETED.

8. MBR Dependency Validation

a. /SMPL/MBR_DEP_VALI_SIGN is called with a Perform/Verify signature type derived from IM_LABEL_ID; its exceptions are mapped back to this FM’s exceptions.

Additional Notes

Error Handling:

•  Raises CONTROL_RECIPE_INITIAL, USER_INITIAL, USER_NOT_SIGNED_INTO_SIG_TAB, USER_SIGNED_PERF_BY, REQ_EBR_STEPS_NOT_COMPLETED and XSTEP_CONFIG as described above.

Dependencies:

•  Tables /SMPL/PPPI_PISST, /SMPL/PPPI_PIBSI, AFKO, COCH; FM /SMPL/MBR_DEP_VALI_SIGN; class /SMPL/MBR_CL_DB_SERVICE.

Assumptions:

•  The signature structure has been built (see /SMPL/PPPI_FM_SIG_POPULATE_CB) and the user is registered on the PI sheet.

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

1. Purpose

Wrapper validation used when a recorded value must fall within a configured range and a supervisor signature is conditionally required. It first checks the value against minimum/target/maximum limits to decide whether a signature is required, then runs the standard signature validation.

2. Input Parameters

•  IM_LABEL_ID, IM_STEP_INDEX, IM_STEP_UUID, IM_CONTROL_RECIPE, IM_PHASE, IM_PO_NR, IM_USERID, IM_CB_VALIDATE, IM_PLANT – Standard signature-validation context (see /SMPL/PPPI_FM_SIG_VALIDATION).

•  IV_MN / IV_MT / IV_MX – Minimum, target and maximum numeric limits.

•  IV_MN_DAY/IV_MN_TIM, IV_MT_DY/IV_MT_TM, IV_MX_DAY/IV_MX_TIM – Optional day-offset and time limits for date/time ranges.

•  IV_INSIDE_RANGE – Indicator (default ‘1’) controlling whether being inside or outside the range requires the signature.

3. Range Check

a. /SMPL/PPPI_FM_VALI_SUPE_CHECK is called with the limit parameters; it returns EV_SIG_REQ (signature required) and may raise range exceptions (VALUE_NOT_SET, RANGE_BELOW_MIN, RANGE_ABOVE_MAX, RANGE_INSIDE_RANGE, DOESNT_WORK_FOR_TIME), which are re-raised to the caller.

4. Signature Validation

a. /SMPL/PPPI_FM_SIG_VALIDATION is called with the signature context; its exceptions (control recipe initial, user initial, user not signed, user signed Done By / Perform By, predecessor / required steps not completed) are mapped and re-raised.

5. Output

•  EV_SIG_REQ – ‘X’ when a supervisor signature is required for the recorded value, otherwise blank.

Additional Notes

•  Combines range evaluation and signature validation so a supervisor signature is only enforced when the value falls in the configured condition.

Error Handling:

•  Re-raises the range and signature exceptions listed above; UNEXPECTED_ERROR for any other failure.

Dependencies:

•  FMs /SMPL/PPPI_FM_VALI_SUPE_CHECK and /SMPL/PPPI_FM_SIG_VALIDATION.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.

b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

•  If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

•  If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

•  This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps.

•  The use of constants (gc_yes, gc_no, gc_inside_range) aligns the logic with broader application settings or standards.

•  Input is optional, so defaulting behavior can be configured outside this function.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR step can be activated:

•  IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

•  IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

•  IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

•  IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

•  IM_PLANT (WERKS_D) – Plant code for process context.

b. The function validates that IM_CONTROL_RECIPE is not initial; if it is, CONTROL_RECIPE_INITIAL is raised. Internal data is initialized and routine LOAD_INTERNAL_DATA populates the order dependency buffers.

2. Signature Data Retrieval

a. Signature records for the process order are retrieved from /SMPL/MBR_OR_SIG (AUFNR = IM_AUFNR) into internal table lt_ord_sigs for evaluation.

3. Dependency Evaluation

a. The function loops through dependencies where TARGET_UUID = IM_STEP_UUID and DEP_RULE is a Perform-Before-Start or Verify-Before-Start rule:

•  For PERFORM BEFORE START, it checks whether the prerequisite step has a pending perform signature.

•  For VERIFY BEFORE START, it checks whether the prerequisite step has a pending verify signature.

b. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive); if all prerequisites are complete, it is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in EV_ACTIVE (active = all prerequisites complete; inactive = at least one prerequisite incomplete).

Additional Notes

•  Evaluates Perform Before Start and Verify Before Start dependency rules using live signature data from /SMPL/MBR_OR_SIG.

•  Error Handling: raises CONTROL_RECIPE_INITIAL if the control recipe is missing; handles empty dependency/signature data without runtime errors.

•  Activation status is recalculated each time the function is called to reflect live signature data.

## Configuration Specification

N/A

## Test Scenarios

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version. | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version. |  |  |
| 2 | Open the EBR and navigate to SMPL: Bioreactor Sampling. | Open the EBR and navigate to SMPL: Bioreactor Sampling. |  |  |
| 3 | The operator enters basic tracking data: selects Sample Day Type (Seed, Production, or Seed Perfusion) from the dropdown and inputs Day # and Sample Time. | Dropdown selection and inputs are accepted. |  |  |
| 4 | The operator checks the conditional visibility of TFF fields when IDB Production is selected vs. when it is not selected. | “TFF Number” and “Time of Permeate Sample” fields are editable only when the IDB Production option is selected. |  |  |
| 5 | The operator enters the Bioreactor Temp within range (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 6 | The operator enters the Bioreactor Temp outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 7 | The operator enters the pOr primary value (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 8 | The operator enters the pO2 primary value outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 9 | The operator enters the Air Sparge within range (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 10 | The operator enters the Air Sparge outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 11 | The operator enters the CO2 Sparge value (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 12 | The operator enters the CO2 Sparge value outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 13 | The operator enters the Agitation value (as authored in SiMPL MBR). | Text entry accepted. |  |  |
| 14 | The operator enters the Agitation value outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 15 | The operator enters the pO2 Secondary value (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 16 | The operator enters the pO2 Secondary value outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 17 | The operator enters the O2 Sparge value (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 18 | The operator enters the O2 Sparge value outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 19 | The operator enters the Base Weight value. | Numeric entry accepted with one decimal. |  |  |
| 20 | The operator selects from the dropdown input the primary O2 probe. | Dropdown input accepted. |  |  |
| 21 | The operator enters the Pressure value (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 22 | The operator enters the Pressure value outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 23 | The operator enters the Overlay Air value (as authored in SiMPL MBR). | Numeric entry accepted. |  |  |
| 24 | The operator enters the Overlay Air value outside the validation range (as authored in SiMPL MBR). | Deviation popup occurs. |  |  |
| 25 | The operator enters the Bioreactor Weight value. | Text entry accepted. |  |  |
| 26 | The operator populates the 6 numeric inputs for the blood gas analyzer (BGA) offline conditions table. Verify that the numeric value displays with one decimal. | Numeric entries for blood gas metrics are accepted. |  |  |
| 27 | The operator selects the primary pH probe (AIT07006/AIT07007) and enters numeric values for Prim, Sec, and Offline pH fields with two decimal numeric values | Dropdown and numeric inputs are accepted; system calculates pH Diff Prim and pH Diff Sec automatically with two decimal point values. |  |  |
| 28 | The operator inputs an Offline pH resulting in a pH difference < 0.05 units. | "One Point Calib Req?" and "2nd Sample Req" auto-populate to show no correction/action required. |  |  |
| 29 | The operator inputs an Offline pH resulting in a pH difference between 0.05 and 0.10 units. | "One Point Calib Req?" auto-populates to indicate a one-point calibration is required using the offline pH. |  |  |
| 30 | The operator inputs an Offline pH resulting in a pH difference > ±0.10 units. | "2nd Sample Req" auto-populates indicating a 2nd pH sample must be collected immediately. |  |  |
| 31 | The operator enters Glucose and Lactate Values and Dilution factors. | Inputs accepted; system correctly determines the Calculated Value output field. |  |  |
| 32 | The operator executes the "Get Equipment" button for the analytical testing area. | The system interfaces with module ZSMPL_FM_GET_ASSIGNED_EQUI_EBR, validates against the Equipment Master, and auto-populates "Device ID". |  |  |
| 33 | The operator completes the remaining analytical field inputs (Vi-Cell Dilution, VCD, Viability %, Osmolality). | Numeric and text entries are successfully captured. |  |  |
| 34 | The operator verifies conditional tables for TFF Pressure Check & Swap and Raman Glucose. | The tables are selectively available/hidden based on the configurations authored in SiMPL MBR. |  |  |
| 35 | The operator inputs P1 and P3 pressure values in the TFF table. | The system automatically calculates and outputs the differential "P1-P3 (psig)". |  |  |
| 36 | The operator populates Raman Glucose Result and Current Offset. | The system displays the "Current Glucose Result" as the calculated sum of the two inputs. |  |  |
| 37 | The operator inputs a value for BGA Permeate Glu. that creates an Update Offset < 50 mg/dL. | A validation error is displayed; the system generates a deviation popup to be signed by a supervisor to continue. |  |  |
| 38 | The operator updates the inputs so that the Update Offset is ≥ 50 mg/dL and interacts with the Complete Offset dropdown. | Validation passes; Raman Sample Time automatically populates with the system timestamp via module /SMPL/PPPI_FM_GET_DATE_TIME. |  |  |
| 39 | The operator attempts to finalize the bioreactor sampling step with a mandatory field left empty. | A validation error is displayed—mandatory fields must be fully captured before sign-off can proceed. |  |  |
| 40 | The operator executes the required signature fields to complete the process. | Signatures are checked against system access configurations, accepted, and the step is fully closed out. |  |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.1 | Initial Document (Functional requirements and Technical specification) | Frieda van der Merwe | 20-05-2026 |