V 1.0

May 2026

XStep Design Specification

SMPL: TFF Swap

Table of Contents

Right-click and choose "Update Field" to build the table of contents.

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: TFF Swap”. The purpose of the SMPL: TFF Swap XStep is to provide a standardized, system-enforced digital workflow within the Electronic Batch Record (EBR) to manage Tangential Flow Filtration (TFF) unit transitions when filters reach predefined production days or exceed a 1.5 psi differential pressure threshold. It ensures rigorous process control, real-time deviation triggering, and end-to-end sample traceability while enforcing strict regulatory compliance via a mandatory two-user digital signature strategy.

## Overview

This XStep facilitates the management of Tangential Flow Filtration (TFF) swaps, a critical operational procedure performed during the continuous bioreactor production phase to maintain consistent perfusion and cell retention. The system utilizes multiple TFF units (TFF No. 1 through No. 4) to effectively separate permeate from the cell culture. As these filters are subject to fouling or reaching processing capacity, they must be "swapped" to preserve validated flow rates and pressures.

The primary objective of this process step is to provide a standardized, validated method for transitioning between filtration units while maintaining rigorous sample traceability and baseline equipment data.

Triggering Events

The requirement for a swap is determined during the preceding bioreactor production sampling step and is initiated based on two primary criteria:

- Scheduled Production Day: Reaching predefined milestones: TFF1 to TFF2 (Day 8), TFF2 to TFF3 (Day 11), and TFF3 to TFF4 (Day 13).

- Pressure Differential Trigger: If the calculated pressure differential (P1 – P3) exceeds 1.5 psi, indicating significant filter fouling.

The XStep contains several main input parameters:

- IV_HEADER - This can be changed in SiMPL MBR to display specific header text for the XStep

- IV_INSTR - This can be changed in SiMPL MBR to display a specific set of instructions for the XStep that will be displayed below the header

- IV_DAY - Indicates the target production day trigger for the specific swap instance.

This xstep contains four tables.

Pre Swap Prot_A_BIN Titer Samples

Captures sample metadata before the transition to ensure batch integrity.

- “Sample ID Type” - Text Input

The operator executes the “Get Time” button:

- Get Time - Function Button

The button is linked to the function module /SMPL/PPPI_FM_GET_DATE_TIME  to timestamp the sample.

- “Swap Time” - Output

The operator enters from the DLIMS Updated? Dropdown input with a subsequent signature in the Performed By field:

- “DLIMS Updated?” - Dropdown Input with option of Yes or No

Pre Swap Values

Provides a snapshot of equipment performance immediately before the transition. This serves to verify that the system was within a safe operating range and confirms the validity of the pressure trigger.

The operator records the pressure values:

- “P1” - Numeric input

- “P2” - Numeric input

- “P3” - Numeric input

- “P1-P3” - Calculate function with numeric output

- “Existing ” - Text Input

Time Table

Ensures the transition occurs within validated timeframes.

The operator executes the “Get Start Time” button:

- Get Start Time - Function Button

The button is linked to the function module /SMPL/PPPI_FM_GET_DATE_TIME which pulls the current time:

- “Swap start Time” - Output

The operator executes the “Get End Time” button:

- Get End Time - Function Button

The button is linked to the function module /SMPL/PPPI_FM_GET_DATE_TIME which pulls the current time:

- “Swap End Time” - Output

Post Swap Prot_A_BIN Titer Samples

Establishes the baseline for the new filter unit.

- “Sample ID Type” - Text Input

The operator executes the “Get Time” button:

- Get Time - Function Button

The button is linked to the function module /SMPL/PPPI_FM_GET_DATE_TIME which pulls the current time:

- “Swap Time” - Output

The operator enters from the DLIMS Updated? Dropdown input with a subsequent signature in the Performed By field:

- “DLIMS Updated?” - Dropdown Input with option of Yes or No

Functional Design

To ensure process compliance, a system-driven validation is active on the P1-P3 calculated field. If the pressure exceeds the 1.5 psi threshold, the system automatically triggers a deviation and generates an error message. This requires the operator to document the exception before the XStep can be finalized, ensuring real-time process monitoring and batch integrity.

## Reasons for developing

The SMPL: TFF TFF Swap xstep was developed to provide a standardized, validated method for transitioning between filtration units, ensuring continuous perfusion remains within defined pressure and flow limits while maintaining rigorous sample traceability and cross-departmental alignment.

## Authorization

The Signature Strategies SS000201 - SiMPL Done By and SS000202 - SiMPL Checked By are created and assigned to the Authorization Groups AGDS0001 PI step Done By SS and AGDS0002 PI step Checked By SS. This ensures that at least two users are required to successfully sign off the process step. The first user signs off in the Performed By field and a second user signs the Witnessed By field.

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | User signing signature fields is an approved user in the system with the correct access configuration |
| 2 | Master Data has been set up correctly |
| 3 | The function modules are available and return the correct system time. |
| 4 | The function module can successfully pull the data into the output fields |

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| User Signature rejected due to an incorrect username or password | User accepts error message and re-enter the correct username and password |

## XStep Layout Design

*EBR screenshot will be inserted once step has been developed

## Function Module(s)

Function: ZSMPL_FM_GET_DATE_AND_TIME

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_USER | CHAR30 | Character 30 (user, optional) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_DATE | /SMPL/STR_DE_START_DATE | Start Date |
| EV_TIME | /SMPL/STR_DE_START_TIME | Start Time |

Function: /SMPL/PPPI_FM_START_TIME

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_START_TIME | /SMPL/STR_DE_START_TIME | Start Time |

Function: /SMPL/PPPI_FM_CALC_EXECUTE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_VAL1 | F | Floating point (value 1) |
| IV_TYPE1 | CHAR1 | Character 1 (operation 1) |
| IV_VAL2 | F | Floating point (value 2) |
| IV_TYPE2 | I | Integer (operation 2) |
| IV_VAL3 | F | Floating point (value 3) |
| IV_RESULT_UOM | ERFME | Unit of measure of entry (result UOM) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_RESULT | F | Floating point (calculated result) |
| EV_RESULT_OUT | CHAR30 | Character 30 (formatted result) |

Function: /SMPL/PPPI_FM_MIN_MAX

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| I_MIN | F | Floating Point Number (minimum) |
| I_MAX | F | Floating Point Number (maximum) |
| I_VALUE | F | Floating Point Number (value to check) |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_OUT_OF_RANGE | Entered value falls outside the defined range |
| MIN_GREATER_THAN_MAX | Minimum value is greater than or equal to the maximum |

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_IDX | INT4 | XStep Step Index Number |
| IM_STEP | CHAR30 | XStep Header |
| IM_SIGTYPE | CHAR20 | Signature Type |
| IM_USERID | XUBNAME | User Name |
| IM_DB_UP_DOWN | CHAR1 | Done By Up/Down |
| IM_VALIDATION_REQ | CHAR3 | Validation Required |
| IM_STEP_UUID | ZSMPL_DE_SMPL_SOG_UUID | XStep SOG UUID |

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_STEP_UUID | /SMPL/STR_DE_SOG_UUID | XStep SOG UUID |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| TABLE_LOCK_NOT_POSSIBLE | Structure table could not be locked |

Function: /SMPL/PPPI_FM_SIG_VALIDATION

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | XStep SOG UUID |
| IM_CB_VALIDATE | CHAR1 | Checked By Validation Flag |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User ID is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not registered on PI sheet |
| USER_SIGNED_DONE_BY | User already signed the Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed the Perform By |
| XSTEP_CONFIG | XStep / structure configuration error |

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | SYSUUID_22 | 16 Byte UUID in 22 Characters |
| IM_CB_VALIDATE | CHAR1 | Checked By Validation Flag |
| IM_PLANT | WERKS_D | Plant |
| IV_MN | F | Minimum Value |
| IV_MN_DAY | I | Minimum Day Offset |
| IV_MN_TIM | TIMS | Minimum Time |
| IV_MT | F | Target Value |
| IV_MT_DY | I | Target Day Offset |
| IV_MT_TM | TIMS | Target Time |
| IV_MX | F | Maximum Value |
| IV_MX_DAY | I | Maximum Day Offset |
| IV_MX_TIM | TIMS | Maximum Time |
| IV_INSIDE_RANGE | CHAR1 | Inside Range Indicator (default 1) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_SIG_REQ | CHAR1 | Signature Required Indicator |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_NOT_SET | Value to validate not set |
| RANGE_BELOW_MIN | Value below minimum |
| RANGE_ABOVE_MAX | Value above maximum |
| RANGE_INSIDE_RANGE | Value inside range |
| DOESNT_WORK_FOR_TIME | Not supported for time values |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User ID is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not registered on PI sheet |
| USER_SIGNED_DONE_BY | User already signed the Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed the Perform By |
| UNEXPECTED_ERROR | Unexpected error |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_ADD_PERFORM

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_AUFNR | AUFNR | N/A |
| IM_USERID | XUBNAME | N/A |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | N/A |
| IM_DEST_NAME | /SMPL/MBR_CMX_XS_DST_NAME | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

## Pseudocode

Function: ZSMPL_FM_GET_DATE_AND_TIME

1. Purpose

Returns the current local date and time for stamping a pH check / measurement entry in the XStep.

2. Input Parameters

•  IV_USER – optional user identifier (not required for the date/time calculation).

3. Export Parameters

•  EV_DATE – current local date.

•  EV_TIME – current local time.

4. Processing Logic

a. EV_DATE = sy-datlo (current local date).

b. EV_TIME = sy-timlo (current local time).

Output

•  Current local date and time.

Additional Notes

Error Handling:

•  No exceptions.

Dependencies:

•  System fields sy-datlo / sy-timlo.

Assumptions:

•  The local (user time-zone) date and time are required for the entry stamp.

Function: /SMPL/PPPI_FM_START_TIME

1. Purpose

Returns the current local system time, used to stamp the start time of a process step (e.g. a TFF swap timer).

2. Export Parameters

•  EV_START_TIME – the current local time.

3. Processing Logic

a. EV_START_TIME = sy-timlo (current local time).

Output

•  The current local time.

Additional Notes

Error Handling:

•  No exceptions.

Dependencies:

•  System field sy-timlo.

Assumptions:

•  The local (user time-zone) time is required for the start stamp.

Function: /SMPL/PPPI_FM_CALC_EXECUTE

1. Purpose

A generic two-step calculation engine for XStep formulas. It applies one arithmetic operation to two values and, optionally, a second operation with a third value, then formats the result with a unit of measure for display.

2. Input Parameters

•  IV_VAL1 / IV_VAL2 – the first operands.

•  IV_TYPE1 – the first operation (add, subtract, multiply, divide, or absolute-difference).

•  IV_TYPE2 / IV_VAL3 – optional second operation and its operand, applied to the intermediate result.

•  IV_RESULT_UOM – optional unit of measure used when formatting the output.

3. Export Parameters

•  EV_RESULT – the numeric calculation result.

•  EV_RESULT_OUT – the formatted result string (value with UOM).

4. Processing Logic

a. If IV_VAL1 > 0, apply the first operation (IV_TYPE1) to IV_VAL1 and IV_VAL2 — add, subtract, multiply, divide (guarding against divide-by-zero → 0), or absolute difference.

b. If a result was produced and IV_TYPE2 is supplied, apply the second operation with IV_VAL3 to the intermediate result (same operation set, same divide-by-zero guard).

c. Call /SMPL/PPPI_FM_OUTPUT_VALUE with the result and IV_RESULT_UOM to produce the formatted display string EV_RESULT_OUT.

Output

•  EV_RESULT (numeric) and EV_RESULT_OUT (formatted with UOM).

Additional Notes

Error Handling:

•  No exceptions; division by zero yields a result of 0.

Dependencies:

•  Function module /SMPL/PPPI_FM_OUTPUT_VALUE; calculation-type constants (plus/minus/multiply/divide/abs-minus).

Assumptions:

•  IV_VAL1 greater than zero indicates a calculation should be attempted; the operation-type codes match the configured calculation constants.

Function: /SMPL/PPPI_FM_MIN_MAX

1. Purpose

Validates a numeric value against an optional minimum and/or maximum limit and raises an exception (after displaying an explanatory message) when the value falls outside the defined range, or when the configured minimum and maximum are inconsistent. Used to enforce the K setpoint and other numeric entry limits.

2. Input Parameters

•  I_MIN – optional lower bound.

•  I_MAX – optional upper bound.

•  I_VALUE – the value to validate.

3. Processing Logic

a. The min, max and value are copied into packed decimals (3 decimal places) for use in the operator messages.

b. If BOTH I_MIN and I_MAX are supplied:

•  If I_MIN >= I_MAX, display "The Minimum value is set larger than the Maximum value" and RAISE min_greater_than_max.

•  Else if I_VALUE lies between I_MIN and I_MAX (inclusive), the value is accepted (no exception).

•  Otherwise display a message stating the entered value should be between the min and max, and RAISE value_out_of_range.

c. If ONLY I_MAX is supplied: accept when I_VALUE <= I_MAX; otherwise display a message and RAISE value_out_of_range.

d. If ONLY I_MIN is supplied: accept when I_VALUE >= I_MIN; otherwise display a message and RAISE value_out_of_range.

e. If neither bound is supplied, no validation is performed.

Output

•  No export parameters; a successful validation is indicated by no exception being raised.

Additional Notes

Error Handling:

•  VALUE_OUT_OF_RANGE – the value is outside the configured bound(s); an explanatory message naming the entered value and the limit(s) is shown first.

•  MIN_GREATER_THAN_MAX – the supplied minimum is greater than or equal to the supplied maximum.

Dependencies:

•  None – no tables or external function modules; messages are issued as type 'S' displayed like an error.

Assumptions:

•  Both bounds are optional; when both are blank the range check is intentionally skipped.

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

•  Signature Type Identification

•  Assign the incoming signature type (IM_SIGTYPE) to local variable lv_sigtype.

•  A CASE structure is used to determine behavior based on the type: ‘ ’ (Done By), ‘C’ (Checked By), ‘O’ (Validation Flag Only).

•  Done By Signature (lv_sigtype = ‘ ’)

•  a. Optional Step Name Update – if IM_STEP is provided, update the XStep header in table /SMPL/PPPI_PISST.

b. Determine Position for Signature Insertion

•  If IM_DB_UP_DOWN is empty: insert the new signature above the current item by fetching MIN(UNIQUE_SEQID) and inserting one row before it.

•  If IM_DB_UP_DOWN = ‘B’: insert the new signature below the current item by fetching MAX(UNIQUE_SEQID) and inserting one row after it.

•  In both cases use IM_USERID as the signer (SIGN_ID), the signature type description from TEXT-001, and insert a new row into /SMPL/PPPI_PISST.

•  Checked By Signature (lv_sigtype = ‘C’)

•  a. Get Signature Start Point – select the UNIQUE_SEQID of the “Checked By” step matching the given input.

b. Determine Signature End Point

•  Fetch all prior ‘C’ type signatures for the same control recipe and determine the highest UNIQUE_SEQID lower than the start point to use as the end.

c. Update Checked By Step – set the SIGN_ID and mark the step as COMPLETED = ‘X’.

d. Complete All Steps Between Start and End

•  Loop over all steps with UNIQUE_SEQID between the start and end boundaries and mark each row as COMPLETED = ‘X’; update the database using MODIFY.

•  Validation Flag Update (lv_sigtype = ‘O’)

•  a. Determine Validation Value – if IM_VALIDATION_REQ = ‘2’, set to ‘No’, else ‘Yes’.

b. Update the VALIDATION_REQUIRED field for the current control recipe, phase, and step index in /SMPL/PPPI_PISST.

Additional Notes:

•  Purpose: dynamically inserts or updates signature records in /SMPL/PPPI_PISST depending on the role of the signer (Done By, Checked By, or Validator).

•  Supports step name overrides and UUID tracking, and allows insertion before or after specific records using IM_DB_UP_DOWN.

•  Ensures Checked By signatures cascade completion to preceding steps and accommodates flexible validation logic.

•  Error Handling: no explicit RAISE statements; relies on CHECK and SY-SUBRC evaluations to exit silently if lookup conditions fail.

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

1. Purpose

Builds (once per control recipe) the EBR signature structure stored in /SMPL/PPPI_PISST that drives the Signature Table, then maintains the “Checked By” header/type for the current step. The structure lists every signature step for the order in sequence with a unique index used for ordering and dependency checks.

2. Input Parameters

•  IM_LABEL_ID – Signature label (‘C’ = Checked By).

•  IM_STEP_NAME – Optional XStep header text to display.

•  IM_STEP_INDEX / IM_STEP_UUID – Step sequence index and SOG UUID.

•  IM_CONTROL_RECIPE – Control recipe (PI sheet) number.

•  IM_PHASE, IM_PO_NR, IM_PLANT – Phase, process order and plant context.

3. Structure Build (first execution only)

a. SELECT COUNT(*) from /SMPL/PPPI_PISST for the control recipe + order. If rows already exist the build is skipped.

b. When empty:

•  Read master recipe (PLNNR/PLNAL from AFKO) and control recipe destination (PHSEQ from COCH).

•  Determine whether the order is Production or Process order (AFKO-PLNAW) to pick the SOG data type.

•  Retrieve the most recent SiMPL SOG data via /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA and filter to the current control recipe destination.

•  Lock /SMPL/PPPI_PISST (ENQUEUE_E_TABLE); on failure raise TABLE_LOCK_NOT_POSSIBLE.

•  Insert one /SMPL/PPPI_PISST row per step (step index, name, UUID, master recipe, control recipe, order, date, phase, VALIDATION_REQUIRED = ‘Yes’); then unlock the table.

•  Re-read the structure, sort by phase + step sequence, and assign a UNIQUE_SEQID in increments of 1000.

4. Checked By Maintenance

a. If IM_LABEL_ID = ‘C’, update the step’s TYPE to the “Checked By” text; if IM_STEP_NAME is supplied also update the displayed step name, keyed by step index + control recipe + phase.

5. Output

No export parameters; the persisted /SMPL/PPPI_PISST structure is the result.

Additional Notes

Error Handling:

•  TABLE_LOCK_NOT_POSSIBLE – the structure table could not be locked for the initial build.

Dependencies:

•  Tables /SMPL/PPPI_PISST, AFKO, COCH; class /SMPL/MBR_CL_DB_SERVICE; lock objects ENQUEUE/DEQUEUE_E_TABLE.

Assumptions:

•  SOG/dependency data has been loaded for the order; the first step to execute builds the structure for all subsequent steps.

Function: /SMPL/PPPI_FM_SIG_VALIDATION

1. Purpose

Validates a Done By / Checked By electronic signature before it is committed. It enforces user validity, predecessor-step completion, signature-table registration, the rule that a Checked By user cannot have performed the Done By in the same range, and a final-step “mass dependency” completion check. It also delegates to the MBR dependency validation FM.

2. Input Parameters

•  IM_LABEL_ID – Signature label (‘C’ Checked By, otherwise Done By/Perform).

•  IM_STEP_INDEX / IM_STEP_UUID – Step identifiers.

•  IM_CONTROL_RECIPE, IM_PHASE, IM_PO_NR – Control recipe, phase and process order.

•  IM_USERID – User performing the signature.

•  IM_CB_VALIDATE – Flag to run the Checked-By-vs-Done-By conflict check.

•  IM_PLANT – Plant.

3. Basic Validation

a. If IM_CONTROL_RECIPE is initial → raise CONTROL_RECIPE_INITIAL.

b. If IM_USERID is initial → raise USER_INITIAL.

4. Predecessor / Configuration Checks

a. Read master recipe (AFKO) and control recipe destination (COCH). If either is missing → raise XSTEP_CONFIG.

b. Determine Production vs Process order and retrieve dependency data via /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA.

c. Filter dependencies to the current destination and step UUID and check predecessor completion in /SMPL/PPPI_PISST (predecessor enforcement is delegated to MBR).

5. Signature Table Validation

a. The user must be registered for the PI sheet in /SMPL/PPPI_PIBSI; otherwise → raise USER_NOT_SIGNED_INTO_SIG_TAB.

6. Checked By vs Done By Conflict

a. When IM_LABEL_ID = ‘C’ and IM_CB_VALIDATE is set, the unique sequence range of the current Checked By is determined and the structure is searched for a Done By (‘Perform’) signature by the same user within that range; if found → raise USER_SIGNED_PERF_BY.

7. Final Step Mass Dependency Check

a. The last step (max UNIQUE_SEQID) of the control recipe is read. If the current signature is that final step, all other required (validation required = ‘Yes’, Checked By) steps must be completed; otherwise → raise REQ_EBR_STEPS_NOT_COMPLETED.

8. MBR Dependency Validation

a. /SMPL/MBR_DEP_VALI_SIGN is called with a Perform/Verify signature type derived from IM_LABEL_ID; its exceptions are mapped back to this FM’s exceptions.

Additional Notes

Error Handling:

•  Raises CONTROL_RECIPE_INITIAL, USER_INITIAL, USER_NOT_SIGNED_INTO_SIG_TAB, USER_SIGNED_PERF_BY, REQ_EBR_STEPS_NOT_COMPLETED and XSTEP_CONFIG as described above.

Dependencies:

•  Tables /SMPL/PPPI_PISST, /SMPL/PPPI_PIBSI, AFKO, COCH; FM /SMPL/MBR_DEP_VALI_SIGN; class /SMPL/MBR_CL_DB_SERVICE.

Assumptions:

•  The signature structure has been built (see /SMPL/PPPI_FM_SIG_POPULATE_CB) and the user is registered on the PI sheet.

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

1. Purpose

Wrapper validation used when a recorded value must fall within a configured range and a supervisor signature is conditionally required. It first checks the value against minimum/target/maximum limits to decide whether a signature is required, then runs the standard signature validation.

2. Input Parameters

•  IM_LABEL_ID, IM_STEP_INDEX, IM_STEP_UUID, IM_CONTROL_RECIPE, IM_PHASE, IM_PO_NR, IM_USERID, IM_CB_VALIDATE, IM_PLANT – Standard signature-validation context (see /SMPL/PPPI_FM_SIG_VALIDATION).

•  IV_MN / IV_MT / IV_MX – Minimum, target and maximum numeric limits.

•  IV_MN_DAY/IV_MN_TIM, IV_MT_DY/IV_MT_TM, IV_MX_DAY/IV_MX_TIM – Optional day-offset and time limits for date/time ranges.

•  IV_INSIDE_RANGE – Indicator (default ‘1’) controlling whether being inside or outside the range requires the signature.

3. Range Check

a. /SMPL/PPPI_FM_VALI_SUPE_CHECK is called with the limit parameters; it returns EV_SIG_REQ (signature required) and may raise range exceptions (VALUE_NOT_SET, RANGE_BELOW_MIN, RANGE_ABOVE_MAX, RANGE_INSIDE_RANGE, DOESNT_WORK_FOR_TIME), which are re-raised to the caller.

4. Signature Validation

a. /SMPL/PPPI_FM_SIG_VALIDATION is called with the signature context; its exceptions (control recipe initial, user initial, user not signed, user signed Done By / Perform By, predecessor / required steps not completed) are mapped and re-raised.

5. Output

•  EV_SIG_REQ – ‘X’ when a supervisor signature is required for the recorded value, otherwise blank.

Additional Notes

•  Combines range evaluation and signature validation so a supervisor signature is only enforced when the value falls in the configured condition.

Error Handling:

•  Re-raises the range and signature exceptions listed above; UNEXPECTED_ERROR for any other failure.

Dependencies:

•  FMs /SMPL/PPPI_FM_VALI_SUPE_CHECK and /SMPL/PPPI_FM_SIG_VALIDATION.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.

b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

•  If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

•  If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

•  This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps.

•  The use of constants (gc_yes, gc_no, gc_inside_range) aligns the logic with broader application settings or standards.

•  Input is optional, so defaulting behavior can be configured outside this function.

Function: /SMPL/MBR_DEP_ADD_PERFORM

1. Initialization

a. Input parameters provide context for recording a Perform signature against a specific Manufacturing Batch Record (MBR) step:

•  IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

•  IM_AUFNR (AUFNR) (Optional) – Process order number associated with the control recipe.

•  IM_USERID (XUBNAME) – SAP user ID of the performer signing the MBR step.

•  IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG UUID) representing the MBR step being signed.

•  IM_DEST_NAME (/SMPL/MBR_CMX_XS_DST_NAME) – Destination or XStep node name indicating the origin of the signature action.

b. The function validates that IM_USERID is not initial; if missing, the exception USER_INITIAL is raised.

c. It then checks whether the performer is authorized to sign within the MBR context; if not registered in the signature authorization table, USER_NOT_SIGNED_INTO_SIG_TAB is raised.

2. Signature Entry Construction

a. A new record representing the Perform signature is constructed with the process order (AUFNR), step UUID (SOG_UUID), signature type (TYPE = gc_perform_signature), performer (SIGN_ID), system date/time for the creation and signature timestamps, and destination name (DEST_NAME).

3. Database Insertion

a. The signature record is inserted into /SMPL/MBR_OR_SIG, creating a permanent audit trail linking the performer to the execution of the MBR step. A database error triggers a system exception to ensure data consistency.

4. Output Assignment

a. The function does not return explicit output parameters; its outcome is the successful insertion of a Perform signature record, which downstream logic (e.g. /SMPL/MBR_DEP_CHECK_ACTIVE) consumes.

Additional Notes

•  Captures performer identification, system date, and time automatically at the moment of signing, creating a permanent and auditable MBR signature record.

•  Error Handling: USER_INITIAL (performer ID missing); USER_NOT_SIGNED_INTO_SIG_TAB (performer not authorized); database insert failure raised by the system.

•  Dependencies: table /SMPL/MBR_OR_SIG; constant gc_perform_signature; system fields SY-DATUM/SY-UZEIT.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR step can be activated:

•  IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

•  IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

•  IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

•  IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

•  IM_PLANT (WERKS_D) – Plant code for process context.

b. The function validates that IM_CONTROL_RECIPE is not initial; if it is, CONTROL_RECIPE_INITIAL is raised. Internal data is initialized and routine LOAD_INTERNAL_DATA populates the order dependency buffers.

2. Signature Data Retrieval

a. Signature records for the process order are retrieved from /SMPL/MBR_OR_SIG (AUFNR = IM_AUFNR) into internal table lt_ord_sigs for evaluation.

3. Dependency Evaluation

a. The function loops through dependencies where TARGET_UUID = IM_STEP_UUID and DEP_RULE is a Perform-Before-Start or Verify-Before-Start rule:

•  For PERFORM BEFORE START, it checks whether the prerequisite step has a pending perform signature.

•  For VERIFY BEFORE START, it checks whether the prerequisite step has a pending verify signature.

b. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive); if all prerequisites are complete, it is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in EV_ACTIVE (active = all prerequisites complete; inactive = at least one prerequisite incomplete).

Additional Notes

•  Evaluates Perform Before Start and Verify Before Start dependency rules using live signature data from /SMPL/MBR_OR_SIG.

•  Error Handling: raises CONTROL_RECIPE_INITIAL if the control recipe is missing; handles empty dependency/signature data without runtime errors.

•  Activation status is recalculated each time the function is called to reflect live signature data.

## Configuration Specification

- N/A

## Test Scenarios

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: Solution Summary - Data Recording | Process Order created and Released containing SMPL: Solution Summary - Data Recording |  |
| 2 | Open the EBR, and navigate to SMPL: TFF Swap | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: TFF Swap is visible | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: TFF Swap is visible |  |
| 3 | The operator inputs the sample ID type. | Text value accepted. | Text value accepted. |  |
| 4 | The operator executed the Get Time Button. | The system accurately captures the system time. | The system accurately captures the system time. |  |
| 5 | The operator confirms whether DLIMS has been updated and executes the Performed By signature. | Signature is accepted | Signature is accepted |  |
| 6 | The operator inputs the three numeric pressure values and executes the calculation function. | A deviation popup is triggered if the pressure value exceeds 1.5 and is accepted if below 1.5 psi. | A deviation popup is triggered if the pressure value exceeds 1.5 and is accepted if below 1.5 psi. |  |
| 7 | The operator presses the Get Start Time at the beginning of the filter swap and Get End Time when the swap is complete. | The system accurately captures the system times. | The system accurately captures the system times. |  |
| 8 | The operator inputs the sample ID type. | Text value accepted. | Text value accepted. |  |
| 9 | The operator executes the Get Time Button as part of the post swap sample. | The system accurately captures the system time. | The system accurately captures the system time. |  |
| 10 | The operator confirms whether DLIMS has been updated and executes the Performed By signature. | Signature is accepted | Signature is accepted |  |
| 11 | Enter a username in the “Witnessed By” field then enter the password in the pop-up window. | Signature is accepted | Signature is accepted |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.1 | Initial Document (Functional requirements and Technical specification) | Frieda van der Merwe | 19-05-2026 |