V 1.0

May 2026

XStep Design Specification

SMPL: Viable Cell Density

Table of Contents

Right-click and choose "Update Field" to build the table of contents.

# Purpose

The purpose of this document is to outline the design and functional requirements for the XStep “SMPL: Viable Cell Density.”

The SMPL: Viable Cell Density XStep is used to record viable cell density and viability percentage results during batch execution. The XStep captures cell density values, viability percentage, configured minimum and maximum limits where applicable, unit of measure, operator signature, and witness signature where required.

# Overview

The SMPL: Viable Cell Density XStep allows operators to record viable cell density and viability percentage in the EBR.

The XStep includes an instruction section that provides operator guidance for viable cell density and viability review. The instructions indicate that when the viable cell density is greater than or equal to 14.60 x 10^6 cells/mL, the operator should prepare to add medium. The instructions also identify scenarios where supervisor/designee contact or comment entry may be required.

The XStep includes a Viability Table where the operator enters the Viable Cell Density and Viability % values. The system displays the applicable Min Cell Density, Max Cell Density, Min Viability %, and UoM values based on the MBR configuration.

The Min Cell Density and Max Cell Density fields are flexible. The system will only validate against Min Cell Density if a minimum value is configured. The system will only validate against Max Cell Density if a maximum value is configured. If either value is not configured, the applicable field will have the value of “N/A” and that validation is skipped.

The Min Viability % field is populated from an MBR parameter. The entered Viability % value is validated against the configured Min Viability %. If the entered viability percentage is below the configured minimum, a validation popup will occur and allow the operator to record a comment before accepting the value.

If the entered Viable Cell Density is outside the configured min/max range, the Witnessed By field is required so a supervisor/designee can accept or discard the entered value.

# Reasons for developing

The process step SMPL: Viable Cell Density was developed to record viable cell density and viability results in the EBR in a consistent and traceable way.

The main goals of this XStep are to:

- Capture viable cell density results.

- Capture viability percentage results.

- Display configured minimum and maximum viable cell density limits where applicable.

- Display configured minimum viability percentage from the MBR.

- Validate entered values against configured limits.

- Allow comment entry when viability is below the configured minimum.

- Require witness review when viable cell density is outside the configured acceptable range.

- Capture operator and witness signatures where required.

# Authorization

N/A

# Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | Master data has been set up correctly. |
| 2 | Min Cell Density may be configured as an MBR parameter, but is not always required. |
| 3 | Max Cell Density may be configured as an MBR parameter, but is not always required. |
| 4 | If Min Cell Density is not configured, the Min Cell Density field remains blank and minimum VCD validation is not performed. |
| 5 | If Max Cell Density is not configured, the Max Cell Density field remains blank and maximum VCD validation is not performed. |
| 6 | Min Viability % is configured as an MBR parameter and displayed in the XStep. |
| 7 | The operator enters the Viable Cell Density and Viability % values. |
| 8 | The UoM value is available and displayed in the XStep. |
| 9 | Validation logic is available to check Viable Cell Density only against configured min and/or max values. |
| 10 | Validation logic is available to check Viability % against the configured Min Viability %. |
| 11 | Comment entry is available when validation requires operator justification. |

# Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Viable Cell Density is blank | Validation triggers stating that Viable Cell Density must be entered. |
| Viability % is blank | Validation triggers stating that Viability % must be entered. |
| Min Cell Density is configured and Viable Cell Density is less than Min Cell Density | Validation popup triggers and requires supervisor/designee acceptance or discard, based on configuration. |
| Max Cell Density is configured and Viable Cell Density is greater than Max Cell Density | Validation popup triggers and requires supervisor/designee acceptance or discard, based on configuration. |
| Min Cell Density is not configured | Minimum VCD validation is not performed and the Min Cell Density field is set to “N/A”. |
| Max Cell Density is not configured | Maximum VCD validation is not performed and the Max Cell Density field is set to “N/A”.. |
| Min Viability % is configured and Viability % is less than Min Viability % | Validation popup triggers and allows the operator to record a comment before accepting the value. |
| Viable Cell Density is greater than 29.2 x 10^6 cells/mL | Validation popup triggers for supervisor/designee review or comment, where applicable. |

# XStep Layout Design

# Function Module(s)

Function: /SMPL/PPPI_FM_MIN_MAX

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| I_MIN | F | Floating Point Number (minimum) |
| I_MAX | F | Floating Point Number (maximum) |
| I_VALUE | F | Floating Point Number (value to check) |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_OUT_OF_RANGE | Entered value falls outside the defined range |
| MIN_GREATER_THAN_MAX | Minimum value is greater than or equal to the maximum |

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_IDX | INT4 | XStep Step Index Number |
| IM_STEP | CHAR30 | XStep Header |
| IM_SIGTYPE | CHAR20 | Signature Type |
| IM_USERID | XUBNAME | User Name |
| IM_DB_UP_DOWN | CHAR1 | Done By Up/Down |
| IM_VALIDATION_REQ | CHAR3 | Validation Required |
| IM_STEP_UUID | ZSMPL_DE_SMPL_SOG_UUID | XStep SOG UUID |

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_STEP_UUID | /SMPL/STR_DE_SOG_UUID | XStep SOG UUID |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| TABLE_LOCK_NOT_POSSIBLE | Structure table could not be locked |

Function: /SMPL/PPPI_FM_SIG_VALIDATION

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | XStep SOG UUID |
| IM_CB_VALIDATE | CHAR1 | Checked By Validation Flag |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User ID is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not registered on PI sheet |
| USER_SIGNED_DONE_BY | User already signed the Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed the Perform By |
| XSTEP_CONFIG | XStep / structure configuration error |

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | SYSUUID_22 | 16 Byte UUID in 22 Characters |
| IM_CB_VALIDATE | CHAR1 | Checked By Validation Flag |
| IM_PLANT | WERKS_D | Plant |
| IV_MN | F | Minimum Value |
| IV_MN_DAY | I | Minimum Day Offset |
| IV_MN_TIM | TIMS | Minimum Time |
| IV_MT | F | Target Value |
| IV_MT_DY | I | Target Day Offset |
| IV_MT_TM | TIMS | Target Time |
| IV_MX | F | Maximum Value |
| IV_MX_DAY | I | Maximum Day Offset |
| IV_MX_TIM | TIMS | Maximum Time |
| IV_INSIDE_RANGE | CHAR1 | Inside Range Indicator (default 1) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_SIG_REQ | CHAR1 | Signature Required Indicator |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_NOT_SET | Value to validate not set |
| RANGE_BELOW_MIN | Value below minimum |
| RANGE_ABOVE_MAX | Value above maximum |
| RANGE_INSIDE_RANGE | Value inside range |
| DOESNT_WORK_FOR_TIME | Not supported for time values |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User ID is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not registered on PI sheet |
| USER_SIGNED_DONE_BY | User already signed the Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed the Perform By |
| UNEXPECTED_ERROR | Unexpected error |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

# Pseudocode

Function: /SMPL/PPPI_FM_MIN_MAX

1. Purpose

Validates a numeric value against an optional minimum and/or maximum limit and raises an exception (after displaying an explanatory message) when the value falls outside the defined range, or when the configured minimum and maximum are inconsistent. Used to enforce the K setpoint and other numeric entry limits.

2. Input Parameters

•  I_MIN – optional lower bound.

•  I_MAX – optional upper bound.

•  I_VALUE – the value to validate.

3. Processing Logic

a. The min, max and value are copied into packed decimals (3 decimal places) for use in the operator messages.

b. If BOTH I_MIN and I_MAX are supplied:

•  If I_MIN >= I_MAX, display "The Minimum value is set larger than the Maximum value" and RAISE min_greater_than_max.

•  Else if I_VALUE lies between I_MIN and I_MAX (inclusive), the value is accepted (no exception).

•  Otherwise display a message stating the entered value should be between the min and max, and RAISE value_out_of_range.

c. If ONLY I_MAX is supplied: accept when I_VALUE <= I_MAX; otherwise display a message and RAISE value_out_of_range.

d. If ONLY I_MIN is supplied: accept when I_VALUE >= I_MIN; otherwise display a message and RAISE value_out_of_range.

e. If neither bound is supplied, no validation is performed.

Output

•  No export parameters; a successful validation is indicated by no exception being raised.

Additional Notes

Error Handling:

•  VALUE_OUT_OF_RANGE – the value is outside the configured bound(s); an explanatory message naming the entered value and the limit(s) is shown first.

•  MIN_GREATER_THAN_MAX – the supplied minimum is greater than or equal to the supplied maximum.

Dependencies:

•  None – no tables or external function modules; messages are issued as type 'S' displayed like an error.

Assumptions:

•  Both bounds are optional; when both are blank the range check is intentionally skipped.

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

•  Signature Type Identification

•  Assign the incoming signature type (IM_SIGTYPE) to local variable lv_sigtype.

•  A CASE structure is used to determine behavior based on the type: ‘ ’ (Done By), ‘C’ (Checked By), ‘O’ (Validation Flag Only).

•  Done By Signature (lv_sigtype = ‘ ’)

•  a. Optional Step Name Update – if IM_STEP is provided, update the XStep header in table /SMPL/PPPI_PISST.

b. Determine Position for Signature Insertion

•  If IM_DB_UP_DOWN is empty: insert the new signature above the current item by fetching MIN(UNIQUE_SEQID) and inserting one row before it.

•  If IM_DB_UP_DOWN = ‘B’: insert the new signature below the current item by fetching MAX(UNIQUE_SEQID) and inserting one row after it.

•  In both cases use IM_USERID as the signer (SIGN_ID), the signature type description from TEXT-001, and insert a new row into /SMPL/PPPI_PISST.

•  Checked By Signature (lv_sigtype = ‘C’)

•  a. Get Signature Start Point – select the UNIQUE_SEQID of the “Checked By” step matching the given input.

b. Determine Signature End Point

•  Fetch all prior ‘C’ type signatures for the same control recipe and determine the highest UNIQUE_SEQID lower than the start point to use as the end.

c. Update Checked By Step – set the SIGN_ID and mark the step as COMPLETED = ‘X’.

d. Complete All Steps Between Start and End

•  Loop over all steps with UNIQUE_SEQID between the start and end boundaries and mark each row as COMPLETED = ‘X’; update the database using MODIFY.

•  Validation Flag Update (lv_sigtype = ‘O’)

•  a. Determine Validation Value – if IM_VALIDATION_REQ = ‘2’, set to ‘No’, else ‘Yes’.

b. Update the VALIDATION_REQUIRED field for the current control recipe, phase, and step index in /SMPL/PPPI_PISST.

Additional Notes:

•  Purpose: dynamically inserts or updates signature records in /SMPL/PPPI_PISST depending on the role of the signer (Done By, Checked By, or Validator).

•  Supports step name overrides and UUID tracking, and allows insertion before or after specific records using IM_DB_UP_DOWN.

•  Ensures Checked By signatures cascade completion to preceding steps and accommodates flexible validation logic.

•  Error Handling: no explicit RAISE statements; relies on CHECK and SY-SUBRC evaluations to exit silently if lookup conditions fail.

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

1. Purpose

Builds (once per control recipe) the EBR signature structure stored in /SMPL/PPPI_PISST that drives the Signature Table, then maintains the “Checked By” header/type for the current step. The structure lists every signature step for the order in sequence with a unique index used for ordering and dependency checks.

2. Input Parameters

•  IM_LABEL_ID – Signature label (‘C’ = Checked By).

•  IM_STEP_NAME – Optional XStep header text to display.

•  IM_STEP_INDEX / IM_STEP_UUID – Step sequence index and SOG UUID.

•  IM_CONTROL_RECIPE – Control recipe (PI sheet) number.

•  IM_PHASE, IM_PO_NR, IM_PLANT – Phase, process order and plant context.

3. Structure Build (first execution only)

a. SELECT COUNT(*) from /SMPL/PPPI_PISST for the control recipe + order. If rows already exist the build is skipped.

b. When empty:

•  Read master recipe (PLNNR/PLNAL from AFKO) and control recipe destination (PHSEQ from COCH).

•  Determine whether the order is Production or Process order (AFKO-PLNAW) to pick the SOG data type.

•  Retrieve the most recent SiMPL SOG data via /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA and filter to the current control recipe destination.

•  Lock /SMPL/PPPI_PISST (ENQUEUE_E_TABLE); on failure raise TABLE_LOCK_NOT_POSSIBLE.

•  Insert one /SMPL/PPPI_PISST row per step (step index, name, UUID, master recipe, control recipe, order, date, phase, VALIDATION_REQUIRED = ‘Yes’); then unlock the table.

•  Re-read the structure, sort by phase + step sequence, and assign a UNIQUE_SEQID in increments of 1000.

4. Checked By Maintenance

a. If IM_LABEL_ID = ‘C’, update the step’s TYPE to the “Checked By” text; if IM_STEP_NAME is supplied also update the displayed step name, keyed by step index + control recipe + phase.

5. Output

No export parameters; the persisted /SMPL/PPPI_PISST structure is the result.

Additional Notes

Error Handling:

•  TABLE_LOCK_NOT_POSSIBLE – the structure table could not be locked for the initial build.

Dependencies:

•  Tables /SMPL/PPPI_PISST, AFKO, COCH; class /SMPL/MBR_CL_DB_SERVICE; lock objects ENQUEUE/DEQUEUE_E_TABLE.

Assumptions:

•  SOG/dependency data has been loaded for the order; the first step to execute builds the structure for all subsequent steps.

Function: /SMPL/PPPI_FM_SIG_VALIDATION

1. Purpose

Validates a Done By / Checked By electronic signature before it is committed. It enforces user validity, predecessor-step completion, signature-table registration, the rule that a Checked By user cannot have performed the Done By in the same range, and a final-step “mass dependency” completion check. It also delegates to the MBR dependency validation FM.

2. Input Parameters

•  IM_LABEL_ID – Signature label (‘C’ Checked By, otherwise Done By/Perform).

•  IM_STEP_INDEX / IM_STEP_UUID – Step identifiers.

•  IM_CONTROL_RECIPE, IM_PHASE, IM_PO_NR – Control recipe, phase and process order.

•  IM_USERID – User performing the signature.

•  IM_CB_VALIDATE – Flag to run the Checked-By-vs-Done-By conflict check.

•  IM_PLANT – Plant.

3. Basic Validation

a. If IM_CONTROL_RECIPE is initial → raise CONTROL_RECIPE_INITIAL.

b. If IM_USERID is initial → raise USER_INITIAL.

4. Predecessor / Configuration Checks

a. Read master recipe (AFKO) and control recipe destination (COCH). If either is missing → raise XSTEP_CONFIG.

b. Determine Production vs Process order and retrieve dependency data via /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA.

c. Filter dependencies to the current destination and step UUID and check predecessor completion in /SMPL/PPPI_PISST (predecessor enforcement is delegated to MBR).

5. Signature Table Validation

a. The user must be registered for the PI sheet in /SMPL/PPPI_PIBSI; otherwise → raise USER_NOT_SIGNED_INTO_SIG_TAB.

6. Checked By vs Done By Conflict

a. When IM_LABEL_ID = ‘C’ and IM_CB_VALIDATE is set, the unique sequence range of the current Checked By is determined and the structure is searched for a Done By (‘Perform’) signature by the same user within that range; if found → raise USER_SIGNED_PERF_BY.

7. Final Step Mass Dependency Check

a. The last step (max UNIQUE_SEQID) of the control recipe is read. If the current signature is that final step, all other required (validation required = ‘Yes’, Checked By) steps must be completed; otherwise → raise REQ_EBR_STEPS_NOT_COMPLETED.

8. MBR Dependency Validation

a. /SMPL/MBR_DEP_VALI_SIGN is called with a Perform/Verify signature type derived from IM_LABEL_ID; its exceptions are mapped back to this FM’s exceptions.

Additional Notes

Error Handling:

•  Raises CONTROL_RECIPE_INITIAL, USER_INITIAL, USER_NOT_SIGNED_INTO_SIG_TAB, USER_SIGNED_PERF_BY, REQ_EBR_STEPS_NOT_COMPLETED and XSTEP_CONFIG as described above.

Dependencies:

•  Tables /SMPL/PPPI_PISST, /SMPL/PPPI_PIBSI, AFKO, COCH; FM /SMPL/MBR_DEP_VALI_SIGN; class /SMPL/MBR_CL_DB_SERVICE.

Assumptions:

•  The signature structure has been built (see /SMPL/PPPI_FM_SIG_POPULATE_CB) and the user is registered on the PI sheet.

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

1. Purpose

Wrapper validation used when a recorded value must fall within a configured range and a supervisor signature is conditionally required. It first checks the value against minimum/target/maximum limits to decide whether a signature is required, then runs the standard signature validation.

2. Input Parameters

•  IM_LABEL_ID, IM_STEP_INDEX, IM_STEP_UUID, IM_CONTROL_RECIPE, IM_PHASE, IM_PO_NR, IM_USERID, IM_CB_VALIDATE, IM_PLANT – Standard signature-validation context (see /SMPL/PPPI_FM_SIG_VALIDATION).

•  IV_MN / IV_MT / IV_MX – Minimum, target and maximum numeric limits.

•  IV_MN_DAY/IV_MN_TIM, IV_MT_DY/IV_MT_TM, IV_MX_DAY/IV_MX_TIM – Optional day-offset and time limits for date/time ranges.

•  IV_INSIDE_RANGE – Indicator (default ‘1’) controlling whether being inside or outside the range requires the signature.

3. Range Check

a. /SMPL/PPPI_FM_VALI_SUPE_CHECK is called with the limit parameters; it returns EV_SIG_REQ (signature required) and may raise range exceptions (VALUE_NOT_SET, RANGE_BELOW_MIN, RANGE_ABOVE_MAX, RANGE_INSIDE_RANGE, DOESNT_WORK_FOR_TIME), which are re-raised to the caller.

4. Signature Validation

a. /SMPL/PPPI_FM_SIG_VALIDATION is called with the signature context; its exceptions (control recipe initial, user initial, user not signed, user signed Done By / Perform By, predecessor / required steps not completed) are mapped and re-raised.

5. Output

•  EV_SIG_REQ – ‘X’ when a supervisor signature is required for the recorded value, otherwise blank.

Additional Notes

•  Combines range evaluation and signature validation so a supervisor signature is only enforced when the value falls in the configured condition.

Error Handling:

•  Re-raises the range and signature exceptions listed above; UNEXPECTED_ERROR for any other failure.

Dependencies:

•  FMs /SMPL/PPPI_FM_VALI_SUPE_CHECK and /SMPL/PPPI_FM_SIG_VALIDATION.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.

b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

•  If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

•  If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

•  This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps.

•  The use of constants (gc_yes, gc_no, gc_inside_range) aligns the logic with broader application settings or standards.

•  Input is optional, so defaulting behavior can be configured outside this function.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR step can be activated:

•  IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

•  IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

•  IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

•  IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

•  IM_PLANT (WERKS_D) – Plant code for process context.

b. The function validates that IM_CONTROL_RECIPE is not initial; if it is, CONTROL_RECIPE_INITIAL is raised. Internal data is initialized and routine LOAD_INTERNAL_DATA populates the order dependency buffers.

2. Signature Data Retrieval

a. Signature records for the process order are retrieved from /SMPL/MBR_OR_SIG (AUFNR = IM_AUFNR) into internal table lt_ord_sigs for evaluation.

3. Dependency Evaluation

a. The function loops through dependencies where TARGET_UUID = IM_STEP_UUID and DEP_RULE is a Perform-Before-Start or Verify-Before-Start rule:

•  For PERFORM BEFORE START, it checks whether the prerequisite step has a pending perform signature.

•  For VERIFY BEFORE START, it checks whether the prerequisite step has a pending verify signature.

b. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive); if all prerequisites are complete, it is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in EV_ACTIVE (active = all prerequisites complete; inactive = at least one prerequisite incomplete).

Additional Notes

•  Evaluates Perform Before Start and Verify Before Start dependency rules using live signature data from /SMPL/MBR_OR_SIG.

•  Error Handling: raises CONTROL_RECIPE_INITIAL if the control recipe is missing; handles empty dependency/signature data without runtime errors.

•  Activation status is recalculated each time the function is called to reflect live signature data.

# Configuration Specifications

N/A

# Test Scenarios

| # | Scenario | Type | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- | --- |
| ID |  |  |  |  |  |
| 1 | Create and release a process order with the pre-configured SiMPL Master Recipe and relevant Production Version. | Positive | Process Order is created and released successfully containing SMPL: Viable Cell Density. |  |  |
| 2 | Open the EBR for the released process order and navigate to the phase containing SMPL: Viable Cell Density. | Positive | The EBR opens successfully and the SMPL: Viable Cell Density XStep is visible in the applicable phase. |  |  |
| 3 | Review the instruction text displayed in the XStep. | Positive | Instructions for viable cell density, viability %, supervisor/designee contact, and witness requirements are displayed. |  |  |
| 4 | Confirm that Min Cell Density, Max Cell Density, Min Viability %, and UoM fields are displayed according to configuration. | Positive | Configured output values display correctly. Any unconfigured Min/Max Cell Density field remains blank. |  |  |
| 5 | Configure both Min Cell Density and Max Cell Density, then enter a Viable Cell Density value within range. | Positive | Viable Cell Density is accepted without validation popup. |  |  |
| 6 | Configure only Min Cell Density, then enter a Viable Cell Density value above the minimum. | Positive | Viable Cell Density is accepted and maximum validation is not performed. |  |  |
| 7 | Configure only Max Cell Density, then enter a Viable Cell Density value below the maximum. | Positive | Viable Cell Density is accepted and minimum validation is not performed. |  |  |
| 8 | Configure no Min Cell Density and no Max Cell Density, then enter a Viable Cell Density value. | Positive | Viable Cell Density is accepted without min/max validation. |  |  |
| 9 | Configure Min Cell Density and leave Max Cell Density blank. | Positive | Min Cell Density displays, Max Cell Density remains blank, and only minimum validation is active. |  |  |
| 10 | Configure Max Cell Density and leave Min Cell Density blank. | Positive | Max Cell Density displays, Min Cell Density remains blank, and only maximum validation is active. |  |  |
| 11 | Enter a Viable Cell Density value below the configured Min Cell Density. | Negative | Validation popup is displayed and Witnessed By is required for supervisor/designee acceptance or discard. |  |  |
| 12 | Enter a Viable Cell Density value above the configured Max Cell Density. | Negative | Validation popup is displayed and Witnessed By is required for supervisor/designee acceptance or discard. |  |  |
| 13 | Enter a Viable Cell Density value greater than 29.2 x 10^6 cells/mL. | Negative | Validation popup is displayed for supervisor/designee review or comment, where applicable. |  |  |
| 14 | Configure Min Viability % and enter a Viability % greater than or equal to the configured value. | Positive | Viability % is accepted without validation popup. |  |  |
| 15 | Configure Min Viability % and enter a Viability % below the configured value. | Negative | Validation popup appears and allows the operator to record a comment before accepting the value. |  |  |
| 16 | Leave Viable Cell Density blank and attempt to complete the Performed By signature. | Negative | Validation triggers stating that Viable Cell Density must be entered. |  |  |
| 17 | Leave Viability % blank and attempt to complete the Performed By signature. | Negative | Validation triggers stating that Viability % must be entered. |  |  |
| 18 | Complete all required fields with valid values and sign the Performed By field. | Positive | Performed By signature is captured successfully and the XStep can proceed. |  |  |

# Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

# Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| SiMPL version | Description of revisions made. Never “Improved language” or anything indicating the previous version was bad/ wrong. Only show versions previously delivered to the client. | First and Last name of Author who made revision | Date the revision was made. MM-DD-YYYY |
| 4.2 | Initial Document | Carlos Redekopp | 05-13-2026 |