V 1.0

May 2026

XStep Design Specification

SMPL: Rocker Bag Daily Sample

Table of Contents

Right-click and choose Update Field to build the table of contents.

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: Rocker Bag Daily Sampling”.

## Overview

This XStep facilitates the routine parameter monitoring and daily biological sampling of the cell culture when grown inside the 50 L Rocker bags. The process step ensures that critical environmental process parameters remain tightly controlled within defined action limits and tracks the growth velocity of the culture to determine if the criteria for a bioreactor transfer are successfully met.

Standardized recording windows ensure strict data accuracy, batch integrity, and real-time visibility into required cell split and inoculation timelines.

The XStep contains several main input parameters:

- IV_HEADER - This can be changed in SiMPL MBR to display specific header text for the XStep

- IV_INSTR - This can be changed in SiMPL MBR to display a specific set of instructions for the XStep that will be displayed below the header

The table header is configurable in SiMPL MBR.

The following fields are auto populated in the Xstep:

- “#” - Output numeric field which auto-populates sequentially with each add table line. The first output value is “1”

The operator inputs the following fields and executes the “Get Time” button:

- “Working Day” - Text Input

- Get Time - Function Button

The button is linked to the function module /SMPL/PPPI_FM_GET_DATE_TIME to timestamp the sample.

- “Sampling Time” - Output Timestamp

The operator enters the following numeric input values:

- “Temp (°C)” - Numeric Input with range validation check as authored from MBR.

- “CO2 (%)” - Numeric Input with range validation check as authored from MBR

- “Speed (RPM)” - Numeric Input with range validation check as authored from MBR

- “Angle (°)” - Numeric Input with range validation check as authored from MBR

- “Airflow (mL/min) ” - Numeric Input with range validation check as authored from MBR

The operator executes the ¨Equipment Select” button:

- Equipment Select - Function Button

The button is linked to the function module ZSMPL_FM_GET_ASSIGNED_EQUI_EBR that verifies the equipment number against the Equipment Master and retrieves the following information:

- “Vi-CELL Blu ID” - Input text

- “Vi-CELL Blu Descr” - Output text

The operator enters the following numeric input values:

- “VCD (x 106 cells/ml)” - Numeric Input

- “Viability (%)” - Numeric Input

- “Is Transfer Required?” - Automatic output of Yes when VCD ⪈ 3.1 x 106 cells/ml and automatic output of No when VCD ⪇ 3.1 x 106 cells/ml.

The operator signs the Performed By signature in the table line and the supervisor signs the Witnessed By signature to close out the table.

Functional Design

## Reasons for developing

The SMPL: Rocker Bag Daily Sampling XStep was developed to provide a standardized, validated system for documenting ongoing cell culture growth kinetics within the single-use rocker arrays. Digitizing this process step ensures rigorous control limit compliance, enforces independent double verification of critical calculation logic, and guarantees flawless cross-departmental traceability before expanding downstream to the large-scale production bioreactor.

## Authorization

The Signature Strategies SS000201 - SiMPL Done By and SS000202 - SiMPL Checked By are created and assigned to the Authorization Groups AGDS0001 PI step Done By SS and AGDS0002 PI step Checked By SS. This ensures that at least two users are required to successfully sign off the process step. The first user signs off in the Performed By field and a second user signs the Witnessed By field.

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | User signing signature fields is an approved user in the system with the correct access configuration |
| 2 | Master Data has been set up correctly |
| 3 | The function modules are available and return the correct system time. |
| 4 | The function module can successfully pull the data into the output fields |

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| User Signature rejected due to an incorrect username or password | User accepts error message and re-enter the correct username and password |
| Equipment item not found | Check Equipment Master |

## XStep Layout Design

*EBR screenshot will be inserted of developed xstep

## Function Module(s)

Function: ZSMPL_FM_GET_ASSIGNED_EQUI_EBR

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_ORDER | AUFNR | Order Number |
| IM_EQUIP_TYPE | EQTYP | Equipment Category |
| IM_ALLOW_NA | BOOLE_D | Boolean variable (allow N/A) |
| IM_VORNR | VORNR | Operation/Activity Number |
| IM_OBJ_TYP | EQART | Object Type of Technical Object |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_EQUI | CHAR30 | Equipment ID |
| EX_EQUI_TXT | CHAR30 | Equipment Description |

Exceptions:

| Exception | Short Text |
| --- | --- |
| INTERNAL_ERROR | Internal processing error |

Function: ZSMPL_FM_RANGE_YES_NO

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_NUM | F | Value to test |
| IM_MIN | F | Minimum bound (optional) |
| IM_MAX | F | Maximum bound (optional) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_RESULT | CHAR10 | Result (Yes/No) |

Function: /SMPL/PPPI_FM_GET_DATE_TIME

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_COMPLETE | CHAR1 | Character 1 |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_DATE | DATS | Date |
| EV_TIME | TIMS | Time |
| EV_DATE_DEFAULT | DATS | Date |
| EV_TIME_DEFAULT | TIMS | Time |

Exceptions:

| Exception | Short Text |
| --- | --- |
| PERIOD_COMPLETED | Raised/handled when the period has already been completed |

Function: /SMPL/PPPI_FM_SET_LINE_IDX

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| ET_INDEX_T1 | /SMPL/STR_TT_VORNR | Numbered index – copy 1 |
| ET_INDEX_T2 | /SMPL/STR_TT_VORNR | Numbered index – copy 2 |

Changing Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| CT_INDEX | /SMPL/STR_TT_VORNR | Table line index column |

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_IDX | INT4 | XStep Step Index Number |
| IM_STEP | CHAR30 | XStep Header |
| IM_SIGTYPE | CHAR20 | Signature Type |
| IM_USERID | XUBNAME | User Name |
| IM_DB_UP_DOWN | CHAR1 | Done By Up/Down |
| IM_VALIDATION_REQ | CHAR3 | Validation Required |
| IM_STEP_UUID | ZSMPL_DE_SMPL_SOG_UUID | XStep SOG UUID |

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Signature Label ID (D/C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | SYSUUID_22 | 16 Byte UUID in 22 Characters |
| IM_CB_VALIDATE | CHAR1 | Checked By Validation Flag |
| IM_PLANT | WERKS_D | Plant |
| IV_MN | F | Minimum Value |
| IV_MN_DAY | I | Minimum Day Offset |
| IV_MN_TIM | TIMS | Minimum Time |
| IV_MT | F | Target Value |
| IV_MT_DY | I | Target Day Offset |
| IV_MT_TM | TIMS | Target Time |
| IV_MX | F | Maximum Value |
| IV_MX_DAY | I | Maximum Day Offset |
| IV_MX_TIM | TIMS | Maximum Time |
| IV_INSIDE_RANGE | CHAR1 | Inside Range Indicator (default 1) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_SIG_REQ | CHAR1 | Signature Required Indicator |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_NOT_SET | Value to validate not set |
| RANGE_BELOW_MIN | Value below minimum |
| RANGE_ABOVE_MAX | Value above maximum |
| RANGE_INSIDE_RANGE | Value inside range |
| DOESNT_WORK_FOR_TIME | Not supported for time values |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User ID is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not registered on PI sheet |
| USER_SIGNED_DONE_BY | User already signed the Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed the Perform By |
| UNEXPECTED_ERROR | Unexpected error |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_ADD_PERFORM

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_AUFNR | AUFNR | N/A |
| IM_USERID | XUBNAME | N/A |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | N/A |
| IM_DEST_NAME | /SMPL/MBR_CMX_XS_DST_NAME | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

## Pseudocode

Function: ZSMPL_FM_GET_ASSIGNED_EQUI_EBR

1. Purpose

Returns the equipment (e.g. welder, room, or additional equipment) currently assigned to the active process order from the SAP Equipment Master / SiMPL equipment logbook. If a single qualifying equipment is assigned it is returned automatically; if multiple are assigned the operator is prompted with a selection pop-up; if none are assigned a warning is displayed.

2. Input Parameters

•  IM_ORDER – Process order number used to look up assigned equipment.

•  IM_EQUIP_TYPE – Equipment category (EQTYP), e.g. ‘R’ room or ‘A’ additional equipment.

•  IM_ALLOW_NA – Optional flag; when set, an ‘N/A’ entry is offered so the operator can record that no equipment applies.

•  IM_VORNR – Optional operation/phase number to restrict the search to an order-phase combination.

•  IM_OBJ_TYP – Optional technical object type (EQART) for further filtering.

3. Status Determination

a. The active “IN USE” and “IN USE - CLEANED” equipment statuses are resolved dynamically from the SiMPL status configuration table /SMPL/ELB_EQSTAT via subroutine GET_EQ_STAT_BY_CLASS (status classes ‘U1’ and ‘U2’), allowing status text translation.

4. Equipment Retrieval

a. Export parameters and working tables are cleared. If IM_ALLOW_NA is set, an ‘N/A’ row is appended to the candidate list.

b. Equipment is selected from /SMPL/ELB_EQUIP joined to the Equipment Master (EQUI) and description table (EQKT) where:

•  order number = IM_ORDER

•  (optional) activity = IM_VORNR

•  equipment category = IM_EQUIP_TYPE

•  the confirmation/update signature has been completed (UPDAT_CB <> space)

•  (optional) object type = IM_OBJ_TYP

c. Adjacent duplicates are removed (latest entry per equipment kept), and rows whose status is neither “IN USE” nor “IN USE - CLEANED” are deleted.

5. Result Handling

a. Single match: the equipment ID and description are returned directly.

b. No match (or only the N/A row): a configured SiMPL enhancement-framework class/method is invoked and a warning pop-up (“No room is currently assigned…” / “…has not been assigned…”) is shown; the export values are set to ‘N/A’.

c. Multiple matches: for the SiMPL EBR (websocket) context a generic value-help dialog is built (/SMPL/EBR_CL_GENERIC_VAL_HELP); for the SAP GUI context an ALV pop-up (CL_SALV_TABLE) lists the equipment for single selection. The selected row is read.

6. Standardization Check

a. The selected equipment number is ALPHA-converted and passed to subroutine STANDARDIZATION_CHECK.

b. If the equipment is not standardized (‘S1’) or has not yet been standardized (‘S2’), a warning pop-up is displayed and the export values are reset to ‘N/A’.

7. Output

•  EX_EQUI – Selected equipment ID (leading zeros removed).

•  EX_EQUI_TXT – Equipment description.

Additional Notes

Key Features:

•  Supports automatic single-equipment return, multi-equipment pop-up selection, and N/A handling.

•  Dynamically resolves equipment status text for translation support.

•  Works in both the SiMPL EBR websocket UI and the classic SAP GUI.

Error Handling:

•  INTERNAL_ERROR is raised if a configured enhancement class/method cannot be called.

Dependencies:

•  Tables /SMPL/ELB_EQUIP, EQUI, EQKT, /SMPL/ELB_EQSTAT; classes /SMPL/EBR_CL_APC_WS, /SMPL/EBR_CL_GENERIC_VAL_HELP, /SMPL/EBR_CL_ENH_FWK.

Assumptions:

•  Equipment has been assigned to the order through the appropriate SiMPL assignment step with a completed confirmation signature.

Function: ZSMPL_FM_RANGE_YES_NO

1. Purpose

Returns "Yes" when a value falls within the supplied bound(s), otherwise "No". Used to drive a Yes/No column from a numeric reading and its configured limits.

2. Input Parameters

•  IM_NUM – the value being tested.

•  IM_MIN / IM_MAX – minimum and/or maximum bound (either is optional).

3. Processing Logic

a. Default the result to "No".

b. If both IM_MIN and IM_MAX are supplied, return "Yes" when IM_NUM is strictly between them.

c. If only IM_MIN is supplied, return "Yes" when IM_NUM is greater than it; if only IM_MAX, return "Yes" when IM_NUM is less than it.

d. If neither bound is supplied, the result stays "No".

Output

•  EX_RESULT – "Yes" or "No".

Additional Notes

Error Handling:

•  No exceptions are defined.

Dependencies:

•  None – pure comparison logic.

Assumptions:

•  Bounds are supplied as floats; a bound left initial (zero) is treated as not configured.

Function: /SMPL/PPPI_FM_GET_DATE_TIME

1. Purpose

Returns the current local date and time for use in a process instruction (XStep) entry, unless the period/XStep has already been completed, in which case it issues an information message and does nothing.

2. Input Parameters

•  IV_COMPLETE – completion flag; '1' means the period has already been completed.

Export Parameters

•  EV_DATE / EV_TIME – current local date and time.

•  EV_DATE_DEFAULT / EV_TIME_DEFAULT – default copies of the same date and time.

3. Processing Logic

a. If IV_COMPLETE = '1', issue information message 008 ('Period has been completed, remove completion before updating') and EXIT without setting any values.

b. Otherwise set EV_DATE = sy-datlo and EV_TIME = sy-timlo (current local date/time).

c. Copy those values into EV_DATE_DEFAULT and EV_TIME_DEFAULT.

Output

•  Current local date/time in both the working and default export fields.

Additional Notes

Error Handling:

•  PERIOD_COMPLETED – declared exception; in practice completion is signalled via information message 008 and an early EXIT rather than RAISE.

Dependencies:

•  System fields sy-datlo / sy-timlo; message class message 008; constant gc_type_i (message type 'I').

Assumptions:

•  IV_COMPLETE = '1' reliably indicates a completed period that must not be re-dated.

Function: /SMPL/PPPI_FM_SET_LINE_IDX

1. Purpose

Numbers each row of a table sequentially (1, 2, 3 …) and returns two copies of the numbered column for XSteps that drive more than one table from the same index.

2. Parameters

•  CT_INDEX – table line index column to number (changing).

•  ET_INDEX_T1 / ET_INDEX_T2 – copies of the numbered column for additional tables.

3. Processing Logic

a. Loop over CT_INDEX; set each row to its current row number (sy-tabix) and left-justify the value.

b. Copy the numbered CT_INDEX into ET_INDEX_T1 and ET_INDEX_T2.

Output

•  CT_INDEX (changing) plus ET_INDEX_T1 / ET_INDEX_T2 – the sequential row numbers.

Additional Notes

Error Handling:

•  No exceptions are defined.

Dependencies:

•  Table type /SMPL/STR_TT_VORNR; no database reads.

Assumptions:

•  The index column has one row per table row to be numbered.

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

•  Signature Type Identification

•  Assign the incoming signature type (IM_SIGTYPE) to local variable lv_sigtype.

•  A CASE structure is used to determine behavior based on the type: ‘ ’ (Done By), ‘C’ (Checked By), ‘O’ (Validation Flag Only).

•  Done By Signature (lv_sigtype = ‘ ’)

•  a. Optional Step Name Update – if IM_STEP is provided, update the XStep header in table /SMPL/PPPI_PISST.

b. Determine Position for Signature Insertion

•  If IM_DB_UP_DOWN is empty: insert the new signature above the current item by fetching MIN(UNIQUE_SEQID) and inserting one row before it.

•  If IM_DB_UP_DOWN = ‘B’: insert the new signature below the current item by fetching MAX(UNIQUE_SEQID) and inserting one row after it.

•  In both cases use IM_USERID as the signer (SIGN_ID), the signature type description from TEXT-001, and insert a new row into /SMPL/PPPI_PISST.

•  Checked By Signature (lv_sigtype = ‘C’)

•  a. Get Signature Start Point – select the UNIQUE_SEQID of the “Checked By” step matching the given input.

b. Determine Signature End Point

•  Fetch all prior ‘C’ type signatures for the same control recipe and determine the highest UNIQUE_SEQID lower than the start point to use as the end.

c. Update Checked By Step – set the SIGN_ID and mark the step as COMPLETED = ‘X’.

d. Complete All Steps Between Start and End

•  Loop over all steps with UNIQUE_SEQID between the start and end boundaries and mark each row as COMPLETED = ‘X’; update the database using MODIFY.

•  Validation Flag Update (lv_sigtype = ‘O’)

•  a. Determine Validation Value – if IM_VALIDATION_REQ = ‘2’, set to ‘No’, else ‘Yes’.

b. Update the VALIDATION_REQUIRED field for the current control recipe, phase, and step index in /SMPL/PPPI_PISST.

Additional Notes:

•  Purpose: dynamically inserts or updates signature records in /SMPL/PPPI_PISST depending on the role of the signer (Done By, Checked By, or Validator).

•  Supports step name overrides and UUID tracking, and allows insertion before or after specific records using IM_DB_UP_DOWN.

•  Ensures Checked By signatures cascade completion to preceding steps and accommodates flexible validation logic.

•  Error Handling: no explicit RAISE statements; relies on CHECK and SY-SUBRC evaluations to exit silently if lookup conditions fail.

Function: /SMPL/PPPI_FM_VALI_SUPE_SIG

1. Purpose

Wrapper validation used when a recorded value must fall within a configured range and a supervisor signature is conditionally required. It first checks the value against minimum/target/maximum limits to decide whether a signature is required, then runs the standard signature validation.

2. Input Parameters

•  IM_LABEL_ID, IM_STEP_INDEX, IM_STEP_UUID, IM_CONTROL_RECIPE, IM_PHASE, IM_PO_NR, IM_USERID, IM_CB_VALIDATE, IM_PLANT – Standard signature-validation context (see /SMPL/PPPI_FM_SIG_VALIDATION).

•  IV_MN / IV_MT / IV_MX – Minimum, target and maximum numeric limits.

•  IV_MN_DAY/IV_MN_TIM, IV_MT_DY/IV_MT_TM, IV_MX_DAY/IV_MX_TIM – Optional day-offset and time limits for date/time ranges.

•  IV_INSIDE_RANGE – Indicator (default ‘1’) controlling whether being inside or outside the range requires the signature.

3. Range Check

a. /SMPL/PPPI_FM_VALI_SUPE_CHECK is called with the limit parameters; it returns EV_SIG_REQ (signature required) and may raise range exceptions (VALUE_NOT_SET, RANGE_BELOW_MIN, RANGE_ABOVE_MAX, RANGE_INSIDE_RANGE, DOESNT_WORK_FOR_TIME), which are re-raised to the caller.

4. Signature Validation

a. /SMPL/PPPI_FM_SIG_VALIDATION is called with the signature context; its exceptions (control recipe initial, user initial, user not signed, user signed Done By / Perform By, predecessor / required steps not completed) are mapped and re-raised.

5. Output

•  EV_SIG_REQ – ‘X’ when a supervisor signature is required for the recorded value, otherwise blank.

Additional Notes

•  Combines range evaluation and signature validation so a supervisor signature is only enforced when the value falls in the configured condition.

Error Handling:

•  Re-raises the range and signature exceptions listed above; UNEXPECTED_ERROR for any other failure.

Dependencies:

•  FMs /SMPL/PPPI_FM_VALI_SUPE_CHECK and /SMPL/PPPI_FM_SIG_VALIDATION.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.

b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

•  If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

•  If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

•  This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps.

•  The use of constants (gc_yes, gc_no, gc_inside_range) aligns the logic with broader application settings or standards.

•  Input is optional, so defaulting behavior can be configured outside this function.

Function: /SMPL/MBR_DEP_ADD_PERFORM

1. Initialization

a. Input parameters provide context for recording a Perform signature against a specific Manufacturing Batch Record (MBR) step:

•  IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

•  IM_AUFNR (AUFNR) (Optional) – Process order number associated with the control recipe.

•  IM_USERID (XUBNAME) – SAP user ID of the performer signing the MBR step.

•  IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG UUID) representing the MBR step being signed.

•  IM_DEST_NAME (/SMPL/MBR_CMX_XS_DST_NAME) – Destination or XStep node name indicating the origin of the signature action.

b. The function validates that IM_USERID is not initial; if missing, the exception USER_INITIAL is raised.

c. It then checks whether the performer is authorized to sign within the MBR context; if not registered in the signature authorization table, USER_NOT_SIGNED_INTO_SIG_TAB is raised.

2. Signature Entry Construction

a. A new record representing the Perform signature is constructed with the process order (AUFNR), step UUID (SOG_UUID), signature type (TYPE = gc_perform_signature), performer (SIGN_ID), system date/time for the creation and signature timestamps, and destination name (DEST_NAME).

3. Database Insertion

a. The signature record is inserted into /SMPL/MBR_OR_SIG, creating a permanent audit trail linking the performer to the execution of the MBR step. A database error triggers a system exception to ensure data consistency.

4. Output Assignment

a. The function does not return explicit output parameters; its outcome is the successful insertion of a Perform signature record, which downstream logic (e.g. /SMPL/MBR_DEP_CHECK_ACTIVE) consumes.

Additional Notes

•  Captures performer identification, system date, and time automatically at the moment of signing, creating a permanent and auditable MBR signature record.

•  Error Handling: USER_INITIAL (performer ID missing); USER_NOT_SIGNED_INTO_SIG_TAB (performer not authorized); database insert failure raised by the system.

•  Dependencies: table /SMPL/MBR_OR_SIG; constant gc_perform_signature; system fields SY-DATUM/SY-UZEIT.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR step can be activated:

•  IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

•  IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

•  IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

•  IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

•  IM_PLANT (WERKS_D) – Plant code for process context.

b. The function validates that IM_CONTROL_RECIPE is not initial; if it is, CONTROL_RECIPE_INITIAL is raised. Internal data is initialized and routine LOAD_INTERNAL_DATA populates the order dependency buffers.

2. Signature Data Retrieval

a. Signature records for the process order are retrieved from /SMPL/MBR_OR_SIG (AUFNR = IM_AUFNR) into internal table lt_ord_sigs for evaluation.

3. Dependency Evaluation

a. The function loops through dependencies where TARGET_UUID = IM_STEP_UUID and DEP_RULE is a Perform-Before-Start or Verify-Before-Start rule:

•  For PERFORM BEFORE START, it checks whether the prerequisite step has a pending perform signature.

•  For VERIFY BEFORE START, it checks whether the prerequisite step has a pending verify signature.

b. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive); if all prerequisites are complete, it is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in EV_ACTIVE (active = all prerequisites complete; inactive = at least one prerequisite incomplete).

Additional Notes

•  Evaluates Perform Before Start and Verify Before Start dependency rules using live signature data from /SMPL/MBR_OR_SIG.

•  Error Handling: raises CONTROL_RECIPE_INITIAL if the control recipe is missing; handles empty dependency/signature data without runtime errors.

•  Activation status is recalculated each time the function is called to reflect live signature data.

## Configuration Specification

N/A

## Test Scenarios

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: Solution Summary - Data Recording | Process Order created and Released containing SMPL: Solution Summary - Data Recording |  |
| 2 | Open the EBR, and navigate to SMPL: Rocker Bag Daily Sample | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Rocker Bag Daily Sample | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Rocker Bag Daily Sample |  |
| 3 | The operator inputs the working day | Text value accepted. | Text value accepted. |  |
| 4 | The operator executes the Get Time function button. | System time accurately captured. | System time accurately captured. |  |
| 5 | The operator inputs the temperature value inside the range as authored from MBR. | Numeric value is accepted. | Numeric value is accepted. |  |
| 6 | The operator inputs the temperature value outside the range as authored from MBR. | Numeric value is rejected and a deviation popup occurs. | Numeric value is rejected and a deviation popup occurs. |  |
| 7 | The operator inputs the CO2 value inside the range as authored from MBR. | Numeric value is accepted. | Numeric value is accepted. |  |
| 8 | The operator inputs the CO2 value outside the range as authored from MBR. | Numeric value is rejected and a deviation popup occurs. | Numeric value is rejected and a deviation popup occurs. |  |
| 9 | The operator inputs the speed value inside the range as authored from MBR. | Numeric value is accepted. | Numeric value is accepted. |  |
| 10 | The operator inputs the speed value outside the range as authored from MBR. | Numeric value is rejected and a deviation popup occurs. | Numeric value is rejected and a deviation popup occurs. |  |
| 11 | The operator inputs the angle value inside the range as authored from MBR. | Numeric value is accepted. | Numeric value is accepted. |  |
| 12 | The operator inputs the angle value outside the range as authored from MBR. | Numeric value is rejected and a deviation popup occurs. | Numeric value is rejected and a deviation popup occurs. |  |
| 13 | The operator inputs the airflow value inside the range as authored from MBR. | Numeric value is accepted. | Numeric value is accepted. |  |
| 14 | The operator inputs the airflow value outside the range as authored from MBR. | Numeric value is rejected and a deviation popup occurs. | Numeric value is rejected and a deviation popup occurs. |  |
| 15 | The operator executes the Equipment Select Button. | The Vi-CELL Blu ID and Vi-CELL Blu Descr fields are correctly populated with the selected equipment. | The Vi-CELL Blu ID and Vi-CELL Blu Descr fields are correctly populated with the selected equipment. |  |
| 16 | The operator enters the VCD and % viability | Numeric values are accepted. | Numeric values are accepted. |  |
| 17 | Verify that a VCD value equal to or greater than 3.1 auto-populates the Transfer Req? to Yes. | Is Transfer Req field auto-populates with Yes | Is Transfer Req field auto-populates with Yes |  |
| 18 | Verify that a VCD value equal to or less than 3.1 auto-populates the Transfer Req? to No. | Is Transfer Req field auto-populates with No | Is Transfer Req field auto-populates with No |  |
| 19 | Enter a username in the “Performed By” field then enter the password in the pop-up window. | Signature is accepted | Signature is accepted |  |
| 20 | Enter a username in the “Witnessed By” field then enter the password in the pop-up window. | Signature is accepted | Signature is accepted |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.1 | Initial Document (Functional requirements and Technical specification) | Frieda van der Merwe | 21-05-2026 |