V 1.2

June 2026

Xstep Design Specification

SMPL: In-Process Parameter Recording

Table of Contents

1	Purpose	3

2	Overview	3

3	Reasons for developing	4

4	Authorization	4

5	Assumptions/ Dependencies	5

6	Validation Checks	5

7	Technical XStep Layout Design	7

8	Function Module(s)	8

9	Pseudocode	13

10	Test Scenarios	21

11	Document References	22

12	Revision History	22

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: In-Process Parameter Recording”.

## Overview

This XStep facilitates the recording of critical process parameters—such as temperature and conductivity for example—during Electronic Batch Record (EBR) execution. It is configured to retrieve data directly from connected process equipment or, where automation is not available, allows for manual data entry. The step is fully integrated with the SAP Equipment Master and equipment logbook records, supporting equipment traceability and compliance.

Operators can select from pre-assigned, qualified equipment, ensuring alignment with manufacturing phase requirements. Parameter limits (minimum and maximum values) are predefined in the SAP Production Version, and built-in system validations verify that recorded values fall within the acceptable range before allowing the process to continue.

This XStep plays a key role in maintaining batch integrity, ensuring traceability, and enabling real-time process monitoring.

The XStep contains two main input parameters:

- IV_HEADER - This can be changed in SiMPL MBR to display specific header text for the XStep

- IV_INSTR - This can be changed in SiMPL MBR to display a specific set of instructions for the XStep that will be displayed below the header

- IV_OBJ_TYP – This can be changed in SiMPL MBR to configure the desired equipment type for retrieval (2000 - Tanks, 2010 - Scales/Balances, 2020 - Meters, etc.)

- IV_DES - This can be changed in SiMPL MBR to display a specific description of the parameter to be recorded

- IV_MIN - This can be changed in SiMPL MBR to configure the Minimum validation range of the parameter to be recorded

- IV_MAX - This can be changed in SiMPL MBR to configure the Maximum validation range of the parameter to be recorded

- IV_UOM_T - This can be changed in SiMPL MBR to display a specific Unit of Measure of the parameter to be recorded

The following fields are auto populated in the Xstep:

- “Min” - Output

- “Max” - Output

- “UoM” - Output

The operator executes the “Select Equipment” button:

- Select Equipment - Function Button

The button is linked to the function module ZSMPL_FM_GET_ASSIGNED_EQUI_EBR that verifies the equipment number against the Equipment Master and retrieves the following information:

- “Equipment ID” - Input

- “Equipment Description” - Output

The operator enters the data reading:

- “Results” - Input

If the result is within the minimum and maximum range, the operator must sign off in the "Performed By" field.

If the result is outside the minimum and maximum range a deviation and error message will occur, and the operator will not be able to sign off in the "Performed By" field.

Functional Design

## Reasons for developing

This SMPL: In-Process Parameter Recording XStep was developed to ensure accurate and GMP-compliant recording of key process parameters, such as temperature and conductivity. It reduces manual errors, enforces validation rules, and ensures traceability through integration with SAP master data and equipment logbook records.

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | User signing signature fields is an approved user in the system with the correct access configuration |
| 2 | Master Data has been set up correctly |
| 3 | The function modules are available and returns the equipment details |
| 4 | The function module can successfully pull the data into the output fields |
| 5 | The function module has access to the Master Data and Logbooks |
| 6 | The XStep validates key user input fields to ensure that recorded results fall within predefined minimum and maximum limits |

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Equipment item not found | Check Equipment Master |
| User Signature rejected due to an incorrect username or password | User accepts error message and re-enter the correct username and password |

## Technical XStep Layout Design

## Function Module(s)

Function: ZSMPL_FM_GET_ASSIGNED_EQUI_EBR

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_ORDER | AUFNR | Order Number |
| IM_EQUIP_TYPE | EQTYP | Equipment Category |
| IM_ALLOW_NA | BOOLE_D | Data element for domain BOOLE: TRUE |
| IM_VORNR | VORNR | Operation/Activity Number |
| IM_OBJ_TYP | EQART | Type of Technical Object |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_EQUI | CHAR30 | Equipment Number |
| EX_EQUI_TXT | CHAR30 | Equipment Description |

Exceptions:

| Exception | Short Text |
| --- | --- |
| INTERNAL_ERROR | Internal error |

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_IDX | INT4 | XStep Step Index Number |
| IM_STEP | CHAR30 | XStep Header |
| IM_SIGTYPE | CHAR20 | Signature Type |
| IM_USERID | XUBNAME | User Name |
| IM_DB_UP_DOWN | CHAR1 | Done By Up/Down |
| IM_VALIDATION_REQ | CHAR3 | Validation Required |
| IM_STEP_UUID | /SMPL/STR_DE_SOG_UUID | XStep SOG UUID |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

Function: /SMPL/ELB_FM_GET_ASS_EQ_VALID

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_ORDER | AUFNR | Order Number |
| IM_EQUIP_TYPE | EQTYP | Equipment Category |
| IM_ALLOW_NA | BOOLE_D | Allow N/A Flag |
| IM_EQUI | CHAR30 | Equipment Number (entered) |
| IM_VORNR | VORNR | Operation/Activity Number |
| IM_MULTI_PO | ABAP_BOOL | Multi-Order (Multi-PO) Flag |
| IM_CRID | POC_DOCID | Control Recipe Number |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EX_EQUI_TXT | CHAR30 | Equipment Description |

Exceptions:

| Exception | Short Text |
| --- | --- |
| EQUI_NOT_ASSIGNED | Equipment not assigned to the order |
| EQUI_NOT_CLEAN | Equipment not clean |
| STATUS_NOT_CONFIGURED | Equipment status not configured |

Function: /SMPL/PPPI_FM_MIN_MAX

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| I_MIN | F | Minimum Value |
| I_MAX | F | Maximum Value |
| I_VALUE | F | Entered Value |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_OUT_OF_RANGE | Value out of range |
| MIN_GREATER_THAN_MAX | Minimum value greater than maximum value |

Function: /SMPL/PPPI_FM_SIG_VALIDATION

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Single-Character Flag (Signature ID) |
| IM_STEP_NAME | CHAR40 | XStep Step Name |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | XStep SOG UUID |
| IM_CB_VALIDATE | CHAR1 | Checked By Validate Flag |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not signed into signature table |
| USER_SIGNED_DONE_BY | User already signed as Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed as Performed By |
| XSTEP_CONFIG | XStep configuration error |

## Pseudocode

Function: ZSMPL_FM_GET_ASSIGNED_EQUI_EBR

1. Initialization

a. Clear export parameters EX_EQUI and EX_EQUI_TXT
b. Dynamic Status Retrieval (MOD4): Call get_eq_stat_by_class to fetch localized values for lc_stat_in_use and lc_iuc (In Use and In Use Cleaned).
c. Dynamic Status Retrieval (MOD4): Call get_eq_stat_by_class to fetch localized values for lc_stat_in_use and lc_iuc (In Use and In Use Cleaned).

d. Optional "N/A" Entry: If IM_ALLOW_NA = abap_true, pre-populate the results table with an "N/A" row using the retrieved "In Use" status.

2. Equipment Retrieval from Database

a. Query Logic (MOD2 & MOD4): Query /SMPL/ELB_EQUIP (Master) joined with EQUI and a left outer join on EQKT (for language-specific descriptions).
b. Filter based on:

- ORDER_NUM = IM_ORDER

- Object Type (MOD2): EQUI~EQART = IM_OBJ_TYP (Note: EQTYP is now ignored per 2026-02 update).

- Phase Filter: ACTIVITY = IM_VORNR (if provided).

- UPDAT_CB must not be initial (Confirmation flag).

c. Latest Entry Logic: Order by EQUI_ID and ENTRY_NUM DESCENDING.

d. Remove Duplicates: Keep only the most recent entry per EQUI_ID.

e. Status Filtering (MOD3): Delete entries where status is neither lv_assigned_status ("In Use") nor lv_iuc ("In Use Cleaned").

3. Single Result Logic

a. If exactly one equipment found and IM_ALLOW_NA is false:

- Assign EQUI_ID to EX_EQUI and EQUI_TXT to EX_EQUI_TXT and return immediately.

4. No Result or Single N/A Case

a. Standardization Check (MOD1): Call Enhancement Framework (/smpl/ebr_cl_enh_fwk=>get_entry) using keys 'SMPL_EBR' and 'GET_ASSIGNED_EQUI'.

b. Dynamic Handling: If an enhancement method is defined, execute custom logic via dynamic class/method call.

c. Warning Popup: If no valid equipment remains (or only N/A exists), display the warning popup with the retrieved icon.

d. Default Return: Return "N/A" (from text symbols) in both EX_EQUI and EX_EQUI_TXT.

5. WebSocket-Based Selection (for PI Sheet integration)

a. If GV_FUNCTION = 'SMPL_EBR' and no equipment selected yet:

- Prepare a semicolon-separated value help string with equipment data

- Trigger frontend equipment dialog via WebSocket

- Return from function until user selection happens

b. If user has selected an equipment via WebSocket:

- Match and return the selected equipment

6. Manual Popup Selection (Fallback)

a. If multiple options and not using WebSocket:

- Use CL_SALV_TABLE to display equipment in a popup

- Set column labels and hide STATUS field

- Let the user select one line

b. After selection, extract and assign:

- EQUI_ID → EX_EQUI

- EQUI_TXT → EX_EQUI_TXT

7. Output Assignment

a. Return selected equipment ID and description in:

- EX_EQUI

- EX_EQUI_TXT

Additional Notes:

- Purpose:
Returns equipment of a specified type (e.g., Room, Additional Equipment) assigned to a process order, with user selection if needed.

- Key Features:

Retrieves latest assigned equipment per order/phase
	Allows optional fallback to “N/A”
	WebSocket integration for UI5-based PI Sheets
	Manual fallback popup for SAP GUI usage
	Enhancement spot for customized warning handling

- Error Handling:

Displays warning popups if no equipment found

Handles exceptions in dynamic method calls

Avoids dumps using robust TRY...CATCH

- Assumptions:

Equipment assignments are stored in /SMPL/ELB_EQUIP
	UPDAT_CB serves as confirmation flag
	WebSocket setup is operational for EBR communication

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

- Signature Type Identification

- Assign the incoming signature type (IM_SIGTYPE) to local variable lv_sigtype.

- A CASE structure is used to determine behavior based on the type:

  - ' ' (Done By)

  - 'C' (Checked By)

  - 'O' (Validation Flag Only)

- Done by Signature(lv_sigtype = '')
 a. Optional Step Name Update

- If IM_STEP is provided, update the XStep header in table /SMPL/PPPI_PISST.

b. Determine Position for Signature Insertion

- If IM_DB_UP_DOWN is empty:

  - Insert the new signature above the current item by fetching MIN(UNIQUE_SEQID) and inserting one row before it.

- If IM_DB_UP_DOWN = 'B':

  - Insert the new signature below the current item by fetching MAX(UNIQUE_SEQID) and inserting one row after it.

- In both cases:

  - Use IM_USERID as the signer (SIGN_ID)

  - Signature type description from TEXT-001

  - Insert new row into /SMPL/PPPI_PISST

- Checked By Signature (lv_sigtype = 'C')
 a. Get Signature Start Point

  - Select the UNIQUE_SEQID of the "Checked By" step matching the given input.

b. Determine Signature End Point

- Fetch all prior 'C' type signatures for the same control recipe (and optionally across all phases).

- Determine the highest UNIQUE_SEQID lower than the start point to use as the end.

c. Update Checked By Step

- Set the SIGN_ID and mark the step as COMPLETED = 'X'.

d. Complete All Steps Between Start and End

- Fetch and loop over all steps with UNIQUE_SEQID between the start and end boundaries.

- Mark each row in that range as COMPLETED = 'X'.

- Update the database using MODIFY.

- Validation Flag Update (lv_sigtype = 'O')
 a. Determine Validation Value

  - If IM_VALIDATION_REQ = '2', set to 'No', else 'Yes'.

b. Update Validation Requirement

- Update the VALIDATION_REQUIRED field for the current control recipe, phase, and step index in /SMPL/PPPI_PISST.

Additional Notes:

- Purpose:
Dynamically inserts or updates signature records in /SMPL/PPPI_PISST depending on the role of the signer (Done By, Checked By, or Validator).

- Features:

Supports step name overrides and UUID tracking.

Allows insertion before or after specific records using IM_DB_UP_DOWN.

Ensures Checked By signatures cascade completion to preceding steps.

Accommodates flexible validation logic.

- Error Handling:

No explicit RAISE statements or exception handling.

Relies on CHECK and SY-SUBRC evaluations to exit silently if lookup conditions fail.

- Extensibility:

Designed with optional parameters (IM_STEP, IM_SIGTYPE, IM_DB_UP_DOWN, IM_VALIDATION_REQ) to support various signing scenarios.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.
b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

- If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.
If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

- This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps

- The use of constants (gc_yes, gc_no, and gc_inside_range) allows the logic to be aligned with broader application settings or standards.

- Helpful in scenarios where input values may vary but need to result in consistent "active" or "inactive" behavior.

- Input is optional, so defaulting behavior can be configured outside this function.

- This function improves clarity and consistency in determining whether an XStep should be active when rendered in the PI Sheet.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR (Manufacturing Batch Record) step can be activated:

- IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

- IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

- IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

- IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

- IM_PLANT (WERKS_D) – Plant code for process context.

b. The function first validates that IM_CONTROL_RECIPE is not initial.

- If it is initial, the exception CONTROL_RECIPE_INITIAL is raised.

c. Internal data is initialized:

- lt_ord_sigs: Internal table for storing order signature data from /SMPL/MBR_OR_SIG.

- ls_step_dep: Work area for step dependency data.

- Routine LOAD_INTERNAL_DATA is called to populate internal buffers (g_ord_data-t_ord_deps) using IM_AUFNR, IM_CONTROL_RECIPE, and IM_STEP_UUID.

2. Signature Data Retrieval

a. The function retrieves signature records for the process order from database table /SMPL/MBR_OR_SIG.
b. Selected fields include: AUFNR, SOG_UUID, TYPE, CREATED_DATE, CREATED_TIME, DEST_NAME, SIGN_ID, SIGN_DATE, and SIGN_TIME.
c. Only signatures related to the specified order (AUFNR = IM_AUFNR) are selected.
d. These records are stored in the internal table lt_ord_sigs for later evaluation.

3. Dependency Evaluation

a. The function loops through dependencies in g_ord_data-t_ord_deps where:

- TARGET_UUID = IM_STEP_UUID, and

- DEP_RULE is either gc_perform_before_start_rule or gc_verify_before_start_rule.

b. For each dependency record:

- If the rule is PERFORM BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending perform signature (gc_perform_signature) with SIGN_ID = abap_false.

- If the rule is VERIFY BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending verify signature (gc_verify_signature) with SIGN_ID = abap_false.

c. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive).
d. If all prerequisite signatures are complete, the step is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in the export parameter EV_ACTIVE.

- gc_step_active – All prerequisite steps are completed; the current step can start.

- gc_step_inactive – At least one prerequisite step is incomplete; the current step remains blocked.

Additional Notes

This function determines whether a dependent step in a Manufacturing Batch Record is eligible to start based on the completion of prerequisite steps.

Key Features:

- Evaluates Perform Before Start and Verify Before Start dependency rules.

- Dynamically validates signature data from /SMPL/MBR_OR_SIG.

- Integrates with existing MBR dependency structures via global table g_ord_data-t_ord_deps.

Error Handling:

- Raises CONTROL_RECIPE_INITIAL if the control recipe input is missing.

- Handles empty dependency or signature data without runtime errors.

Dependencies:

- Tables: /SMPL/MBR_OR_SIG (Signatures), g_ord_data-t_ord_deps (Step Dependencies).

- Constants: gc_perform_before_start_rule, gc_verify_before_start_rule, gc_perform_signature, gc_verify_signature, gc_step_active, gc_step_inactive.

- Routine: LOAD_INTERNAL_DATA for initializing order context.

Assumptions:

- All prerequisite steps are recorded in the dependency table.

- Signature records are current and synchronized with MBR step progression.

- Activation status is recalculated each time the function is called to reflect live signature data.

Function: /SMPL/ELB_FM_GET_ASS_EQ_VALID

1. Initialization

a. Clear EX_EQUI_TXT and the working tables; define the status-class constants ('U1' = In Use, 'U2' = In Use Cleaned).

b. Retrieve the configured In Use status via GET_EQ_STAT_BY_CLASS (class 'U1'); if none is configured, raise STATUS_NOT_CONFIGURED. When IM_MULTI_PO = ABAP_TRUE, also retrieve the In Use Cleaned status (class 'U2'); if none, raise STATUS_NOT_CONFIGURED.

2. Optional N/A Entry

a. If IM_ALLOW_NA = ABAP_TRUE, pre-append an 'N/A' row to the results using the In Use status.

3. Retrieve and Match Equipment

a. CONVERSION_EXIT_ALPHA_INPUT on IM_EQUI; SELECT from /SMPL/ELB_EQUIP joined to EQUI and LEFT JOIN EQKT (SY-LANGU) WHERE ORDER_NUM = IM_ORDER, (ACTIVITY = IM_VORNR if supplied), (CR_NR = IM_CRID if supplied), EQUI~EQTYP = IM_EQUIP_TYPE and EQUI~EQUNR = the entered equipment; order by entry number and keep the latest entry per equipment (delete adjacent duplicates).

b. Delete results whose status is neither the In Use nor the In Use Cleaned status.

4. Validate Assignment

a. Read the first remaining result; if none, MESSAGE and raise EQUI_NOT_ASSIGNED. Otherwise set EX_EQUI_TXT to the equipment description.

5. Standardization Check (MOD6)

a. PERFORM STANDARDIZATION_CHECK; if the equipment is not standardized ('S1') or has not yet been standardized ('S2'), show a warning popup and set EX_EQUI_TXT to 'N/A'.

Additional Notes

Validator on the Equipment ID / Select Equipment column: confirms the entered equipment is actually assigned to the order (status In Use / In Use Cleaned) and returns its description.

EQUI_NOT_CLEAN is declared but not raised in the current source. Status values are configuration-driven via GET_EQ_STAT_BY_CLASS.

Function: /SMPL/PPPI_FM_MIN_MAX

1. Range Validation

a. If both I_MIN and I_MAX are provided: if I_MIN >= I_MAX raise MIN_GREATER_THAN_MAX; otherwise accept when I_MIN <= I_VALUE <= I_MAX, else raise VALUE_OUT_OF_RANGE.

b. If only I_MAX is provided: accept when I_VALUE <= I_MAX, otherwise raise VALUE_OUT_OF_RANGE.

c. If only I_MIN is provided: accept when I_VALUE >= I_MIN, otherwise raise VALUE_OUT_OF_RANGE.

d. If neither limit is provided, no range check is performed.

Additional Notes

Validator on the 'Results' input column; enforces that the recorded value falls within the configured minimum / maximum range (from the SAP Production Version).

All operator messages are issued TYPE 'S' DISPLAY LIKE 'E'; the out-of-range message reports the entered value and the expected bounds.

Function: /SMPL/PPPI_FM_SIG_VALIDATION

1. Initialize and Validate Mandatory Inputs

a. Declare locals for master-recipe info and step dependencies and the signature constants. Clear / refresh all working data.

b. If IM_CONTROL_RECIPE is initial, raise CONTROL_RECIPE_INITIAL; if IM_USERID is initial, raise USER_INITIAL.

2. Resolve Master Recipe, Destination and Order Type

a. SELECT SINGLE PLNNR / PLNAL FROM AFKO; if not found, raise XSTEP_CONFIG.

b. SELECT SINGLE PHSEQ (control recipe destination) FROM COCH; if not found, raise XSTEP_CONFIG.

c. SELECT SINGLE PLNAW FROM AFKO to derive the order type (production vs process).

3. Retrieve and Filter SiMPL Dependency Data

a. Call /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA; errors (/SMPL/MBR_CL_T100) are caught and ignored.

b. Keep dependency rows where DEST_NAME matches the control recipe destination and DEP_UUID = IM_STEP_UUID.

4. Predecessor-Step Completion Check (currently inert)

a. The inline RAISE PREDECESSOR_NOT_COMPLETED is commented out ("MBR handles the dependencies"); enforcement is deferred to the MBR delegation in step 7.

5. Signature-Table Validation

a. SELECT SINGLE from /SMPL/PPPI_PIBSI WHERE PI_SHEET = IM_CONTROL_RECIPE AND SIGNATURE = IM_USERID; if not found, raise USER_NOT_SIGNED_INTO_SIG_TAB.

6. Checked By Segregation-of-Duties Check

a. Only when IM_LABEL_ID = 'C' and IM_CB_VALIDATE = ABAP_TRUE: determine the signature range for the step and, if the same user already holds a Performed / Done By signature in that range, raise USER_SIGNED_PERF_BY.

7. Final Required-Steps Check and MBR Delegation

a. Identify the last EBR step (MAX UNIQUE_SEQID); if missing, raise XSTEP_CONFIG.

b. If this is the last step, ensure all required EBR steps are completed; otherwise raise REQ_EBR_STEPS_NOT_COMPLETED.

c. Call /SMPL/MBR_DEP_VALI_SIGN and map SY-SUBRC 1:1 to the corresponding exception (CONTROL_RECIPE_INITIAL, USER_INITIAL, USER_NOT_SIGNED_INTO_SIG_TAB, USER_SIGNED_DONE_BY, PREDECESSOR_NOT_COMPLETED, REQ_EBR_STEPS_NOT_COMPLETED, USER_SIGNED_PERF_BY).

Additional Notes

Validates the Performed By signature captured in the main step (and the optional Witnessed By), including signature-table membership, segregation-of-duties and required-step completion.

USER_SIGNED_DONE_BY and PREDECESSOR_NOT_COMPLETED are only raised via the SY-SUBRC mapping from /SMPL/MBR_DEP_VALI_SIGN; XSTEP_CONFIG is the catch-all configuration / structure failure. Source: DE1 client 100, group /SMPL/PPPI_FG_EBR_FUNC_SIG.

## Test Scenarios

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: In-Process Parameter Recording | Process Order created and Released containing SMPL: In-Process Parameter Recording |  |
| 2 | Open the EBR, and navigate to SMPL: In-Process Parameter Recording | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: In-Process Parameter Recording is visible | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: In-Process Parameter Recording is visible |  |
| 3 | The operator executes the Select Equipment Button and selects an equipment from the options available. The operator verifies that the Equipment ID and Equipment Description is populated once an equipment item is selected. | Equipment Item is selected and Equipment ID and Equipment Description are retrieved and displayed. | Equipment Item is selected and Equipment ID and Equipment Description are retrieved and displayed. |  |
| 4 | The operator enters a result value. | Result value accepted and in range. | Result value accepted and in range. |  |
| 5 | Enter a username in the “Performed By” field then enter the password in the pop-up window. | Signature is accepted | Signature is accepted |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.1 | Initial Document (Functional requirements and Technical specification) | Frieda van der Merwe | 18-06-2025 |
| 1.2 | The following Technical Spec sections added to spec: Function Module(s), and Pseudocode. | Juna Jaouhari | 22-06-2026 |
| 1.3 | Updated to align the specification with the current XStep version (v0003). Added Function Module(s) and Pseudocode sections for the previously undocumented main-step function modules: the Select Equipment validator /SMPL/ELB_FM_GET_ASS_EQ_VALID, the Results min/max range validator /SMPL/PPPI_FM_MIN_MAX, and the signature validator /SMPL/PPPI_FM_SIG_VALIDATION. Added the INTERNAL_ERROR exception to ZSMPL_FM_GET_ASSIGNED_EQUI_EBR and corrected the /SMPL/PPPI_FM_SIG_ADD_DB_CB IM_STEP_UUID type to /SMPL/STR_DE_SOG_UUID. (Function modules used only by the embedded Conditional Header / Optional Signature references are not documented here.) | [Author] | 23-06-2026 |