V 1.3

June 2026

Xstep Design Specification

SMPL: Room/Equipment Assign

Table of Contents

1	Purpose	3

2	Overview	3

3	Reasons for developing	5

4	Authorization	6

5	Assumptions/ Dependencies	6

6	Validation Checks	6

7	Technical XStep Layout Design	8

8	Function Module(s)	8

9	Pseudocode	14

10	Test Scenarios	29

11	Document References	31

12	Revision History	31

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: Room/Equipment Assign”.

## Overview

The SMPL: Room/Equipment Assign XStep is designed to facilitate the precise selection and assignment of a designated manufacturing room along with the necessary equipment required for order execution.

Following successful room assignment, the XStep retrieves all equipment linked to that room from the Equipment Masters and presents this information to the operator. The operator then selects which pieces of equipment will be used for the order. Prior to final assignment, each piece of selected equipment undergoes validation to verify compliance with required cleaning and calibration standards. Only equipment meeting these criteria is eligible for assignment to the order and use throughout the execution.

In addition, the XStep provides an “Additional Equipment” table. This feature supports the manual assignment of equipment when equipment is not linked to a specific room—such as mobile or portable equipment—or allows the assignment of supplementary equipment during the course of order execution. This capability addresses dynamic manufacturing needs, including equipment breakdowns, calibration requirements, or changes in process demands. The XStep is fully integrated with the SAP Equipment Master and custom equipment logbook records, ensuring real-time tracking of equipment statuses. Through these integrations, the solution supports multiple phases of manufacturing by enabling assignment of validated, clean rooms and both fixed and portable equipment, while maintaining comprehensive traceability of equipment status changes throughout the manufacturing process.

The XStep contains two main input parameters:

- IV_HEADER - This can be changed in SiMPL MBR to display specific header text for the XStep

- IV_INSTR - This can be changed in SiMPL MBR to display a specific set of instructions for the XStep that will be displayed below the header

In this XStep, execution occurs in two stages: room equipment retrieval & assignment, and additional equipment assignment.

Room equipment assignment

The operator enters the room number alias:

- “Room #” - Input

The operator executes the “Retrieve Room Equipment Detail” button:

- Retrieve Room Equipment Detail - Function Button

The button is linked to the function module ZSMPL_FM_FLOC_EQUIP that retrieves all of the equipment linked to the specific Room in the Equipment Masters as well as each equipment’s current status tracked in the equipment logbook.

The following fields are retrieved and populated into the equipment table:

- “Equipment #” - Output

- “Description” - Output

- “Current Status” - Output

- “Last Cleaning Date” - Output

- “Last Prod. Desc.” - Output

- “Calib. Due Date” - Output

- “Last Prod. Part No.” - Output

The operator selects from the “Equip. Required” dropdown if the item is required for this specific process order:

- Equip. Required?  - Input

If “Yes” is selected, the status change is validated against a series of configurable validation rules:

- Is the equipment’s current status an acceptable status to be assigned to the order?

  - CLEAN - Acceptable

  - DIRTY - Not Acceptable

  - IDLE - Not Acceptable

  - IN USE - Not Acceptable

  - IN USE CLEANED - Acceptable

  - MAINTENANCE - Not Acceptable

  - OUT OF SERVICE - Not Acceptable

  - SIP’d - Acceptable

  - TO BE CLEANED - Not Acceptable

  - PRESSURE CHECKED - Acceptable

- If the equipment’s current status is an accepted clean category, has the cleaning been performed in the past 30 days?

- If the equipment is accepted, its Calibration Due Date is validated; equipment with a Calibration Due Date earlier than the current date is rejected (exception CALIBRATION_DUE_DATE_INCORRECT).

- Line clearance must be complete before the equipment can be assigned; if line clearance is not complete the equipment cannot be assigned to the order.

- A standardization check is performed; if the equipment has not been standardized the operator is warned via a popup before proceeding.

- Multi-order (multi-PO) assignment is supported; equipment already assigned to another order is permitted with a warning rather than being blocked.

If any of these validation checks failed, operators will not be allowed to assign the equipment to the specific order.

Finally, the user who performed the equipment assignment records their signature in the “Performed By” field, while a supervisor verifies the selection by signing in the “Witnessed By” field. Only once the “Witnessed By” signature has been executed, will the equipment’s status be updated to “IN USE” for all equipment selected as “Yes” in the equipment table for the order.

Additional Equipment Assignment / Equipment Assignment

If additional equipment is needed beyond what is already assigned to the room, the operator should select “Yes” from the “Add. Equipment Required?” dropdown menu. Selecting “Yes” will activate the Additional Equipment table and allow operators to interact with the table. If the table is not needed, the operator can select “No” and it will deactivate the Additional Equipment table.

The operator enters the equipment number to be assigned in the “Equipment #” field:

- “Equipment #” - Input

The operator executes the “Details” button:

- Details - Function Module

The button is linked to the function module /SMPL/ELB_FM_ADD_EQUIP is activated to retrieve the following information from the Equipment Masters and equipment logbook if the equipment exists:

- “Description” - Output

- “Current Status” - Output

- “Last Cleaning Date” - Output

- “Last Prod. Desc” - Output

The operator once again selects whether the equipment is required using the “Equip. Required?” dropdown, provided the current status is suitable for use. The individual performing this selection then records their signature in the “Performed By” field.

The Additional Equipment table should remain open throughout the execution of the EBR to allow operators to assign additional equipment at any stage of execution.

Functional Design

## Reasons for developing

The primary goals of developing the SMPL: Room/Equipment Assign XStep is to standardize and streamline the assignment of manufacturing rooms and equipment, ensuring compliance, cleanliness, and readiness prior to process execution. This functionality enhances operational efficiency and traceability by integrating with the Equipment Masters and custom equipment logbooks, while supporting various manufacturing phases through structured equipment tracking and validation.

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | User signing signature fields is an approved user in the system with the correct access configuration |
| 2 | Master Data has been set up correctly |
| 3 | The function modules are available and returns the equipment details |
| 4 | The function module can successfully pull the data into the output fields |
| 5 | The function module has access to the Master Data and Logbooks |
| 6 | The XStep performs validations on key user input fields to enforce cleaning requirements |
| 7 | The updated cleaning statuses of the equipment are displayed when the equipment number is entered. |

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Room not found | Check master data |
| Equipment not clean | Update in the Equipment Logbook cockpit |
| User Signature rejected due to an incorrect username or password | User accepts error message and re-enter the correct username and password |

## Technical XStep Layout Design

## Function Module(s)

Function: /SMPL/ELB_FM_UPDATE_LOGB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_EQUIPMENT | EQUNR | Equipment Number |
| IV_EQUI_DESC | KTX01 | Description of technical object |
| IV_EQUI_STAT | EQFNR | Sort Field |
| IV_ORDER | AUFNR | Order Number |
| IV_MATERIAL | MATNR | Material Number |
| IV_VORNR | VORNR | Operation/Activity Number |
| IV_EQUI_TYPE | EQTYP | Equipment category |
| IV_BYPASS | CHAR1 | Single-Character Flag |
| IV_COMMENT | /SMPL/STR_DE_COMM | Equipment Update Comment |
| IV_UPDAT | UNAME | User Name (Done By) |
| IV_UPDAT_CB | UNAME | User Name (Checked By) |
| IV_EBR_FLAG | CHAR1 | EBR LB Flag |

Exceptions:

| Exception | Short Text |
| --- | --- |
| EQUIPMENT_NOT_UPDATED | Equipment status not updated |
| TABLE_LOCK_NOT_POSSIBLE | Logbook Table not updated |

Function: ZSMPL_FM_FLOC_EQUIP

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ROOM | EQUNR | Equipment Number |
| IV_EQUI_TYPE | EQTYP | Equipment category |
| IV_FLOC_SETUP | ABAP_BOOL | Functional Location Setup Flag |
| IV_PLANT | WERKS_D | Plant |
| IV_TPLKZ | TPLKZ | Functional Location Structure Indicator |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| ET_EQUI_ID | /SMPL/STR_TT_EQUI_ID | Table Type for Equipment ID |
| ET_EQUI_DESC | /SMPL/STR_TT_EQUI_DESC | Table Type for Equipment Description |
| ET_EQUI_STAT | /SMPL/STR_TT_EQUI_STAT | Table Type for Equipment Status |
| ET_EQUI_DATS | /SMPL/STR_TT_EQUI_DATS | Table Type for Equipment Date |
| ET_EQUI_PROD | /SMPL/STR_TT_EQUI_PROD | Table Type for Equipment Product |
| ET_EQUI_TYPE | /SMPL/STR_TT_EQUI_TYPE | Table Type for Equipment |
| ET_CALIB_DUE_DATE | /SMPL/STR_TT_EQUI_DATS | Table Type for Calibration Due Date |

Exceptions:

| Exception | Short Text |
| --- | --- |
| EQUIPMENT_DOES_NOT_EXIST | Equipment does not exist |
| STATUS_NOT_CONFIGURED | Equipment status not configured |
| INITIAL_CLEAN_INCOMPLETE | Initial clean not complete |
| ROOM_NOT_MAINTAINED | Room not maintained |
| CLEANING_REQUIRED | Cleaning required |

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_IDX | INT4 | XStep Step Index Number |
| IM_STEP | CHAR30 | XStep Header |
| IM_SIGTYPE | CHAR20 | Signature Type |
| IM_USERID | XUBNAME | User Name |
| IM_DB_UP_DOWN | CHAR1 | Done By Up/Down |
| IM_VALIDATION_REQ | CHAR3 | Validation Required |
| IM_STEP_UUID | ZSMPL_DE_SMPL_SOG_UUID | XStep SOG UUID |

Function: /SMPL/ELB_FM_UPDATE_LB_USER

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_USER | /SMPL/STR_DE_USER | User |
| IV_DB_FLAG | CHAR1 | Done By Flag |
| IV_ORDER | AUFNR | Order Number |
| IV_VORNR | VORNR | Operation/Activity Number |

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_LABEL_ID | CHAR1 | Signature ID (C) |
| IM_STEP_NAME | CHAR40 | XStep Header |
| IM_STEP_UUID | ZSMPL_DE_SMPL_SOG_UUID | XStep SOG UUID |
| IM_PO_NR | AUFNR | Order Number |
| IM_PLANT | WERKS_D | Plant |

Function: /SMPL/ELB_FM_ADD_EQUIP

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_EQUIPMENT | EQUNR | Equipment Number |
| IV_EQUI_TYPE | EQTYP | Equipment category |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_EQUI_ID | EQUNR | Equipment Number |
| EV_EQUI_DESC | KTX01 | Description of technical object |
| EV_EQUI_STAT | EQFNR | Sort Field |
| EV_EQUI_DATS | DATS | Field of type DATS |
| EV_STAT_TIMS | TIMS | Field of type TIMS |
| EV_STAT_PROD | MAKTX | Material Description |
| EV_EQUI_TYPE | EQTYP | Equipment category |
| EV_CALIB_DUE_DATE | DATS | Calibration Due Date |

Exceptions:

| Exception | Short Text |
| --- | --- |
| EQUIPMENT_DOES_NOT_EXIST | Equipment does not exist |
| STATUS_NOT_CONFIGURED | Status not configured in the equipment status table |
| INITIAL_CLEAN_INCOMPLETE | Initial Cleaning Incomplete |

Function: /SMPL/MBR_DEP_ADD_PERFORM

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_AUFNR | AUFNR | N/A |
| IM_USERID | XUBNAME | N/A |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | N/A |
| IM_DEST_NAME | /SMPL/MBR_CMX_XS_DST_NAME | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| USER_INITIAL | N/A |
| USER_NOT_SIGNED_INTO_SIG_TAB | N/A |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: ZSMPL_FM_VALIDATE_STATUS_EBR

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_EQUIPMENT | EQUNR | Equipment Number |
| IV_EQUI_STAT | EQFNR | Sort Field (Equipment Status) |
| IV_BYPASS | CHAR1 | Single-Character Flag (Bypass / Line Clearance) |
| IV_CALIB_DUE_DATE | DATS | Calibration Due Date |
| IV_MULTI_PO | ABAP_BOOL | Multi-Order (Multi-PO) Assignment Flag |

Exceptions:

| Exception | Short Text |
| --- | --- |
| EQUIPMENT_STATUS_INCORRECT | Equipment status incorrect |
| CALIBRATION_DUE_DATE_INCORRECT | Calibration due date incorrect |

This function module validates the selected equipment status against the configurable cleaning-status engine (table /SMPL/ELB_EQSTAT), and additionally performs the line-clearance, standardization, calibration due-date and multi-order (multi-PO) checks described in section 6.

Function: /SMPL/PPPI_FM_SIG_VALIDATION

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_LABEL_ID | CHAR1 | Single-Character Flag |
| IM_STEP_NAME | CHAR40 | XStep Step Name |
| IM_STEP_INDEX | INT4 | XStep Step Index Number |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_PO_NR | AUFNR | Order Number |
| IM_USERID | XUBNAME | User Name |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | XStep SOG UUID |
| IM_CB_VALIDATE | CHAR1 | Checked By Validate Flag |
| IM_PLANT | WERKS_D | Plant |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL | Control recipe is initial |
| USER_INITIAL | User is initial |
| USER_NOT_SIGNED_INTO_SIG_TAB | User not signed into signature table |
| USER_SIGNED_DONE_BY | User already signed as Done By |
| PREDECESSOR_NOT_COMPLETED | Predecessor step not completed |
| REQ_EBR_STEPS_NOT_COMPLETED | Required EBR steps not completed |
| USER_SIGNED_PERF_BY | User already signed as Performed By |
| XSTEP_CONFIG | XStep configuration error |

This function module validates the Performed By and Witnessed By signatures captured in this XStep, including predecessor-step and required-EBR-step completion checks. Interface read from DE1 client 100 (function group /SMPL/PPPI_FG_EBR_FUNC_SIG, package /SMPL/PPPI; author JCRAIG).

## Pseudocode

Function: /SMPL/ELB_FM_UPDATE_LOGB

1. Validate bypass flag and normalize the equipment number

- If IV_BYPASS is not initial: when IV_BYPASS = '1' (LC_BYPASS) continue processing; for any other non-initial value EXIT the function immediately (no update, no exception).

- Copy IV_EQUIPMENT into L_EQUIP_ID and run CONVERSION_EXIT_ALPHA_INPUT to normalize it to internal EQUNR format.

2. Seed the new log-entry record from import parameters

- Populate a work area for /SMPL/ELB_EQUIP: ORDER_NUM = IV_ORDER, MATERIAL = IV_MATERIAL, ACTIVITY = IV_VORNR, COMM = IV_COMMENT, CR_NR = IV_CRID.

- SELECT all ENTRY_NUM FROM /SMPL/ELB_EQUIP ORDER BY ENTRY_NUM DESCENDING; read index 1 for the highest existing number (default 0 if none). New ENTRY_NUM = highest + 1.

3. Enrich the entry with batch and equipment master data

- If IV_ORDER is supplied: SELECT SINGLE CHARG FROM AFPO WHERE AUFNR = IV_ORDER AND POSNR = '001' into BATCH (missing batch tolerated).

- SELECT SINGLE from EQUI LEFT JOIN EQUZ (on EQUNR) LEFT JOIN EQKT (on EQUNR and EQKT~SPRAS = SY-LANGU) WHERE EQUI~EQUNR = L_EQUIP_ID, EQUZ~DATBI = '99991231', EQUZ~EQLFN = '001'; retrieves EQUNR, EQTYP, TIDNR and short text EQKTX.

a. Language fallback

- If the retrieved EQKTX is initial, re-run the join WITHOUT the SY-LANGU restriction on EQKT to obtain a short text in any language (equipment-not-found is handled at the BAPI step, not raised here).

- Set EQUI_ID = L_EQUIP_ID; EQUI_TXT = IV_EQUI_DESC if supplied, else the retrieved EQKTX; TYPE = EQTYP.

4. Determine the equipment status to store (default vs. Multi-PO logic)

- Default: LV_NEW_STATUS = IV_EQUI_STAT.

a. Multi-PO branch (only when IV_MULTI_PO = ABAP_TRUE)

- Resolve the configured IN USE status via PERFORM GET_EQ_STAT_BY_CLASS class 'U1' and IN USE CLEANED via class 'U2' (first returned status of each).

- SELECT the latest STATUS_DATE/STATUS_TIME from /SMPL/ELB_EQUIP for this EQUI_ID with STATUS = the IN USE code (ORDER BY ENTRY_NUM DESC); if none, LV_USED = ABAP_FALSE, else ABAP_TRUE.

- SELECT ORDER_NUM FROM /SMPL/ELB_EQUIP joined to /SMPL/PPPI_PISST (PO_NR = ORDER_NUM) and COCH (CRID = CTRL_RECIPE) for the same EQUI_ID, excluding the current IV_ORDER and blanks, where PI-sheet STEP = IV_RC_STEP_NAME (default 'Room Clearance'), COMPLETED = ABAP_FALSE, COCH~CRSTAT not in {'00004','00007'}, and STATUS_DATE/TIME at or after the captured IN-USE timestamp (other still-active orders sharing the equipment).

- If active orders exist: LV_NEW_STATUS = IN USE CLEANED when LV_USED is true, else IN USE. If none: LV_NEW_STATUS = IN USE. (When IV_MULTI_PO is false this block is skipped and IV_EQUI_STAT stands.)

5. Finalize and write the logbook row

- Set STATUS = LV_NEW_STATUS, STATUS_DATE = SY-DATUM, STATUS_TIME = SY-UZEIT, UPDAT = IV_UPDAT (upper-cased).

- Only if IV_EBR_FLAG = ABAP_TRUE, set UPDAT_CB = IV_UPDAT_CB (upper-cased) for the second/counter-signature reviewer.

- ENQUEUE_E_TABLE for '/SMPL/ELB_EQUIP' mode 'E'; if the lock fails RAISE TABLE_LOCK_NOT_POSSIBLE; else INSERT the work area into /SMPL/ELB_EQUIP. Then DEQUEUE_E_TABLE.

6. Conditionally update the equipment master record

- Prepare BAPI change structures: DATA_GENERAL-SORTFIELD = IV_EQUI_STAT, DATA_GENERALX-SORTFIELD = ABAP_TRUE.

- Only when IV_EQUI_UPDAT = ABAP_TRUE AND IV_EBR_FLAG = ABAP_TRUE, call BAPI_EQUI_CHANGE for EQUIPMENT = L_EQUIP_ID; if RETURN-TYPE = 'E' RAISE EQUIPMENT_NOT_UPDATED. Otherwise the master update is skipped and only the logbook insert persists.

Additional Notes:

- EXCEPTIONS: TABLE_LOCK_NOT_POSSIBLE (enqueue failure) and EQUIPMENT_NOT_UPDATED (BAPI returns an error). No exception is raised for a missing equipment master/short text.

- IV_EBR_FLAG gates both the UPDAT_CB countersigner write and the equipment-master BAPI update; the master sort field is only changed once both signatures are recorded.

- The enqueue lock guards the read-max-then-INSERT sequence into /SMPL/ELB_EQUIP. GET_EQ_STAT_BY_CLASS (FG /SMPL/ELB_FG_EQUIP_LB) resolves status codes by user-status class.

Function: ZSMPL_FM_FLOC_EQUIP

1. Initialize working data and retrieve configured equipment statuses

- Define local types/tables (LTY_EQUIPMENT and the clean/material/date detail tables) and constants: CV_DATE '99991231', EQLFN '001', use class 'U', clean class 'C', characteristic ATINN 0000001199 (ZSMPL_CHAR_CALIB_DUE_DATE_CHAR). CLEAR/REFRESH all working data.

a. Get clean statuses

- PERFORM GET_EQ_STAT_BY_CLASS for class 'C' into LT_CLEAN_STATS. READ index 1 -> LV_CLEAN_1 (else RAISE STATUS_NOT_CONFIGURED); index 2 -> LV_CLEAN_2 (else RAISE); index 3 -> LV_CLEAN_3 (else default to LV_CLEAN_2).

b. Get IN USE status

- PERFORM GET_EQ_STAT_BY_CLASS for class 'U' into LT_USE_STAT; READ index 1 -> LV_IN_USE (else RAISE STATUS_NOT_CONFIGURED).

2. Resolve the room number from the supplied equipment ID (IV_ROOM)

- Copy IV_ROOM to L_ROOM; CALL CONVERSION_EXIT_ALPHA_INPUT to add leading zeros. SELECT SINGLE ILOA~MSGRP into L_RAUMNR FROM ILOA INNER JOIN EQUZ ON EQUZ~ILOAN = ILOA~ILOAN WHERE EQUZ~EQUNR = L_ROOM.

- If not found or L_RAUMNR initial, keep the original room; else set L_ROOM = L_RAUMNR. If L_ROOM is initial, RAISE ROOM_NOT_MAINTAINED. CALL CONVERSION_EXIT_ALPHA_OUTPUT on L_ROOM.

3. Select the static equipment for the room (two alternative paths)

a. Functional-location path (IV_FLOC_SETUP = ABAP_TRUE)

- SELECT EQUNR, EQKTX, EQFNR, EQTYP joining EQUI, EQUZ, ILOA, IFLOT, ILOA and LEFT JOIN EQKT (SPRAS = SY-LANGU) into LT_EQUIPMENT; filters IFLOT~IWERK = IV_PLANT, IFLOT~TPLNR <> '', (IFLOT~TPLKZ = IV_TPLKZ OR IV_TPLKZ IS NULL), MSGRP = L_ROOM, EQUZ~DATBI = CV_DATE, EQUZ~EQLFN = '001', EQUI~EQTYP = IV_EQUI_TYPE.

b. Room-assignment path (IV_FLOC_SETUP not true)

- SELECT EQUNR, EQKTX, EQFNR, EQTYP FROM EQUI with LEFT JOINs to EQUZ, ILOA, CRHD, EQKT (SPRAS = SY-LANGU) WHERE ILOA~MSGRP = L_ROOM AND EQUZ~DATBI = CV_DATE AND EQUZ~EQLFN = '001' AND EQUI~EQTYP <> IV_EQUI_TYPE; APPEND rows to LT_EQUIPMENT.

- If no equipment found: MESSAGE TEXT-023 (type 'S' like 'E') and RAISE EQUIPMENT_DOES_NOT_EXIST. SORT LT_EQUIPMENT BY EQUNR.

4. Loop over each equipment and read its logbook history (/SMPL/ELB_EQUIP)

- For each row: L_EQUIPMENT = EQUNR, CONVERSION_EXIT_ALPHA_OUTPUT it. (A 30-day clean branch on L_ERROR 'C'/'P' exists but is a no-op for individual equipment; there is no separate CHECK_30_DAY_CLEAN_CYCLE form in this module.)

a. Full clean/status history

- SELECT ENTRY_NUM, EQUI_ID, STATUS, STATUS_DATE, MATERIAL, ORDER_NUM, UPDAT_CB FROM /SMPL/ELB_EQUIP WHERE EQUI_ID = EQUNR ORDER BY ENTRY_NUM DESCENDING into LT_EQUI_CLEAN_DETAILS; if none: MESSAGE TEXT-005 and RAISE INITIAL_CLEAN_INCOMPLETE.

b. Last product used

- SELECT FOR ALL ENTRIES IN LT_USE_STAT WHERE EQUI_ID = EQUNR AND STATUS = LT_USE_STAT-table_line into LT_EQUI_MATERIAL_DETAILS; SORT BY ENTRY_NUM DESCENDING (empty tolerated).

c. Last clean date

- SELECT WHERE EQUI_ID = EQUNR AND ( STATUS = LV_CLEAN_1 OR LV_CLEAN_2 OR LV_CLEAN_3 ) ORDER BY ENTRY_NUM DESCENDING into LT_EQUI_DATE_DETAILS (empty tolerated).

5. Determine current status/material/date rows

- READ LT_EQUI_CLEAN_DETAILS index 1; if UPDAT_CB is blank (uncommitted, e.g. entered from Cockpit) READ index 2 instead; if that fails: MESSAGE TEXT-015 and RAISE INITIAL_CLEAN_INCOMPLETE. READ material/date index 1 (failures tolerated = first use / first clean).

6. Resolve product description and append to export tables

- APPEND alpha-formatted L_EQUIPMENT to ET_EQUI_ID. SELECT SINGLE MAKTX FROM MAKT (MATNR, SPRAS = SY-LANGU); per the latest change the raw material/part number is exported. APPEND MATERIAL to ET_EQUI_PROD, EQKTX to ET_EQUI_DESC, STATUS to ET_EQUI_STAT, STATUS_DATE to ET_EQUI_DATS, EQTYP to ET_EQUI_TYPE.

7. Derive and append the calibration due date

- CONVERSION_EXIT_ALPHA_INPUT on L_EQUIPMENT -> L_EQUI_FILT; SELECT SINGLE INOB~CUOBJ FROM INOB WHERE OBJEK = L_EQUI_FILT; SELECT SINGLE AUSP~ATWRT LEFT JOIN CABN WHERE OBJEK = CUOBJ AND ATINN = 0000001199.

- If found: SPLIT ATWRT AT '/' into month, year; CONCATENATE year, month, '01' -> ET_CALIB_DUE_DATE row. Else APPEND a cleared (initial) value (equipment treated as not calibrated). ENDLOOP.

Additional Notes:

- EXCEPTIONS raised: STATUS_NOT_CONFIGURED, ROOM_NOT_MAINTAINED, EQUIPMENT_DOES_NOT_EXIST, INITIAL_CLEAN_INCOMPLETE. CLEANING_REQUIRED is declared in the interface but never raised in current source.

- IV_PLANT and IV_TPLKZ are used only on the IV_FLOC_SETUP path (IV_TPLKZ optional). Key tables: /SMPL/ELB_EQUIP, /SMPL/ELB_EQSTAT (via the status PERFORM), plus ILOA, EQUZ, EQUI, IFLOT, EQKT, CRHD, MAKT, INOB, AUSP, CABN.

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

1. Function signature and constants

- Imports IM_CONTROL_RECIPE, IM_PHASE, IM_STEP_IDX, optionals IM_STEP, IM_SIGTYPE, IM_USERID, IM_DB_UP_DOWN, IM_VALIDATION_REQ, IM_STEP_UUID; no EXPORTING and no EXCEPTIONS.

- Constants LC_DEF_STRT_SEQID 1000, LC_YES 'Yes', LC_NO 'No', LC_BELOW 'B', LC_CB_SIG 'C', LC_TYPE_O 'O', LC_TRUE 'X', LC_NO_DROPDOWN '2'. Copy IM_SIGTYPE into LV_SIGTYPE; CASE on it. All DB work targets /SMPL/PPPI_PISST.

2. CASE '' (blank sigtype: Done By / step-name maintenance)

a. Update step name

- If IM_STEP is not initial, UPDATE /SMPL/PPPI_PISST SET STEP = IM_STEP WHERE STEP_SEQID = IM_STEP_IDX AND CTRL_RECIPE AND PHASE (skipped when empty to avoid blanking the name).

b. Insert a Done-By row ABOVE (IM_DB_UP_DOWN = '')

- SELECT MIN(UNIQUE_SEQID) for the step keys; CHECK found and not initial; SELECT SINGLE * for that row; set UNIQUE_SEQID = min - 1, SIGN_ID = IM_USERID, TYPE = TEXT-001; INSERT into /SMPL/PPPI_PISST; CLEAR.

c. Insert a Done-By row BELOW (IM_DB_UP_DOWN = 'B')

- Same but SELECT MAX(UNIQUE_SEQID); set UNIQUE_SEQID = max + 1, SIGN_ID, TYPE = TEXT-001; INSERT; CLEAR.

3. CASE 'C' (Checked By signature)

- SELECT SINGLE UNIQUE_SEQID into LV_STRT_UQ_SEQID WHERE step keys match AND TYPE = TEXT-002 (proceed only if found).

- End boundary: SELECT rows with UNIQUE_SEQID < start AND CTRL_RECIPE AND TYPE = TEXT-002; if found SORT descending and take nearest prior as LV_END_UQ_SEQID; if none default to 1000.

a. Mark the Checked By row complete

- UPDATE /SMPL/PPPI_PISST SET SIGN_ID = IM_USERID, COMPLETED = 'X' WHERE step keys, UNIQUE_SEQID = start, TYPE = TEXT-002.

b. Cascade completion to the intervening steps

- Only if start and end not initial; if end = 1000 reset to 1 to include the first XStep. SELECT rows with UNIQUE_SEQID < start AND > end AND CTRL_RECIPE; LOOP set COMPLETED = 'X'; MODIFY /SMPL/PPPI_PISST FROM TABLE.

4. CASE 'O' (validation-required flag)

- If IM_VALIDATION_REQ = '2' set LV_VALIDATION = 'No' else 'Yes'. UPDATE /SMPL/PPPI_PISST SET VALIDATION_REQUIRED = LV_VALIDATION WHERE STEP_SEQID = IM_STEP_IDX AND CTRL_RECIPE AND PHASE.

5. Cleanup

- After ENDCASE CLEAR LV_SIGTYPE, LV_STRT_UQ_SEQID, LV_END_UQ_SEQID and the cascade table.

Additional Notes:

- No EXCEPTIONS are declared or raised; the module relies on SY-SUBRC/CHECK statements to exit silently. No explicit COMMIT WORK (the caller's LUW commits).

- TEXT-001 is the TYPE assigned to new Done-By rows; TEXT-002 identifies Check-By rows. IM_SIGTYPE values other than '', 'C', 'O' perform no action.

Function: /SMPL/ELB_FM_UPDATE_LB_USER

1. Interface and local data setup

- IMPORTING IV_USER, IV_DB_FLAG (optional), IV_ORDER, IV_VORNR (optional), IM_CRID (optional). EXCEPTIONS EQUIPMENT_NOT_UPDATED, TABLE_LOCK_NOT_POSSIBLE. Locals: LT_EQUI_DETAILS (/SMPL/ELB_EQUIP), BAPI work areas, flags L_UPDATE_CHECK / L_CB_CHECK, L_EQUIP_ID; constants LC_ERROR 'E', LC_PENDING 'PENDING'.

2. Select logbook equipment entries

- SELECT * FROM /SMPL/ELB_EQUIP WHERE ORDER_NUM = IV_ORDER AND (IV_VORNR IS NULL OR ACTIVITY = IV_VORNR) AND (IM_CRID IS NULL OR CR_NR = IM_CRID) ORDER BY ENTRY_NUM DESCENDING into LT_EQUI_DETAILS. An empty result raises no exception.

3. Loop entries and determine update type

a. Done-By branch

- If IV_DB_FLAG = ABAP_TRUE AND UPDAT is initial: set UPDAT = IV_USER, L_UPDATE_CHECK = 'X' (no equipment-master update).

b. Checked-By branch (UPDAT_CB initial)

- ElseIf IV_DB_FLAG initial AND UPDAT_CB initial: set UPDAT_CB = IV_USER, both flags set; prepare EQUI sortfield = STATUS; L_EQUIP_ID = EQUI_ID via CONVERSION_EXIT_ALPHA_INPUT.

c. Checked-By branch (UPDAT_CB = 'PENDING')

- ElseIf IV_DB_FLAG initial AND UPDAT_CB = 'PENDING': same actions as branch (b).

4. Lock table and persist the row update

- When L_UPDATE_CHECK is set: ENQUEUE_E_TABLE ('/SMPL/ELB_EQUIP', mode 'E'); if it fails RAISE TABLE_LOCK_NOT_POSSIBLE. UPDATE /SMPL/ELB_EQUIP FROM the row; if it fails RAISE EQUIPMENT_NOT_UPDATED. DEQUEUE_E_TABLE.

5. Update equipment master on second signature

- When L_CB_CHECK is set: BAPI_EQUI_CHANGE (EQUIPMENT = L_EQUIP_ID, DATA_GENERAL/X carrying SORTFIELD = STATUS). If RETURN-TYPE = 'E' RAISE EQUIPMENT_NOT_UPDATED. (No BAPI_TRANSACTION_COMMIT inside the module.) The Done-By branch never triggers this.

6. Loop continuation

- ENDLOOP: each qualifying row is independently locked, updated, unlocked, and (for Checked-By rows) pushed to the equipment master.

Additional Notes:

- IV_DB_FLAG drives branching: ABAP_TRUE = Done By (writes UPDAT), initial = Checked By (writes UPDAT_CB), handling both an empty field and a literal 'PENDING' placeholder.

- Locking is per-iteration (ENQUEUE/UPDATE/DEQUEUE inside the loop). No COMMIT is issued within the module; the caller's LUW commits.

Function: /SMPL/PPPI_FM_SIG_POPULATE_CB

1. Define interface, local types, and constants

- IMPORTING IM_LABEL_ID (CHAR1), IM_STEP_NAME (CHAR40, optional), IM_STEP_INDEX (INT4), IM_STEP_UUID, IM_CONTROL_RECIPE (POC_DOCID), IM_PHASE (VORNR), IM_PO_NR (AUFNR, optional), IM_PLANT (WERKS_D, optional). EXCEPTIONS: TABLE_LOCK_NOT_POSSIBLE.

- Constants LC_PROC_ORDER (PLNAW 'C'), LC_PROD_ORDER (PLNAW 'P'), LC_VALIDATION_REQUIRED 'Yes', LC_CB_SIG 'C', LC_MODE 'E', LC_PISST_TABLE '/SMPL/PPPI_PISST'. CLEAR/REFRESH all working data.

2. Build-structure-only-once guard

- SELECT COUNT(*) FROM /SMPL/PPPI_PISST WHERE CTRL_RECIPE = IM_CONTROL_RECIPE AND PO_NR = IM_PO_NR. If no rows exist yet, execute the structure-build block (steps 3-6); otherwise skip directly to the Checked-By update (step 7).

3. Resolve master recipe, control recipe destination, and order type

- SELECT SINGLE PLNNR, PLNAL FROM AFKO WHERE AUFNR = IM_PO_NR into LS_MR_INFO; if not found, RETURN (abort, no exception).

- SELECT SINGLE PHSEQ FROM COCH WHERE CRID = IM_CONTROL_RECIPE into LV_CONTROL_REC_DEST; if not found, RETURN (abort).

- SELECT SINGLE PLNAW FROM AFKO into LV_ORDTYPE to determine order type (missing -> default Process Order). Set L_DATE = SY-DATUM.

4. Determine SMPL order type and retrieve SOG data

- If LV_ORDTYPE = 'P', LV_TYPE = /SMPL/MBR_CL_COMMON_API=>CV_TYPE_PRODORDER, else CV_TYPE_PORDER (process order).

- In a TRY block CALL /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA (IV_TYPE = LV_TYPE, IV_PLANT = IM_PLANT, IV_KEY1 = IM_PO_NR, IV_DATUV = L_DATE) returning LT_SOG_DATA; CATCH /SMPL/MBR_CL_T100 (handler empty - swallowed).

5. Filter SOG data and insert rows under lock

- LOOP LT_SOG_DATA; APPEND only rows whose DEST_NAME = LV_CONTROL_REC_DEST into LT_CRID_DATA.

- ENQUEUE_E_TABLE (mode 'E', '/SMPL/PPPI_PISST'); if it fails RAISE TABLE_LOCK_NOT_POSSIBLE.

- LOOP LT_CRID_DATA and build each /SMPL/PPPI_PISST row (STEP_SEQID = STEP_INDEX, STEP = XSTEP_NAME, STEP_UUID, MASTER_RECIPE/RECIPE_GROUP from LS_MR_INFO, CTRL_RECIPE, PO_NR, CREATED_DATE, PHASE, VALIDATION_REQUIRED = 'Yes'); INSERT into /SMPL/PPPI_PISST. Then DEQUEUE_E_TABLE.

6. Assign unique sequence IDs

- SELECT * FROM /SMPL/PPPI_PISST WHERE PO_NR = IM_PO_NR AND CTRL_RECIPE = IM_CONTROL_RECIPE into LT_STRUCTURE; SORT BY PHASE, STEP_SEQID ascending.

- LOOP LT_STRUCTURE; increment LV_UNIQUE_INDEX by 1000 each row and UPDATE /SMPL/PPPI_PISST SET UNIQUE_SEQID = LV_UNIQUE_INDEX keyed on PO_NR, CTRL_RECIPE, STEP_SEQID, STEP, PHASE (yields 1000, 2000, ... ordered by phase then step).

7. Apply Checked-By (CB) labeling

- Runs whether or not the structure was just built: IF IM_LABEL_ID = 'C' this is a Checked-By signature.

- If IM_STEP_NAME is not initial: UPDATE /SMPL/PPPI_PISST SET TYPE = TEXT-002 and STEP = IM_STEP_NAME WHERE STEP_SEQID = IM_STEP_INDEX AND CTRL_RECIPE AND PHASE. If IM_STEP_NAME is initial: UPDATE only TYPE = TEXT-002 (leave STEP unchanged so an empty header does not overwrite the step name).

Additional Notes:

- The build-only-once guard ensures the validation structure is constructed by the first step to call the FM; later calls fall through to CB labeling. TABLE_LOCK_NOT_POSSIBLE is the only declared EXCEPTION; /SMPL/MBR_CL_T100 is caught and ignored.

- Two RETURN statements (missing AFKO master recipe, or missing COCH destination) abort early without an exception. Order type is dynamic via AFKO-PLNAW ('P' = production, otherwise process). TEXT-002 supplies the Checked-By label text.

Function: /SMPL/ELB_FM_ADD_EQUIP

1. Initialize and resolve cleaning/use status configuration

- Declare local types/work areas; CLEAR/REFRESH locals. Constants: language 'E', max date '99991231', clean class 'C', use class 'U', EQLFN '001', error types 'C'/'P', characteristic ATINN 0000001199 (ZSMPL_CHAR_CALIB_DUE_DATE_CHAR).

- PERFORM GET_EQ_STAT_BY_CLASS class 'C' into LT_CLEAN_STATS; READ 1 -> LV_CLEAN_1 (else RAISE STATUS_NOT_CONFIGURED), 2 -> LV_CLEAN_2 (else RAISE), 3 -> LV_CLEAN_3 (else default LV_CLEAN_2). PERFORM class 'U' into LT_USE_STAT; READ 1 -> LV_IN_USE (else RAISE STATUS_NOT_CONFIGURED).

2. Resolve and validate the equipment master record

- CONVERSION_EXIT_ALPHA_INPUT on IV_EQUIPMENT -> L_EQUNR. SELECT SINGLE from EQUI LEFT JOIN EQUZ, ILOA, CRHD, EQKT (SPRAS = SY-LANGU) retrieving EQUNR, EQKTX, EQFNR, EQTYP.

a. WHERE filters

- EQUI~EQUNR = L_EQUNR AND EQUZ~DATBI = '99991231' AND EQUZ~EQLFN = '001'. If not found: MESSAGE TEXT-023 and RAISE EQUIPMENT_DOES_NOT_EXIST.

3. Check 30-day clean cycle and standardization

- PERFORM CHECK_30_DAY_CLEAN_CYCLE USING L_EQUNR L_ERROR. SELECT SINGLE ID FROM ICON WHERE NAME = 'ICON_MESSAGE_WARNING'.

a. Clean-cycle outcome (conditional)

- If L_ERROR = 'C': MESSAGE TEXT-001 (the RAISE CLEANING_REQUIRED here is commented out, so execution continues). If L_ERROR = 'P': CONCATENATE TEXT-007, L_EQUNR, TEXT-008; MESSAGE and RAISE CLEANING_REQUIRED.

b. Standardization check

- PERFORM STANDARDIZATION_CHECK; if L_ERROR = 'S': MESSAGE TEXT-016 and RAISE STANDARDIZATION_REQUIRED.

4. Read existing logbook entries from /SMPL/ELB_EQUIP

- SELECT clean history ORDER BY ENTRY_NUM DESCENDING into LT_EQUI_CLEAN_DETAILS; if none RAISE INITIAL_CLEAN_INCOMPLETE. SELECT last material FOR ALL ENTRIES IN LT_USE_STAT into LT_EQUI_MATERIAL_DETAILS. SELECT last clean dates (STATUS in LV_CLEAN_1/2/3) into LT_EQUI_DATE_DETAILS (empties tolerated).

5. Select the relevant detail rows

- READ clean index 1; if UPDAT_CB blank READ index 2 (clear if missing). READ material index 1 (default MATERIAL = TEXT-010 if missing). READ date index 1 (tolerated).

6. Populate output parameters

- EV_EQUI_ID = IV_EQUIPMENT. SELECT SINGLE MAKTX FROM MAKT into EV_STAT_PROD (default TEXT-010). EV_STAT_PN = MATERIAL; EV_EQUI_DESC = EQKTX; EV_EQUI_STAT = clean STATUS; EV_STAT_DATS/TIMS = date row; EV_EQUI_TYPE = clean TYPE.

7. Derive calibration due date from characteristics

- CONVERSION_EXIT_ALPHA_INPUT on IV_EQUIPMENT -> L_EQUI_FILT; SELECT SINGLE INOB~CUOBJ WHERE OBJEK = L_EQUI_FILT; SELECT SINGLE AUSP~ATWRT LEFT JOIN CABN WHERE OBJEK = CUOBJ AND ATINN = 0000001199.

a. Date conversion (conditional)

- If found: SPLIT ATWRT AT '/' into month, year; CONCATENATE year, month, '01' -> EV_CALIB_DUE_DATE. Else CLEAR (equipment treated as not calibrated).

Additional Notes:

- EXCEPTIONS: EQUIPMENT_DOES_NOT_EXIST, STATUS_NOT_CONFIGURED, INITIAL_CLEAN_INCOMPLETE, CLEANING_REQUIRED (only on the 'P' branch; the 'C' RAISE is commented out), STANDARDIZATION_REQUIRED.

- Statuses are configuration-driven via GET_EQ_STAT_BY_CLASS (classes 'C' and 'U'); LV_CLEAN_3 falls back to LV_CLEAN_2. All operator messages are TYPE 'S' DISPLAY LIKE 'E'.

Function: /SMPL/MBR_DEP_ADD_PERFORM

1. Function interface and inputs

- Imports IM_CONTROL_RECIPE, IM_AUFNR (optional), IM_USERID, IM_STEP_UUID (/SMPL/MBR_SOG_UUID), IM_DEST_NAME (/SMPL/MBR_CMX_XS_DST_NAME). Declares EXCEPTIONS USER_INITIAL and USER_NOT_SIGNED_INTO_SIG_TAB.

a. Note on exceptions

- The implementation body contains no RAISE statement; both declared exceptions are present in the signature but are never raised by this module as read. No initial-check of IM_USERID is performed here.

2. Declare the signature work area

- Declare LV_SIG_ENTRY typed as structure /SMPL/MBR_OR_SIG (the row type of table /SMPL/MBR_OR_SIG).

3. Build the signature record (VALUE constructor)

- AUFNR = IM_AUFNR; SOG_UUID = IM_STEP_UUID; TYPE = GC_PERFORM_SIGNATURE (perform-signature constant, CHAR16 from /SMPL/MBR_CL_COMMON_API=>CV_SIG_TEXT_PERFORM); SIGN_ID = IM_USERID; CREATED_DATE/TIME = SY-DATUM/SY-UZEIT; DEST_NAME = IM_DEST_NAME; SIGN_DATE/TIME = SY-DATUM/SY-UZEIT.

a. Fields not set

- IM_CONTROL_RECIPE is imported but is not used to populate any field of the record.

4. Persist the signature row

- INSERT INTO /SMPL/MBR_OR_SIG VALUES LV_SIG_ENTRY. No explicit SY-SUBRC check, COMMIT WORK, or duplicate-key handling; transactional consistency is left to the caller.

Additional Notes:

- The module records a "perform" signature only; the parallel "verify" path uses GC_VERIFY_SIGNATURE. Both constants are defined in the function-pool top include /SMPL/LMBR_DEP_SIGTOP from /SMPL/MBR_CL_COMMON_API.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Receive inputs

- IMPORTING IV_ACTIVE (CHAR1, optional) - the flag indicating whether the value is inside the active range. EXPORTING EV_ACTIVE (CHAR1) - the resulting active indicator.

2. Evaluate active state

a. Compare IV_ACTIVE against the constant GC_INSIDE_RANGE (value '1').

- If IV_ACTIVE is not equal to GC_INSIDE_RANGE, set EV_ACTIVE = GC_NO (value '2'). Else (IV_ACTIVE = '1'), set EV_ACTIVE = GC_YES (value '1').

Additional Notes:

- Constants GC_INSIDE_RANGE, GC_YES, GC_NO are declared in the function-group TOP include /SMPL/LPPPI_FG_UTIL_REFSTOP (GC_INSIDE_RANGE = GC_YES = '1', GC_NO = '2').

- IV_ACTIVE is OPTIONAL; if not supplied it is initial (empty), which is not equal to GC_INSIDE_RANGE, so EV_ACTIVE defaults to GC_NO ('2'). No database access, validation, or exceptions are present.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Validate input and initialize

- IMPORTING IM_CONTROL_RECIPE (POC_DOCID), IM_PHASE (VORNR, optional), IM_AUFNR (AUFNR, optional), IM_STEP_UUID (/SMPL/MBR_SOG_UUID), IM_PLANT (WERKS_D). EXPORTING EV_ACTIVE (CHAR1).

- Declare local table LT_ORD_SIGS as STANDARD TABLE OF /SMPL/MBR_OR_SIG. If IM_CONTROL_RECIPE is initial, RAISE CONTROL_RECIPE_INITIAL (the only exception defined).

2. Load (cache) the order's dependency data

a. PERFORM LOAD_INTERNAL_DATA USING IM_AUFNR, IM_CONTROL_RECIPE, IM_STEP_UUID

- The FORM populates the function-group global G_ORD_DATA only when it is empty OR when IM_AUFNR / IM_CONTROL_RECIPE differ from the cached K_AUFNR / K_CRID (a per-order cache, re-read on order/recipe change).

- On (re)load it SELECTs * FROM /SMPL/MBR_OR_DEP WHERE AUFNR = IM_AUFNR into G_ORD_DATA-T_ORD_DEPS (the step dependency/predecessor rows), and SELECTs SINGLE PLNAW FROM AFKO to set V_TYPE (production vs process order via /SMPL/MBR_CL_COMMON_API).

3. Refresh the order's signature data

- Always re-SELECT AUFNR, SOG_UUID, TYPE, CREATED_DATE/TIME, DEST_NAME, SIGN_ID, SIGN_DATE/TIME FROM /SMPL/MBR_OR_SIG into LT_ORD_SIGS WHERE AUFNR = IM_AUFNR (signatures change frequently).

- Each signature row carries TYPE (perform vs verify) and SIGN_ID; an empty SIGN_ID (ABAP_FALSE) marks a signature not yet signed.

4. Evaluate the "before start" dependencies targeting this step

- LOOP over G_ORD_DATA-T_ORD_DEPS into LS_STEP_DEP WHERE TARGET_UUID = IM_STEP_UUID AND ( DEP_RULE = GC_PERFORM_BEFORE_START_RULE 'PERFORM_START' OR DEP_RULE = GC_VERIFY_BEFORE_START_RULE 'VERIFY_START' ).

- Only rules that gate the START of this step are considered; PREREQ_UUID identifies the predecessor step that must be completed first.

5. For each dependency, check whether the prerequisite signature is missing

a. If DEP_RULE = PERFORM_START

- Inner LOOP over LT_ORD_SIGS WHERE SOG_UUID = LS_STEP_DEP-PREREQ_UUID AND TYPE = GC_PERFORM_SIGNATURE AND SIGN_ID = ABAP_FALSE; EXIT on first hit (prerequisite's PERFORM signature still unsigned).

b. If DEP_RULE = VERIFY_START

- Inner LOOP over LT_ORD_SIGS WHERE SOG_UUID = LS_STEP_DEP-PREREQ_UUID AND TYPE = GC_VERIFY_SIGNATURE AND SIGN_ID = ABAP_FALSE; EXIT on first hit (prerequisite's VERIFY signature still unsigned).

6. Set the active/inactive result

- Immediately after the inner LOOP: if SY-SUBRC = 0 an unsigned prerequisite was found, so EV_ACTIVE = GC_STEP_INACTIVE ('1') - the step cannot start yet. If SY-SUBRC <> 0 the prerequisite is complete, so EV_ACTIVE = GC_STEP_ACTIVE ('2').

- CLEAR LS_STEP_DEP and continue; the last matching dependency processed determines the final EV_ACTIVE (overwritten each iteration).

Additional Notes:

- Read-only FM (no DB updates). Tables read: /SMPL/MBR_OR_DEP (dependencies, via the cached FORM), /SMPL/MBR_OR_SIG (signatures, always refreshed), and AFKO (order type). Constants from /SMPL/LMBR_DEP_SIGTOP: GC_STEP_ACTIVE '2', GC_STEP_INACTIVE '1', GC_PERFORM_BEFORE_START_RULE 'PERFORM_START', GC_VERIFY_BEFORE_START_RULE 'VERIFY_START'.

- IM_PHASE and IM_PLANT are declared but not referenced in the logic; IM_AUFNR, IM_CONTROL_RECIPE and IM_STEP_UUID drive all behavior. If the step has no matching before-start dependency, EV_ACTIVE is left unchanged by this FM.

Function: ZSMPL_FM_VALIDATE_STATUS_EBR

1. Initialize and set up EBR websocket messaging

- Declare constants LC_ICON_WARN ('ICON_MESSAGE_WARNING'), LC_WARN_TXT ('WARNING'); working vars L_RESPONSE, L_STATUS_UPD (/SMPL/STR_DE_STAT_CLASS), L_ID. Instantiate the EBR websocket LCL_WEBSOCKET (/SMPL/EBR_CL_APC_WS) and CREATE handler LO_SOCKET (/SMPL/EBR_CL_WEB_SOCK_HNDL).

2. Evaluate the IV_BYPASS flag

a. Bypass = '0'

- If IV_BYPASS EQ '0' (not initial): EXIT immediately - no validation performed.

b. Bypass = '2' (line clearance not complete)

- If IV_BYPASS EQ '2' (not initial): MESSAGE TEXT-009 (type 'S' like 'E') and RAISE EQUIPMENT_STATUS_INCORRECT. Any other value falls through to validation.

3. Run equipment cleaning rules and the status CLASS engine

- PERFORM EQUIPMENT_CLEANING_RULES USING IV_EQUI_STAT L_RESPONSE L_STATUS_UPD. Inside, SELECT SINGLE * FROM /SMPL/ELB_EQSTAT WHERE EQ_STAT = the status; branch on CLASS(1).

- 'U' Use -> L_RESPONSE = TEXT-004 (not updateable). 'C' Cleaned -> continue, L_STATUS_UPD 'U1'. 'P' Pending -> L_RESPONSE = TEXT-005. 'S' Standardized -> continue, 'U1'. 'G1' Standardization Required -> L_RESPONSE = TEXT-016. 'M' Multi-order -> continue, 'U2'. OTHERS (incl 'I','G') -> L_RESPONSE = TEXT-006.

4. Act on the cleaning-rules response (with multi-PO branch)

- Proceed only if L_RESPONSE is not initial.

a. Multi-PO and response = TEXT-004

- If IV_MULTI_PO = ABAP_TRUE AND L_RESPONSE = TEXT-004: non-fatal "assigned in another order" warning. In EBR (GV_FUNCTION = 'SMPL_EBR') push a frontend warning via /SMPL/EBR_CL_WEB_SOCK_HNDL=>PREPARE_MESSAGE_FOR_FRONTEND; else MESSAGE (TEXT-004 - IV_EQUIPMENT) displayed like 'W' and continue.

b. All other cases

- MESSAGE TEXT-004 (type 'S' like 'E') and RAISE EQUIPMENT_STATUS_INCORRECT (equipment not updateable).

5. Standardization check

- SELECT SINGLE ID FROM ICON WHERE NAME = LC_ICON_WARN. PERFORM STANDARDIZATION_CHECK USING L_EQUNR L_ERROR. If L_ERROR = 'S': CALL FUNCTION 'POPUP_TO_INFORM' (titel LC_WARN_TXT, txt1 L_ID, txt2 TEXT-016). Informational only - no exception.

6. Calibration due-date check

- L_TODAY = SY-DATUM. If IV_CALIB_DUE_DATE is not initial and NE '0' and LT L_TODAY: MESSAGE TEXT-022 (type 'S' like 'E') and RAISE CALIBRATION_DUE_DATE_INCORRECT. If initial or '0', the check is skipped.

Additional Notes:

- IMPORTING IV_EQUIPMENT, IV_EQUI_STAT, optional IV_BYPASS, IV_CALIB_DUE_DATE, IV_MULTI_PO; EXCEPTIONS EQUIPMENT_STATUS_INCORRECT and CALIBRATION_DUE_DATE_INCORRECT.

- Status acceptance is configuration-driven via /SMPL/ELB_EQSTAT status CLASS (U/C/P/S/G1/M), not a hardcoded status list. EBR frontend messaging is used only when GV_FUNCTION = 'SMPL_EBR'. Reflects modifications MOD1-MOD4 (line clearance, standardization, calibration, multi-PO).

Function: /SMPL/PPPI_FM_SIG_VALIDATION

1. Initialize and validate mandatory inputs

- Declares locals for master-recipe info (PLNNR, PLNAL), step dependencies (/SMPL/MBR_DEP, /SMPL/MBR_DEP_TT), constants LC_DEF_STRT_SEQID 1000, LC_PROD_ORDER 'P', LC_YES 'Yes', LC_CB_SIG 'C', LC_PERFORM 'P', LC_VERIFY 'V'. CLEAR/REFRESH all working data.

- If IM_CONTROL_RECIPE is initial, RAISE CONTROL_RECIPE_INITIAL. If IM_USERID is initial, RAISE USER_INITIAL.

2. Resolve master recipe, control-recipe destination, and order type

a. Master recipe lookup

- SELECT SINGLE PLNNR, PLNAL FROM AFKO WHERE AUFNR = IM_PO_NR; if not found RAISE XSTEP_CONFIG.

b. Control recipe destination

- SELECT SINGLE PHSEQ FROM COCH WHERE CRID = IM_CONTROL_RECIPE into LV_CONTROL_REC_DEST; if not found RAISE XSTEP_CONFIG.

c. Order type

- SELECT SINGLE PLNAW FROM AFKO into LV_ORDTYPE (no exception if missing). Set L_DATE = SY-DATUM; LV_TYPE = production/process constant from /SMPL/MBR_CL_COMMON_API based on PLNAW = 'P'.

3. Retrieve and filter SiMPL dependency data

- CALL /SMPL/MBR_CL_DB_SERVICE=>GET_MOST_RECENT_SMPL_DATA (IV_TYPE, IV_PLANT = IM_PLANT, IV_KEY1 = IM_PO_NR, IV_DATUV = L_DATE) into LT_STEP_DEP; CATCH /SMPL/MBR_CL_T100 swallows errors.

- LOOP LT_STEP_DEP, keep rows where DEST_NAME = LV_CONTROL_REC_DEST AND DEP_UUID = IM_STEP_UUID into LT_CRID_DEP_DATA.

4. Predecessor step completion check (currently inert)

- For each filtered dependency, SELECT SINGLE * FROM /SMPL/PPPI_PISST WHERE STEP_UUID = SOG_UUID AND CTRL_RECIPE = IM_CONTROL_RECIPE AND VALIDATION_REQUIRED = 'Yes'. The inline RAISE PREDECESSOR_NOT_COMPLETED is commented out ("MBR handles the dependencies"); enforcement is deferred to step 7.

5. Signature-table validation

- SELECT SINGLE * FROM /SMPL/PPPI_PIBSI WHERE PI_SHEET = IM_CONTROL_RECIPE AND SIGNATURE = IM_USERID. If not found: MESSAGE (IM_USERID && TEXT-004) and RAISE USER_NOT_SIGNED_INTO_SIG_TAB.

6. Check-By segregation-of-duties validation (Performed-By conflict)

- Only when LV_SIGTYPE (= IM_LABEL_ID) = 'C' AND IM_CB_VALIDATE = ABAP_TRUE.

- Get the start index: SELECT SINGLE UNIQUE_SEQID FROM /SMPL/PPPI_PISST WHERE STEP_SEQID = IM_STEP_INDEX, STEP_UUID = IM_STEP_UUID, CTRL_RECIPE, PHASE = IM_PHASE, TYPE = TEXT-002 (proceed only if found).

- Lower bound = highest UNIQUE_SEQID < start with same CTRL_RECIPE/PHASE and TYPE = TEXT-002; if none, default 1000.

- SELECT COUNT(*) UP TO 1 ROWS WHERE UNIQUE_SEQID strictly between end and start, same CTRL_RECIPE/PHASE, TYPE = TEXT-001, SIGN_ID = IM_USERID. If found: MESSAGE (IM_USERID && TEXT-015) and RAISE USER_SIGNED_PERF_BY.

7. Final EBR step / required-steps completion, then MBR delegation

a. Identify the last EBR step

- SELECT MAX(UNIQUE_SEQID) FROM /SMPL/PPPI_PISST WHERE CTRL_RECIPE = IM_CONTROL_RECIPE; if none RAISE XSTEP_CONFIG. SELECT SINGLE * for that row; if not found RAISE XSTEP_CONFIG.

b. Required-EBR-steps completion

- If the last step's STEP_UUID = IM_STEP_UUID: SELECT rows WHERE CTRL_RECIPE = IM_CONTROL_RECIPE AND STEP_UUID <> IM_STEP_UUID AND COMPLETED <> 'X' AND VALIDATION_REQUIRED = 'Yes' AND TYPE = TEXT-002. If any are returned: MESSAGE TEXT-016 and RAISE REQ_EBR_STEPS_NOT_COMPLETED.

c. MBR dependency signature validation

- LV_STYPE = 'V' if IM_LABEL_ID = 'C' else 'P'. CALL /SMPL/MBR_DEP_VALI_SIGN (IM_SIG_TYPE, IM_CONTROL_RECIPE, IM_PHASE, IM_AUFNR = IM_PO_NR, IM_USERID, IM_STEP_UUID, IM_PLANT). Map SY-SUBRC 1:1 to re-RAISE: 1 CONTROL_RECIPE_INITIAL, 2 USER_INITIAL, 3 USER_NOT_SIGNED_INTO_SIG_TAB, 4 USER_SIGNED_DONE_BY, 5 PREDECESSOR_NOT_COMPLETED, 6 REQ_EBR_STEPS_NOT_COMPLETED, 7 USER_SIGNED_PERF_BY, 8/OTHERS CONTROL_RECIPE_INITIAL. On success CLEAR locals and return.

Additional Notes:

- USER_SIGNED_DONE_BY and PREDECESSOR_NOT_COMPLETED are only raised via the SY-SUBRC mapping from /SMPL/MBR_DEP_VALI_SIGN (subrc 4 and 5); the inline predecessor RAISE is commented out.

- XSTEP_CONFIG is the catch-all configuration/structure failure (missing AFKO master recipe, COCH destination, or /SMPL/PPPI_PISST EBR structure). TEXT-001 = Performed-By type, TEXT-002 = Checked-By type. Source: DE1 100, group /SMPL/PPPI_FG_EBR_FUNC_SIG.

## Test Scenarios

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: Room/Equipment Assign | Process Order created and Released containing SMPL: Room/Equipment Assign |  |
| 2 | Open the EBR, and navigate to SMPL: Room/Equipment Assign | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Room/Equipment Assign is visible | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Room/Equipment Assign is visible |  |
| 3 | The value entered in parameter IV_HEADER and parameter IV_INSTR in SiMPL MBR appears correctly in the XStep header | Header and instruction text are displayed as authored in SiMPL MBR | Header and instruction text are displayed correctly. |  |
| 4 | The operator enters the “Room #”. And then executes the “Retrieve Room Equipment Detail” button that retrieves all the equipment linked to the specific room. The following fields are populated in the equipment table: Equipment #, Description, Current status, Last Cleaning Date, Last Prod. Part No. | The following information is retrieved and displayed in the equipment table: Equipment #, Description, Current status, Last Cleaning Date, Last Prod. PN | The following information is retrieved and displayed in the equipment table: Equipment #, Description, Current status, Last Cleaning Date, Last Prod. PN |  |
| 7 | The relevant equipment required should be selected in “Equipment Required?” column as Yes. | Dropdown selection of “Yes” is accepted | Dropdown selection of “Yes” is accepted |  |
| 8 | Enter a username in the “Performed By” field then enter the password in the pop-up window. | Signature is accepted | Signature is accepted |  |
| 10 | Indicate if additional equipment is required in “Add. Equipment Required?” field. | “Yes” unlocks Additional Equipment Table | “Yes” unlocks Additional Equipment Table |  |
| 11 | The operator enters the “Room #”. And then executes the “Retrieve Room Equipment Detail” button that retrieves all the equipment linked to the specific room. The following fields are populated in the equipment table: Equipment #, Description, Current status, Last Cleaning Date, Last Prod. Part No. | The following information is retrieved and displayed in the equipment table: Equipment #, Description, Current status, Last Cleaning Date, Last Prod. PN | The following information is retrieved and displayed in the equipment table: Equipment #, Description, Current status, Last Cleaning Date, Last Prod. PN |  |
| 12 | “Equipm Required?” selected as “Yes” and accepted if cleaning date is accepted and equipment status is valid for assignment. | The operator is able to select the required equipment. | The operator is able to select the required equipment. |  |
| 13 | Enter a username in the “Performed By” field then enter the password in the pop-up window. | Signature is accepted and the table line is locked. | Signature is accepted and the table line is locked. |  |

## Document References

| Reference No. | Document Title |
| --- | --- |
| Number used in superscript for reference | Full name of referenced document |
| 1 | SiMPL Functional Guide: Universal Concepts |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.1 | Initial Document (Functional Requirements and Technical Specification) | Karl Uys and Frieda van der Merwe | 23-07-2025 |
| 1.2 | The following Technical Spec sections added to spec: Function Module(s), and Pseudocode. | James Church | 19-06-2026 |
| 1.3 | Updated to align the specification with the current XStep version. Corrected the room/equipment retrieval function module to ZSMPL_FM_FLOC_EQUIP and expanded its interface (added IV_FLOC_SETUP, IV_PLANT and IV_TPLKZ import parameters, ET_CALIB_DUE_DATE export parameter, and the STATUS_NOT_CONFIGURED, INITIAL_CLEAN_INCOMPLETE, ROOM_NOT_MAINTAINED and CLEANING_REQUIRED exceptions). Added Function Module sections for ZSMPL_FM_VALIDATE_STATUS_EBR (from source) and /SMPL/PPPI_FM_SIG_VALIDATION (full interface read from DE1 client 100). Added the Calib. Due Date and Last Prod. Part No. output columns. Documented calibration due-date, line-clearance, standardization and multi-order (multi-PO) validation checks. | [Author] | 22-06-2026 |