V 1.2

June 2026

XStep Design Specification

SMPL: Component Addition

1	Purpose	3

2	Overview	3

3	Reasons for developing	4

4	Authorization	4

5	Assumptions/ Dependencies	4

6	Validation Checks	5

7	Configuration Specification	6

8	XStep Layout Design	6

9	Function Module(s)	7

10	Pseudocode	10

11	Test Scenarios	17

12	Revision History	19

## Purpose

This document outlines the functional and technical design, along with the configuration specifications for the XStep “SMPL: Component Addition”.

## Overview

The SMPL: Component Addition XStep is designed to guide the operator through the component addition (charge-in) process, ensuring that each previously weighed material is added to the designated mixing vessel in the specified sequence. The step is dynamically linked to SAP Master Data, which governs the display order and verification of each component. The sequential addition is enforced via a structured table within the EBR, promoting consistency, traceability, and compliance with GMP standards.

This step is dependent on the visibility condition authored in SiMPL MBR, which is based on the BOM Item Numbers, ensuring that only relevant components appear for the operator at the correct point in the process.

The XStep contains two main input parameters:

- IV_HEADER - This can be changed in SiMPL MBR to display specific header text for the XStep

- IV_INSTR - This can be changed in SiMPL MBR to display a specific set of instructions for the XStep that will be displayed below the header

- IV_BOM_BEG – This is authored in SiMPL MBR to determine the weighed components from which BOM Item range for retrieval.  This parameter indicates the lower limit of the current steps of the BOM item range.

- IV_BOM_END – This is authored in SiMPL MBR to determine the weighed components from which BOM Item range for retrieval.  This parameter indicates the upper limit of the current steps of the BOM item range.

The operator executes the “Component Addition” button:

- Component Addition - Function Button

This button is linked to the function module - ZSMPL_FM_COMP_ADDITION_ITEMS that verifies the component details (as weighed out previously in the EBR) with the BOM or any additional weigh outs and retrieves the following information:

- “Component Number” - Output

- “Component Description” – Output

- “Amount Added” - Output

- “UoM” - Output

The individual performing this component addition step then records their signature in the “Performed By” field.

Functional Design

## Reasons for developing

The SMPL: Component Addition XStep is being developed to ensure accurate, traceable, and GMP-compliant component addition of previously weighed materials. By integrating with SAP Master Data, the XStep guides operators through the component charge-in process in the correct sequence, reducing the risk of errors and ensuring alignment with the approved recipe.

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | User signing signature fields is an approved user in the system with the correct access configuration |
| 2 | Master Data has been set up correctly |
| 3 | The function module are available and returns the correct information |
| 4 | The function module can successfully pull the data into the output fields |

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| User Signature rejected due to an incorrect username or password | User accepts error message and re-enter the correct username and password |
| Component Numbers not found | Check Master Data |

## Configuration Specification

N/A

## XStep Layout Design

## Function Module(s)

Function: ZSMPL_FM_COMP_ADDITION_ITEMS

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_BOM_START | CHAR4 | BOM Item Start |
| IV_BOM_END | CHAR4 | BOM Item End |
| IV_CONTROL_RECIPE | POC_DOCID | Domain Model: Document Identifier |
| IV_ORDER | AUFNR | Order Number |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_MATERIAL | /SMPL/STR_TT_MATNR | Table type for Materials |
| EV_MATERIAL_DESC | /SMPL/STR_TT_MAKTX | Table type for Material Description |
| EV_QTY | /SMPL/STR_TT_QTY | Table type for batch quantity |
| EV_UOM | /SMPL/STR_TT_UOM | Table type for UOM |

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | Control Recipe Number |
| IM_PHASE | VORNR | Activity Number |
| IM_STEP_IDX | INT4 | XStep Step Index Number |
| IM_STEP | CHAR30 | XStep Header |
| IM_SIGTYPE | CHAR20 | Signature Type |
| IM_USERID | XUBNAME | User Name |
| IM_DB_UP_DOWN | CHAR1 | Done By Up/Down |
| IM_VALIDATION_REQ | CHAR3 | Validation Required |
| IM_STEP_UUID | ZSMPL_DE_SMPL_SOG_UUID | XStep SOG UUID |

Function: /SMPL/MBR_DEP_ADD_PERFORM

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_AUFNR | AUFNR | N/A |
| IM_USERID | XUBNAME | N/A |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID | N/A |
| IM_DEST_NAME | /SMPL/MBR_CMX_XS_DST_NAME | N/A |

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

## Pseudocode

Function: ZSMPL_FM_COMP_ADDITION_ITEMS

1. Purpose

This function module is used to retrieve and return materials that have already been weighed during a Component Addition XStep, based on a filtered range of BOM item numbers. It enhances /SMPL/PPPI_FM_GET_WGT_MAT by introducing range-based filtering through two additional inputs: IV_BOM_START and IV_BOM_END.

It must be used in conjunction with MBR visibility configuration via ZSMPL_FM_GET_MBR_CONF_BOM_ITEM.

2. Input Parameters

- IV_BOM_START / IV_BOM_END
 Character fields that define the inclusive start and end range of BOM item numbers for filtering which components are considered during this step.

- IV_CONTROL_RECIPE
 The control recipe ID (POC_DOCID) associated with the current manufacturing instruction and weight capture context.

- IV_ORDER
 The process order number used to retrieve BOM component details.

3. Data Initialization

- Internal structures and tables declared for:

  - Material weighing records: lt_weight, ls_weight from /SMPL/PPPI_WT_MT

  - BOM components: lt_components, lt_comp_step of type BAPI_ORDER_COMPONENT

  - Order objects: st_objects to enable selective retrieval of components

- Instance of class ZCL_SMPL_EBR_APC_WS is created, though not used directly in logic.

4. BOM Component Filtering

a. Retrieve the full component list for the order using:

CALL FUNCTION 'BAPI_PROCORD_GET_DETAIL'

EXPORTING

number        = iv_order

order_objects = st_objects

TABLES

component     = lt_components.

b. Filter lt_components based on the configured BOM item number range:

LOOP AT lt_components INTO ls_components.

IF ls_components-item_number GE iv_bom_start

AND ls_components-item_number LE iv_bom_end.

APPEND ls_components TO lt_comp_step.

ENDIF.

ENDLOOP.

5. Match with Weighed Materials

For each filtered component (lt_comp_step), check if the component has already been weighed:

- Query /SMPL/PPPI_WT_MT using:

  - CONTROL_RECIPE = IV_CONTROL_RECIPE

  - MATERIAL = Component material number

- If a matching entry exists:

  - Append material, description, quantity, and unit of measure to the corresponding export parameters.

SELECT SINGLE * FROM /smpl/pppi_wt_mt

WHERE control_recipe = iv_control_recipe

AND material = ls_comp_step-material

INTO ls_check.

IF sy-subrc = 0.

APPEND ls_check-weighed_qty TO ev_qty.

APPEND ls_check-material TO ev_material.

APPEND ls_check-material_desc TO ev_material_desc.

APPEND ls_check-uom TO ev_uom.

ENDIF.

6. Output Parameters

Returns the following data as parallel internal tables (structure types are from /SMPL/STR_TT_*):

- EV_MATERIAL – List of material numbers already weighed within the defined BOM range.

- EV_MATERIAL_DESC – Corresponding material descriptions.

- EV_QTY – Actual weighed quantities per material.

- EV_UOM – Unit of measure for each weighed quantity.

Additional Notes

Key Features:

- Enables granular control of material visibility in XSteps based on BOM item number ranges.

- Dynamically filters weighed materials specific to a sub-step in the process (e.g., multiple additions in same phase).

- Designed for integration with XSteps and MBR configuration in MES environments.

Error Handling:

- No exceptions are raised; missing entries are silently excluded from export parameters.

- No validation on input values; relies on calling layer for integrity checks.

Dependencies:

- Table /SMPL/PPPI_WT_MT must contain pre-weighed material data.

- Order BOM data retrieved via BAPI_PROCORD_GET_DETAIL.

- XStep must configure IV_BOM_START and IV_BOM_END through MBR visibility logic.

Assumptions:

- BOM item numbers are unique and consistently formatted for range comparison (CHAR4).

- Weighed materials have been captured prior to execution of this step.

Function: /SMPL/PPPI_FM_SIG_ADD_DB_CB

- Signature Type Identification

- Assign the incoming signature type (IM_SIGTYPE) to local variable lv_sigtype.

- A CASE structure is used to determine behavior based on the type:

  - ' ' (Done By)

  - 'C' (Checked By)

  - 'O' (Validation Flag Only)

- Done by Signature(lv_sigtype = '')
 a. Optional Step Name Update

- If IM_STEP is provided, update the XStep header in table /SMPL/PPPI_PISST.

b. Determine Position for Signature Insertion

- If IM_DB_UP_DOWN is empty:

  - Insert the new signature above the current item by fetching MIN(UNIQUE_SEQID) and inserting one row before it.

- If IM_DB_UP_DOWN = 'B':

  - Insert the new signature below the current item by fetching MAX(UNIQUE_SEQID) and inserting one row after it.

- In both cases:

  - Use IM_USERID as the signer (SIGN_ID)

  - Signature type description from TEXT-001

  - Insert new row into /SMPL/PPPI_PISST

- Checked By Signature (lv_sigtype = 'C')
 a. Get Signature Start Point

  - Select the UNIQUE_SEQID of the "Checked By" step matching the given input.

b. Determine Signature End Point

- Fetch all prior 'C' type signatures for the same control recipe (and optionally across all phases).

- Determine the highest UNIQUE_SEQID lower than the start point to use as the end.

c. Update Checked By Step

- Set the SIGN_ID and mark the step as COMPLETED = 'X'.

d. Complete All Steps Between Start and End

- Fetch and loop over all steps with UNIQUE_SEQID between the start and end boundaries.

- Mark each row in that range as COMPLETED = 'X'.

- Update the database using MODIFY.

- Validation Flag Update (lv_sigtype = 'O')
 a. Determine Validation Value

  - If IM_VALIDATION_REQ = '2', set to 'No', else 'Yes'.

b. Update Validation Requirement

- Update the VALIDATION_REQUIRED field for the current control recipe, phase, and step index in /SMPL/PPPI_PISST.

Additional Notes:

- Purpose:
Dynamically inserts or updates signature records in /SMPL/PPPI_PISST depending on the role of the signer (Done By, Checked By, or Validator).

- Features:

Supports step name overrides and UUID tracking.

Allows insertion before or after specific records using IM_DB_UP_DOWN.

Ensures Checked By signatures cascade completion to preceding steps.

Accommodates flexible validation logic.

- Error Handling:

No explicit RAISE statements or exception handling.

Relies on CHECK and SY-SUBRC evaluations to exit silently if lookup conditions fail.

- Extensibility:

Designed with optional parameters (IM_STEP, IM_SIGTYPE, IM_DB_UP_DOWN, IM_VALIDATION_REQ) to support various signing scenarios.

Function: /SMPL/MBR_DEP_ADD_PERFORM

1. Initialization

a. Input parameters provide context for recording a Perform signature against a specific Manufacturing Batch Record (MBR) step:

- IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

- IM_AUFNR (AUFNR) (Optional) – Process order number associated with the control recipe.

- IM_USERID (XUBNAME) – SAP user ID of the performer signing the MBR step.

- IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG UUID) representing the MBR step being signed.

- IM_DEST_NAME (/SMPL/MBR_CMX_XS_DST_NAME) – Destination or XStep node name indicating the origin of the signature action.

b. The function first validates that the input parameter IM_USERID is not initial.
 If the performer ID is missing, the exception USER_INITIAL is raised.

c. The function then checks whether the performer is authorized to sign within the MBR context.
 If the user is not registered in the signature authorization table, the exception USER_NOT_SIGNED_INTO_SIG_TAB is raised.

d. Internal structures are initialized to construct the signature entry. The local work area ls_sig_entry (of type /SMPL/MBR_OR_SIG) is prepared to store the signature details before insertion into the database.

2. Signature Entry Construction

a. The function constructs a new record representing the Perform signature for the specified MBR step.

b. The following field values are assigned:

- The process order number (AUFNR) is set from IM_AUFNR.

- The step UUID (SOG_UUID) is set from IM_STEP_UUID.

- The signature type (TYPE) is set to the constant gc_perform_signature, which identifies the record as a Perform signature.

- The performer user ID (SIGN_ID) is assigned from IM_USERID.

- The system date (SY-DATUM) and time (SY-UZEIT) are used to populate both the creation and signature timestamps (CREATED_DATE, CREATED_TIME, SIGN_DATE, SIGN_TIME).

- The destination name (DEST_NAME) is populated from IM_DEST_NAME.

c. This process ensures that the performer, the signed step, and the precise system timestamp are all captured in a single audit record.

3. Database Insertion

a. Once the signature record is fully constructed, it is inserted into the database table /SMPL/MBR_OR_SIG.
b. This creates a permanent audit trail linking the performer to the execution of the corresponding MBR step.
c. If a database insert error occurs (for example, due to duplicate keys or locking issues), the function triggers a system exception to ensure data consistency.

4. Output Assignment

a. The function does not return explicit output parameters.
b. Its outcome is the successful insertion of a Perform signature record into the /SMPL/MBR_OR_SIG table.

c. The resulting database entry serves as the basis for downstream logic, including dependency checks and status updates performed by functions such as /SMPL/MBR_DEP_CHECK_ACTIVE.

Additional Notes

This function module records the Perform signature for a Manufacturing Batch Record (MBR) step, ensuring traceability and compliance with GMP (Good Manufacturing Practice) requirements. It establishes an electronic record of the operator performing a specific step within the manufacturing process.

Key Features

- Captures performer identification, system date, and time automatically at the moment of signing.

- Creates a permanent and auditable record in the MBR signature table.

- Supports integration with MBR dependency and verification functions.

- Provides a simple interface requiring minimal inputs for execution.

Error Handling

- USER_INITIAL – Raised if the performer ID (IM_USERID) is missing.

- USER_NOT_SIGNED_INTO_SIG_TAB – Raised if the performer is not authorized or not registered in the signature authorization table.

- Database Insert Failure – Triggered by the SAP system if the insertion into /SMPL/MBR_OR_SIG fails due to technical issues.

Dependencies

- Table: /SMPL/MBR_OR_SIG – Stores all Perform and Verify signature entries for MBR steps.

- Constants:

  - gc_perform_signature – Identifies the signature type for Perform actions.

- System Fields:

  - SY-DATUM and SY-UZEIT – Provide system date and time values for timestamp creation.

Assumptions

- The performer is a valid SAP user authorized to sign MBR steps.

- The signature authorization table is up to date and synchronized with the MBR system.

- The /SMPL/MBR_OR_SIG table supports direct insertion of new signature records without key conflicts.

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.
b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

- If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

- If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

- This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps

- The use of constants (gc_yes, gc_no, and gc_inside_range) allows the logic to be aligned with broader application settings or standards.

- Helpful in scenarios where input values may vary but need to result in consistent "active" or "inactive" behavior.

- Input is optional, so defaulting behavior can be configured outside this function.

- This function improves clarity and consistency in determining whether an XStep should be active when rendered in the PI Sheet.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR (Manufacturing Batch Record) step can be activated:

- IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

- IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

- IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

- IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

- IM_PLANT (WERKS_D) – Plant code for process context.

b. The function first validates that IM_CONTROL_RECIPE is not initial.

- If it is initial, the exception CONTROL_RECIPE_INITIAL is raised.

c. Internal data is initialized:

- lt_ord_sigs: Internal table for storing order signature data from /SMPL/MBR_OR_SIG.

- ls_step_dep: Work area for step dependency data.

- Routine LOAD_INTERNAL_DATA is called to populate internal buffers (g_ord_data-t_ord_deps) using IM_AUFNR, IM_CONTROL_RECIPE, and IM_STEP_UUID.

2. Signature Data Retrieval

a. The function retrieves signature records for the process order from database table /SMPL/MBR_OR_SIG.
b. Selected fields include: AUFNR, SOG_UUID, TYPE, CREATED_DATE, CREATED_TIME, DEST_NAME, SIGN_ID, SIGN_DATE, and SIGN_TIME.
c. Only signatures related to the specified order (AUFNR = IM_AUFNR) are selected.
d. These records are stored in the internal table lt_ord_sigs for later evaluation.

3. Dependency Evaluation

a. The function loops through dependencies in g_ord_data-t_ord_deps where:

- TARGET_UUID = IM_STEP_UUID, and

- DEP_RULE is either gc_perform_before_start_rule or gc_verify_before_start_rule.

b. For each dependency record:

- If the rule is PERFORM BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending perform signature (gc_perform_signature) with SIGN_ID = abap_false.

- If the rule is VERIFY BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending verify signature (gc_verify_signature) with SIGN_ID = abap_false.

c. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive).
d. If all prerequisite signatures are complete, the step is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in the export parameter EV_ACTIVE.

- gc_step_active – All prerequisite steps are completed; the current step can start.

- gc_step_inactive – At least one prerequisite step is incomplete; the current step remains blocked.

Additional Notes

This function determines whether a dependent step in a Manufacturing Batch Record is eligible to start based on the completion of prerequisite steps.

Key Features:

- Evaluates Perform Before Start and Verify Before Start dependency rules.

- Dynamically validates signature data from /SMPL/MBR_OR_SIG.

- Integrates with existing MBR dependency structures via global table g_ord_data-t_ord_deps.

Error Handling:

- Raises CONTROL_RECIPE_INITIAL if the control recipe input is missing.

- Handles empty dependency or signature data without runtime errors.

Dependencies:

- Tables: /SMPL/MBR_OR_SIG (Signatures), g_ord_data-t_ord_deps (Step Dependencies).

- Constants: gc_perform_before_start_rule, gc_verify_before_start_rule, gc_perform_signature, gc_verify_signature, gc_step_active, gc_step_inactive.

- Routine: LOAD_INTERNAL_DATA for initializing order context.

Assumptions:

- All prerequisite steps are recorded in the dependency table.

- Signature records are current and synchronized with MBR step progression.

- Activation status is recalculated each time the function is called to reflect live signature data.

## Test Scenarios

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: Component Addition | Process Order created and Released containing SMPL: Component Addition |  |
| 2 | Open the EBR, and navigate to SMPL: Component Addition | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Component Addition is visible | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: Component Addition is visible |  |
| 3 | The value entered in parameter IV_HEADER and parameter IV_INSTR in SiMPL MBR appears correctly in the XStep header | Header and instruction text are displayed as authored in SiMPL MBR | Header and instruction text are displayed correctly. |  |
| 4 | Execute the ‘Component Addition’ button and verify that the Component Number, Component Description, Amount Added and UoM correctly populates in the output fields. | Component Number, Component Description, Amount Added and UoM correctly populates in the output fields. Ensure that the visibility condition based on BOM Item Numbers is configured correctly so that only the correct material components are displayed in the correct XStep. | Component Number, Component Description, Amount Added and UoM correctly populates in the output fields. Ensure that the visibility condition based on BOM Item Numbers is configured correctly so that only the correct material components are displayed in the correct XStep. |  |
| 5 | Execute the Component Addition button and verify that the Component Number, Component Description, Amount Added and UoM correctly populates in the output fields. | Component Number, Component Description, Amount Added and UoM correctly populates in the output fields. Ensure that the visibility condition based on BOM Item Numbers is configured correctly so that only the correct material components are displayed in the correct XStep. | Component Number, Component Description, Amount Added and UoM correctly populates in the output fields. Ensure that the visibility condition based on BOM Item Numbers is configured correctly so that only the correct material components are displayed in the correct XStep. |  |
| 6 | Enter a username in the “Performed By” fields then enter the password in the pop-up window. | Signature is accepted. Weighed Components table locks. | Signature is accepted. Weighed Components table locks. |  |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.1 | Initial Document (Functional Requirements and Technical Specifications) | Frieda van der Merwe | 23-07-2025 |
| 1.2 | The following Technical Spec sections added to spec: Function Module(s), and Pseudocode. | Juna Jaouhari | 22-06-2026 |