V 1.2

June 2026

XStep Design Specification

SMPL: QS

Table of Contents

1	Purpose	3

2	Overview	3

3	Reasons for developing	4

4	Authorization	4

5	Assumptions/ Dependencies	4

6	Validation Checks	4

7	Configuration Specification	4

8	XStep Layout Design	5

9	Function Module(s)	6

10	Pseudocode	8

11	Test Scenarios	11

12	Revision History	13

## Purpose

This document outlines the design, along with the configuration specifications for the XStep “SMPL: QS”.

## Overview

This XStep guides the operator in adjusting the volume of the solution to a predefined target using a diluent, such as Water for Injection (WFI). The operator is prompted to add the diluent to the mixing vessel and record key process parameters, including the temperature set point of the added diluent and the gross weight after addition.

The SMPL: QS XStep ensures that the correct quantity of diluent is added at the end of the mixing process to achieve the final target batch weight. This supports consistent batch preparation and ensures process accuracy and compliance.

The XStep contains 7 main input parameters:

- IV_HEADER - This can be changed in SiMPL MBR to display specific header text for the XStep

- IV_INSTR - This can be changed in SiMPL MBR to display a specific set of instructions for the XStep that will be displayed below the header

- IV_COMP - This can be changed in SiMPL MBR to display the specific component description

- IV_MIN - This can be changed in SiMPL MBR to configure the specific minimum temperature for the WFI

- IV_MAX - This can be changed in SiMPL MBR to configure the specific maximum temperature for the WFI

- IV_WUOM & IV_GUOM - This can be changed in SiMPL MBR to configure the specific Unit of Measure fields.

The following fields will be pre-populated by default from the input parameters:

- “Component” - Output

- “Min” - Output

- “Max” - Output

- “UoM” - Output

- “Required Qty” - Output (Equal to the Batch Quantity)

The operator follows the instructions and enters the temperature set point and gross weight after QS:

- “Temperature set point” - Input

- “Gross Weight After QS” - Input

System Validations and Constraints:

- Temperature Monitoring: If the recorded Temperature Set Point deviates from the established Min/Max range, the system will capture a deviation.

- Weight Accuracy: The Gross Weight After QS is validated against the Required Quantity using a ±2% tolerance.

- Error Handling: Any value recorded outside these validated limits will trigger an error message with a deviation popup, halting the process until the exception is addressed.

Functional Design

## Reasons for developing

The SMPL: QS XStep was developed to ensure precise and consistent adjustment of the final batch volume during manufacturing. By guiding the operator through the addition of a diluent and capturing key data such as temperature and gross weight, this step standardizes the QS process, minimizes the risk of error, and ensures compliance with GMP requirements. It also provides traceability and supports accurate batch documentation within the EBR system.

## Authorization

N/A

## Assumptions/ Dependencies

|  | Assumption |
| --- | --- |
| 1 | Master Data has been set up correctly |

## Validation Checks

| Error Handling |  |
| --- | --- |
| Error | Action |
| Min and Max temperature fields not pre-populated by default | Check MBR Configuration |

## Configuration Specification

N/A

## XStep Layout Design

## Function Module(s)

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_ACTIVE | CHAR1 | N/A |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IM_CONTROL_RECIPE | POC_DOCID | N/A |
| IM_PHASE | VORNR |  |
| IM_AUFNR | AUFNR |  |
| IM_STEP_UUID | /SMPL/MBR_SOG_UUID |  |
| IM_PLANT | WERKS_D |  |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ACTIVE | CHAR1 | N/A |

Exceptions:

| Exception | Short Text |
| --- | --- |
| CONTROL_RECIPE_INITIAL |  |

Function: /SMPL/PPPI_FM_MIN_MAX

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| I_MIN | F | Minimum value (optional) |
| I_MAX | F | Maximum value (optional) |
| I_VALUE | F | Value to validate |

Exceptions:

| Exception | Short Text |
| --- | --- |
| VALUE_OUT_OF_RANGE | Value is outside the min/max range |
| MIN_GREATER_THAN_MAX | Minimum is greater than maximum |

Function: ZSMPL_FM_VALIDATE_ISSUED_QTY

Import Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| IV_CURRENT_QTY | ERFMG | Current (entered) quantity |
| IV_DEFAULT_QTY | ERFMG | Default / required quantity |
| IV_PREVIOUS_QTY | ERFMG | Previous quantity (optional) |
| IV_REQ_QTY | ERFMG | Required quantity (optional) |
| IV_PCT | INT1 | Tolerance percentage (optional, default 2) |
| IV_ISS_DIFF | ABAP_BOOL | Issue difference flag (optional) |
| IV_ADD_ISS | ERFMG | Additional issue (optional) |
| IV_DEVI_UPDOWN | FLAG | Deviation up/down (optional) |

Export Parameters:

| Parameter Name | Associated Type | Short Text |
| --- | --- | --- |
| EV_ISS_DIFF | ERFMG | Issued quantity difference |

Exceptions:

| Exception | Short Text |
| --- | --- |
| ISSUED_QUANTITY_CHANGED | Issued quantity changed from previous value |
| QUANTITY_OUT_OF_TOLERANCE | Quantity outside the tolerance range |

## Pseudocode

Function: /SMPL/PPPI_FM_INITIAL_ACTIVE

1. Initialization

a. Input parameter IV_ACTIVE is optionally provided by the calling logic.
b. Constants gc_yes, gc_no, and gc_inside_range are assumed to be defined globally in the context of the function module.

2. Active Flag Evaluation

a. The function checks if the input value IV_ACTIVE falls within the expected valid range, represented by gc_inside_range.

b. Based on the result:

- If the input value is not equal to gc_inside_range, the XStep is marked as inactive by setting EV_ACTIVE = gc_no.

- If the input value is equal to gc_inside_range, the XStep is marked as active by setting EV_ACTIVE = gc_yes.

3. Output Assignment

a. The evaluated result is returned in the EV_ACTIVE export parameter for use by the calling logic (e.g., XStep configuration, PI sheet rendering).

Additional Notes

- This is a simple normalization function that abstracts how activation flags are interpreted for PPPI XSteps

- The use of constants (gc_yes, gc_no, and gc_inside_range) allows the logic to be aligned with broader application settings or standards.

- Helpful in scenarios where input values may vary but need to result in consistent "active" or "inactive" behavior.

- Input is optional, so defaulting behavior can be configured outside this function.

- This function improves clarity and consistency in determining whether an XStep should be active when rendered in the PI Sheet.

Function: /SMPL/MBR_DEP_CHECK_ACTIVE

1. Initialization

a. Input parameters provide context for determining whether an MBR (Manufacturing Batch Record) step can be activated:

- IM_CONTROL_RECIPE (POC_DOCID) – Control recipe ID identifying the batch instruction.

- IM_PHASE (VORNR) (Optional) – Operation or phase within the process order.

- IM_AUFNR (AUFNR) (Optional) – Process order number associated with the step.

- IM_STEP_UUID (/SMPL/MBR_SOG_UUID) – Unique step identifier (SOG).

- IM_PLANT (WERKS_D) – Plant code for process context.

b. The function first validates that IM_CONTROL_RECIPE is not initial.

- If it is initial, the exception CONTROL_RECIPE_INITIAL is raised.

c. Internal data is initialized:

- lt_ord_sigs: Internal table for storing order signature data from /SMPL/MBR_OR_SIG.

- ls_step_dep: Work area for step dependency data.

- Routine LOAD_INTERNAL_DATA is called to populate internal buffers (g_ord_data-t_ord_deps) using IM_AUFNR, IM_CONTROL_RECIPE, and IM_STEP_UUID.

2. Signature Data Retrieval

a. The function retrieves signature records for the process order from database table /SMPL/MBR_OR_SIG.
b. Selected fields include: AUFNR, SOG_UUID, TYPE, CREATED_DATE, CREATED_TIME, DEST_NAME, SIGN_ID, SIGN_DATE, and SIGN_TIME.
c. Only signatures related to the specified order (AUFNR = IM_AUFNR) are selected.
d. These records are stored in the internal table lt_ord_sigs for later evaluation.

3. Dependency Evaluation

a. The function loops through dependencies in g_ord_data-t_ord_deps where:

- TARGET_UUID = IM_STEP_UUID, and

- DEP_RULE is either gc_perform_before_start_rule or gc_verify_before_start_rule.

b. For each dependency record:

- If the rule is PERFORM BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending perform signature (gc_perform_signature) with SIGN_ID = abap_false.

- If the rule is VERIFY BEFORE START, it checks whether the prerequisite step (PREREQ_UUID) has a pending verify signature (gc_verify_signature) with SIGN_ID = abap_false.

c. If an incomplete signature is found, the step is marked inactive (EV_ACTIVE = gc_step_inactive).
d. If all prerequisite signatures are complete, the step is marked active (EV_ACTIVE = gc_step_active).

4. Output Assignment

a. The evaluated activation status is returned in the export parameter EV_ACTIVE.

- gc_step_active – All prerequisite steps are completed; the current step can start.

- gc_step_inactive – At least one prerequisite step is incomplete; the current step remains blocked.

Additional Notes

This function determines whether a dependent step in a Manufacturing Batch Record is eligible to start based on the completion of prerequisite steps.

Key Features:

- Evaluates Perform Before Start and Verify Before Start dependency rules.

- Dynamically validates signature data from /SMPL/MBR_OR_SIG.

- Integrates with existing MBR dependency structures via global table g_ord_data-t_ord_deps.

Error Handling:

- Raises CONTROL_RECIPE_INITIAL if the control recipe input is missing.

- Handles empty dependency or signature data without runtime errors.

Dependencies:

- Tables: /SMPL/MBR_OR_SIG (Signatures), g_ord_data-t_ord_deps (Step Dependencies).

- Constants: gc_perform_before_start_rule, gc_verify_before_start_rule, gc_perform_signature, gc_verify_signature, gc_step_active, gc_step_inactive.

- Routine: LOAD_INTERNAL_DATA for initializing order context.

Assumptions:

- All prerequisite steps are recorded in the dependency table.

- Signature records are current and synchronized with MBR step progression.

- Activation status is recalculated each time the function is called to reflect live signature data.

Function: /SMPL/PPPI_FM_MIN_MAX

1. Range Validation

a. If both I_MIN and I_MAX are supplied and I_MIN is greater than I_MAX, raise MIN_GREATER_THAN_MAX.

b. If I_VALUE falls outside the range [I_MIN, I_MAX], raise VALUE_OUT_OF_RANGE.

Additional Notes

Validator on the Temperature Set Point; enforces that the recorded temperature is within the configured minimum/maximum (IV_MIN / IV_MAX). Exceptions: VALUE_OUT_OF_RANGE, MIN_GREATER_THAN_MAX.

Function: ZSMPL_FM_VALIDATE_ISSUED_QTY

1. Purpose

Validates the entered Gross Weight After QS against the required/default quantity within a percentage tolerance (the QS step uses a +/-2% tolerance).

2. Tolerance Check

a. Compare IV_CURRENT_QTY against IV_DEFAULT_QTY (and IV_REQ_QTY) using the tolerance IV_PCT.

b. If the entered quantity is outside tolerance, raise QUANTITY_OUT_OF_TOLERANCE (a deviation for both under- and over-weight).

3. Change Detection

a. If the quantity has changed from a previous value (IV_PREVIOUS_QTY / IV_ISS_DIFF), raise ISSUED_QUANTITY_CHANGED and return the difference in EV_ISS_DIFF.

Additional Notes

Validator on the Gross Weight After QS field. Exceptions: ISSUED_QUANTITY_CHANGED, QUANTITY_OUT_OF_TOLERANCE.

## Test Scenarios

| # | Scenario | Expected Result | Actual Result | Pass/ Fail |
| --- | --- | --- | --- | --- |
| 1 | Create and Release a Process Order with the pre-configured SiMPL Master Recipe and the relevant Production Version | Process Order created and Released containing SMPL: QS | Process Order created and Released containing SMPL: QS |  |
| 2 | Open the EBR, and navigate to Phase 0030 and then navigate to SMPL: QS | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: QS is visible in Phase 0030 | The EBR for the selected Process Order and Control Recipe opens successfully without any errors and the SMPL: QS is visible in Phase 0030 |  |
| 3 | Verify that the value/text entered in parameter IV_HEADER and parameter IV_INSTR in SiMPL MBR appears correctly in the XStep header | Header and instruction text are displayed as authored in SiMPL MBR | Header and instruction text are displayed as authored in SiMPL MBR |  |
| 4 | The operator verifies that the Component, Min and Max temperatures, UoM and Required Quantity fields are pre-populated in the QS Table. | The operator verifies that the Component, Min and Max temperatures, UoM and Required Quantity fields are pre-populated in the QS Table. | The operator verifies that the Component, Min and Max temperatures, UoM and Required Quantity fields are pre-populated in the QS Table. . |  |
| 5 | The operator inputs a Temperature Set Point. | The temperature set point is in range and accepted. | The temperature set point is in range and accepted. |  |
| 6 | The operator inputs Gross Weight After QS | Gross weight after QS weight is accepted. | Gross weight after QS weight is accepted. |  |
| 7 | Enter a username in the “Performed By” field then enter the password in the pop-up window. | Signature is accepted | Signature is accepted |  |
| 8 | Enter a username in the “Witnessed By” field then enter the password in the pop-up window. | Signature is accepted and the xstep locks. | Signature is accepted and the xstep locks. |  |

## Revision History

| Version No. | Description | Author | Date |
| --- | --- | --- | --- |
| 1.1 | Initial Document | Frieda van der Merwe | 23-07-2025 |
| 1.2 | The following Technical Spec sections added to spec: Function Module(s), and Pseudocode. | Juna Jaouhari | 22-06-2026 |
| 1.3 | Updated to align the specification with the current XStep version (v0004). Added Function Module and Pseudocode sections for the two previously undocumented main-step validators: /SMPL/PPPI_FM_MIN_MAX (Temperature Set Point min/max range validation) and ZSMPL_FM_VALIDATE_ISSUED_QTY (Gross Weight After QS validated against the Required Quantity within a +/-2% tolerance). (Function modules used only by the embedded Conditional Header / Optional Signature references are not documented here.) | [Author] | 23-06-2026 |