| PRODUCT DATA |  |  |
| --- | --- | --- |
| PN: 8012441 / AZD0543 - Virus Inactivation (Low pH, GPF, 2000 L Scale, Process 1) |  |  |
| MPR Process Order No. | General Supplies Process Order No. | Recorded by date: |

| This record is an accurate reproduction of the Master. | Issued by: | Date Issued: |
| --- | --- | --- |

| Completed Record Reviewed By: | Production: | Date: |
| --- | --- | --- |
|  | Quality Assurance: | Date: |

# Process Summary

This master production record (MPR) describes the procedure for the Viral Inactivation and Neutralization of the AZD0543 product in the downstream process outlined in this MPR.

# Downstream Process Flowchart

| MABR-0027568 / PN: 8012412 / AZD0543 - Centrifuge Harvest (GPF, 2000 L Scale, Process 1) |
| --- |
|  |  |
| MABR-0027569 / PN: 8012440 / AZD0543 - Affinity Chromatography (DurA Cycle A50, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027570 / PN: 8012441 / AZD0543 - Virus Inactivation (Low pH, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027573 / PN: 8012472 / AZD0543 - Anion Exchange Chromatography (CaptoQ, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027574 / PN: 8012473 / AZD0543 - Cation Exchange Chromatography (Poros HS50, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027575 / PN: 8012474 / AZD0543 - Virus Filtration (VPro, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027576 / PN: 8012475 / AZD0543 - Concentration and Diafiltration (UF Skid, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027577 / PN: 8012477 / AZD0543 - Formulation and Bulk Filtration (Cryovault, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| Storage Final Sterile Filtration and Vialing (off-site fill/finish manufacturer) |

# Referenced Documents

## SOP-0080023:  Operation of the Powerblanket BHT System

## SOP-0080165: Equipment and Material Flow in the Gaithersburg Pilot Facility

## SOP-0080185:  WFI Point of Use Operations

## SOP-0080294: Floor Scale Operation

## SOP-0080295:  DLIMS Sample Submission for Manufacturing Sciences

## SOP-0080297:  Benchtop pH and Conductivity Meter Operation

## SOP-0080303: WFI Hold Tank Operation (TK-1458)

## SOP-0080390:  Operation, Maintenance and Cleaning of the QuattroFlow 4400S, 4000S and 1200S Pumps

## SOP-0080402:  Operation of the GE Bioprocess Chromatography System in the Gaithersburg Manufacturing Facility

## SOP-0080423:  Operation and Maintenance of the LevTech and LevMixer Mixing Systems

## SOP-0080438:  Operation and Cleaning of the Endress and Hauser Promass 80 Flow Meter

## SOP-0080503:  BriskHeat Full-Coverage Drum Heater Set-up, Operation, and Cleaning

## SOP-0080613:  Operation of the Millistak+ HC Pod System

## SOP-0080839:  Peristaltic Pump Operation

## SOP-0080999:  Handling and Exterior Cleaning of Pressure Gauges

## SOP-0081015:  Operation of the MagMix Bag System for Low pH Viral Inactivation

## SOP-0107056:  cGMP Labeling Practices for Controlled Materials, Equipment and Product

## SOP-0107091:  Operation and Maintenance of the SoloVPE System for Protein Concentration Determination

## SOP-0107095:  Operation of the Product Hold Fixed Tanks in the Gaithersburg Pilot Facility

## SOP-0107097:  Sampling from Process Vessels

## SOP-0107099: Transfer Line Configuration for Purification

## SOP-0107102:  Operation and Maintenance of the Integrity Magnetic Mixer Single-Use System

## SOP-0107117:  Thermometer Use

## SOP-0110853: Gowning And Personnel Flow for the Gaithersburg Pilot Facility

## SOP-0081641: Manufacturing Sciences SAP Usage at Gaithersburg Pilot Facilities

## MABR-0027570  PN: 8012440 / AZD0543 - Affinity Chromatography (DurA Cycle A50, GPF, 2000 L Scale, Process 1)

## MABR-0027573  PN: 8012472 / AZD0543 - Anion Exchange Chromatography (CaptoQ, GPF, 2000 L Scale, Process 1)

# Bill of Materials – Equipment and Components

| Part No./ Equipment ID | Quantity | Description |
| --- | --- | --- |
| PK4000 | 1 | Chromatography Skid |
|  | As Needed | Mixer Drive |
|  | As Needed | Peristaltic Pump |
|  | As Needed | Quattroflow Pump |
|  | 1 | pH / Conductivity meter |
|  | 1 | Thermometer |
|  | As Needed | Floor Scale / Benchtop Balance |
|  | As Needed | Flow Meter |
|  | As Needed | Dip tube |
|  | As Needed | Clean Hosing |
|  | As Needed | Autoclaved Class VI Tubing |
| TK-1186 | As Needed | 200 L Portable Tank |
| TK-1926 | As Needed | 200 L Portable Tank |

|  | As Needed | Fixed Tanks |
| --- | --- | --- |
| TK-2095 |  | 1500 L Product Hold Tank |
| TK-2100 |  | 2000 L Product Hold Tank |
| TK-2105 |  | 2000 L Product Hold Tank |
| TK-2110 |  | 2500 L Product Hold Tank |
| TK-2115 |  | 4000 L Product Hold Tank |

|  | As Needed | Product Filters for Portable/Fixed Product Tanks |
| --- | --- | --- |
| 4101130 |  | Filter – 10” SHC Cartridge 0.2 μm Hydrophilic |
| 4101765 |  | Filter – 5” SHC Cartridge, 0.2 μm Hydrophilic |
| 6001331 |  | Filter: Fluorodyne EX Grade EDF Cartridges (10 in.) |
|  | As Needed | Vent Filters for Portable/Fixed Product Tanks |
| 4000214 |  | Filter – Aervent 0.2 μm, Hydrophobic, PTFE Membrane (5 in) |
| 4102547 |  | Filter – Aervent 0.2 μm, Hydrophobic, PTFE Membrane (10 in) |

4	Bill of Materials – Equipment and Components (continued)

| Part No./ Equipment ID | Quantity | Description |
| --- | --- | --- |
|  | As Needed | Product Filters for Bags |
| 4101737 |  | Filter – SHC 0.5+0.2 μm Capsules with C-Flex (3”) |
| 4101736 |  | Filter – SHC 0.5+0.2 μm Capsules with C-Flex (10”) |
| 4100987 * |  | Filter – 30 in., Opticap® XLT 30 Capsule Filter with Millipore Express® SHC Hydrophilic Membrane, 0.5/0.2 μm |
| 4101032 * |  | Filter – 10 in., Opticap® XLT 10 Capsule filter with Millipore Express® SHC membrane, 0.5/0.2 μm |
|  | As Needed | Mixing Bags for Product Filtration Only |
| 4102059 |  | LevTech Mix Bag (50 L) |
| 4102062 |  | LevTech Mix Bag (100 L) |
| 4102060 |  | LevTech Mix Bag (200 L) |
| 4102032 |  | LevTech Mix Bag (300 L) |
| 4102061 |  | LevTech Mix Bag (560 L) |
|  | As Needed | Mixing Bags for Product Filtration and Viral Inactivation Treatment |
| 4102700 |  | Custom MagMix Bag (200 L) |
| 4102701 |  | Custom MagMix Bag (300 L) |
| 4102702 |  | Custom MagMix Bag (370 L) |
| 4102703 |  | Custom MagMix Bag (560 L) |
|  | As Needed | Bags for WFI |
| 4101684 |  | Bags - Stedim 3D Bags with C-Flex (500 L) |
| 4101685 |  | Bags - Stedim 3D Bags with C-Flex (300 L) |
| 4101687 |  | Bags - Stedim 3D Bags with C-Flex (200 L) |

* = Materials with bolded filter part numbers require autoclaving prior to use.

Additional Manufacturing Supplies Used in Process

If additional materials (filter, bag, etc.) need to be used during processing:

- Prior to use, ensure the material is acceptable for use based on Section 4.

- Record the applicable material information.

- Document the reason for the change in Attachment 7: Comments Section.

| MPR Step No. | Material Description | Part No. | Batch No. | Exp. Date | Autoclave No., if applicable | Autoclave Exp. Date, if applicable | Performed by/Date | Witnessed by/Date | SAP Consumption Performed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Document Process Flow Sheet

| Starting Material | PN: 8012440: AZD0543 Affinity Product |
| --- | --- |
| Batch No. |  |



| Acid Addition | PN: A-005168: 1M Acetic Acid |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Batch No. |  |  |  | Exp. Date |  |  |
|  |  |  |  |  |  |  |
| VI Incubation (Target 75 minutes; NOR 60-240 minutes) |  |  |  |  |  |  |
|  |  |  |  |  |  |  |
| Base Addition | PN: A0101-D: 1M Tris Base |  |  |  |  |  |
| Batch No. |  |  |  | Exp. Date |  |  |
| Recorded by / date |  |  | Verified by / date |  |  |  |

Additional Solution Batches Used in Process

If a solution (of a different batch noted in Section 5) needs to be replaced during processing:

- Indicate the switch at the step it is occurring. Ensure the new solution is the same part number as the solution being replaced.

- Record the solution part number, batch number (and/or process order number) and expiration date below.

- Document the change in Attachment 7: Comments Section.

| MPR Step No. | Part No. | Batch No. | Exp. Date | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |

# Process Notes

## All activities are performed at 15 C to 27 C.

## For all calculations, assume 1 L = 1 kg and 1 mL = 1 g unless otherwise noted.

## Verify that all solutions, raw materials, and standards are within expiration prior to use.

## When an equipment ID is written in the MPR, the operator is indicating they have visually inspected and verified calibration prior to use.

## If additional mixing/recirculation is required after initial sampling of the product, record the mixing information in Attachment 6: Product Recirculation Worksheet.

## If a non-routine sample is requested by memo, complete Attachment 2:  Non-Routine Sampling Record.

## If Non-Routine samples are taken, re-adjust the weight of the product sampled.

## All parameters listed in this document are Targets/Alert Limits unless otherwise noted.  When alert limits are exceeded, contact Supervisor and Technical Transfer Group. Document any actions taken in the Attachment 7: Comments Section of the MPR.

# Affinity Product Information / Pre-Processing Activities

| Step | Description |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- |
|  | Record the room where this process will occur. |  |  |  |
|  | Room Number |  |  |  |
|  | If the Affinity Product was warmed up (from cold storage), ensure the time the product was removed from cold storage is recorded in Attachment 3: Hold Times. |  |  |  |
|  | - Confirm that the pH meter has been standardized between pH 2.0 and pH 7.0 per SOP-0080297. - Confirm that the conductivity meter has been standardized within the acceptable range per SOP-0080297. |  |  |  |
|  | Meter ID |  |  |  |
|  | Confirm that the thermometer is within its calibration interval. Record the equipment ID of the thermometer used. |  |  |  |
|  | Thermometer ID |  |  |  |
|  | Record the tare weight and net weight of the AZD0543 – Affinity Product vessel. |  |  |  |
|  | Tare Weight of the Affinity Product Vessel | kg = L |  |  |
|  | Net Weight of the Affinity Product | kg = L |  |  |
|  | Affinity Product SoloVPE Concentration (8012440-30) | g/L |  |  |
|  | DLIMS Project Number |  |  |  |
|  | DLIMS Sample Number |  |  |  |

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mix the product for at least 15 minutes. - Follow SOP-0080423 for the LevTech mixer, if applicable. - Follow SOP-0107102 for the Magnetic Mixer, if applicable. - Follow SOP-0107095 for fixed tanks, if applicable. NOTE: If the product requires recirculation to mix, N/A rows below and complete Attachment 6: Product Recirculation Worksheet. |  |  |  |  |  |
|  | Mixer ID/Tank ID |  |  |  |  |  |
|  | Agitation Setting | RPM |  |  |  |  |
|  | Mixing Start Time |  |  |  |  |  |
|  | Mixing End Time |  |  |  |  |  |
|  | Mixing Duration (≥ 15 minutes) | min |  |  |  |  |
|  | Remove at least the sample volume per Attachment 1: Sampling Record and SOP-0107097. - If a non-routine sample is requested by memo, complete Attachment 2: Non-Routine Sampling Record. |  |  |  |  |  |
|  | Sampling Date and Time |  | Date: | Time: |  |  |
|  | - Aliquot the sample (8012441-10) per Attachment 1: Sampling Record. - If applicable, aliquot non-routine samples per Attachment 2: Non-Routine Sampling Record. - Label the sample per SOP-0107056. |  |  |  |  |  |
|  | - Submit the sample to the storage location indicated in DLIMS per SOP-0080295. - Ensure the sampling time and location are updated in DLIMS. |  |  |  |  |  |

| Step | Description |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Measure the pH, conductivity, and temperature of the Affinity product per SOP-0080297 and SOP-0107117. Record the results below. |  |  |  |  |  |  |  |  |
|  | pH |  |  |  |  |  |  |  |  |
|  | Conductivity |  |  | mS/cm |  |  |  |  |  |
|  | Temperature |  |  | oC |  |  |  |  |  |
|  | Obtain the gross weight of the Affinity product vessel after sampling. NOTE: Ensure the vessel is in the same conformation as when the tare weight was obtained. |  |  |  |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |  |
|  | Calibration Due Date |  |  |  |  |  |  |  |  |
|  | Gross Weight/Volume of the Affinity Product vessel after sampling |  |  |  | kg=L |  |  |  |  |
|  | Calculate the net weight of the Affinity product after sampling: |  |  |  |  |  |  |  |  |
|  | Gross Weight/Volume of the Affinity Product vessel after sampling (Step 7.11) | − | Tare Weight of the Affinity Product Vessel (Step 7.5) |  |  | = | Affinity Weight/Volume after sampling |  |  |
|  | kg |  | kg |  |  |  | kg=L |  |  |

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | - If the product will be transferred to a different adjustment vessel, N/A Step 7.14 and go to Section 8. - Otherwise: - N/A Section 8 and re-label the Viral Inactivation (VI) vessel “VI Treatment Vessel” and record the tare weight on the label. - Continue to Step 7.14. |  |  |  |  |
|  | Starting Product Temperature Check - Proceed per the table below. |  |  |  |  |
|  | Check One | Temperature of the sample at Step 7.10 is: | Action |  |  |
|  |  | Within 18 – 25 oC | - Transcribe the pH, conductivity, and temperature results into Step 10.9 “Initial pH / Conductivity / Temperature.” - N/A all of Section 9 and continue to Step 10.1. |  |  |
|  |  | Less than 18 oC: | - Turn on the treatment vessel’s agitator and document the mixing start time in Step 9.1. * - Begin warming the vessel per SOP-0080503/SOP-0080023 (for a bag) or SOP-0107095 (for a tank) to 18 – 25 oC. ** - Continue to Section 9. |  |  |
|  |  | Greater than 25 oC | Contact a supervisor and document actions taken in Attachment 7: Comments Section. |  |  |
|  | * NOTE: The product warming range is 18 – 25 oC. ** NOTE: If using a tank, activating the glycol loop may alter the tank weight. For this reason, the glycol loop must remain on through neutralization to minimize weight fluctuations during acid and base additions. |  |  |  |  |

# Transfer Vessel Information and Set up

| Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | NOTE: If the Affinity product vessel will be used as the VI Treatment Vessel, N/A all of Section 8, then continue to Section 9 or Section 10, depending on the Action selected in Step 7.14. |  |  |
|  | Obtain the product vessel and label the vessel with “AZD0543”, “VI Treatment Vessel”, batch number, part number, initials, and date. |  |  |
|  | For Step 8.4: VI Treatment Vessel Information: Obtain the tare weight of the Treatment Vessel and record the the information in Step 8.4. |  |  |

| ## VI Treatment Vessel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bag Information (if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Part No. |  |  | Batch No. |  |  |  | Exp. Date |  |  |  | SAP Consumption Performed by/Date |  |
| Bag Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mark the applicable attachments connected to the mixer bag during the vessel tare weight |  | Attached: |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Mixer Drive |  |  |  | Top Base |  |  |  |  |  | Magnetic Clamp |  |  |
| Tank Information (if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tank ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CIP Batch ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CIP Exp. Date / Time |  | Date: |  |  |  |  |  | Time: |  |  |  |  |  |  |
| Pressure Test ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pressure Test Date |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIP Batch ID |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Filter Information (See Section 4 for acceptable filters and autoclaving requirements.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Part No. |  |  | Batch No. |  |  | Exp. Date |  |  | Autoclave Cycle, if applicable |  |  |  | Autoclave Exp., if applicable |
| Vent Filter (if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product Filter 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product Filter 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product Filter 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SAP Consumption Performed by/Date |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tare Weight Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| * (NOTE: If collection vessel is a bag, tare weight will be obtained prior to attachment of the product filter.) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scale/ Balance ID |  |  |  |  |  |  | Calibration Due Date |  |  |  |  |  |  |  |
| Tare Weight of the VI Treatment Vessel * |  |  | kg |  |  |  |  |  |  |  |  |  |  |  |
| Mark filters attached when tare weight is obtained. |  |  | Product filter |  |  | Vent filter |  |  |  | No filters, vessel is a bag |  |  |  |  |
| Recorded by / Date |  |  |  |  |  | Witnessed by / Date |  |  |  |  |  |  |  |  |

| ## Step | Description | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | - Obtain transfer tubing or a clean flex hose to transfer product from the Affinity Product Vessel to the VI Treatment Vessel. - Record the tubing or hose information in Attachment 5: Transfer Tubing/Hosing Information. |  |  |
|  | - Connect the transfer tubing/hose to the outlet of the Affinity Product Vessel and to the inlet of the VI Treatment Vessel. - Thread the transfer tubing through a peristaltic pump, if applicable. - Begin transferring the Affinity product to the VI Treatment Vessel. |  |  |
|  | - After all of the Affinity product has been transferred to the VI Treatment Vessel, disconnect the transfer line from the VI Treatment Vessel. |  |  |

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mix the product for at least 15 minutes. - Follow SOP-0107102 for the MagMix, if applicable. - Follow SOP-0107095 for fixed tanks, if applicable. NOTE: If the product requires recirculation to mix, N/A rows below and complete Attachment 6: Product Recirculation Worksheet. |  |  |  |  |  |
|  | Mixer ID/Tank ID |  |  |  |  |  |
|  | Agitation Setting | RPM |  |  |  |  |
|  | Mixing Start Time |  |  |  |  |  |
|  | Mixing End Time |  |  |  |  |  |
|  | Mixing Duration (≥15 minutes) | min |  |  |  |  |
|  | - Aliquot a 10 mL product sample from the treatment vessel into a 50 mL conical and measure the sample temperature. - Record the results below. |  |  |  |  |  |
|  | Temperature |  | oC |  |  |  |
|  | Obtain the gross and net weight of the Affinity product after sampling. |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |
|  | Calibration Due Date |  |  |  |  |  |
|  | Gross Weight/Volume of the Affinity Product after sampling |  |  | kg |  |  |
|  | Net Weight/Volume of the Affinity Product after sampling * |  |  | kg |  |  |
|  | * Net Weight/Volume of the Affinity Product after sampling = Gross Weight/Volume of the Affinity Product after sampling (above) – Tare weight of the Product Vessel (Step 8.4) |  |  |  |  |  |

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | Starting Product Temperature Check - Proceed per the table below. |  |  |  |  |
|  | Check One | Temperature of the sample at Step 8.9 is: | Action |  |  |
|  |  | Within 18 – 25 oC | - Transcribe the pH, conductivity, and temperature results into Step 10.9 “Initial pH / Conductivity / Temperature.” - N/A all of Section 9 and continue to Section 10. |  |  |
|  |  | Less than 18 oC: | - Turn on the treatment vessel’s agitator and document the mixing start time in Step 9.1. * - Begin warming the vessel per SOP-0080503/SOP-0080023 (for a bag) or SOP-0107095 (for a tank) to reach 18 – 25oC. ** - Continue to Section 9. |  |  |
|  |  | Greater than 25 oC | Contact a supervisor and document actions taken in Attachment 7: Comments Section. |  |  |
|  | * NOTE: The product warming range is 18 – 25 oC. ** NOTE: If using a tank, activating the glycol loop may alter the tank weight. For this reason, the glycol loop must remain on through neutralization to minimize weight fluctuations during acid and base additions. |  |  |  |  |

# Additional Mixing and Pre-Treatment Product Warming (if applicable)

| Step | Description |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Start mixing the product per SOP-0107102 (for a bag) or SOP-0107095 (for a tank) and record the mixing start time. - If the product requires recirculation to mix, N/A rows below and complete Attachment 6: Product Recirculation Worksheet. |  |  |  |  |  |  |  |
|  | Mixer ID/Tank ID |  |  |  |  |  |  |  |
|  | Agitation Setting |  |  |  | RPM |  |  |  |
|  | Mixing Start Time |  |  |  |  |  |  |  |
|  | Aliquot a 10 mL product sample from the treatment vessel into a 50 mL conical and measure the sample temperature. Record the results below. |  |  |  |  |  |  |  |
|  | Temperature |  |  | oC |  |  |  |  |
|  | Product Temperature Check - Proceed based on the table below. |  |  |  |  |  |  |  |
|  | Check One | Temperature of the sample at Step 9.2 is: | Action |  |  |  |  |  |
|  |  | Within 18 – 25 oC | - Measure the pH, conductivity, and temperature of the sample and transcribe results into Step 10.9 “Initial pH / Conductivity / Temperature.” - Document the mixing end time in Step 9.4, then continue to Section 10. |  |  |  |  |  |
|  |  | Less than 18 oC | - Continue to warm and agitate the product. * - Resample the product regularly until the product is within 18 – 25 oC. - When the product is within 18 – 25 oC, measure the pH, conductivity, and temperature of the sample and transcribe results into Step 10.9 “Initial pH / Conductivity / Temperature.” - Document the mixing end time in Step 9.4, then continue to Section 10. |  |  |  |  |  |
|  |  | Greater than 25 oC | Contact a supervisor and document actions taken in Attachment 7: Comments Section. |  |  |  |  |  |
|  | * If using a tank, activating the glycol loop may alter the tank weight. For this reason, the glycol loop must remain on through neutralization to minimize weight fluctuations during acid and base additions. |  |  |  |  |  |  |  |
| Step | Description |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
|  | Record the mixing end time. Calculate the mixing duration from the time the mixing was started in Step 9.1. |  |  |  |  |  |  |  |
|  | Mixing End Time |  |  |  |  |  |  |  |
|  | Mixing Duration (≥15 minutes) |  |  |  |  | min |  |  |

# Setup and Acid Addition

| Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | - As needed, obtain tubing and/or hosing for the transfer of the acid to the starting material. - Record the tubing / hosing information in Attachment 5: Transfer Tubing/Hosing Information. |  |  |
|  | If using a tank, confirm that the VI Treatment Vessel has a dip tube on the inlet and that the dip tube outlet is below the surface of the product in the treatment vessel. |  |  |
|  | - Attach tubing/hosing from the outlet of the acid buffer vessel (A-005168) to the inlet of the VI Treatment Vessel, as applicable. Follow SOP-0081015 for a MagMix bag. - If applicable, thread the tubing through a peristaltic pump following SOP-0080839. - Prime the tubing/hosing leading to the VI Treatment Vessel with (A-005168) pump per SOP-0080839. |  |  |
|  | Record the gross weight of the VI Treatment Vessel after priming and the scale/balance used in Step 10.9. |  |  |
|  | Ensure agitation is on. |  |  |

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the expected amount of acid to be added based on the addition ratio. * Use Step 8.10 if the product was transferred. |  |  |  |  |  |  |
|  | Net Weight of Affinity after sampling (Step 7.12 or 8.10 *) | x | Acid Addition Ratio (kg acid/ kg Affinity Product) | = | Recommended weight of acid |  |  |
|  | kg |  | 0.013 |  | kg |  |  |
|  | Determine the initial acid addition weight. After initial addition, more acid may be added until the target pH is reached. |  |  |  |  |  |  |
|  | Recommended weight of acid (Step 10.6) | x | Initial Addition Ratio | = | Target Weight for Initial Acid Addtion |  |  |
|  | kg |  | 0.50 |  | kg |  |  |

| ## Step | Description | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | Step 10.9: Acid Treatment - Begin transferring 1M Acetic Acid (A-005168) into the VI Treatment Vessel: - Do not stop agitation during Treatment. - Follow SOP-0081015 for performing VI in a MagMix bag. If the VI Treatment Vessel is a tank, follow SOP-0107095. - After each addition of A-005168, mix for at least ten minutes before sampling for pH measurement. - Continue adding A-005168 until the product pool is within the following ranges: - pH Range (NOR): 3.4 - 3.6 (Target pH is 3.5) - Temperature Range: 18 – 25 oC - If necessary, warm the product per SOP-0080503/SOP-0080023 (if using a bag) or SOP-0107095 (if using a tank). - Record the Final pH, Final conductivity, Temperature, and Start Time for Incubation (pH and temperature in spec) in Step 10.9. |  |  |

| ## Acid Treatment |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Initial Measurements |  | Initial pH |  | Initial Conductivity |  | Initial Temperature |  | Start time of addition (End of storage time) |  |  |  | Performed by / Date | Witnessed by / Date |
|  |  |  |  | mS/cm |  | ____________°C (18 – 25 oC) |  |  |  |  |  |  |  |
| Scale/Balance ID |  |  |  |  |  | Calibration due date |  |  |  |  |  |  |  |
| Gross weight of Treatment Vessel before addition (kg) | End time of acid addition |  | Gross weight of Treatment Vessel after addition (kg) |  | Net weight of buffer added (kg) a | Total net weight of buffer added (kg) b | Time of pH sampling |  | Adjusted pH | Sample Temperature (oC) |  | Performed by / Date | Witnessed by / Date |
| Note: Do not exceed the volume calculated in Step 10.7 on the initial addition. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table continued onto next page

| # 10.9 Acid Treatment Cont. |  |  |  |  |
| --- | --- | --- | --- | --- |
| SAP Consumption Performed by/Date |  |  |  |  |
| Time of pH sample confirmed at 3.5 c, d | ________________ |  |  |  |
| Final pH (3.4 – 3.6) c | ________________ |  |  |  |
| Final Conductivity | mS/cm |  |  |  |
| Temperature (18 – 25 oC) e | _________________C |  |  |  |
| Start Time for Incubation (pH and temperature in-spec) f | ________________ |  |  |  |

- Net weight of buffer added (kg) = Gross weight of Treatment Vessel after addition (kg) – Gross weight of Treatment Vessel before addition (kg).

- Total net weight of buffer added = Net weight of buffer added (kg) +Total net weight of buffer added (kg) from the previous addition. For the first addition only—the previous net weight is zero.

- The target pH is 3.5. The acid addition may be continued after the pH is within range to be closer to the target pH.

- This time should match the time of the first sample where pH is confirmed within 3.4 – 3.6.

- The NOR temperature range for the duration of incubation is 18 – 25 oC.

- Start Time for Incubation (pH and temperature in spec) should match the time of the first sample when the pH is confirmed to be within 3.4 – 3.6 and temperature 18 – 25 oC.

# Incubation

| Step | Description |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Incubation Temperature Check for a Bag Vessel If the treatment vessel is a tank, N/A this step. Otherwise, continue below. - Incubate the product for a minimum of 38 minutes from Step 10.9 “Start Time for Incubation (pH and temperature in spec),” then sample 10 mL from the treatment vessel. - Measure the sample temperature. - If the sample is outside the range of 18 – 25 oC, contact a supervisor and document actions taken in Attachment 7: Comments Section. |  |  |  |  |  |  |  |  |
|  | Sampling Time |  |  |  |  |  |  |  |  |
|  | Temperature |  |  |  | oC |  |  |  |  |
|  | Incubated Product Information - Confirm that the product has been incubating for a minimum of 60 minutes. - Take a 25 mL sample from the incubated product. - Measure the pH, conductivity, and temperature of the sample and record the results below. |  |  |  |  |  |  |  |  |
|  | Time of Sampling |  |  |  |  |  |  |  |  |
|  | pH (3.4 – 3.6) |  |  |  |  |  |  |  |  |
|  | Conductivity |  |  | mS/cm |  |  |  |  |  |
|  | Temperature (18 – 25 oC) |  |  | °C |  |  |  |  |  |
|  | Calculate the duration that the product has been incubating. |  |  |  |  |  |  |  |  |
|  | Time of Sampling (Step 11.2) | − | Start Time for Incubation (pH and temperature in spec) (Step 10.9) |  |  | = | Duration the product has been incubating |  |  |
|  |  |  |  |  |  |  | min |  |  |

# Acid Addition- Extension

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | Choose an option based on the Step 11.2 sample result: |  |  |  |  |
|  | Check One | Condition | Action |  |  |
|  |  | The pH is above 3.6 | Continue to the next step. |  |  |
|  |  | The pH is within 3.4 – 3.6 and temperature is within 18 – 25 oC | - N/A the remainder of this section and Section 13 - Go to Section 14. |  |  |
|  |  | The pH is below 3.4 or the temperature is not within 18 – 25 oC | Contact an area manager and document actions taken in Attachment 7: Comments Section. |  |  |
|  |  | Name of Supervisor Contacted, if applicable |  |  |  |
|  | Ensure agitation is on. |  |  |  |  |
|  | Step 12.4: Acid Treatment Extension - Continue transferring 1M Acetic Acid (A-005168) into the VI Treatment Vessel: - Do not stop agitation during Treatment. - Follow SOP-0081015 for performing VI in a MagMix bag. If the VI Treatment Vessel is a tank, follow SOP-0107095. - After each addition of A-005168, mix for at least ten minutes before sampling for pH measurement. - Continue adding A-005168 until the product pool is within the following ranges: - pH Range (NOR): 3.4 - 3.6 (Target pH is 3.5) - Temperature Range: 18 – 25 oC - If necessary, warm the product per SOP-0080503/SOP-0080023 (if using a bag) or SOP-0107095 (if using a tank). - Record the Final pH, Final conductivity, Temperature, and Start Time for Incubation (pH and temperature in spec) in Step 12.4. |  |  |  |  |

| ## Acid Treatment - Extension |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | Performed by / Date | Witnessed by / Date |
| Scale/Balance ID |  |  | Calibration Due Date |  |  |  | Start time of addition |  |  |  |  |  |
| Gross weight of Treatment Vessel before addition (kg) | End time of acid addition | Gross weight of Treatment Vessel after addition (kg) |  | Net weight of buffer added (kg) a |  | Total net weight of buffer added (kg) b |  | Time of pH sampling | Adjusted pH | Sample Temperature (oC) | Performed by / Date | Witnessed by / Date |
| SAP Consumption Performed by/Date |  |  |  |  |  |  |  |  |  |  |  |  |
| Time of pH sample confirmed at 3.5 c, d |  |  |  |  |  | ________________ |  |  |  |  |  |  |
| Final pH (3.4 – 3.6) c |  |  |  |  |  |  |  |  |  |  |  |  |
| Final Conductivity |  |  |  |  |  | ------------------------------ mS/cm |  |  |  |  |  |  |
| Temperature (18 – 25 oC) e |  |  |  |  |  | _________________C |  |  |  |  |  |  |
| Start Time for Incubation (pH and Temperature in spec) f |  |  |  |  |  | ________________ |  |  |  |  |  |  |

- Net weight of buffer added (kg) = Gross weight of Treatment Vessel after addition (kg) – Gross weight of Treatment Vessel before addition (kg).

- Total net weight of buffer added = Net weight of buffer added (kg) +Total net weight of buffer added (kg) from the previous addition. For the first addition only—use the total amount of buffer added in Step 10.9 as the “Total Net Weight of Buffer added.”

- The target pH is 3.5. The acid addition may be continued after the pH is within range to be closer to the target pH.

- This time should match the time of the first sample where pH is confirmed within 3.4 – 3.6.

- The NOR temperature range for the duration of incubation is 18 – 25 oC.

- Start Time for Incubation (pH and Temperature in spec) should match the time of the first sample that pH is confirmed to be within 3.4 – 3.6 and temperature 18 – 25 oC.

# Incubation - Extension

| Step | Description |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mid-Incubation Temperature Check for a Bag Vessel: - If the treatment vessel is a tank, N/A this step. Otherwise, continue below. - Incubate the product for a minimum of 38 minutes from Step 12.4 “Start Time for Incubation (pH and Temperature in spec),” then sample 10 mL from the treatment vessel. Measure the sample temperature. - If the sample is outside the range of 18 – 25 oC, contact a supervisor and document actions in Attachment 7: Comments Section. |  |  |  |  |  |  |  |  |
|  | Sampling Time |  |  |  |  |  |  |  |  |
|  | Temperature |  |  |  | oC |  |  |  |  |
|  | Incubated Product Information - Confirm that the product has been incubating for a minimum of 60 minutes. - Take a 25 mL sample from the incubated product. - Measure the pH, conductivity, and temperature of the sample and record the results below. |  |  |  |  |  |  |  |  |
|  | Time of Sampling |  |  |  |  |  |  |  |  |
|  | pH (3.4 – 3.6) |  |  |  |  |  |  |  |  |
|  | Conductivity |  |  | mS/cm |  |  |  |  |  |
|  | Temperature (18 – 25 oC) |  |  | °C |  |  |  |  |  |
|  | Calculate the duration that the product has been incubating. |  |  |  |  |  |  |  |  |
|  | Time of Sampling (Step 13.2) | − | Start Time for Incubation (pH and Temperature in spec) (Step 12.4) |  |  | = | Duration that the product has been incubating |  |  |
|  |  |  |  |  |  |  | min |  |  |

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | Choose an option based on the Step 13.2 sample result. |  |  |  |  |
|  | Check One | Condition | Action |  |  |
|  |  | The pH is within 3.4 – 3.6 and the temperature is within 18 – 25 oC. | Continue to Section 14 |  |  |
|  |  | The pH is not within 3.4 – 3.6 or the temperature is not within 18 – 25 oC. | Contact an area manager and document actins taken in Attachment 7: Comments Section. |  |  |
|  |  | Name of Supervisor Contacted, if applicable |  |  |  |

# Setup and Base Addition

| Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | - Disconnect the acid transfer tubing from the VI Treatment vessel. - Drain the tubing used for acid addition. |  |  |
|  | - If using a tank and the glycol loop was activated to adjust the treatment vessel temperature, keep the glycol loop on during neutralization to prevent fluctuations to the vessel weight during base addition. - If using a bag and a heating blanket was used to adjust the vessel temperature, remove the blanket before proceeding with base addition. - If the product did not need to be temperature adjusted, N/A this step and continue to Step 14.3. |  |  |
|  | - Attach tubing/hosing from the outlet of the base buffer vessel 1M Tris Base (A0101-D) to the inlet of the VI Treatment Vessel, as applicable. If using a MagMix bag, follow SOP-0081015. - If applicable, thread the tubing through a peristaltic pump following SOP-0080839. - Prime the tubing/hosing leading to the VI Treatment Vessel with (A0101-D). |  |  |
|  | Record the gross weight of the VI Treatment Vessel after priming in Step 14.11. |  |  |
|  | Ensure agitation is on. |  |  |

| Step | Description |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the expected amount of base to be added based on the addition ratio. |  |  |  |  |  |  |  |  |  |
|  | Total Net Weight of Acid added (Step 10.9 or 12.4 *) | x |  | Base Addition Ratio (kg base / kg acid) |  | = |  | Recommended weight of base |  |  |
|  | kg |  |  | 1.20 |  |  |  | kg |  |  |
|  | * If additional acid addition was needed, use Step 12.4. Ensure the acid from the first addition is included in the total net weight of acid added. |  |  |  |  |  |  |  |  |  |
|  | Determine the initial base addition weight. After initial addition, more base may be added to reach the target pH. |  |  |  |  |  |  |  |  |  |
|  | Recommended weight of base (Step 14.6) |  | x | Initial Addition Ratio | = |  | Target Weight of Initial Base Addition |  |  |  |
|  | kg |  |  | 0.50 |  |  | kg |  |  |  |

| ## Step | Description | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | - Steps 14.9 and 14.10 provide instructions for filling out the table in Step 14.11 and performing the base additions. - Review the instructions in Steps 14.9 and 14.10 before proceeding. |  |  |
|  | For Step 14.11 - End of Incubation Information: - Take a final sample of the incubation product and measure the pH and temperature. - In Step 14.11: - Record the (A) ‘pH at the end time of Incubation’ and (B) “Temperature at the end of Incubation’ - Record the (C) ‘End time for Incubation’, which is also the start time of the first base addition. - Calculate the (D) ‘Total Duration of Incubation’ NOTE: Contact a supervisor if the Total Duration of Incubation exceeds 240 minutes and document actions taken in Attachment 7: Comments Section. |  |  |
|  | For Step Step 14.11 - Neutralization Treatment: - After a target incubation time of 75 minutes, within a range of 60 – 240 minutes, begin transferring 1M Tris Base (A0101-D) into the VI Treatment Vessel: - Do not stop agitation during treatment. - For the base additions, follow SOP-0081015 if the VI Treatment Vessel is a MagMix bag. If the vessel is a tank, follow SOP-0107095. - After each addition of A0101-D, mix the product for at least ten minutes prior to taking a sample for pH measurement. - When pH rises above 4.0, recalibrate the pH meter to pH 4.0 and pH 10.0 and record the calibration information in Step 14.12. - Continue adding A0101-D until the product pool is within pH range 7.2 – 7.6. - Record the Final pH, Final conductivity, Temperature and End Time For Incubation in Step 14.11. |  |  |

| ## Neutralization Information |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| End of Incubation Information |  |  |  |  |  | Performed by / Date | Witnessed by / Date |
| (A) pH at the end time of Incubation (3.4 – 3.6) |  |  | (B) Temperature at the end of Incubation (18 – 25 oC) |  | oC |  |  |
| (C) End time for Incubation (Start Time of base addition) | − | Start Time of Incubation (Step 10.9 or 12.4) a |  | = | (D) Total Duration of Incubation ( 60-240 minutes ) |  |  |
|  |  |  |  |  | minutes |  |  |

| # Neutralization Treatment |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scale/Balance ID |  |  |  |  | Calibration Due Date |  |  |  |  |  |  |  |
| Gross weight of Treatment Vessel before addition (kg) | End time of base addition |  | Gross weight of Treatment Vessel after addition (kg) | Net weight of buffer added (kg) b |  | Total net weight of buffer added (kg) c |  | Time of pH sampling d |  | Adjusted pH e | Performed by / Date | Witnessed by / Date |
| Note: Do not exceed the volume calculated in Step 14.7 on the initial addition. |  |  |  |  |  |  |  |  |  |  |  |  |
| SAP Consumption Performed by/Date |  |  |  |  |  |  |  |  |  |  |  |  |
| Final pH (7.2 – 7.6) e |  |  |  |  |  |  |  |  |  |  |  |  |
| Final Conductivity |  |  |  |  |  |  | mS/cm |  |  |  |  |  |
| Temperature (15 – 27 °C) |  |  |  |  |  |  | C |  |  |  |  |  |

- Use Step 12.4 if the acid addition was extended.

- Net weight of buffer added (kg) = Gross weight of Treatment Vessel after addition (kg) – Gross weight of Treatment Vessel before addition (kg).

- Total net weight of buffer added = Net weight of buffer added (kg) +Total net weight of buffer added (kg) from the previous addition. For the first addition only—the previous net weight is zero.

- Re-standardization of the benchtop pH meter is needed at Step 14.12 when the pH is ≥ 4.0.

- The target pH after neutralization is 7.4. The NOR pH after neutralization is 7.2 – 7.6.

| Step | Description |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Record the standardization of the benchtop pH meter with pH 4.0 and pH 10.0 standards. |  |  |  |  |  |  |  |  |  |
|  | pH Meter ID |  |  |  |  |  |  |  |  |  |
|  | Once the product pH has been confirmed to be in neutralization range: - Close the outlet of the base vessel and the inlet of the VI Treatment Vessel. - Disconnect the base transfer tubing from the VI Treatment Vessel. - Drain the tubing used for base addition. |  |  |  |  |  |  |  |  |  |
|  | Obtain and record the gross weight of the VI Treatment Vessel after the neutralization. NOTE: Ensure the VI Treatment Vessel is in the same configuration as when the tare weight was obtained in Step 7.5 (or Step 8.4 if product was transferred to a treatment vessel). |  |  |  |  |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |  |  |
|  | Calibration Due Date |  |  |  |  |  |  |  |  |  |
|  | Gross Weight after Neutralization |  |  |  | kg |  |  |  |  |  |
|  | Calculate the net weight of the Neutralized VI product. * Use Step 8.10 if the Affinity Product was transferred to a treatment vessel. ** Use Step 12.4 if additional acid was needed. Ensure that the acid from the first addition is included in the total net weight of acid added. |  |  |  |  |  |  |  |  |  |
|  | Net Weight of Affinity Product after sampling (Step 7.12 or 8.10 *) | + | Total Net Weight of Acid added (Step 10.9 or 12.4 **) | + |  | Total Net Weight of Base added (Step 14.11) | = | Net Weight/Volume of the Neutralized VI Product |  |  |
|  | kg |  | kg |  |  | kg |  | kg=L |  |  |

# VIral-Inactivated Product – Sampling

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Mix the product for at least 15 minutes. - Follow SOP-0080423 for the LevTech mixer, if applicable. - Follow SOP-0107102 for the Magnetic Mixer, if applicable. - Follow SOP-0107095 if using a product hold tank, if applicable. NOTE: If the product is in a non-mixing vessel and requires recirculation to mix, N/A the step below and complete Attachment 6: Product Recirculation Worksheet. |  |  |  |  |  |
|  | Mixer ID/Tank ID |  |  |  |  |  |
|  | Agitation Setting |  | RPM |  |  |  |
|  | Mixing Start Time |  |  |  |  |  |
|  | Mixing End Time |  |  |  |  |  |
|  | Mixing Duration (≥ 15 minutes) |  | min |  |  |  |
|  | Remove at least the sample volume per Attachment 1: Sampling Record and SOP-0107097. - If a non-routine sample is requested by memo, complete Attachment 2: Non-Routine Sampling Record. |  |  |  |  |  |
|  | Sampling Date and Time | Date: |  | Time: |  |  |
|  | - Aliquot the sample (8012441-30) per Attachment 1: Sampling Record. - If applicable, aliquot non-routine samples per Attachment 2: Non-Routine Sampling Record. - Label the sample per SOP-0107056. |  |  |  |  |  |
|  | - Submit the sample to the storage location indicated in DLIMS per SOP-0080295. - Ensure the sampling time and location are updated in DLIMS. |  |  |  |  |  |

| Step | Description |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Measure the pH, conductivity, and temperature of the sample. NOTE: Contact Supervisor if the pH of the VI Product is not 7.2 – 7.6 and document actions taken in Attachment 7: Comments Section. |  |  |  |  |  |  |  |
|  | pH (7.2 – 7.6) |  |  |  |  |  |  |  |
|  | Conductivity |  |  | mS/cm |  |  |  |  |
|  | Temperature |  |  | °C |  |  |  |  |
|  | Obtain the gross and net weights of the collected product after sampling and record the values. |  |  |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |
|  | Calibration Due Date |  |  |  |  |  |  |  |
|  | Gross Weight of the VI Treatment Vessel after sampling |  |  | kg |  |  |  |  |
|  | Calculate the net weight/volume of the VI Product after sampling. * NOTE: Use Step 8.4 as tare weight if the Affinity product was transferred to a treatment vessel in Section 8. Otherwise, use Step 7.5 as tare weight. |  |  |  |  |  |  |  |
|  | Gross Weight of the VI Treatment Vessel after sampling (Step 15.6) | − | Tare Weight of the VI Product Vessel (Step 7.5 or 8.4)* |  | = | Net Weight/Volume of the VI Product after sampling |  |  |
|  | kg |  | kg |  |  | kg = L |  |  |

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | Store the product and document storage information below. - For product stored at 2 – 8ºC, the storage start time is the time when the product is transferred into 2 – 8ºC storage. - For product stored at 15 – 25ºC, the storage start time is the time when final sampling occurred in Step 15.2. Initial Storage Condition; 2 – 8°C 15 – 25°C. Storage Start Time. |  |  |
|  | Ensure that Attachment 3: Hold Times is filled out with all hold times for the Affinity product. |  |  |  |  |
|  | Name in MPR | Name in Attachment | Step No. |  |  |
|  | Start time of addition | Affinity Product Storage End (Time/Date) | Step 10.9 |  |  |

# Batch Production Record Review

| Step | Description | Production Initial/Date |
| --- | --- | --- |
|  | Ensure any additional supporting documentation is included (e.g. additional Pi Trends, Memos, Logbook pages, Non-Routine sample requests, etc.). |  |
|  | Verify that the product net weight after sampling (from Step 15.7) is correctly inputted into SAP. |  |
|  | Review the Batch Production Record for completeness and submit for review. |  |

# Attachment 1: Sampling Record

| Viral Inactivation |  |  |  |  |  | (8012441) |
| --- | --- | --- | --- | --- | --- | --- |
| Dev Samples DLIMS Project # |  |  |  |  |  |  |
| Pre-Load Performed By/Date |  |  |  |  |  |  |
| DLIMS Test Name | Volume | Location | Tube Type | Intermediate Storage Temp b | DLIMS Storage Temp | Sample Number |
| -10 (Affinity Product) |  |  |  |  |  |  |
| BIOBURDEN_IP | 25 mL | QC | PP | 2-8 oC | 2-8 oC |  |
| ENDOTOXIN_IP_GPF1 | 10 mL |  | PET | 2-8 oC | 2-8 oC |  |
| -30 (VI Product) |  |  |  |  |  |  |
| SOLO_A280 a | 10 mL | MS | PP | 2-8 oC | 2-8 oC |  |
| CIEF_NATIV | 10 mL | AS | PP | 2-8 oC | 2-8 oC |  |
| CHO_SYBR | 10 mL | AS | PP | 2-8 oC | 2-8 oC |  |
| EL_CHO_GEN | 10 mL | AS | PP | 2-8 oC | 2-8 oC |  |
| SEC_PUR | 10 mL | AS | PP | 2-8 oC | 2-8 oC |  |
| CGE_N_RED | 10 mL | AS | PP | 2-8 oC | 2-8 oC |  |
| ABT Retain | 5 x 14 mL | AS | PP | 2-8 oC | ≤-70 oC |  |
| PPS Small Retain | 3 x 14 mL | AS | PP | 2-8 oC | ≤-70 oC |  |

- Test SOLO_A280 sample per SOP-0107091. If sample is not tested immediately, store sample at 2 - 8°C.

- The intermediate storage temperature applies to both the shipment and the storage of the sample prior to reaching the sample management team.

Definitions/Terminology:

- Use GREENFORM Template for Development DLIMS sample submissions.

- Sample Designation “-10”:  For sampling of starting material.

- Sample Designation “-30”:  For product quality samples.

- Bold (STAT): Bold text indicates STAT sample submission.

# Attachment 2:  Non-Routine Sampling Record

- If non-routine samples are taken, re-adjust the weight of the product sample.

- Log into DLIMS per SOP-0080295 as required by the memo. Record DLIMS Project number and Sample number if submitting via DLIMS.

| Pre-Load Performed By/Date |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Process Section No. | Contents | DLIMS Form | DLIMS Project Number | Container Type | Quantity | DLIMS Tests | Sample Number | Designation | Receiving Group | Storage Temp (°C) | Sample Performed By/Date | Sample Witnessed By/Date |

# Attachment 3: Hold Times

Record the storage start time, end time, and date for the Affinity Product.

- For product stored at 2 – 8°C, the storage start time is the time when the product is transferred into 2 – 8°C storage, and the storage end time is the time when the product is transferred from 2 – 8°C storage.  A new entry for storage at R. T. (15 – 25°C) begins upon removal from 2 – 8°C.

- Calculate the storage duration for each temperature where the product was stored as well as the combined storage duration, if applicable.

| Hold | Location | Temp. | Affinity Product Storage Start (Time/Date) | Performed by/Date | Verified by/Date |  | Affinity Product Storage End (Time/Date) | Hold Times |  | Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | R. T. | 2 - 8 °C |  |  |
| 1 |  | R.T. 2-8 °C |  |  |  |  |  | ______hrs _____mins | ______hrs _____mins |  |  |
| 2 |  | R.T. 2-8 °C |  |  |  |  |  | ______hrs _____mins | ______hrs _____mins |  |  |
| 3 |  | R.T. 2-8 °C |  |  |  |  |  | ______hrs _____mins | ______hrs _____mins |  |  |
| Total R. T. Hold Time: If R.T. Total Hold Time > 72 hrs, contact a supervisor |  |  |  |  |  |  |  | hrs |  |  |  |
| Total 2-8 °C Hold Time |  |  |  |  |  |  |  |  | hrs |  |  |
| Total 2-8 °C + RT Hold Time: If Total Combined Hold > 168 hrs, contact a supervisor |  |  |  |  |  |  |  | hrs |  |  |  |

# Attachment 4: Yield Calculations

| # Load Mass Pre-Sampling |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # Load Volume Pre-Sampling | # X | # Load Concentration | # = | # Load Mass Pre-Sampling | # Performed by/Date | # Verified by/Date |
|  |  |  |  | # (a) |  |  |
| # ___________L_ Step 7.5 |  | # ________g/L__ Step 7.5 |  | # g |  |  |
| # Load Mass Post-Sampling |  |  |  |  |  |  |
| # Load Volume Post-Sampling | # X | # Load Concentration | # = | # Load Mass Post-Sampling | # Performed by/Date | # Verified by/Date |
|  |  |  |  | # (b) |  |  |
| # ___________L_ Step 7.12 or Step 8.10 |  | # ________g/L__ Step 7.5 |  | # g |  |  |
| # * Use Step 8.10 volume if Affinity product was transferred to a new VI Treatment Vessel |  |  |  |  |  |  |
| # Final Product Mass Pre-Sampling |  |  |  |  |  |  |
| # Final Product Volume Pre-Sampling | # X | # Final Product Concentration | # = | # Final Product Mass Pre-Sampling | # Performed by/Date | # Verified by/Date |
|  |  |  |  | # (c) |  |  |
| # ___________L_ Step 14.15 |  | # ________g/L__ ( 8012441 -30) |  | # g |  |  |
| # Final Product Mass Post-Sampling |  |  |  |  |  |  |
| # Final Product Volume Post-Sampling | # X | # Product Concentration | # = | # Final Product Mass Post-Sampling | # Performed by/Date | # Verified by/Date |
|  |  |  |  | # (d) |  |  |
| # ___________L_ Step 15.7 |  | # ________g/L__ ( 8012441 -30) |  | # g |  |  |

Attachment 4: Yield Calculations (continued)

Step Yield

| # Final Product Mass Pre-Sampling |  | # Load Mass Post-Sampling | # = | # Yield | # x 100 = | # % Yield | # Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # (c) |  | # (b) |  |  |  |  |  |  |
| # g |  | # g |  |  | # x 100 = | # % |  |  |

MPR Yield

| # Final Product Mass Post-Sampling |  | # Load Mass Pre-Sampling | # = | # Yield | # x 100 = | # % Yield | # Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # (d) |  | # (a) |  |  |  |  |  |  |
| # g |  | # g |  |  | # x 100 = | # % |  |  |

# Attachment 5:  Transfer Tubing/Hosing Information

|  | Tubing/Hose 1 | Tubing/Hose 2, if applicable | Tubing/Hose 3, if applicable |
| --- | --- | --- | --- |
| Applicable MPR Step |  |  |  |
| Tubing Part No, if applicable |  |  |  |
| Tubing Batch No, if applicable |  |  |  |
| Tubing Exp. Date, if applicable |  |  |  |
| Tubing Autoclave Cycle No., if applicable |  |  |  |
| Tubing Autoclave Exp. Date, if applicable |  |  |  |
| Hose Serial No., if applicable |  |  |  |
| Hose Cleaning Batch ID, if applicable |  |  |  |
| Hose Cleaning Exp. Date, if applicable |  |  |  |
| Recorded By / Date |  | Witnessed By / Date |  |

# Attachment 6: Product Recirculation Worksheet

| - Obtain a piece of tubing for recirculation of the product. Record the applicable product name and tubing information. - Start mixing the product by recirculation. Record the start time. - Mix the product by recirculation per SOP-0107097 for at least 40 minutes and record the mixing information. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Section 7 | Section 8 | Section 9 | Section 15 |
| Product Name |  |  |  |  |  |
| Tubing Part No. |  |  |  |  |  |
| Tubing Batch No. |  |  |  |  |  |
| Tubing Exp. Date |  |  |  |  |  |
| Tubing Autoclave cycle No. |  |  |  |  |  |
| Tubing Autoclave Exp. Date |  |  |  |  |  |
| Mixing Start Time |  |  |  |  |  |
| Mixing End Time |  |  |  |  |  |
| Mixing Duration * (≥ 40 minutes) |  |  |  |  |  |
| ## Performed By / Date |  |  |  |  |  |
| ## Witnessed By / Date |  |  |  |  |  |

* The duration of mixing is the time that elapses between the mixing end time and the mixing start time in minutes.

# Attachment 7: Comments Section

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 7: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 7: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 7: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

# MASTER PRODUCTION RECORD REVISION HISTORY

The following is included for reference only and is not part of the executable record.

| Version | Change | Reason |
| --- | --- | --- |
| 1.0 | This is a new master production record (MPR) for use in the downstream processing of AZD0543 Viral Inactivation. | The information contained in this document is derived from technical report REP-0494621 and REP-0414101 and sampling and testing plan PLAN-0124866. |