| PRODUCT DATA |  |  |
| --- | --- | --- |
| PN: 8012475 / AZD0543 - Concentration and Diafiltration (UF Skid, GPF, 2000L Scale, Process 1) |  |  |
| Process Order No.: | General Supplies Process Order No. | Recorded By/Date: |

| This record is an accurate reproduction of the Master. | Issued by: | Date Issued: |
| --- | --- | --- |

| Completed Record Reviewed By: | Production: | Date: |
| --- | --- | --- |
|  | Quality Assurance: | Date: |

# Process Summary

This master production record (MPR) describes the procedure for the Concentration, Diafiltration, Recovery and Dilution, of the AZD0543 product in the downstream process outlined below.

# Downstream Process Flow Sheet

| MABR-0027568 / PN: 8012412 / AZD0543 - Centrifuge Harvest (GPF, 2000 L Scale, Process 1) |
| --- |
|  |  |
| MABR-0027569 / PN: 8012440 / AZD0543 - Affinity Chromatography (DurA Cycle A50, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027570 / PN: 8012441 / AZD0543 - Virus Inactivation (Low pH, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027573 / PN: 8012472 / AZD0543 - Anion Exchange Chromatography (CaptoQ, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027574 / PN: 8012473 / AZD0543 - Cation Exchange Chromatography (Poros HS50, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027575 / PN: 8012474 / AZD0543 - Virus Filtration (VPro, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027576 / PN: 8012475 / AZD0543 - Concentration and Diafiltration (Large Skid, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| MABR-0027577 / PN: 8012477 / AZD0543 - Formulation and Bulk Filtration (Cryovault, GPF, 2000 L Scale, Process 1) |  |
|  |  |
| Storage Final Sterile Filtration and Vialing (off-site fill/finish manufacturer) |

# Referenced Documents

## FORM-0071789:  Skid UFDF Cassette Installation and Pre-Sanitization Data Sheet

## SOP-0080153:  Operation of Formulation Portable Tank (TK-1927)

## SOP-0080185:  WFI Point of Use Operations

## SOP-0080273:  Operation of Advanced MicroOsmometer

## SOP-0080295:  DLIMS Sample Submission for Manufacturing Sciences

## SOP-0080297:  Benchtop pH and Conductivity Meter Operation

## SOP-0080303:  WFI Hold Tank Operation (TK-1458)

## SOP-0080423:  Operation and Maintenance of the LevTech and LevMixer Mixing Systems

## SOP-0080509:  Osmometer Operation

## SOP-0080525:  Operation and Exterior Cleaning of Agitator Controlled 100 L Portable Formulation
Tanks: TK-2149 and TK-2150

## SOP-0080539:  Operation, Cleaning and Maintenance of Magnetic Stirplates

## SOP-0080839:  Peristaltic Pump Operation

## SOP-0081194:  Laminar Flow Cabinet Operation and Cleaning

## SOP-0107056:  cGMP Labeling Practices for Controlled Materials, Equipment and Product

## SOP-0107091: Operation and Maintenance of the SoloVPE System for Protein Concentration Determination.

## SOP-0107095: Operation of the Product Hold Fixed Tanks in the Gaithersburg Pilot Facility

## SOP-0107097:  Sampling from Process Vessels

## SOP-0107102: Operation and Maintenance of the Integrity Magnetic Mixer Single-Use Mixing System

## SOP-0107111:  Ultrafiltration Skid Operation, Cleaning, and Storage

## SOP-0107117:  Thermometer Use

## SOP-0107420:  Pressure Hold Test

## MABR-0027575 / PN: 8012474: AZD0543 – PN: 8012474 / AZD0543 - Virus Filtration (VPro, GPF, 2000 L Scale, Process 1)

## MABR-0027577 / PN: 8012477: AZD0543 – PN: 8012477 / AZD0543 - Formulation and Bulk Filtration (Cryovault, GPF, 2000 L Scale, Process 1)

# Bill of Materials – Equipment and Components

| Part No./ Equipment ID | Quantity | Description |
| --- | --- | --- |
| DF-1244 | 1 | PALL Large Scale UF/DF Skid |
|  | As Needed | Mixer Drive |
|  | As Needed | Peristaltic Pump |
|  | As Needed | Stir Plate |
|  | 1 | Conductivity Meter |
|  | 1 | pH meter |
|  | 1 | Thermometer |
|  | As Needed | Floor Scale/ Benchtop Balance |
|  | 1 | Filter Integrity Tester |
|  | As Needed | Autoclaved Class 6 Tubing |
|  | As Needed | Stainless Steel Dip tubes |
|  | As Needed | Sight glasses |
|  | As Needed | 5 L Glass Bottle |
|  |  | 10 L Glass Bottle |
|  |  | 20 L Glass Bottle |

| Part No./ Equipment ID | Quantity | Description |
| --- | --- | --- |
|  | As Needed | UFDF Membranes |
| 6001576 |  | PELLICON 3 30kDa (C Screen) 0.11 m2 |
| 4102051 |  | PELLICON 3 30kDa (C Screen) 0.57 m2 |
| 4102050 6001350 |  | PELLICON 3 30kDa (C Screen) 1.14 m2 Pellicon Manifold Support Plate |
|  | As Needed | Portable Formulation Tanks |
| TK-1523 |  | 100 L Diafiltration / Concentration Tank |
| TK-1927 |  | 200 L Diafiltration / Concentration Tank |
| TK-2149 |  | 100 L Diafiltration / Concentration Tank |
| TK-2150 |  | 100 L Diafiltration / Concentration Tank |
|  | As Needed | Vent Filters for the Diafiltration / Concentration Tanks |
| 4000214 |  | Filter – Aervent, 0.2 µm, Hydrophobic, PTFE Membrane (5 in.) |
| 4102547 |  | Filter – Aervent, 0.2 μm, Hydrophobic PTFE Membrane (10 in.) |
|  | As Needed | Product Filters for Filtration |
| 6004083 |  | Filter – 10-inch Opticap XLT Capsule w/ SHC Membrane, Sterile |
| 4101032 * |  | Filter- 10 in., Opticap XLT 10 Capsule Filter with Millipore Express SHC Hydrophilic Membrane, 0.5/0.2 m |
| 4101736 |  | Filter – SHC 10” 0.5+0.2 um Capsule with C-flex |
| 4101737 |  | Filter – SHC 3” 0.5+0.22 um Capsule with C-flex |
| 4100987 * |  | Filter – 30 in., Opticap XLT 30 Capsule Filter with Millipore Express SHC Hydrophilic Membrane, 0.5/0.2 m |
| 4101010 |  | Filter- 4 in., Opticap XL Capsule Filter with Hydrophilic Durapore Membrane, 0.22 m |
| 4102542 |  | Filter- Millipak 200 0.22 μm, Hydrophilic Gamma Irradiated |
|  | As Needed | Bags for Product Mixing/Collection |
| 4102059 |  | LevTech Mix Bag (50 L) |
| 4102062 |  | LevTech Mix Bag (100 L) |
| 4102060 |  | LevTech Mix Bag (200 L) |
| 4102032 |  | LevTech Mix Bag (300 L) |
| 4102061 |  | LevTech Mix Bag (560 L) |
|  | As Needed | Bags for WFI |
| 4101685 |  | Bags – Stedim 3D Bags with C-flex (300 L) |
| 4101687 |  | Bags – Stedim 3D Bags with C-flex (200 L) |
| 4101688 |  | Stedim Bag- 50 L Flexel 2D Bag with C-flex |

* = Materials with bolded filter part numbers require autoclaving prior to use

Additional Manufacturing Supplies Used in Process:

If additional materials (filter, bag, etc.) need to be used during processing:

- Prior to use, ensure the material is acceptable for use based on Section 4.

- Record the applicable material information.

- Document the reason for the change in Attachment 9: Comments Section.

| MPR Step No. | Material Description | Part No. | Batch No. | Serial No., if applicable | Exp. Date | Autoclave No., if applicable | Autoclave Exp. Date, if applicable | Performed by/Date | Witnessed by/Date | SAP Consumption Performed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Process Flowchart

| WFI Flush prior to Equilibration | Water for Injection (WFI) |
| --- | --- |



| Equilibration | 8012555: 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 |  |  |
| --- | --- | --- | --- |
| Batch No. |  | Exp. Date |  |



| Concentration 1 of the Virus Filtration (VF) Product (8012474) to 100 g/L |  |
| --- | --- |
| Batch No. |  |



| Diafiltration | 8012555: 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 |
| --- | --- |



Concentration 2 to 210 g/L



| Recovery flush to 190 g/L | 8012555: 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 |
| --- | --- |



| Dilution to 180 g/L | 8012555: 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 |
| --- | --- |



| Filtration, Sampling and Storage |  |  |
| --- | --- | --- |
|  |  |  |
| System Sanitization |  | A0013-D: 0.5 N Sodium Hydroxide |
| Sanitization buffer batch number and exp date are documented in Step 20.3. |  |  |
|  |  |  |
| System Storage |  | A0014-D: 0.1 Sodium Hydroxide |
| Storage buffer batch number and exp. date are documented in Step 20.3. |  |  |

| Recorded by/ Date |  | Verified by/ Date |  |
| --- | --- | --- | --- |

Additional Solution Batches Used in Process

If a solution (of a different batch noted in Section 5) needs to be replaced during processing:

- Indicate the switch at the step it is occurring. Ensure the new solution is the same part number as the solution being replaced.

- Record the solution part number, batch number (and/or process order number) and expiration date below.

- Document the change in Attachment 9: Comments Section.

| Applicable Step Number | Part Number | Batch Number | Exp. Date | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |

# Process Notes

## All activities are performed at 15 C to 27 C.

## For all calculations, assume 1 L = 1 kg and 1 mL = 1 g unless otherwise noted.

## Verify that all solutions, raw materials, and standards are within expiration prior to use.

## When an equipment ID is written in the MPR, the operator is indicating they have visually inspected and verified calibration prior to use.

## If additional mixing/recirculation is required after initial sampling of the product, record the mixing information in Attachment 9: Comments Section.

## CAUTION:  Sodium Hydroxide is corrosive. A face shield must be worn when the solution is being connected and/or disconnected from the UF/DF system.

## If additional WFI is pulled from the WFI drop during processing, record the WFI valve ID in Attachment 9: Comments Section.

## If a non-routine sample is requested by memo, complete Attachment 4: Non-Routine Sampling Record.

# Initial Product and UF Membrane Information

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Record the room number where this process will occur. |  |  |  |  |  |
|  | Room Number |  |  |  |  |  |
|  | Obtain and record the following information from the AZD0543 CIPDS (FORM-0071789) and Skid Logbook. |  |  |  |  |  |
|  | CIPDS part number |  |  |  |  |  |
|  | CIPDS lot number |  |  |  |  |  |
|  | Skid ID |  |  |  |  |  |
|  | Cassette installation date |  |  |  |  |  |
|  | No. of cassettes used |  |  |  |  |  |
|  | Cassette part number |  | 6001576 4102051 4102050 |  |  |  |
|  | Cassette batch number |  |  |  |  |  |
|  | Cassette expiration date |  |  |  |  |  |
|  | Additional cassette batch number, if applicable |  |  |  |  |  |
|  | Additional cassette expiration date, if applicable |  |  |  |  |  |
|  | Hold-up volume of the UF/DF Skid |  | L=kg |  |  |  |
|  | Total surface area of cassettes |  | m2 |  |  |  |
|  | Support Plate part number |  |  |  |  |  |
|  | Support Plate batch number |  | Plate #1 | Plate #2 |  |  |
|  | Support Plate expiration date |  | Plate #1 | Plate #2 |  |  |
|  | Logbook Number |  |  |  |  |  |
|  | CIPDS SAP Consumption Performed by/Date |  |  |  |  |  |

| Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | Pre-Processing Activities for the UF/DF Skid - Verify the data for the UF/DF Skid to be used. - From the logbook, record in Step 7.4 the logbook number, the CIPDS batch number, and the date when the actions were documented. - Ensure that the UF/DF Skid’s most immediate storage after sanitization has not exceeded 30 days storage. - If the storage duration is greater than 30 days, contact a supervisor and document actions taken in Attachment 9: Comments Section. |  |  |

| ## Pre-Processing Activities for the UF/DF Skid |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Action | Logbook Number | CIPDS Batch Number | Date | Performed By/Date | Witnessed By/Date |
| Sanitization |  |  |  |  |  |
| NWP |  |  |  |  |  |
| Storage |  |  |  |  |  |
| DF-1244 Skid Storage Duration (≤ 30 days) |  |  |  |  |  |

| Step | Description | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- |
|  | # In the Step 7.6 “Virus Filtration Product Information” table: - If a tank was used, N/A Vessels 2 and 3. For bag(s), N/A any unused Vessel fields. - Record the net weight of each Virus Filtration Product from (MABR-0027575) MPR or the label of each product transport vessel. - Record the tare weight of each Virus Filtration Product Vessel. - Record the product concentration of each Virus Filtration Product Vessel. # Record the corresponding DLIMS project number and sample number for the Virus Filtration Product(s) vessels. |  |  |

| ## Virus Filtration Product Information |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Vessel 1 | Vessel 2, if applicable |  | Vessel 3, if applicable |  | SAP Consumption Performed by/Date |
| Virus Filtration Product Net Weight | kg | kg |  | kg |  |  |
| Tare Weight | kg | kg |  | kg |  |  |
| VF Product Concentration | g/L | g/L |  | g/L |  |  |
| DLIMS Sample Number |  |  |  |  |  |  |
| DLIMS Project Number |  |  |  |  |  |  |
| Recorded By/Date |  |  | Verified By/ Date |  |  |  |

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the Virus Filtration Product volume in each vessel. NOTE: N/A any unused Vessel fields below. |  |  |  |  |  |  |
|  | Virus Filtration Product Net Weight (Step 7.6) |  | Density of VF Product | = | Virus Filtration (VF) Product Volume |  |  |
|  | Vessel 1 |  | 1.014 kg/L |  | Vessel 1 |  |  |
|  | kg |  |  |  | L |  |  |
|  | Vessel 2, if applicable |  |  |  | Vessel 2, if applicable |  |  |
|  | kg |  |  |  | L |  |  |
|  | Vessel 3, if applicable |  |  |  | Vessel 3, if applicable |  |  |
|  | kg |  |  |  | L |  |  |
|  | Calculate the total grams of Virus Filtration Product in each vessel. NOTE: N/A any unused Vessel fields below. |  |  |  |  |  |  |
|  | Virus Filtration (VF) Product Volume (Step 7.7) | X | Virus Filtration Product Concentration (Step 7.6) | = | Total grams of VF Product per Vessel |  |  |
|  | Vessel 1 |  | Vessel 1 |  | Vessel 1 |  |  |
|  | L |  | g/L |  | g |  |  |
|  | Vessel 2, if applicable |  | Vessel 2, if applicable |  | Vessel 2, if applicable |  |  |
|  | L |  | g/L |  | g |  |  |
|  | Vessel 3, if applicable |  | Vessel 3, if applicable |  | Vessel 3, if applicable |  |  |
|  | L |  | g/L |  | g |  |  |
|  | Total grams of VF Product before sampling (Vessel 1 grams + Vessel 2 grams + Vessel 3 grams) |  |  |  | g |  |  |

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the total membrane surface area needed for the UFDF process: |  |  |  |  |  |  |
|  | Total grams of VF Product before sampling (Step 7.8) | ¸ | Maximum Load (g of Product / m2) | = | Minimum membrane surface area |  |  |
|  | g |  | 1100 g/m2 |  | m2 |  |  |
|  | Verify the total installed surface area of cassettes (Step 7.2) is greater than or equal to the minimum membrane surface area calculated in Step 7.9. |  |  |  |  |  |  |

# WFI Flush prior to Equilibration

| Step | Description |  |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set up per SOP-0107111. WFI collected/obtained directly from the WFI Hold Tank (TK-1458) per SOP-0080303: - Verify the SIP of the WFI Hold Tank was completed and the tank status is “Clean.” - Before connecting to the UF skid, WFI Flush the TK-1458 WFI port on the transfer panel per the following instructions: - Ensure an appropriately sized Tee with bleed valve is connected to the manual valve leading from TK-1458 on the transfer panel, then attach a clean hose to the other end of the Tee. - Connect the other end of the hose to a process drain port. - Ensure any valves on the process drain port are open, if applicable. - Open the manual valve leading from the TK-1458 port. Ensure the valve is completely open. - Flush TK-1458 through the transfer panel port manual valve to process drain for ≥30 seconds, then close the manual valve on the TK-1458 transfer panel port. - N/A the bag information. WFI was/will be collected into a bag: - Record the WFI collection vessel information. - Verify that the WFI port has been flushed per SOP-0080185. - Charge into the vessel and record the charge information. |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Part No. |  |  | Batch No. |  |  | Exp. Date |  |  |
|  | Bag, if applicable |  |  |  |  |  |  |  |  |  |  |
|  | WFI Valve ID/Tank ID |  |  |  |  |  |  |  |  |  |  |
|  | WFI Charge Date/Time |  | Date: |  |  |  | Time: |  |  |  |  |
|  | WFI Expiration Date/Time* |  | Date: |  |  |  | Time: |  |  |  |  |
|  | * The WFI Expiration Date/Time is 24 hours from the WFI Charge Date/Time. |  |  |  |  |  |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |  |  |  |  |  |
| Step |  | Description |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
|  |  | - Confirm that the pH meter has been standardized per SOP-0080297. - Confirm that the conductivity meter has been standardized per SOP-0080297. |  |  |  |  |  |  |  |  |  |
|  |  | Meter ID |  |  |  |  |  |  |  |  |  |
|  |  | Confirm that the thermometer is within its calibration interval. Record the equipment ID of the thermometer used. |  |  |  |  |  |  |  |  |  |
|  |  | Thermometer ID |  |  |  |  |  |  |  |  |  |
|  |  | Ensure that the Ultrafiltration (UF) system is set up for WFI flush of the installed cassettes according to SOP-0107111. |  |  |  |  |  |  |  |  |  |
|  |  | On the UF/DF Skid HMI, enter the total installed cassette surface area in m2 (Step 7.2): - Click on the membrane Area Setpoint input box. - Enter Total Surface Area of Cassettes in m2 (Step 7.2) and press ENTER. - Click ACCEPT on the Membrane Area Change pop-up window. |  |  |  |  |  |  |  |  |  |
|  |  | - Connect the WFI supply inlet to the UF/DF Skid feed line. - Ensure that the retentate and permeate lines on the UF/DF Skid are directed to waste. |  |  |  |  |  |  |  |  |  |
|  |  | Calculate the target volume of solution needed for a 22.0 L/m2 WFI and Equilibration flushes. |  |  |  |  |  |  |  |  |  |
|  |  | Target Flush Volume | x |  | Total Surface Area of Cassettes (Step 7.2) |  | = | Target WFI and Equilibration Flush Volume |  |  |  |
|  |  | 22.0 L/m2 |  |  | m2 |  |  | L |  |  |  |

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Download and run DF1244_Flush according to SOP-0107111 to flush the UF/DF Skid with WFI. - Record the batch ID and start time. The batch ID should follow this format: MPR Part Number_PrepWFI_Batch number_Processing Date NOTE: The membranes should be flushed no more than 24 hours before use. |  |  |  |  |  |
|  | Batch ID |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |
|  | At the user prompt, input the flush volume set points per the table below. |  |  |  |  |  |
|  | Process Step |  | Input Title | Entered Set Point |  |  |
|  | Permeate Flush |  | PERMEATE_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | Retentate Flush |  | RET_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | - Go to the Recipe Control screen on the UF/DF Skid HMI, and push START to initiate the pre-use WFI flush. - Perform the WFI flush of the UF/DF Skid per SOP-0107111. - Sample and document the flush conductivities per Step 8.12. |  |  |  |  |  |
|  | At the completion of the recipe, the status changes from “Running” to “Complete” and the pump stops. |  |  |  |  |  |

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | At the end of the recipe, take a sample of the retentate and permeate and record the results below. |  |  |  |
|  | Retentate Values |  |  |  |
|  | Conductivity (≤ 1 mS/cm) | mS/cm |  |  |
|  | Temperature (15 – 25°C) | °C |  |  |
|  | Permeate Values |  |  |  |
|  | Conductivity (≤ 1 mS/cm) | mS/cm |  |  |
|  | Temperature (15 – 25°C) | °C |  |  |
|  | - If the conductivity is in range, N/A the remainder of Section 8 and go to Section 9. - If the conductivity is not in range, go to the next step. |  |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Download and run DF1244_Flush according to SOP-0107111 to flush the UF/DF Skid with WFI. - Record the batch ID and start time. The batch ID should follow this format: MPR Part Number_PrepWFI2_Batch Number_Processing Date. NOTE: The membranes should be flushed no more than 24 hours prior to use. |  |  |  |  |  |
|  | Batch ID |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |
|  | At the user prompt, input the flush volumes per the table below. |  |  |  |  |  |
|  | Process Step |  | Input Title | Entered Set Point |  |  |
|  | Permeate Flush |  | PERMEATE_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | Retentate Flush |  | RET_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | - Go to the Recipe Control screen on the UF/DF Skid HMI, and push START to initiate the WFI flush of the UF/DF Skid. - Perform the WFI flush of the UF/DF Skid per SOP-0107111. - Sample and document the flush conductivities per Step 8.18. |  |  |  |  |  |
|  | At the completion of the recipe, the status changes from “Running” to “Complete” and the pump stops. |  |  |  |  |  |

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | - Take a sample of the retentate and permeate and record the results below. - If the conductivities are not within the specified range after repeating the recipe, contact an area supervisor and document actions taken in Attachment 9: Comments Section. |  |  |  |
|  | Retentate Values |  |  |  |
|  | Conductivity (≤ 1 mS/cm) | mS/cm |  |  |
|  | Temperature (15 – 25°C) | °C |  |  |
|  | Permeate Values |  |  |  |
|  | Conductivity (≤ 1 mS/cm) | mS/cm |  |  |
|  | Temperature (15 – 25°C) | °C |  |  |

# End of Virus Filtration Product Storage

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## If the Virus Filtration Product was warmed up (from cold storage), ensure the time the product was removed from cold storage is recorded in Attachment 5: Hold Times. |  |  |  |  |  |  |
|  | Mix the Virus Filtration product for at least 15 minutes and record mixing information below: - Follow SOP-0080423 for the LevTech mixer. - Follow SOP-0107095 for agitation in fixed tanks. - If mixing by recirculation, N/A this step and document mixing in Attachment 7: Product Recirculation Work Sheet. - N/A any unused Vessel fields below. |  |  |  |  |  |  |
|  | Vessel 1 |  |  |  |  |  |  |
|  | Mixer ID (if applicable) |  |  |  |  |  |  |
|  | Agitation Speed |  | RPM |  |  |  |  |
|  | Start Time |  |  | End Time |  |  |  |
|  | Duration (≥ 15 min) |  | min |  |  |  |  |
|  | Vessel 2, if applicable |  |  |  |  |  |  |
|  | Mixer ID (if applicable) |  |  |  |  |  |  |
|  | Agitation Speed |  | RPM |  |  |  |  |
|  | Start Time |  |  | End Time |  |  |  |
|  | Duration (≥ 15 min) |  | min |  |  |  |  |
|  | Vessel 3, if applicable |  |  |  |  |  |  |
|  | Mixer ID (if applicable) |  |  |  |  |  |  |
|  | Agitation Speed | RPM |  |  |  |  |  |
|  | Start Time |  |  | End Time |  |  |  |
|  | Duration (≥ 15 min) | min |  |  |  |  |  |

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Remove at least the sample volume per Attachment 3: Sampling Record from each of the product collection vessel per SOP-0107097. - If non-routine samples are requested by memo, complete Attachment 4: Non-Routine Sampling Record. - N/A any unused Vessel fields below. |  |  |  |  |  |
|  | Sampling Date and Time, Vessel 1 | Date: |  | Time: |  |  |
|  | Sampling Date and Time, Vessel 2, if applicable | Date: |  | Time: |  |  |
|  | Sampling Date and Time, Vessel 3, if applicable | Date: |  | Time: |  |  |
|  | - Aliquot the sample (sample designation: 8012475-10-1, 8012475-10-2, or 8012475-10-3, as applicable) per Attachment 3: Sampling Record. - If applicable, aliquot non-routine samples per Attachment 4: Non-Routine Sampling Record. - Label the sample per SOP-0107056. |  |  |  |  |  |
|  | - Submit the sample to the storage location per SOP-0080295. - Ensure the sampling time and location are updated in DLIMS. |  |  |  |  |  |
|  | - Place the remaining sample in a 50 ml conical tube and measure the temperature of the sample per SOP-0107117. - Record the reading below. - N/A any unused Vessel fields below. |  |  |  |  |  |
|  | Vessel 1 Temperature |  | °C |  |  |  |
|  | Vessel 2 Temperature, if applicable |  | °C |  |  |  |
|  | Vessel 3 Temperature, if applicable |  | °C |  |  |  |
|  | If the VF product is in a bag or portable tank, N/A this step. Otherwise, continue below. In Purification, ensure the following: - All the necessary transfer panel connections are made for transfer of the VF product to Formulation per SOP-0107099. - The condensate flex hose on the transfer panel is removed and the manual valve that the flex hose was attached to is closed. Use SOP-0080548 for Large Purification fixed tanks and SOP-0080323 for Small Purification fixed tanks. |  |  |  |  |  |

| Step | Description |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Weigh the product vessel(s) and record below. N/A any unused Vessel fields. - Calculate the Net Weight for each vessel. - The “Total Net Weight after sampling” is the sum of the net weights for each vessel. |  |  |  |  |  |  |  |  |  |
|  | Vessel | 1 |  |  | 2, if applicable |  |  | 3, if applicable |  |  |
|  | Gross Weight | kg |  |  | kg |  |  | kg |  |  |
|  | Virus Filtration Product Vessel Tare Weight (Step 7.6) | kg |  |  | kg |  |  | kg |  |  |  |
|  | Net Weight a | kg |  |  | kg |  |  | kg |  |  |
|  | Total Net Weight of VF Product after sampling (Vessel 1 + Vessel 2 + Vessel 3) |  |  |  | kg |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |  |  |
|  | Cal. Due Date |  |  |  |  |  |  |  |  |  |
|  | - Net Weight = Gross weight – Virus Filtration Product Vessel Tare Weight |  |  |  |  |  |  |  |  |  |
|  | Calculate the Virus Filtration Product Volume after sampling. NOTE: N/A any unused Vessel fields below. |  |  |  |  |  |  |  |  |  |
|  | Net Weight (Step 9.8) |  |  | Density of VF Product |  | = | Virus Filtration Product Volume after sampling |  |  |  |
|  | Vessel 1 |  |  | 1.014 kg/L |  |  | Vessel 1 |  |  |  |
|  | kg |  |  |  |  |  | L |  |  |  |
|  | Vessel 2, if applicable |  |  |  |  |  | Vessel 2, if applicable |  |  |  |
|  | kg |  |  |  |  |  | L |  |  |  |
|  | Vessel 3, if applicable |  |  |  |  |  | Vessel 3, if applicable |  |  |  |
|  | kg |  |  |  |  |  | L |  |  |  |

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the Total Grams of Virus Filtration Product after sampling. NOTE: N/A any unused Vessel fields below. |  |  |  |  |  |  |
|  | Virus Filtration Product Volume after sampling (Step 9.9) | x | Virus Filtration Product concentration (Step 7.6) | = | Total Grams of Virus Filtration Product after sampling per Vessel |  |  |
|  | Vessel 1 |  | Vessel 1 |  | Vessel 1 |  |  |
|  | L |  | g/L |  | g |  |  |
|  | Vessel 2, if applicable |  | Vessel 2, if applicable |  | Vessel 2, if applicable |  |  |
|  | L |  | g/L |  | g |  |  |
|  | Vessel 3, if applicable |  | Vessel 3, if applicable |  | Vessel 3, if applicable |  |  |
|  | L |  | g/L |  | g |  |  |
|  | Total grams of Virus Filtration Product after sampling (Vessel 1 grams + Vessel 2 grams + Vessel 3 grams) |  |  |  | g |  |  |

# Equilibration

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | After the skid WFI flush is completed and prior to proceeding to Equilibration, standardized the skid’s UV meter per SOP-0107111. |  |  |  |  |
|  | - Obtain the equilibration buffer, 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 (PN: 8012555). - Connect the equilibration buffer to the UF/DF Skid feed line. |  |  |  |  |
|  | Remove a sample of the equilibration buffer and record the results below. |  |  |  |  |
|  | pH (5.7 - 6.1) |  |  |  |  |
|  | Conductivity |  | mS/cm |  |  |
|  | Temperature (15 - 25°C) |  | °C |  |  |
|  | - Download and run DF1244_EQUIL according to SOP-0107111 to flush the UF membrane(s). - Record the batch ID and start time. The batch ID should follow this format: MPR Part Number_Equil_Batch Number_Processing Date. |  |  |  |  |
|  | Batch ID |  |  |  |  |
|  | Start Time |  |  |  |  |

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | At the user prompt, input the values per the table below. |  |  |  |  |  |
|  | Process Step | Input Title |  | Entered Set Point |  |  |
|  | Permeate Flush | PERMEATE_FLUSH_VOL |  | ≥ Step 8.7 |  |  |
|  | Retentate Flush | RET_FLUSH_VOL |  | ≥ Step 8.7 |  |  |
|  | Trans-Membrane Pressure (psid) | EQUIL_TMP |  | 20 a |  |  |
|  | Cross Flow (L/min/ft2) c | EQUIL_Crossflow |  | 0.5 b, c |  |  |
|  | - The TMP can be set as needed up to NOR of ≤ 35 psid. If the TMP is adjusted, record the value in Attachment 9: Comments Section. Ensure the feed pressure does not exceed NOR of ≤ 60 psig. - Cross flow can be adjusted within the NOR range of 0.3 – 0.7 L/min/ft2 as needed to ensure feed pressure does not exceed NOR of ≤ 60 psig. If the cross flow is adjusted, record the new value in Attachment 9: Comments Section. - The UF/DF Skid measures the cross flow in L/min/ft2. For conversion reference, L/min/ft2 = L/min/m2 10 |  |  |  |  |  |
|  | Go to the Recipe Control screen on the UF/DF Skid HMI, and push START to initiate the equilibration of the UF/DF Skid. NOTE: Ensure the pH meets the range required per Step 10.8. |  |  |  |  |  |
|  | - At the completion of the recipe, the status changes from “Running” to “Complete” and the pump stops. - Record the amount of equilibration buffer 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 flushed using the UF/DF skid’s retentate and permeate totalizers. NOTE: The totalizer HMI tags are FQI-301 (retentate) and FQI-201 (permeate). |  |  |  |  |  |
|  | Retentate Totalizer (FQI-301) |  | L |  |  |  |
|  | Permeate Totalizer (FQI-201) |  | L |  |  |  |
|  | SAP Consumption of 8012555 Performed by/Date |  |  |  |  |  |

| Step | Description |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- |
|  | At the end of the recipe, take a sample of the permeate and record the results below. |  |  |  |
|  | Permeate Values |  |  |  |
|  | pH (5.7 - 6.1) |  |  |  |
|  | Conductivity | mS/cm |  |  |
|  | Temperature (15 - 25°C) | °C |  |  |
|  | - If the pH is in range, mark Steps 10.10 through 10.15 as N/A, then proceed to Section 11. - If the pH is not in range, go to the next step. |  |  |  |

| Step | Description |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Download and run DF1244_Equil according to SOP-0107111 to flush the UF membrane(s). - Record the batch ID and start time. The batch ID should follow this format: MPR Part Number_Equil2_Batch Number_Processing Date. |  |  |  |  |  |
|  | Batch ID |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |
|  | At the user prompt, input the values per the table below. |  |  |  |  |  |
|  | Process Step |  | Input Title | Entered Set Point |  |  |
|  | Permeate Flush |  | PERMEATE_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | Retentate Flush |  | RET_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | Trans-Membrane Pressure (psid) |  | EQUIL_TMP | 20 a |  |  |
|  | Cross Flow (L/min/ft2) c |  | EQUIL_Crossflow | 0.5 b |  |  |
|  | - The TMP can be set as needed up to NOR of ≤ 35 psid. If the TMP is adjusted, record the value in Attachment 9: Comments Section. Ensure the feed pressure does not exceed NOR of ≤ 60 psig. - Cross flow can be adjusted within the NOR range of 0.3 – 0.7 L/min/ft2 as needed to ensure feed pressure does not exceed NOR of ≤ 60 psig. If the cross flow is adjusted, record the new value in Attachment 9: Comments Section. - The UF/DF Skid measures the cross flow in L/min/ft2. For conversion reference, L/min/ft2 = L/min/m2 10 |  |  |  |  |  |
|  | Go to the Recipe Control screen on the UF/DF Skid HMI, and push START to initiate the equilibration of the UF/DF Skid. NOTE: Ensure the pH meets the range required per Step 10.14. |  |  |  |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - At the completion of the recipe, the status changes from “Running” to “Complete” and the pump stops. - Record the amount of equilibration buffer 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 flushed using the UF/DF skid’s retentate and permeate totalizers. NOTE: The totalizer HMI tags are FQI-301 (retentate) and FQI-201 (permeate). |  |  |  |  |  |
|  | Retentate Totalizer (FQI-301) | L |  |  |  |  |
|  | Permeate Totalizer (FQI-201) | L |  |  |  |  |
|  | SAP Consumption of 8012555 Performed by/Date |  |  |  |  |  |
|  | At the end of the recipe, take a sample from the permeate. Record the results below. |  |  |  |  |  |
|  | Permeate Values |  |  |  |  |  |
|  | pH (5.7 - 6.1) |  |  |  |  |  |
|  | Conductivity |  |  | mS/cm |  |  |
|  | Temperature (15 - 25°C) |  |  | °C |  |  |
|  | - If the pH is in range, go to the next step. - If the pH is not in range, contact an area supervisor and document actions in Attachment 9: Comments Section. |  |  |  |  |  |
|  | Supervisor Contacted |  |  |  |  |  |

# Vessel Set-up

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | - Obtain the Retentate Vessel. - Label the vessel “Retentate Vessel”. - Record the vessel’s tank ID, clean expiration date and time, sterilization expiration date and time, Cleaning and SIP Batch IDs and date of passed pressure test. - Record the volume of the Retentate Vessel. |  |  |  |
|  | Tank ID | TK-1927 |  |  |
|  | Cleaning Exp. Date / Time |  |  |  |
|  | Tank Date of Passed Pressure Test |  |  |  |
|  | Cleaning Batch ID |  |  |  |
|  | Sterilization Exp. Date/ Time |  |  |  |
|  | Sterilization Batch ID |  |  |  |
|  | Retentate Vessel Volume | L |  |  |
|  | Vent Filter Information |  |  |  |
|  | Part Number |  |  |  |
|  | Batch Number |  |  |  |
|  | Expiration Date |  |  |  |
|  | Serial Number |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |
|  | Verify that the vent filter is open on the Retentate Vessel. |  |  |  |

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Retentate Vessel Tare Set-Up - Label the manual valve on the dip tube of the tank as valve A. - Endcap and clamp valve A. - Label the tank outlet valve as valve B. - End cap the elbow connected to valve B. - Label the valve on the tank’s retentate tube as valve C. - End cap valve C. |  |  |  |
|  | - Ensure that the Retentate Tank vent filter is open per Step 11.2, then open Valves A, B, and C all the way (while they are end capped) to relieve pressure between the endcap and the valve outlets. - Then, close Valves A, B, and C. |  |  |  |
|  | - Record the tare weight of the vessel before it is attached to the UF/DF System. - Record the scale/balance equipment ID and calibration due date. |  |  |  |
|  | Scale / Balance ID |  |  |  |
|  | Scale/Balance Calibration Due Date |  |  |  |
|  | Tare weight of Retentate Vessel before attachment | kg |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Per Attachment 6: Retentate Tank , ensure a sight glass is installed on the following: - On the VF Product transfer panel manual valve outlet (valve H) If the VF Product is in a bag, N/A the “Transfer Panel (Valve H)” sight glass fields in this step. - Prior to valve A (which is attached to the Retentate Vessel inlet). - Prior to valve C (which is attached to the retentate tube on the Retentate Vessel). - Record the sight glass information below. |  |  |  |  |  |
|  | Sight Glass Information |  |  |  |  |  |
|  |  | Transfer Panel (Valve H), if applicable | Valve A | Valve C |  |  |
|  | Autoclave Cycle No. |  |  |  |  |  |
|  | Autoclave Exp. Date |  |  |  |  |  |

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Retentate Vessel and UF/DF System Set-Up - Set up the Retentate Vessel and DF-1244 system per Attachment 6: Retentate Tank and DF-1244 Set-up. - Record the tubing/hose information in Attachment 1: Process Tubing/Hose Information. - For tubing, record the tubing part number, batch number, expiration date, autoclave cycle number, and autoclave expiration date. - For hoses, record the hose serial number. NOTE: If hoses are used and a serial number is not available, just record that a hose was used in Attachment 1: Process Tubing/Hose Information. - Obtain elbows, tees, and manual valves as needed and connect these components to the Retentate Vessel and the UF/DF skid per Attachment 6: Retentate Tank and DF-1244 Set-up. - Ensure that manual valves A, B, C, D, E, F, G, H, and I are labeled. - Label the Permeate, Retentate, Feed, Recovery, and Charge/Diafiltration (DF) lines. |  |  |  |
|  | Prepare for system priming. Ensure the following valve states: |  |  |  |
|  | Valve States | UF/DF System |  |  |
|  | Closed | A, B, C, D, E, F, G, H, I |  |  |
|  | Opened | N/A |  |  |

| Step | Description | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | - Ensure that valves E and H are closed. - Initiate the transfer phase on the PCS HMI for the VF Product and all the buffers and the that are coming through the transfer panel. NOTE: Confirm that connections are made in Large Purification before proceeding. |  |  |
|  | System Prime: Recovery Line - Prime the Recovery Line: - Ensure valves A, B, C, D and F are closed. - Open bleed valve B-2. Ensure the tubing is directed to waste. - Open valves E and G to prime the Recovery Line with the DF Buffer. - If necessary, use a pump to prime the Recovery Line. - Once no more air is observed exiting bleed valve B-2, close valve E then close bleed valve B-2. - Close valve G. NOTE: Buffer charging is performed using diafiltration buffer (PN: 8012555 : 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9) |  |  |
|  | System Prime: Charge/Diafiltration (DF) Line - Prime the Charge/DF Line: - Ensure valves A, B, C, D, and I are closed. - Open bleed valve B-1. Ensure the tubing is directed to waste. - Open valves E and F from the Diafiltration Buffer to the Charge/DF Line. - Use the pump to prime the Charge/DF Line, as necessary. - Once no more air is observed exiting bleed valve B-1, close valve E, then close bleed valve B-1. - Close valve F. NOTE: Buffer charging is performed using diafiltration buffer (PN: 8012555 : 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9) |  |  |

| Step | Description |  | Performed by/Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | - Record the tare weight of the vessel after it is attached to the UF/DF System. - Record the scale/balance equipment ID and calibration due date. |  |  |  |
|  | Scale / Balance ID |  |  |  |
|  | Scale / Balance Calibration Due Date |  |  |  |
|  | Tare weight of Retentate Vessel after attachment | kg |  |  |
|  | Secure the wheels of the Retentate Vessel. |  |  |  |
|  | Tare (zero) the scale holding the Retentate Vessel. |  |  |  |

# Process Input Calculations

| Step | Description |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the volume at 80% Retentate Vessel Capacity. |  |  |  |  |  |  |  |  |
|  | Retentate Vessel Volume (Step 11.1) |  | X | Capacity Limit |  | = | Retentate Vessel Charge Volume/Weight at 80% Capacity |  |  |
|  | L |  |  | 0.8 |  |  | L = kg |  |  |
|  | Calculate the volume at 40% Retentate Vessel Capacity. |  |  |  |  |  |  |  |  |
|  | Retentate Vessel Volume (Step 11.1) | X |  | Buffer Charge Target | = |  | Retentate Vessel Charge Volume/Weight at 40% Capacity |  |  |
|  | L |  |  | 0.4 |  |  | L = kg |  |  |

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the product volume in the UF/DF Skid after Concentration 1. |  |  |  |  |  |  |
|  | Total Grams of VF Product after sampling (Step 9.10) |  | Concentration 1 Range | = | Product volume in the UF/DF System after Concentration 1 |  |  |
|  | g |  | 110 g/L (max) |  | L (Min) |  |  |
|  |  |  | 100 g/L (Target) |  | L (Target) |  |  |
|  |  |  | 90 g/L (min) |  | L (Max) |  |  |
|  | Calculate the product volume in the Retentate Vessel after Concentration 1. |  |  |  |  |  |  |
|  | Product volume in the UF/DF System after Concentration 1 (Step 12.3) | − | Hold-up volume of the UF/DF Skid (Step 7.2) | = | Product volume in the Retentate Vessel after Concentration 1 |  |  |
|  | L (Min) |  | L |  | L (Min) |  |  |
|  | L (Target) |  |  |  | L (Target) |  |  |
|  | L (Max) |  |  |  | L (Max) |  |  |
|  | Calculate the product weight in the Retentate Vessel after Concentration 1. |  |  |  |  |  |  |
|  | Product volume in the Retentate Vessel after Concentration 1 (Step 12.4) | X | Density of the Concentration 1 Product | = | Product weight in the Retentate Vessel after Concentration 1 |  |  |
|  | L (Min) |  | 1.041 kg/L |  | kg (Min) |  |  |
|  | L (Target) |  |  |  | kg (Target) |  |  |
|  | L (Max) |  |  |  | kg (Max) |  |  |

| Step | Description |  |  |  |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the permeate volume range for Diafiltration. |  |  |  |  |  |  |  |  |  |  |  |
|  | Diafiltration Volume Range (DVs) | X |  | Product Volume in the UF/DF System after Concentration 1 (Step 12.3) |  |  | = |  |  | Permeate volume range after Diafiltration |  |  |
|  | 5.5 (min) |  |  | L |  |  |  |  |  | L (Min) |  |  |
|  | 6.0 (target) |  |  |  |  |  |  |  |  | L (Target) |  |  |
|  | Calculate the product volume range in the UF/DF System after Concentration 2 |  |  |  |  |  |  |  |  |  |  |  |
|  | Total Grams of VF Product after sampling (Step 9.10) |  |  | Concentration 2 Range |  | = |  |  | Product volume in the UF/DF System after Concentration 2 |  |  |  |
|  | g |  |  | 210 g/L (target) |  |  |  |  | L (Target) |  |  |  |
|  |  |  |  | 171 g/L (min) |  |  |  |  | L (Max) |  |  |  |
|  | Calculate the product volume range in the Retentate Vessel after Concentration 2. |  |  |  |  |  |  |  |  |  |  |  |
|  | Product volume in the UF/DF System after Concentration 2 (Step 12.7) |  | − |  | Hold-up volume of the UF/DF Skid (Step 7.2) | = |  | Product volume in the Retentate Vessel after Concentration 2 |  |  |  |  |
|  | L (Min) |  |  |  | L |  |  | L (Min) |  |  |  |  |
|  | L (Target) |  |  |  |  |  |  | L (Target) |  |  |  |  |
|  | L (Max) |  |  |  |  |  |  | L (Max) |  |  |  |  |

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the net weight range of the product in the Retentate Vessel after Concentration 2. |  |  |  |  |  |  |
|  | Product volume in the Retentate Vessel after Concentration 2 (Step 12.8) | X | Density of the product after Concentration 2 at target | = | Net Weight of product in the Retentate Vessel after Concentration 2 |  |  |
|  | L (Min) |  | 1.071 kg/L |  | kg (Min) |  |  |
|  | L (Target) |  |  |  | kg (Target) |  |  |
|  | L (Max) |  |  |  | kg (Max) |  |  |

# Concentration 1

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Ensure the Retentate Vessel is in the same configuration as it was when its Tare Weight after attachment was obtained in Step 11.12. |  |  |  |  |
|  | To prepare for buffer charge, verify the following valve states on the Retentate Vessel. |  |  |  |  |
|  | Valve State | UF/DF System |  |  |  |
|  | Closed | A, B, C, D, E, F, G, H, I |  |  |  |
|  | Opened | N/A |  |  |  |
|  | Ensure that skid’s hydraulic valves to the feed (XV101), retentate (XV301 and XV302) and permeate (XV201) lines are closed. |  |  |  |  |
|  | - Open valves A and F. - Slowly open valve E from 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 (PN: 8012555) and charge the Retentate Vessel with buffer to achieve the Vessel Charging Volume at 40% (Step 12.2). - Once the weight from Step 12.2 has been reached, immediately close valves A, E, and F. - • Document the Weight of the Retentate Vessel after buffer charge below. |  |  |  |  |
|  | Scale ID |  |  |  |  |
|  | Cal. Due Date |  |  |  |  |
|  | Weight of Retentate Vessel after buffer charge |  | kg |  |  |
|  | SAP Consumption of 8012555 Performed by/Date |  |  |  |  |

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | To prepare for recirculation, ensure the following: |  |  |  |  |  |  |
|  | Valve State | UF/DF System |  |  |  |  |  |
|  | Closed | A, D, E, F, G, H, I |  |  |  |  |  |
|  | Opened | B, C |  |  |  |  |  |
|  | On the UF/DF Skid HMI: - Open the valves to the feed (XV101), and retentate (XV301 and XV302). - Ensure the permeate valve (XV201) is closed. - Ensure all drain valves (XV103, XV102, and XV202) are closed. NOTE: Drain valves XV102, XV103, and XV202 should not be opened at any time during processing. |  |  |  |  |  |  |
|  | Recirculate the charging buffer, click on the “Semi-Automatic” mode on the UF/DF skid screen and input the following initial parameters in the order listed: |  |  |  |  |  |  |
|  | Parameter |  | Valve ID | Setpoint | Alarm |  |  |
|  | Feed Pressure (psig) |  |  | 25 a | > 60 |  |  |
|  | Feed Valve |  | XV101 | Open |  |  |  |
|  | Retentate Valves |  | XV301 | Open |  |  |  |
|  |  |  | XV302 |  |  |  |  |
|  | Permeate Valve |  | XV201 | Closed |  |  |  |
|  | Drain Valves |  | XV103 | Closed |  |  |  |
|  |  |  | XV102 |  |  |  |  |
|  |  |  | XV202 |  |  |  |  |
|  | - The feed pressure can be adjusted as needed to maintain a target TMP of 20 psid and a maximum AL feed pressure ≤ 60 psig. Record changes to transmembrane pressure and/or cross flow setpoint(s) in Attachment 2: Operation Parameters. Record any additional changes in Attachment 9: Comments Section. |  |  |  |  |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Record the start time as the time when the is pump turned on. |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |
|  | - Recirculate for at least 5 minutes. - At the completion of recirculation, stop the skid pump and record the end time and duration below. |  |  |  |  |  |
|  | End time for recirculation |  |  |  |  |  |
|  | Duration of recirculation (≥ 5 minutes) * |  |  | min |  |  |
|  | * NOTE: The duration is the time that elapses between the start time for recirculation (Step 13.8) and the end time for recirculation (this step). |  |  |  |  |  |
|  | Once buffer recirculation is complete, ensure the following valves on the Retentate Vessel are closed per the table below. |  |  |  |  |  |
|  | Valve State |  | UF/DF System |  |  |  |
|  | Closed |  | A, B, C, D, E, F, G, H, I |  |  |  |
|  | Opened |  | N/A |  |  |  |
|  | - Ensure that valves E and F from the charge buffer 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 (PN: 8012555) vessel are closed. - Ensure that the valve A, H, and I on the Charge/DF Line are closed. - Ensure valves to feed (XV101) and retentate (XV301 and XV302) are open. - Ensure the permeate (XV201) line is closed. |  |  |  |  |  |

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Charge the Retentate Vessel with the appropriate amount of Virus Filtered Product according to the charging condition below. - Step 9.8: Total VF Product Net Weight after Sampling - Step 12.2: Retentate Vessel Charge Volume/Weight at 40% Capacity |  |  |  |  |  |  |
|  | Condition | Check one | Action to take |  |  |  |  |
|  | Step 12.2 ≥ Step 9.8 |  | Charge in all the VF product (Step 9.8) |  |  |  |  |
|  | Step 12.2 ≤ Step 9.8 |  | Charge to the Retentate Vessel Charge Volume/Weight at 80% Capacity (Step 12.1) |  |  |  |  |
|  | Prior to charging in the VF product, ensure the VF Product line and the Charge/DF Line are bled of air by ensuring the following valve states: |  |  |  |  |  |  |
|  | Valve State |  |  | UF/DF System |  |  |  |
|  | Closed |  |  | A, B, C, D, E, F, G |  |  |  |
|  | Opened |  |  | H, I, B1 |  |  |  |
|  | - Once no more air is observed exiting from B-1, close B-1 and then close valves H and I. - Open valve A. |  |  |  |  |  |  |
|  | - Open the valve H, then slowly open valve I to charge the appropriate weight of VF product into the Retentate Vessel per Step 13.12, then close valves H and I. - Record the weight of the Retentate Vessel after charging and the scale information below. - Record the start time of product charge below. |  |  |  |  |  |  |
|  | Start Time of Product Charge |  |  |  |  |  |  |
|  | Weight of the Retentate Vessel after product charge |  |  |  | kg |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |
|  | Scale/Balance Calibration Due Date |  |  |  |  |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | - Turn on the agitator. Record the agitator speed setting. - Adjust the agitation speed as necessary minimizing foaming. Record changes in speed in Attachment 2: Operation Parameters, and rationale for change in Attachment 9: Comments Section. |  |  |  |  |
|  | Agitator Setting |  | RPM |  |  |
|  | Prior to product recirculation, ensure the following valve states: |  |  |  |  |
|  | Valve State | UF/DF System |  |  |  |
|  | Closed | A, D, E, F, G, H, I |  |  |  |
|  | Opened | B, C |  |  |  |
|  | On the skid HMI: - Open the valves to the feed (XV101), and Retentate (XV301 and XV302). - Ensure the permeate valve (XV201) is closed. - Ensure all drain valves (XV103, XV102, and XV202) are closed. NOTE: Drain Valves XV102, XV103, and XV202 should not be opened at any time during processing. |  |  |  |  |

| ## Step | Description | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | To recirculate the charged product, click on the “Semi-Automatic” mode on the UF/DF Skid screen and input the following initial parameters in the order listed: Parameter; Valve ID; Setpoint; Alarm. Feed Pressure (psig); 25 a; > 60 b. Feed Valve; XV101; Open. Retentate Valve; XV301; Open. XV302. Permeate Valve; XV201; Closed. Drain Valves; XV103; Closed. XV102. XV202. - The feed pressure can be adjusted as needed to maintain a target TMP of 20 psid and a maximum feed pressure of ≤ 60 psig. Record changes to TMP and/or cross flow setpoint(s) in Attachment 2: Operation Parameters. Record any additional changes in Attachment 9: Comments Section. - Max feed pressure NOR is ≤60 psig. |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | The start time for recirculation is the time the pump turned on. |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |
|  | - Continue to recirculate for at least five (5) minutes. - Record the end time and the duration of recirculation. - Stop the pump. - Ensure valve D is closed. |  |  |  |  |  |
|  | End time for recirculation |  |  |  |  |  |
|  | Duration of recirculation (≥ 5 minutes) * |  |  | min |  |  |
|  | * NOTE: The duration is the time that elapses between the start time for recirculation (Step 13.19) and the end time for recirculation (this step). |  |  |  |  |  |
|  | To prepare for Concentration 1: - Open the permeate valve (XV201) on the skid HMI. - Ensure the manual valves on the Retentate Vessel are open or closed per the table below. |  |  |  |  |  |
|  | Valve State |  | UF/DF System |  |  |  |
|  | Closed |  | D, E, F, G, I |  |  |  |
|  | Opened |  | A, B, C, H |  |  |  |
|  | - Click on the “Add Recipe” button in Recipe Control screen. - Find and Select DF1244_CONC from the list. - Record the Concentration 1 batch ID and start time from the system overview screen. The batch ID should follow this format: MPR Part Number_Conc_1 _Product Lot Number_Processing Date. |  |  |  |  |  |
|  | Batch ID |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | At the user prompt, insert the following values. |  |  |  |  |  |
|  | Process Step | Input Title |  | Set Points |  |  |
|  | Trans-Membrane Pressure (psid) | Initial_Conc_TMP |  | 20 a |  |  |
|  | Cross Flow (L/min/ft2) | Initial_Conc_Crossflow |  | 0.5 b |  |  |
|  | - The transmembrane pressure can be set up to ≤ 35 psid as needed. If the value is adjusted, record the value in Attachment 9: Comments Section. Ensure the feed pressure is ≤ 60 psig. - The cross flow can be adjusted within the range of 0.3 – 0.7 L/min/ft2, as needed. If the value is adjusted, record the new value in Attachment 9: Comments Section. |  |  |  |  |  |
|  | - Push START to initiate the Concentration 1 process. - The Start Time is the time the pump turns on. |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Concentration 1 - Once the skid pump starts, carefully adjust manual valve I on the VF Product Line or adjust the pump speed to control the flow of VF Product into the Retentate Vessel. Maintain the gross charging weight from Step 13.14 until the Virus Filtration Product vessel(s) is empty. - Adjust the crossflow and transmembrane pressure (TMP) setpoint(s) from their initial values, as necessary. See Step 13.23, for appropriate ranges. - NOTE: If adjustment occurs, record the adjusted values, the time of adjustment, and the step number in Attachment 2: Operation Parameters. - Record additional changes to operational setpoints and the time of the change in Attachment 9: Comments Section. - Record the process data in Attachment 2: Operation Parameters approximately every 20 minutes. NOTE: If any of the maximum values below are exceeded, stop process immediately, contact an area supervisor, and document actions taken in Attachment 9: Comments Section. |  |  |  |  |
|  | Parameter | Target | Maximum |  |  |
|  | TMP (psid) | 20 | 35 |  |  |
|  | Feed Flow (L/min/ft2) | 0.5 | 0.7 |  |  |
|  | Feed Pressure (psig) | NA | 60 |  |  |
|  | UV (OD) | NA | 0.075 |  |  |

| Step | Description |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Once the Virus Filtration product vessel is confirmed empty, - Close manual valve H on the VF Product outlet. - Ensure the following valve states: |  |  |  |  |  |  |  |  |
|  | Valve State |  |  | UF/DF System |  |  |  |  |  |
|  | Closed |  |  | D, E, F, G, H, I |  |  |  |  |  |
|  | Opened |  |  | A, B, C |  |  |  |  |  |
|  | - Continue Concentration 1 until the Target product weight in the Retentate Vessel after Concentration 1 (Step 12.5) is achieved. - Once the Retentate Vessel reaches the weight in Step 12.5: - Stop the pump on the UF/DF skid. - Open the retentate valves (XV301 and XV302) completely. - Close the permeate valve (XV201). |  |  |  |  |  |  |  |  |
|  | At the end of Concentration 1, ensure the following: |  |  |  |  |  |  |  |  |
|  | Valve State |  | UF/DF System |  |  |  |  |  |  |
|  | Closed |  | A, D, E, F, G, H*, I* |  |  |  |  |  |  |
|  | Open |  | B, C |  |  |  |  |  |  |
|  | * NOTE: Valves H and I will remain closed for the rest of the UF/DF process. |  |  |  |  |  |  |  |  |
|  | Record the volume of permeate from the permeate totalizer at the end of Concentration 1. |  |  |  |  |  |  |  |  |
|  | Permeate Volume |  |  |  | L |  |  |  |  |
|  | Record the net weight of the product at the end of Concentration 1 from the retentate scale and scale information. |  |  |  |  |  |  |  |  |
|  | Net Weight of Product after Concentration 1 |  |  |  |  | kg |  |  |  |
|  | Scale ID |  |  |  |  | Cal. Due Date |  |  |  |

# Diafiltration

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Diafiltration (DF) Line Priming. Prime the Charge/DF Line with Diafiltration buffer 20 mM histidine/histidine-HCl, 120 mM arginine-HCl, pH 5.9 (8012555) by performing the following: - First, ensure valves A, D, E, F, and G are closed. - Open bleed valve B-1. Ensure the tubing is directed to waste. - Open valves E and F to prime the Charge/DF Line. - If necessary, use a pump to prime the Charge/DF Line. - Once no more air is observed exiting bleed valve B-1, close valve E, then close bleed valve B-1. - Close valve F. - Ensure that valves B and C are open. NOTE: Ensure no Diafiltration (DF) buffer enters the Retentate Vessel during the prime. |  |  |  |
|  | - Click on the “Add Recipe” button in Recipe Control screen. - Find and Select DF1244_Diafiltration from the list. - Record the Diafiltration batch ID and start time from the system overview screen. The batch ID should follow this format: MPR Part Number_Diafiltration_Product Lot Number_Processing Date. |  |  |  |
|  | Batch ID |  |  |  |
|  | Start Time |  |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | At the user prompt, enter the following values. |  |  |  |  |
|  | Process Step | Input Title | Set Points |  |  |
|  | Target Permeate Volume after Diafiltration | Diafiltration_Perm_Vol | Step 12.6 |  |  |
|  | Trans-Membrane Pressure (psid) | Diafiltration_TMP | 20 a |  |  |
|  | Cross Flow (L/min/ft2) | Diafiltration_Crossflow | 0.5 b |  |  |
|  | - The TMP can be set to ≤ 35 psid as needed. If the TMP is adjusted, record the value in Attachment 9: Comments Section. Ensure the feed pressure is ≤ 60 psig. - The cross flow can be adjusted within the range of 0.3 – 0.7 L/min/ft2, as needed. If the value is adjusted, record the new value in Attachment 9: Comments Section. |  |  |  |  |
|  | - Ensure that valves E and F remain closed, then open valve A to prepare for Diafiltration. - Push START to initiate the Diafiltration recipe |  |  |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Diafiltration: Once the skid pump starts: - Open valve E, then slowly open valve F to allow diafiltration buffer to enter the Retentate Vessel. - Adjust valve F or the buffer pump speed to control the flow of DF buffer to the Retentate Vessel to maintain the weight from Step 13.30. - Record the process data in Attachment 2: Operation Parameters approximately every 20 minutes. - Prior to the end of diafiltration, obtain samples from the retentate and permeate. Record the appropriate readings in Step 14.10. NOTE: If any of the maximum values below are exceeded, stop process immediately, contact an area supervisor, and document actions taken in Attachment 9: Comments Section. |  |  |  |  |
|  | Parameter | Target | Maximum |  |  |
|  | TMP (psid) | 20 | 35 |  |  |
|  | Feed Flow (L/min/ft2) | 0.5 | 0.7 |  |  |
|  | Feed Pressure (psig) | NA | 60 |  |  |
|  | UV (OD) | NA | 0.075 |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | At the completion of the Diafiltration recipe once the skid pump stops, close valves E and F. |  |  |  |  |  |
|  | Record the permeate volume from the permeate totalizer after Diafiltration and ensure it is within the range calculated in Step 12.6. |  |  |  |  |  |
|  | Permeate Volume |  | L |  |  |  |
|  | SAP Consumption of 8012555 Performed by/Date |  |  |  |  |  |
|  | Record the net weight of the product at the end of Diafiltration from the retentate scale and scale information. |  |  |  |  |  |
|  | Net Weight of Product after Diafiltration |  | kg |  |  |  |
|  | Scale ID |  | Cal. Due Date |  |  |  |

| Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the product volume in the Retentate Vessel after Diafiltration. |  |  |  |  |  |  |  |
|  | Net Weight of Product after Diafiltration (Step 14.8) |  | Density of the product after Diafiltration |  | = | Product Volume in the Retentate Vessel after Diafiltration |  |  |
|  | kg |  | 1.036 kg/L |  |  | L |  |  |
|  | Collect a sample from the permeate and retentate lines for pH, conductivity, and temperature measurement. Record the results below. NOTE: If the Retentate and Permeate pH values are not within range, contact a supervisor prior to proceeding and document actions taken in Attachment 9: Comments Section. |  |  |  |  |  |  |  |
|  | Retentate Values |  |  |  |  |  |  |  |
|  | pH (5.7 - 6.1) |  |  |  |  |  |  |  |
|  | Conductivity |  |  | mS/cm |  |  |  |  |
|  | Temperature (15 – 25°C) |  |  | °C |  |  |  |  |
|  | Permeate Values |  |  |  |  |  |  |  |
|  | pH |  |  |  |  |  |  |  |
|  | Conductivity |  |  | mS/cm |  |  |  |  |
|  | Temperature (15 – 25°C) |  |  | °C |  |  |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Proceed based on the Step 14.10 sample results: |  |  |  |  |  |
|  | Check One | The pH of the Step 14.10 retentate samples are: | Actions to take |  |  |  |
|  |  | In-range | - Stop the transfer phase on the PCS HMI for the Diafiltration buffer (PN: 8012555. - Close valves A and F. - N/A Steps 14.12 through 14.22, then go to Section 15. |  |  |  |
|  |  | Out-of-range | - Ensure Valves D, E, F, and G are closed. - Ensure Valves A, B and C are opened. - Go to the next step. |  |  |  |
|  | Contact area supervisor to determine additional volume to be used for coninuted Diafiltration. NOTE: Additional Volume = Target Permeate Volume after Continued Diafiltration NOTE: There is no NOR maximum volume for Diafiltration. |  |  |  |  |  |
|  | Additional Volume |  |  | L |  |  |
|  | Supervisor Contacted |  |  |  |  |  |

| Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Click on the “Add Recipe” button in Recipe Control screen. - Find and Select DF1244_Diafiltration from the list. - Record the Diafiltration batch ID and start time from the system overview screen. The batch ID should follow this format: MPR Part Number_Diafiltration2_Product Lot Number_Processing Date. |  |  |  |  |  |  |  |
|  | Batch ID |  |  |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |  |  |
|  | At the user prompt, insert the following values. |  |  |  |  |  |  |  |
|  |  | Process Step |  | Input Title | Set Points |  |  |  |
|  |  | Target Permeate Volume after Diafiltration |  | Diafiltration_Perm_Vol | Step 14.12 |  |  |  |
|  |  | Trans-Membrane Pressure (psid) |  | Diafiltration_TMP | 20 a |  |  |  |
|  |  | Cross Flow (L/min/ft2) |  | Diafiltration_Crossflow | 0.5 b |  |  |  |
|  | - The TMP can be set to ≤ 35 psid as needed. If the value is adjusted, record the value in Attachment 9: Comments Section. Ensure the feed pressure is ≤ 60 psig. - The cross flow can be adjusted within the range of 0.3 – 0.7 L/min/ft2, as needed. If the value is adjusted, record the new the value in Attachment 9: Comments Section |  |  |  |  |  |  |  |
|  | - Ensure that valves E and F remain closed, then open valve A to prepare for Diafiltration. - Push START to initiate the Diafiltration recipe. |  |  |  |  |  |  |  |

| Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Additional Diafiltration: Once the skid pump starts: - Open valve E, then slowly open valve F to allow the diafiltration buffer to enter the Retentate Vessel. - Adjust valve F or the buffer pump speed to control the flow of DF buffer to the Retentate Vessel to maintain the weight from Step 13.30. - Record the process data in Attachment 2: Operation Parameters approximately every 20 minutes. - Prior to the end of Diafiltration, obtain samples from the retentate and permeate. Record the appropriate readings in Step 14.21. NOTE: If any of the maximum values below are exceeded, stop process immediately, contact an area supervisor, and document actions taken in Attachment 9: Comments Section. |  |  |  |  |  |  |  |
|  | Parameter |  | Target |  | Maximum |  |  |  |
|  | TMP (psid) |  | 20 |  | 35 |  |  |  |
|  | Feed Flow (L/min/ft2) |  | 0.5 |  | 0.7 |  |  |  |
|  | Feed Pressure (psig) |  | NA |  | 60 |  |  |  |
|  | UV (OD) |  | NA |  | 0.075 |  |  |  |
|  | At the completion of the Diafiltration recipe once the skid pumps stops, close valves E and F. |  |  |  |  |  |  |  |
|  | Record the permeate volume after continued diafiltration and ensure it is less than or equal to the volume calculated in Step 14.12. |  |  |  |  |  |  |  |
|  | Permeate Volume |  |  | L |  |  |  |  |
|  | SAP Consumption of 8012555 Performed by/Date |  |  |  |  |  |  |  |
|  | Record the net weight of the product at the end of diafiltration from the retentate scale and scale information. |  |  |  |  |  |  |  |
|  | Net Weight of Product after Diafiltration |  |  | kg |  |  |  |  |
|  | Scale ID |  |  | Cal. Due Date |  |  |  |  |

| Step | Description |  |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the product volume in the Retentate Vessel after the additional Diafiltration. |  |  |  |  |  |  |  |  |  |
|  | Net Weight of Product after Diafiltration (Step 14.19) |  |  | Estimated Density of the Product after Diafiltration |  |  | = | Product Volume in the Retentate Vessel after Diafiltration |  |  |
|  | kg |  |  | 1.036 kg/L |  |  |  | L |  |  |
|  | Collect a sample from the permeate and retentate lines for pH, conductivity, and temperature measurement. Record the results below. |  |  |  |  |  |  |  |  |  |
|  | Retentate Values |  |  |  |  |  |  |  |  |  |
|  | pH (5.7 - 6.1) |  |  |  |  |  |  |  |  |  |
|  | Conductivity |  |  |  |  | mS/cm |  |  |  |  |
|  | Temperature (15 – 25°C) |  |  |  |  | °C |  |  |  |  |
|  | Permeate Values |  |  |  |  |  |  |  |  |  |
|  | pH |  |  |  |  |  |  |  |  |  |
|  | Conductivity |  |  |  |  | mS/cm |  |  |  |  |
|  | Temperature (15 – 25°C) |  |  |  |  | °C |  |  |  |  |
|  | Proceed based on the Step 14.21 sample results: |  |  |  |  |  |  |  |  |  |
|  | Check One | The pH of the Step 14.21 samples are: |  |  | Actions to take |  |  |  |  |  |
|  |  | In-range |  |  | - Stop the transfer phase on the PCS HMI for the Diafiltration buffer. - Close valves E and F. - Go to Section 15. |  |  |  |  |  |
|  |  | Out-of-range |  |  | Contact an area supervisor and document actions take in Attachment 9: Comments Section. |  |  |  |  |  |
|  |  | Name of supervisor contacted |  |  |  |  |  |  |  |  |

# Concentration 2

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Prior to starting Concentration 2: - Reset the permeate totalizer. - Ensure the valve states below. |  |  |  |
|  | Valve State | UF/DF System |  |  |
|  | Closed | A, D, E, F, G |  |  |
|  | Opened | B, C |  |  |
|  | To begin Concentration 2, click on the “Semi-Automatic” mode on the UF/DF Skid screen and input the following initial parameters in the order listed. NOTE: Drain valves XV102, XV103, XV202 should not opened at any time during processing. Process Step; Valve ID; Setpoint; Alarm. Feed Pressure (psig) c; N/A a; > 60. Trans-Membrane Pressure (TMP); 20 psid; 35 psid b. Feed Valve; XV101; Open. Retentate Valve; XV301; Open. XV302. Permeate Valve; XV201; Open. Drain Valves; XV103; Closed. XV102. XV202. Feed Flow/Crossflow (L/min/ft2); 0.5. - The feed pressure can be adjusted to as needed to a maximum AL feed pressure ≤ 60 psig. Record changes to transmembrane pressure and/or cross flow setpoint(s) in Attachment 2: Operation Parameters. Record any additional changes in Attachment 9: Comments Section. - The TMP can be set to ≤ 35 psid as needed. If the value is adjusted, notify supervisor if TMP is ≥ 35 psid record the value in Attachment 9: Comments Section. Ensure the feed pressure is ≤ 60 psig. - Feed pressure NOR is ≤ 60 psig. |  |  |

| ## Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | - Push START to initiate the Concentration 2 process. - Record the start time as the time the pump turns on. |  |  |  |
|  | Start Time |  |  |  |

|  | Concentration 2: - The crossflow and transmembrane pressure (TMP) setpoint(s) may be adjusted from their initial values as necessary as long as they do not exceed the maximum values below. - Record the adjusted values, the time of adjustment, and the step number in Attachment 2: Operation Parameters. - Record additional changes to operational setpoints and the time of the change in Attachment 9: Comments Section. - Record the process data and step number in Attachment 2: Operation Parameters approximately every 20 minutes. - Concentration 2 is complete when the retentate weight reaches the Target net weight calculated in Step 12.9. - Proceed to the next step for instructions to end Concentration 2. NOTE: If any of the maximum values below are exceeded, stop process immediately, contact an area supervisor, and document actions taken in Attachment 9: Comments Section. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Parameter | Target | Maximum |  |  |
|  | TMP (psid) | 20 | 35 |  |  |
|  | Feed Flow (L/min/ft2) | 0.5 | 0.7 |  |  |
|  | Feed Pressure (psig) | NA | 60 |  |  |
|  | UV (OD) | NA | 0.075 |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | When the target weight of the Retentate Vessel after Concentration 2 is reached (Step 12.9): - Stop the pump. - Ensure the valve states below: |  |  |  |  |
|  | Valve State | UF/DF System |  |  |  |
|  | Closed | A, D, F, G Permeate (XV201) |  |  |  |
|  | Opened | B, C, Retentate (XV301 and XV302) |  |  |  |
|  | Record the end time, retentate weight and the actual volume of permeate from the permeate totalizer after Concentration 2. * NOTE: Ensure the Retentate Weight is within the range calculated in Step 12.9 |  |  |  |  |
|  | Concentration 2 End Time |  |  |  |  |
|  | Retentate Weight * |  | kg |  |  |
|  | Permeate Volume |  | L |  |  |

| Step | Description | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | Recirculation: To recirculate the concentrated product, click on the “Semi-Automatic” mode on the UF/DF Skid screen and input the following initial parameters in the order listed: NOTE: Drain Valves XV102, XV103, XV202 should not opened at any time during processing. Parameter; Valve ID; Setpoint; Alarm. Feed Pressure (psig); 25 a,b; > 60. Feed Valve; XV101; Open. Retentate Valves; XV301; Open. XV302. Permeate Valve; XV201; Closed. Drain Valves; XV103; Closed. XV102. XV202. Feed Flow/Cross Flow (L/min/ft2); ≤ 0.7 b. - The feed pressure can be adjusted as needed to maintain a target of 25 psig and a maximum feed pressure ≤ 60 psig. Record changes to TMP and/or cross flow setpoint(s) in Attachment 2: Operation Parameters. Record any additional changes in Attachment 9: Comments Section - The cross flow can be adjusted, as long as crossflow is ≤ 0.7 L/min/ft2, to obtain the target feed pressure. Do not exceed 60 psig for the feed pressure. If the value is adjusted, record the new value in Attachment 9: Comments Section. |  |  |

| ## Step |  | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | - Recirculate for 5 - 120 minutes (max NOR for recirculation is 120 mins). - Record the start and end time of recirculation below. - Once recirculation is complete: - Stop the pump. - Close valves B and C. - Close the Feed Valve (XV101) on the HMI. |  |  |  |  |  |  |  |
|  |  | Recirculation start time |  |  |  |  |  |  |  |
|  |  | Recirculation end time |  |  |  |  |  |  |  |
|  |  | Duration of recirculation (5 - 120 min) |  |  | min |  |  |  |  |
|  | Ensure the Retentate Vessel is in the same configuration as it was when its tare weight after attachments was obtained in Step 11.12. |  |  |  |  |  |  |  |  |
|  | Record the weight of the Retentate Vessel after Recirculation. |  |  |  |  |  |  |  |  |
|  | Retentate Weight after Recirculation |  |  | kg |  |  |  |  |  |
|  | Scale ID |  |  | Cal. Due Date |  |  |  |  |  |

# Recovery

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the Recovery Product volume range. |  |  |  |  |  |  |
|  | Total grams of Virus Filtration Product after sampling (Step 9.10) |  | Recovery Concentration Range | = | Recovery Product Volume Range |  |  |
|  | g |  | 190 g/L |  | L (Target) |  |  |
|  |  |  | 171 g/L |  | L (Max) |  |  |
|  | Calculate the Recovery Product weight range. |  |  |  |  |  |  |
|  | Recovery Product Volume Range (Step 16.1) | X | Density of the Product at Recovery | = | Recovery Product Net Weight Range |  |  |
|  | L (Target) |  | 1.065 kg/L |  | kg (Target) |  |  |
|  | L (Max) |  |  |  | kg (Max) |  |  |
|  | Calculate the amount of Recovery buffer to add. |  |  |  |  |  |  |
|  | Recovery Product Net Weight range (Step 16.2) | − | Retentate Weight after Recirculation (Step 15.10) | = | Weight of Recovery buffer to add |  |  |
|  | kg (Target) |  | kg |  | kg (Target) |  |  |
|  | kg (Max) |  |  |  | kg (Max) |  |  |
|  | Calculate the volume of Recovery buffer to add. |  |  |  |  |  |  |
|  | Target Weight of Recovery buffer to add (Step 16.3) |  | Density of the Recovery buffer | = | Volume of Recovery buffer to add |  |  |
|  | kg |  | 1.008 kg/L |  | L |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | Prior to Recovery, ensure the valve states listed below: |  |  |  |  |  |
|  | Valve State |  | UF/DF System |  |  |  |
|  | Closed |  | A, B, F, G, Feed (XV101), Permeate (XV201) |  |  |  |
|  | Opened |  | C, D, E, Retentate (XV301 and XV302) |  |  |  |
|  | Choose below whether Recovery will be performed manually or by recipe control: |  |  |  |  |  |
|  | Check One | Condition |  | Action to take |  |  |
|  |  | Recovery will be run manually |  | - N/A Step 16.7 to Step 16.11. - Go to Step 16.12. |  |  |
|  |  | Recovery will be run by recipe control |  | Go to the next step. |  |  |

| Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Recovery by Recipe Control (Steps 16.7 – 16.11) - Click on the “Add Recipe” button on the Recipe Control screen. - Select DF1244_RECOVERY_2 from the list. - Record the recovery batch ID and start time from the system overview screen. The batch ID should follow this format: MPR part Number_Recovery _Product Batch Number_Processing Date |  |  |  |  |  |  |  |
|  | Batch ID |  |  |  |  |  |  |  |
|  | Start Time |  |  |  |  |  |  |  |
|  | At the user prompt, insert the following values. |  |  |  |  |  |  |  |
|  | Process Step |  |  | Input Title |  | Set Points |  |  |
|  | Retentate Volume |  |  | Recovery_Ret_Vol |  | Step 16.4 |  |  |
|  | Max Feed Pressure |  |  | Feed_Pressure_Max |  | 60 psig a |  |  |
|  | - Target feed pressure is 15 psig. |  |  |  |  |  |  |  |
|  | Push START to initiate the Recovery process. |  |  |  |  |  |  |  |
|  | - Once the pump starts, slowly open manual valve G to allow Recovery buffer to flow into the Retentate Vessel and to maintain a feed flow of less than or equal to 0.7 L/min/ft2. - If the feed pressure exceeds 60 psig, stop the process, contact area management, and document actions taken in Attachment 9: Comments Section. Otherwise mark the box below as “NA” |  |  |  |  |  |  |  |
|  | Supervisor Contacted |  |  |  |  |  |  |  |
|  | - At the completion of Recovery when the skid pump stops, immediately close valves C and G to stop the flow of Recovery buffer into the Retentate Vessel. - Ensure the remaining manual valves on the UF/DF system are closed per the table below, then N/A Steps 16.12 – 16.13 and proceed to Step 16.14. |  |  |  |  |  |  |  |
|  | Valve State |  | UF/DF System |  |  |  |  |  |
|  | Closed |  | A, B, C, D, E, F, G |  |  |  |  |  |

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | Manual Recovery (Steps 16.12 – 16.13) Prior to the start of Recovery, verify the valve states below on the UF/DF System: |  |  |  |
|  | Valve State | UF/DF System |  |  |
|  | Closed | A, B, F, G, Feed (XV101), Permeate (XV201) |  |  |
|  | Opened | C, D, E, Retentate (XV301 and XV302) |  |  |
|  | - Open Feed valve XV101 on the skid HMI, then slowly open manual valve G to allow Recovery buffer to flow into the Retentate Vessel and to maintain a feed flow of less than or equal to NOR max of 0.7 L/min/ft2. - Monitor the weight of the Retentate Vessel until the target Recovery Product Net Weight (Step 16.2) is achieved, then immediately close valves C and G to stop the flow of Recovery buffer into the Retentate Vessel. - Ensure the remaining manual valves on the UF/DF system are closed per the table below. |  |  |  |
|  | Valve State | UF/DF System |  |  |
|  | Closed | A, B, C, D, E, F, G |  |  |

| Step | Description |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Obtain the weight of the product after Recovery (with attachments). - Record the scale/balance equipment ID and calibration due date. |  |  |  |  |  |  |  |  |
|  | Weight of product after Recovery (with attachments) |  |  | kg |  |  |  |  |  |
|  | Scale/Balance ID and Cal Due Date |  |  | ID |  |  | Due Date |  |  |
|  | Calculate the amount of Recovery buffer 8012555 used for the Recovery flush. |  |  |  |  |  |  |  |  |
|  | Retentate Vessel Weight after Recovery (with attachments) (Step 16.14) | − | Retentate Weight after Recirculation (Step 15.10) |  | = | Amount of Recovery buffer 8012555 used for Recovery flush |  |  |  |
|  | kg |  | kg |  |  | kg |  |  |  |
|  | SAP Consumption of 8012555 Performed by/Date |  |  |  |  |  |  |  |  |

| Step | Description | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- |
|  | - Close all the valves on the Retentate Vessel. - Disconnect the following parts: - The Charge/Diafiltration Line with sight glass, elbow, and tee connected to valve A. NOTE: Keep valve A connected to the tank. - The Retentate Line with sight glass and tee connected to valve C. NOTE: Keep valve C connected to the tank. - The Recovery and Feed Lines including the reducer and tee connected to the valve B tank outlet. NOTE: This will include disconnecting valve D. |  |  |
|  | - Place end caps on valves A, B, and C. - The Retentate Vessel should be in the configuration per the diagram below. |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | - Obtain a gross weight of the Retentate Vessel, with no attachments, after Recovery. - Ensure that the retentate vessel is the same configuration as in Step 16.17. - If necessary, remove the Retentate Vessel from the scale and tare (zero) the scale again before obtaining the gross weight. - Record the scale/balance equipment ID and calibration due date. |  |  |  |  |
|  | Gross Weight of Retentate Vessel after Recovery (no attachments) | kg |  |  |  |
|  | Scale/Balance ID and Cal Due Date | ID | Due Date |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Obtain a mixing bag for the transfer of the Pre-Formulated Intermediate (PFI) product. - Record the part number, batch number, and expiration date for the bag. - Ensure the vessel is labeled with “AZD0543”, “Pre-Formulated Intermediate Dilution Vessel”, batch number, 8012475, and initials/date. |  |  |  |  |  |
|  | Dilution Vessel Info |  |  |  |  |  |
|  | Part No |  |  |  |  |  |
|  | Batch No |  |  |  |  |  |
|  | Exp. Date |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |
|  | - Obtain the tare weight for the Dilution Vessel. - Record the equipment ID and calibration due date for the scale and the tare weight of the bag. - Check if the Mixer Drive is attached when the tare weight is obtained. |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |
|  | Scale/Balance Calibration Due Date |  |  |  |  |  |
|  | Tare Weight of Dilution Vessel before attachment |  | kg |  |  |  |
|  | Mixer Drive attached at Tare Weight? (check one) |  | □ Yes | □ No |  |  |

| Step | Description |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- |
|  | - Obtain tubing/hose(s) for the transfer of the product to the Dilution Vessel. - Record the tubing part number, batch number, expiration date, autoclave cycle number, and autoclave expiration date. - Record the hose cleaning cycle number and expiration date, if applicable. |  |  |  |
|  | Tubing Info, if applicable |  |  |  |
|  | Part No. |  |  |  |
|  | Batch No. |  |  |  |
|  | Exp. Date |  |  |  |
|  | Autoclave Cycle No. * |  |  |  |
|  | Autoclave Exp. Date * |  |  |  |
|  | Hose Info, if applicable |  |  |  |
|  | Hose Cleaning Cycle No. |  |  |  |
|  | Hose Cleaning Exp. Date |  |  |  |
|  | * Autoclave information is not required if tubing is gamma irradiated. |  |  |  |
|  | If the transfer line is tubing, thread the tubing through the peristaltic pump following SOP-0080839. |  |  |  |
|  | Turn off the agitator on the Retentate Vessel. |  |  |  |
|  | Connect the Recovery line from the outlet of Retentate Vessel elbow to the inlet of the Dilution Vessel. NOTE: Ensure the Dilution Vessel bottom outlet clamp is closed and clamped as close to the bag as possible. |  |  |  |
|  | - Once the Dilution Vessel is connected to the outlet of the Retentate Vessel, open the outlet of the Retentate Vessel at valve B. - Pump the entire contents of the Retentate Vessel into the Dilution Vessel and record the end time of transfer below. |  |  |  |
|  | Transfer End Time |  |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Begin to mix the product by agitation per SOP-0107097. Ensure the agitation is set to mix the product while minimize foaming. - Mix for at least 20 minutes. - Once a steady agitation rate has been achieved, record the mixer ID, agitation setting, mixing start time, mixing end time and the mixing duration below. |  |  |  |  |  |
|  | Mixer ID |  |  |  |  |  |
|  | Agitation Setting |  | RPM |  |  |  |
|  | Mixing Start Time |  |  |  |  |  |
|  | Mixing End Time |  |  |  |  |  |
|  | Mixing Duration (≥20 mins) |  | min |  |  |  |
|  | Remove product from the Dilution Vessel via a sample port for STAT protein determination (-20) per Attachment 3: Sampling Record. Follow procedures in SOP-0107097. |  |  |  |  |  |
|  | Sampling Date and Time | Date: |  | Time: |  |  |
|  | - Aliquot the sample (sample designation: 8012475-20) per Attachment 3: Sampling Record. - If applicable, aliquot non-routine samples per Attachment 4: Non-Routine Sampling Record. - Label the sample with 8012475-20, per SOP-0107056. |  |  |  |  |  |
|  | - Submit 8012475-20 according to SOP-0080295. - Ensure the sampling time and location are updated in DLIMS. |  |  |  |  |  |

| Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Remove from the Dilution Vessel the tubing/hosing that was used for transfer. NOTE: Ensure the vessel is in the same configuration as it was when the tare weight before attachment was obtained in Step 16.20. |  |  |  |  |  |  |  |
|  | - Record the gross weight of the Dilution Vessel after transfer. - Record the floor scale/balance ID number and calibration due date. |  |  |  |  |  |  |  |
|  | Gross weight of the Dilution Vessel |  |  | kg |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |
|  | Scale/Balance Calibration Due Date |  |  |  |  |  |  |  |
|  | Calculate the net weight of the Recovery Product after transfer. |  |  |  |  |  |  |  |
|  | Gross weight of the Dilution Vessel (Step 16.31) | − | Tare weight of Dilution Vessel before attachment (Step 16.20) |  | = | Net weight of Recovery Product after Transfer |  |  |
|  | kg |  | kg |  |  | kg |  |  |
|  | Calculate the volume of the Recovery Product after Transfer. |  |  |  |  |  |  |  |
|  | Net weight of Recovery Product after Transfer (Step 16.32) |  | Density of the product at 190 g/L |  | = | Volume of Recovery Product after Transfer |  |  |
|  | kg |  | 1.065 kg/L |  |  | L |  |  |

| Step | Description |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- |
|  | - Record the protein concentration and DLIMS Project and Sample ID of the Recovery Product (8012475-20). - Based on the 8012475-20 concentration result, indicate below if the product is within the Pre-Formulated Intermediate (PFI) concentration range of 171 – 189 g/L or if it requires dilution. |  |  |  |  |  |
|  | Concentration of the Recovery Product |  | g/L |  |  |  |
|  | DLIMS Project ID |  |  |  |  |  |
|  | DLIMS Sample ID |  |  |  |  |  |
|  | Check One | Concentration |  | Action |  |  |
|  |  | > 189 g/L |  | Dilution is required: Go to Section 17. |  |  |
|  |  | 171 – 189 g/L |  | Dilution is not required: N/A Steps 17.1 - 17.16, then go to Step 17.17. |  |  |
|  |  | < 171 g/L |  | Contact a supervisor and document actions taken in Attachment 9: Comments Section. |  |  |
|  |  | Supervisor Contacted, if applicable |  |  |  |  |

# Dilution

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the total grams of product in the Retentate Vessel. |  |  |  |  |  |  |
|  | Volume of Recovery Product after Transfer (Step 16.33) | X | Concentration of the Recovery Product (Step 16.34) | = | Total grams of the Recovery Product |  |  |
|  | L |  | g/L |  | g |  |  |
|  | Calculate the AZD0543 Dilution Product Volume Range. |  |  |  |  |  |  |
|  | Total grams of the Recovery Product (Step 17.1) |  | PFI Concentration Range | = | AZD0543 Dilution Product Volume Range |  |  |
|  | g |  | 189 g/L (max) |  | L (Min) |  |  |
|  |  |  | 180 g/L (target) |  | L (Target) |  |  |
|  |  |  | 171 g/L (min) |  | L (Max) |  |  |
|  | Calculate the AZD0543 Dilution Product Net Weight Range. |  |  |  |  |  |  |
|  | AZD0543 Dilution Product Volume Range (Step 17.2) | X | Density of the Pre-Formulated Intermediate (PFI) | = | AZD0543 PFI Net Weight Range |  |  |
|  | L (Min) |  | 1.062 kg/L |  | kg (Min) |  |  |
|  | L (Target) |  |  |  | kg (Target) |  |  |
|  | L (Max) |  |  |  | kg (Max) |  |  |

| Step | Description |  |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Calculate the weight of Dilution Buffer to Add. |  |  |  |  |  |  |  |  |  |  |
|  | AZD0543 PFI Net Weight Range (Step 17.3) |  | − | Net weight of the Recovery Product after Transfer (Step 16.32) |  |  | = |  | Weight of Dilution Buffer to Add |  |  |
|  | kg (Min) |  |  | kg |  |  |  |  | kg (Min) |  |  |
|  | kg (Target) |  |  |  |  |  |  |  | kg (Target) |  |  |
|  | kg (Max) |  |  |  |  |  |  |  | kg (Max) |  |  |
|  | Convert the Weight of Dilution Buffer to Add to volume. |  |  |  |  |  |  |  |  |  |  |
|  | Weight of Dilution Buffer to Add (Step 17.4) |  |  | Density of the Dilution Buffer (PN: 8012555) |  | = |  | Volume of Dilution Buffer to Add |  |  |  |
|  | kg (Min) |  |  | 1.008 kg/L |  |  |  | L (Min) |  |  |  |
|  | kg (Target) |  |  |  |  |  |  | L (Target) |  |  |  |
|  | kg (Max) |  |  |  |  |  |  | L (Max) |  |  |  |
|  | Check the box for the method for how the dilution buffer will be transferred. |  |  |  |  |  |  |  |  |  |  |
|  | Check One | Dilution Method |  |  | Action |  |  |  |  |  |  |
|  |  | If using a pipette, grad cylinder, carboy, or bottle |  |  | Proceed to the next step. |  |  |  |  |  |  |
|  |  | If connecting the dilution buffer directly to the Dilution Vessel |  |  | - N/A Steps 17.7 – 17.9. - Go to Step 17.10. |  |  |  |  |  |  |

| Step | Description |  |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Obtain a vessel for the transfer of the dilution buffer. Label the vessel as “Intermediate Vessel”, and initial and date. NOTE: N/A any unused spaces. |  |  |  |  |  |  |  |  |  |  |
|  |  | Part No | Batch No |  | Exp. Date | Qty |  | Autoclave Cycle No. | Autoclave Exp. Date |  |  |
|  | Carboy |  |  |  |  |  |  |  |  |  |  |
|  | Grad. Cylinder |  |  |  |  |  |  |  |  |  |  |
|  | Bottle |  |  |  |  |  |  |  |  |  |  |
|  | Sterile Pipette |  |  |  |  |  |  |  |  |  |  |
|  | Sterile Pipette |  |  |  |  |  |  |  |  |  |  |
|  | Tubing |  |  |  |  |  |  |  |  |  |  |
|  | SAP Consumption Performed By/Date |  |  |  |  |  |  |  |  |  |  |
|  | - Check the box for the method of addition. - If adding using a pump, record the pump ID, if applicable. |  |  |  |  |  |  |  |  |  |  |
|  | Pump ID, if applicable |  |  |  |  |  |  |  |  |  |  |
|  | Measurement Method (range reference listed) |  |  | Volumetrically Step 17.5 |  |  | Weight Step 17.4 |  |  |  |  |
|  | Transfer the dilution buffer (8012555) to the Intermediate vessel. |  |  |  |  |  |  |  |  |  |  |

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | - If transferring using a pump, connect either the dilution buffer (8012555) or the Intermediate Vessel to the inlet of the PFI Dilution vessel. - Thread tubing through a peristaltic pump. - Prime the tubing. - Otherwise, mark this step as “NA.” |  |  |  |  |
|  | Tubing Information |  |  |  |  |
|  | Part Number |  |  |  |  |
|  | Batch Number |  |  |  |  |
|  | Expiration Date |  |  |  |  |
|  | Autoclave Cycle Number * |  |  |  |  |
|  | Autoclave Exp. Date * |  |  |  |  |
|  | * Autoclave information is not required if tubing is gamma irradiated. |  |  |  |  |
|  | Tare (zero) the scale holding the Dilution Vessel. |  |  |  |  |
|  | - Add dilution buffer to the Dilution Vessel until the target Weight of Dilution Buffer to Add (Step 17.4) is reached. - Once target weight is reached, turn off the transfer pump. - Record weight after dilution. |  |  |  |  |
|  | Net Weight of dilution buffer 8012555 added |  | kg |  |  |
|  | Scale/Balance ID |  |  |  |  |
|  | Cal. Due Date |  |  |  |  |
|  | SAP Consumption of 8012555 Performed by/Date |  |  |  |  |

| Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Disconnect the buffer line from the Dilution Vessel, if applicable. - Remove the dilution vessel from the scale and tare (zero) the scale. - Obtain the gross weight of the Dilution Vessel after dilution. NOTE: Ensure the Dilution Vessel is in the same configuration as it was when its tare weight was obtained in Step 16.20. |  |  |  |  |  |  |  |
|  | Gross weight of Dilution Vessel, with attachments removed |  |  | kg |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |
|  | Calibration Due Date |  |  |  |  |  |  |  |
|  | Calculate the net weight of PFI. |  |  |  |  |  |  |  |
|  | Gross weight of Dilution Vessel, with attachments removed (Step 17.13) | – | Tare weight of Dilution Vessel before attachment (Step 16.20) |  | = | PFI Net Weight |  |  |
|  | kg |  | kg |  |  | kg |  |  |

| Step | Description |  |  |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - If the PFI Net Weight is above the max net weight range calculated in Step 17.3,contact a supervisor and document actions taken in Attachment 9: Comments Section. - Otherwise, mark this step as “NA”. |  |  |  |  |  |  |  |  |  |  |
|  | Supervisor Contacted |  |  |  |  |  |  |  |  |  |  |
|  | Calculate the volume of PFI after dilution and before sampling. |  |  |  |  |  |  |  |  |  |  |
|  | PFI Net Weight (Step 17.14) |  |  | Density of the PFI |  |  |  | = | PFI Volume before sampling |  |  |
|  | kg |  |  | 1.062 kg/L |  |  |  |  | L |  |  |
|  | - Begin to mix the product per SOP-0107097 by agitation. - If product is mixed by recirculation, complete Attachment 7: Product Recirculation Work Sheet. - Mix the product for at least 20 minutes by agitation. Adjust agitation setting to minimize foaming, as applicable. NOTE: Agitation may already be on. |  |  |  |  |  |  |  |  |  |  |
|  | Mixer/Stir Plate ID, if applicable |  |  |  |  |  |  |  |  |  |  |
|  | Agitation Setting |  |  |  |  |  | RPM |  |  |  |  |
|  | Mixing Start Time |  |  |  |  |  |  |  |  |  |  |
|  | Mixing End Time |  |  |  |  |  |  |  |  |  |  |
|  | Duration of Mixing (≥ 20 minutes) |  |  |  |  |  | min |  |  |  |  |
|  | Select one of the options below. |  |  |  |  |  |  |  |  |  |  |
|  | Check One | Condition |  |  | Actions to take |  |  |  |  |  |  |
|  |  | If there was no dilution |  |  | - Mark Steps 17.19 – 17.23 as “NA.” - Mark sample (8012475 – 30) in Attachment 3: Sampling Record as “N/A”. - Proceed to Step 17.24. |  |  |  |  |  |  |
|  |  | If there was dilution |  |  | Proceed to the next step |  |  |  |  |  |  |

| Step | Description |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Remove product from the product collection vessel for (-30) samples per Attachment 3: Sampling Record. Sample per SOP-0107097. NOTE: Ensure the sample line is flushed prior to taking the sample. |  |  |  |  |  |  |
|  | Sampling Time and Date | Time |  |  | Date |  |  |
|  | - Aliquot the sample (sample designation: 8012475-30) per Attachment 3: Sampling Record - If applicable, aliquot non-routine samples per Attachment 4: Non-Routine Sampling Record. - Label the sample per SOP-0107056 |  |  |  |  |  |  |
|  | - Submit the sample in DLIMS per SOP-0080295. - Ensure the sampling time and location are updated in DLIMS. |  |  |  |  |  |  |
|  | - Record the results from sample designation, 8012475–30. - If the results are not in range, contact supervisor and document actions taken in Attachment 9: Comments Section. |  |  |  |  |  |  |
|  | PFI concentration (171 – 189 g/L) |  |  | g/L |  |  |  |
|  | - Once the Pre-Formulated Intermediate has confirmed to be in range, begin to mix the product per SOP-0107097 by agitation. - If using a stir plate, record the equipment ID for the stir plate used. NOTE: Agitation may already be on. |  |  |  |  |  |  |
|  | Mixer/Stir Plate ID |  |  |  |  |  |  |
|  | Agitation Setting |  | RPM |  |  |  |  |
|  | Mixing Start Time |  |  |  |  |  |  |
|  | Mixing End Time |  |  |  |  |  |  |
|  | Mixing Duration (≥ 15 minutes) |  | min |  |  |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Remove product from the product collection vessel for (-40) samples per Attachment 3: Sampling Record. Follow procedures in SOP-0107097 for mixing bags. |  |  |  |  |
|  | Sampling Time and Date | Time | Date |  |  |
|  | - Aliquot the sample (sample designation: 8012475-40) per Attachment 3: Sampling Record. - If applicable, aliquot non-routine samples per Attachment 4: Non-Routine Sampling Record. |  |  |  |  |
|  | - Label with 8012475 and additional information per SOP-0107056 - Submit the sample in DLIMS per SOP-0080295. NOTE: Ensure the sampling time and location are updated in DLIMS. |  |  |  |  |

|  | Aliquot product into a 50 ml conical and measure the pH, conductivity, and temperature. NOTE: If the pH value is not in range, contact a supervisor and document actions taken in Attachment 9: Comments Section. |  |  |  |
| --- | --- | --- | --- | --- |
|  | pH (5.7 - 6.1) |  |  |  |
|  | Conductivity | mS/cm |  |  |
|  | Temperature (15 – 25 °C) | °C |  |  |

| Step | Description |  |  |  |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Record the gross weight of the Dilution Vessel after sampling. NOTE: Ensure the vessel is in the same configuration as it was when the tare weight was obtained in Step 16.20. |  |  |  |  |  |  |  |  |  |  |  |
|  | Dilution Vessel Gross Weight after sampling |  |  |  |  | kg |  |  |  |  |  |  |
|  | Calculate the net weight of the Pre-Formulated Intermediate (PFI) after sampling. |  |  |  |  |  |  |  |  |  |  |  |
|  | Dilution Vessel Gross Weight after sampling (Step 17.30) |  | – |  | Tare weight of Dilution Vessel (Step 16.20) |  |  | = |  | PFI Net Weight after sampling |  |  |
|  | kg |  |  |  | kg |  |  |  |  | kg |  |  |
|  | Calculate the volume of the PFI after sampling. |  |  |  |  |  |  |  |  |  |  |  |
|  | PFI Net Weight after sampling (Step 17.31) |  |  | Density of the PFI |  |  | = |  | PFI Volume after sampling |  |  |  |
|  | kg |  |  | 1.062 kg/L |  |  |  |  | L |  |  |  |

# Filtration and Storage of the Pre-Formulated Intermediate

| Step | Description |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Obtain a mixing bag or bottle for the filtration and storage of the PFI. - Record the bag or bottle information below and ensure the vessel is labeled with “AZD0543”, “Pre-Formulated Intermediate”, batch number, part number, and initials/date. NOTE: A mixing bag is preferred to use for storage. |  |  |  |  |  |  |  |
|  | Bag information (if applicable) |  |  |  |  |  |  |  |
|  | Bag Part No. |  |  |  |  |  |  |  |
|  | Bag Batch No. |  |  |  |  |  |  |  |
|  | Bag Exp. Date |  |  |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |  |  |
|  | Glass Bottle Information (if applicable) |  |  |  |  |  |  |  |
|  | Glass Bottle Description |  |  |  |  |  |  |  |
|  | Glass Bottle Autoclave Cycle No. |  |  |  |  |  |  |  |
|  | Glass Bottle Autoclave Exp. Date |  |  |  |  |  |  |  |
|  | - Obtain a tare weight of the PFI storage vessel. - Record the equipment ID and calibration due date for the scale. |  |  |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |
|  | Scale/Balance Calibration Due Date |  |  |  |  |  |  |  |
|  | Tare Weight of Storage Vessel |  |  | kg |  |  |  |  |
|  | Obtain an SHC filter per Section 4 and record the filter information below. NOTE: The bolded filter part number(s) in Section 4 require autoclaving. |  |  |  |  |  |  |  |
|  | Pre-Formulated Intermediate Product Filter Information |  |  |  |  |  |  |  |
|  | ## Part Number |  |  |  |  |  |  |  |
|  | ## Batch No. |  |  |  |  |  |  |  |
|  | ## Exp. Date |  |  |  |  |  |  |  |
|  | Autoclave Information, if applicable |  |  |  |  |  |  |  |
|  | ## Autoclave Cycle No. |  | Autoclave Expiration Date |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | - Obtain tubing/hose(s) for the transfer of the product to the storage vessel. - Record the tubing part number, batch number, expiration date, autoclave cycle number, and autoclave expiration date. - Record the hose cleaning cycle number and expiration date, if applicable. |  |  |  |  |
|  | Tubing Information, if applicable |  |  |  |  |
|  | Part No. |  |  |  |  |
|  | Batch No. |  |  |  |  |
|  | Exp. Date |  |  |  |  |
|  | Autoclave Cycle No. * |  |  |  |  |
|  | Autoclave Exp. Date * |  |  |  |  |
|  | Hose Information, if applicable |  |  |  |  |
|  | Hose Cleaning Cycle No. |  |  |  |  |
|  | Hose Cleaning Exp. Date |  |  |  |  |
|  | * Autoclave information is not required if tubing is gamma irradiated. |  |  |  |  |
|  | - Attach the outlet of the PFI product filter to the inlet of the PFI storage vessel. - Connect the transfer line between the outlet of the Dilution Vessel and the inlet of the PFI product filter. - If the transfer line is tubing, thread the tubing through the peristaltic pump following SOP-0080839. - Pump the entire contents of the Dilution Vessel into the PFI storage vessel and record the end time of transfer below. |  |  |  |  |
|  | Transfer End Time |  |  |  |  |
|  | - Drain the hold-up of the PFI product filter and transfer line into the PFI storage vessel. - Remove the transfer tubing from the PFI storage vessel and remove the PFI product filter(s). |  |  |  |  |

| Step | Description |  |  |  |  |  |  |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | - Obtain a gross weight of the PFI storage vessel after the transfer. - Record the scale/balance equipment ID and calibration due date. NOTE: Ensure the storage vessel is in the same configuration as when the tare weight was obtained in Step 18.2 |  |  |  |  |  |  |  |  |  |  |
|  | Gross Weight of PFI Storage Vessel after transfer |  |  |  |  | kg |  |  |  |  |  |
|  | Scale/Balance ID |  |  |  |  |  |  |  |  |  |  |
|  | Scale/Balance Calibration Due Date |  |  |  |  |  |  |  |  |  |  |
|  | Calculate the net weight of the PFI after sampling and transfer |  |  |  |  |  |  |  |  |  |  |
|  | Gross Weight of PFI Storage Vessel after transfer (Step 18.7) | − |  | Tare Weight of Storage Vessel (Step 18.2) |  |  | = | Net Weight of the PFI after sampling and transfer |  |  |  |
|  | kg |  |  | kg |  |  |  | kg |  |  |  |
|  | Calculate the volume of the PFI after sampling and transfer. |  |  |  |  |  |  |  |  |  |  |
|  | Net Weight of the PFI after sampling and transfer (Step 18.8) |  |  |  | Density of the Product after Dilution |  | = |  | Volume of the PFI after sampling and transfer |  |  |
|  | kg |  |  |  | 1.062 kg/L |  |  |  | L |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Record the net weight of the PFI (Step 18.8), the tare weight of the PFI storage vessel (Step 18.2), and the gross weight of the PFI storage vessel after transfer (Step 18.7) on the storage vessel label. |  |  |  |  |
| ## ` | Store the product and document storage information below. - For product stored at 2 – 8°C, the storage start time is the time when the product is transferred into 2 – 8°C storage. - For product stored at 15 – 25°C, the storage start time is the time when final sampling occurred in Step 17.24. Initial Storage Condition; 2 – 8°C 15 – 25°C. Storage Start Time. |  |  |  |
|  | Record the following values in Attachment 5: Hold Times. |  |  |  |  |  |
|  | MPR Name | Name in Attachment | Step No. |  |  |  |
|  | Start Time/Date of Product Charge | Virus Filtered Product Storage End (Time/Date) | Step 13.14 |  |  |  |

# Post-Use WFI Flush

| Step | Description |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set up per SOP-0107111 using one of the options below. For WFI collected directly from the WFI Hold Tank (TK-1458) - Verify the SIP of the WFI Hold Tank was completed and the tank status is “Clean” and that the WFI port has been flushed per SOP-0080303. - Mark the bag information as “N/A.” For WFI collected into a bag - If the same WFI vessel from Step 8.1 is used: - Transcribe from Step 8.1 the WFI Valve/Tank ID, WFI Charge Time/Date and WFI Expiration Date/Time below. - N/A the bag information boxes and N/A the SAP consumption box. - Record the WFI collection vessel information. - Verify that the WFI port has been flushed per SOP-0080185. - Charge into the vessel and record the WFI Charge Date/Time and Expiration. |  |  |  |  |  |  |
|  |  | Part No. | Batch No. |  | Exp. Date |  |  |
|  | Bag, if applicable |  |  |  |  |  |  |
|  | WFI Valve ID/Tank ID |  |  |  |  |  |  |
|  | WFI Charge Date/Time | Date: |  | Time: |  |  |  |
|  | WFI Expiration Date/Time * | Date: |  | Time: |  |  |  |
|  | * The charged WFI Expiration Date/Time is 24 hours from the WFI Charge Date/Time. |  |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |  |

| Step | Description |  |  | Performed by/ Date | Witnessed by/ Date |
| --- | --- | --- | --- | --- | --- |
|  | Confirm that the conductivity meter has been standardized per SOP-0080297. |  |  |  |  |
|  | Meter ID |  |  |  |  |
|  | Ensure that the Ultrafiltration (UF) system is set up according to SOP-0107111. |  |  |  |  |
|  | On the UF/DF Skid HMI, enter the total cassette surface area in m2 (Step 7.2): - Click on the membrane Area Setpoint input box. - Enter surface area in m2 (Step 7.2) and press ENTER. - Click ACCEPT on the Membrane Area Change pop-up window. |  |  |  |  |
|  | - Connect the WFI supply inlet to the UF/DF Skid feed line. - Ensure that the retentate and permeate lines to the UF/DF Skid are directed to waste. |  |  |  |  |
|  | - Download and run DF1244_Flush according to SOP-0107111 to flush the UF/DF Skid with WFI. - Record the batch ID and start time. The batch ID should follow this format: MPR Part Number_CleanWFI_Batch Number_Processing Date |  |  |  |  |
|  | Batch ID |  |  |  |  |
|  | Start Time |  |  |  |  |

| Step | Description |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- |
|  | At the user prompt, input the values per the table below. |  |  |  |  |
|  | Processing Step | Input Title | Entered Set Point |  |  |
|  | Permeate Flush | PERMEATE_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | Retentate Flush | RET_FLUSH_VOL | ≥ Step 8.7 |  |  |
|  | Go to the Recipe Control screen on the UF/DF Skid HMI, and push START to initiate the post-use WFI flush. |  |  |  |  |
|  | - At the completion of the recipe, the status changes from “Running” to “Complete” and the pump stops. - Continue to Section 20. |  |  |  |  |

- Post-Processing Activities for the UF/DF Skid

| Step | Description |  |  |  |  |  |  |  | Performed by/Date | Witnessed by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Post-Processing Activities for the UF/DF Skid Check which action will be taken for the cassettes. |  |  |  |  |  |  |  |  |  |
|  | Check One | Decision |  |  |  | Actions to take |  |  |  |  |
|  |  | The cassettes will not be re-used: - Discard cassettes * |  |  |  | - N/A Step 20.2, then continue to Step 20.3 to clean and store the UF/DF skid. * NOTE: If the cassettes are to be discarded, NWP is not required. |  |  |  |  |
|  |  | The cassettes will be re-used: - Clean, WFI Flush, perform NWP, and store the cassettes |  |  |  | Continue to the next step. |  |  |  |  |
|  | - Verify the data for the UF/DF Skid to be used. - From the logbook, record the logbook number, CIPDS batch number and date when the actions were documented below. |  |  |  |  |  |  |  |  |  |
|  | Logbook Number |  |  |  |  |  |  |  |  |  |
|  | CIPDS Batch Number |  |  |  |  |  |  |  |  |  |
|  | Action |  |  | Date |  |  |  |  |  |  |
|  | Sanitization |  |  |  |  |  |  |  |  |  |
|  | NWP |  |  |  |  |  |  |  |  |  |
|  | Storage |  |  |  |  |  |  |  |  |  |
|  | Record the Sanitization and Storage Buffer Batch Number and amount used based on SOP-0107111 * Amount Used = Retentate Totalizer + Permeate Totalizer volume |  |  |  |  |  |  |  |  |  |
|  | Step |  | Buffer Info |  | Batch Number |  | Exp. Date | Amount Used (L) * |  |  |
|  | Sanitization |  | PN: A0013-D 0.5 N Sodium Hydroxide – DV |  |  |  |  |  |  |  |
|  | Storage |  | PN: A0014-D 0.1 N Sodium Hydroxide |  |  |  |  |  |  |  |
|  | SAP Consumption Performed by/Date |  |  |  |  |  |  |  |  |  |

# Batch Production Record Review

| Step | Description | Production Initial/Date |
| --- | --- | --- |
|  | If tanks were used, ensure that the SIP, CIP, and Pressure Test Reports for the product tank are attached to the MPR. |  |
|  | Verify that the PFI product net weight (from Step 18.8) is correctly inputted into SAP. |  |
|  | Review the Batch Production Record for completeness and submit for review. |  |

# Attachment 1: Process Tubing/Hose Information

| Hose information for the Retentate Vessel, if applicable |  |  |  |
| --- | --- | --- | --- |
|  | Hose Serial Number |  | Cleaning Exp. Date |
| Feed Line |  |  |  |
| Additional Feed Line, as needed |  |  |  |
| Retentate Line |  |  |  |
| Additional Retentate Line, as needed |  |  |  |
| Permeate Line |  |  |  |
| Additional Permeate Line, as Needed |  |  |  |
| Charge/Diafiltration Line |  |  |  |
| Additional Charge/Diafiltration Line, if needed |  |  |  |
| Additional Charge/Diafiltration Line, if needed |  |  |  |
| Recovery Line |  |  |  |
| Additional Recovery Line, if needed |  |  |  |
| Performed by/Date |  | Witnessed/Date |  |

Attachment 1:  Process Tubing/Hose Information (continued)

| Tubing for the Retentate Vessel, if applicable |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tubing Use | Part Number | Batch Number | Expiration Date | Autoclave Cycle Number | Autoclave Cycle Exp. Date |
| Feed Line |  |  |  |  |  |
| Additional Feed Line, as needed |  |  |  |  |  |
| Retentate Line |  |  |  |  |  |
| Additional Retentate Line, as needed |  |  |  |  |  |
| Permeate Line |  |  |  |  |  |
| Additional Permeate Line, as Needed |  |  |  |  |  |
| Charge/Diafiltration Line |  |  |  |  |  |
| Additional Charge/Diafiltration Line, if needed |  |  |  |  |  |
| Peristaltic Pump Tubing |  |  |  |  |  |
| Additional Peristaltic Pump Tubing, if needed |  |  |  |  |  |
| Performed by/Date |  |  | Witnessed/Date |  |  |

# Attachment 2: Operation Parameters

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

Attachment 2: Operation Parameters (continued)

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

Attachment 2: Operation Parameters (continued)

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

Attachment 2: Operation Parameters (continued)

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

Attachment 2: Operation Parameters (continued)

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

Attachment 2: Operation Parameters (continued)

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

Attachment 2: Operation Parameters (continued)

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

Attachment 2: Operation Parameters (continued)

| Time |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step |  |  |  |  |
| PFeed (psig) @ PT-101 |  |  |  |  |
| PRetentate (psig) @ PT-301 |  |  |  |  |
| PPermeate (psig) @ PT-201 |  |  |  |  |
| ∆P (psid) (= PFeed - PRetentate) |  |  |  |  |
| Diafiltration Pump Settings, if applicable |  |  |  |  |
| % Retentate Open (%) @ BPRV-301 |  |  |  |  |
| Pump Speed (RPM) @ P-101 |  |  |  |  |
| Crossflow Flux (L/min/ft2) |  |  |  |  |
| TMP (psid) @ TMP-100 |  |  |  |  |
| Permeate Flow Rate (L/min) @ FT-201 |  |  |  |  |
| Permeate Flux (LMH) |  |  |  |  |
| Total Permeate Volume (L) @ FQI-201 |  |  |  |  |
| Permeate UV Reading (OD) @ AIT-201 |  |  |  |  |
| Permeate Conductivity, offline (mS/cm) * |  |  |  |  |
| Permeate pH, offline * |  |  |  |  |
| Retentate Vessel Agitator Setting | rpm | rpm | rpm | rpm |
| Product weight in Retentate Vessel (kg) |  |  |  |  |
| Performed by/ Date |  |  |  |  |
| Witnessed by/Date |  |  |  |  |

* = The conductivity meter used must match the meter used in Step 8.2, including temperature compensation during equilibration and sections forward. Otherwise, annotate in Attachment 9: Comments Section.

# Attachment 3: Sampling Record

| Concentration and Diafiltration |  |  |  |  |  |  |  |  | (8012475) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dev Samples DLIMS Project # |  |  |  |  |  |  |  |  |  |  |  |
| CoA GxP Samples Project # |  |  |  |  |  |  |  |  |  |  |  |
| WHITEFORM GxP Samples Project # |  |  |  |  |  |  |  |  |  |  |  |
| Pre-Load Performed By/Date |  |  |  |  |  |  |  |  |  |  |  |
| DLIMS Test Name | Sample Volume | Location | Tube Type | DLIMS Form | Intermediate Storage Temp (oC) |  |  | DLIMS Storage Temp. (oC) | Sample Number |  |  |
| -10 (Virus Filtered Product) |  |  |  |  |  |  |  |  | -10 -1 | -10-2 | -10-3 |
| BIOBURDEN_IP_GPF1 | 25 mL | QC | PP | G | 2-8oC |  |  | 2-8oC |  |  |  |
| ENDOTOXIN_IP_GPF1 | 10 mL | QC | PET | G | 2-8oC |  |  | 2-8oC |  |  |  |
| -20 (Recovered Product) |  |  |  |  |  |  |  |  |  |  |  |
| SOLO_A280 a | 10 mL | MS | PP | G | 2-8oC |  |  | 2-8oC |  |  |  |
| - 30 (Diluted Product, if applicable) |  |  |  |  |  |  |  |  |  |  |  |
| SOLO_A280 a | 10 mL | MS | PP | G | 2-8oC |  |  | 2-8oC |  |  |  |
| -40 (Pre-Formulated Intermediate - Product Quality Samples) |  |  |  |  |  |  |  |  |  |  |  |
| SEC_PUR | 10 mL | AS | PP | G | 2-8oC | 2-8oC |  |  |  |  |  |
| CGE_N_RED | 10 mL | AS | PP | G | 2-8oC | 2-8oC |  |  |  |  |  |
| SOLO_A280 a | 10 mL | MS | PP | G | 2-8oC | 2-8oC |  |  |  |  |  |
| ABT Retain c | 5 x 14 mL | AS | PP | G | 2-8oC |  | 2-8oC |  |  |  |  |
| PPS Small Retain b | 3 x 14 mL | AS | PP | G | 2-8oC |  | -80oC |  |  |  |  |
| BIOBURDEN_IP_GPF2 | 25 mL | QC | PP | G | 2-8oC |  | 2-8oC |  |  |  |  |
| ENDOTOXIN_IP_GPF1 | 10 mL | QC | PET | G | 2-8oC |  | 2-8oC |  |  |  |  |

- Test SOLO_A280 sample per SOP-0107091.  If sample not tested immediately, store sample at 2 - 8°C.

- For PPS Small Retains, select “Retain – PPS Small” for the purpose field when loading into DLIMS.

- For AS Retains, select “Retain – ABT” for the purpose field when loading into DLIMS.

NOTE:  Bolded tests are STAT samples.

# Attachment 4: Non-Routine Sampling Record

- If non-routine samples are taken, re-adjust weight of the product sampled.

- Log into DLIMS per SOP-0080295 as required by the memo. Record DLIMS Project number and sample number if submitted per DLIMS.

| Pre-Load Performed by/Date |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Process Section No. | Contents | DLIMS Form | DLIMS Project Number | Container Type | Sample Volume | DLIMS Test Name | Sample Number | Storage Temp. (oC) | Location | Performed By/Date | Witnessed By/Date |

# Attachment 5: Hold Times

Record the storage start time, end time, and date for the Virus Filtered Product.

- For product stored at 2 – 8°C, the storage start time is the time when the product is transferred into 2 – 8°C storage, and the storage end time is the time when the product is transferred from 2 – 8°C storage. A new entry for storage at 15 – 25°C begins upon removal from 2 – 8°C.

- Calculate the storage duration for each temperature where the product was stored as well as the combined storage duration, if applicable.

| Hold | Location | Temp. | Virus Filtered Product Storage Start (Time/Date) | Performed by/Date | Verified by/Date |  | Virus Filtered Product Storage End (Time/Date) | Hold Times (hrs and mins) |  | Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | R. T. | 2-8 °C |  |  |
| 1 |  | R.T. 2-8 °C |  |  |  |  |  | ______hrs _____mins | ______hrs _____mins |  |  |
| 2 |  | R.T. 2-8 °C |  |  |  |  |  | ______hrs _____mins | ______hrs _____mins |  |  |
| 3 |  | R.T. 2-8 °C |  |  |  |  |  | ______hrs _____mins | ______hrs _____mins |  |  |
| Virus Filtered Product Total R. T. Hold Time: If R.T. Total Hold Time > 72 hrs, contact a supervisor |  |  |  |  |  |  |  | hrs |  |  |  |
| Virus Filtered Product Total 2-8 °C Hold Time: |  |  |  |  |  |  |  |  | hrs |  |  |
| Virus Filtered Product Total 2-8 °C + RT Hold Time: If Total 2-8 °C + RT Hold Time > 168 hrs, contact a supervisor |  |  |  |  |  |  |  | hrs |  |  |  |

# Attachment 6: Retentate Tank and DF-1244 Set-up

# Attachment 7: Product Recirculation Work Sheet

| - Obtain a piece of tubing for recirculation of the product. Record the applicable product name and tubing information. - Start mixing the product by recirculation. Record the start time. - Mix the product by recirculation per SOP-0107097 for at least 40 minutes and record the mixing information. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Applicable MPR Step |  |  |  |  |  |
| Product Name |  |  |  |  |  |
| Tubing Part No. |  |  |  |  |  |
| Tubing Batch No. |  |  |  |  |  |
| Tubing Exp. Date |  |  |  |  |  |
| Tubing Autoclave Cycle No. |  |  |  |  |  |
| Tubing Autoclave Exp. Date |  |  |  |  |  |
| Mixing Start Time |  |  |  |  |  |
| Mixing End Time |  |  |  |  |  |
| Mixing Duration * (≥ 40 minutes) | min | min | min | min | min |
| Performed By / Date |  |  |  |  |  |
| Witnessed by / date |  |  |  |  |  |

* The duration of mixing is the time that elapses between the mixing end time and the mixing start time in minutes.

# Attachment 8: Yield Calculations

Final Product Mass, Pre-Sampling

| PFI Volume before sampling | ## X | Final Product Concentration | = | Final Product Mass, Pre-Sampling (a) | Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- |
| ___________L_ (Step 17.16 or Step 16.33 if no dilution) |  | ________g/L (8012475-20 or 8012475-30 *) |  | g |  |  |
| Volume of the PFI after sampling and after transfer | ## X | Final Product Concentration | = | Final Product Mass, Post-Sampling (b) | Performed by/Date | Verified by/Date |
| ___________L_ (Step 18.9) |  | ________g/L (8012475-20 or 8012475-30 *) |  | g |  |  |

* Use (-30) if product was diluted.

Step and MPR Yield

| Final Product Mass, Pre-Sampling (a) |  | Total Grams of Virus Filtration Product after sampling (Step 9.10) | = | Yield | X 100 | % Step Yield | Performed by/Date | Verified by/Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| g |  | g |  |  |  | % |  |  |
| Final Product Mass, Post-Sampling (b) |  | Total grams of Virus Filtration Product before sampling (Step 7.8) | = | Yield | X 100 | % MPR Yield | Performed by/Date | Verified by/Date |
| g |  | g |  |  |  | % |  |  |

# Attachment 9: Comments Section

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 9: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 9: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 9: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 9: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

Attachment 9: Comments Section (continued)

| Step No. | Comments Please initial and date each entry. | Initials/ Date |
| --- | --- | --- |

# MASTER PRODUCTION RECORD REVISION HISTORY

The following is included for reference only and is not part of the executable record.

| Version | Change | Reason |
| --- | --- | --- |
| 1.0 | - This is a new master production record (MPR) | - The information contained in this document is derived from Technical report REP-0494621 and sampling and testing plan PLAN-0124866. Additional information may be found in REP-0414101. |